3GPP TSG RAN WG1 #103-e			R1-2008540

e-Meeting, October 26th – November 13th, 2020

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary on UE features for URLLC/IIoT

Agenda Item:	7.2.11

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the discussions and proposals in AI 7.2.11 regarding UE features for URLLC/IIoT.

Based on the discussions summarized in Section 2, followings are parts of the suggested email discussions/approvals for AI 7.2.11.

FL proposal of email discussion/approval:

[103-e-NR-UEFeature-URLLCIIoT-01] Email discussion/approval on UE features for URLLC/IIoT (26th Oct – 3rd Nov)

- Whether or not to confirm Working assumption on new FG for Rel-16 PDCCH monitoring capability with restriction that the same span pattern repeats in every slot for a given CC

- How to define a new FG for Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission according to conclusion made at RAN#89-e

- Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG 11-7a/7b/9/9a and 12-2a

- Whether or not to add components for the restriction on the number of different start symbol indices of PDCCH monitoring occasions per slot/half-slot in FG11-2

- Whether or not to add component for the supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot in FG11-3

- Whether or not to remove the note regarding relationship between FG12-1 and the feature of up to two HARQ-ACK codebooks of 11-4 and 11-4x

Companies are encouraged to check above FL proposal and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Qualcomm | We are fine to discuss all the FL proposed topics during this meeting. |
| Intel | We are fine with FL proposal. |
| Moderator | Thank you very much for the feedbacks. I see that the proposed email discussion scope seems fine for all. |
| Apple | We are fine with the FL proposal. |
| Moderator | Thank you very much for further feedbacks. Based on all the feedbacks, it seems ok to keep the original FL proposal on email discussion scope for UE features for URLLC/IIoT. |

- Discussion on NR Rel-16 UE features

  - Working assumption on a new FG for Rel-16 PDCCH monitoring capability with restriction that the same span pattern repeats in every slot for a given CC

Following working assumption was made at RAN1#102-e.

Working assumption:

- Add a new FG11-2d “Rel-16 PDCCH monitoring capability with restriction that the same span pattern repeats in every slot for a given CC”.

| 11. NR_L1enh_URLLC | 11-2d | Rel-16 PDCCH monitoring capability with restriction that the same span pattern repeats in every slot for a given CC | - Supported combination(s) of (X, Y, ). For each reported combination, the UE supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span - Maximum number of DL and UL unicast DCI formats in a span For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - For a given CC, support only that same span pattern [TBD definition of span pattern] repeats in every slot |  | Yes | N/A |  | Per FS for component 1 Note: Indicating support of this capability in a band in a BC implies that only rel-16 monitoring can be configured in a CA configuration for the BC if the CA configuration includes the band and if rel-16 monitoring is configured for the band | N/A | N/A | N/A | This capability is signaled for SCS 15 kHz and 30 kHz. For =0 and 1, candidate value set for (X, Y, ): {(7, 3, ), (4, 3, ), (2, 2, )} For component 1, a list of separate UE capabilities (X, Y, )for processing capability #1; For component 1, a list of separate UE capabilities (X, Y, )for processing capability #2; | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposals are made in contributions.

| [2] | According to the span definition in section 10 of TS 38.213 copied below, “PDCCH monitoring occasion” in existing spec can be interpreted as actual monitoring occasions in each slot,. In other words, it is not the synthetic monitoring occasions across slots because there is no additional explanation. 10 UE procedure for receiving control information A UE can indicate a capability to monitor PDCCH according to one or more of the combinations = (2, 2), (4, 3), and (7, 3) per SCS configuration of =0 and =1. A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination , the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to Y.. As a result,, as shown in Figure 1, spans are determined in each slot for FG 11-2, resulting in the “span pattern” in different slot can be different. However, a single same combination (2,2) applies to all slots. Figure 1 An example for span pattern based on FG 11-2 For FG 11-2d, span pattern in different slots should be the same and a single same combination (X,Y) applies across slots. There are two interpretations to guarantee this restriction. - Interpretation 1: all the PDCCH monitoring occasions are the same across slots because it is assumed “PDCCH monitoring occasion” is actual monitoring occasions. There is no need to define what is span pattern but will strongly limit the configuration of PDCCH monitoring occasions. - Interpretation 2: “PDCCH monitoring occasion” in existing spec is interpreted as synthetic monitoring occasions across slots. The span patterns are exactly the same across slots. One way is to introduce synthetic monitoring occasions in the specification as in FG 3-5b. That is, in order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. As shown in Figure 2, the configuration of monitoring occasions in Figure 1 with Interpretation 2 will guarantee the same span pattern across slots. Figure 2 An example for span pattern based on FG 11-2d Above all, Interpretation 1 will strongly limit the configuration of PDCCH monitoring occasions while Interpretation 2 will introduce the definition of span pattern. We prefer the same span pattern across slots is only defined in FG 11-2d and not restricted in FG 11-2. The definition of span pattern as in FG 3-5b can be only added in FG 11-2d. As a result, the span defined in TS 38.213 can be regarded used for FG 11-2 with no further modification. Proposal 7 (URLLC): Confirm the working assumption of adding a new FG11-2d and introduce the definition of span pattern as in FG 3-5b. |
| --- | --- |
| [3] | First, we think that the WA can be confirmed. Further, the phrase “span pattern” can be defined by the (X,Y) combination that is applicable for the PDCCH MOs in the slot. Proposal 1: - Confirm the WA with updating the FG index to FG #11-2f - To define a span pattern, add the following as to the Note column: - In a given slot, a “span pattern” is defined by the applicable (X,Y) combination based on the configured PDCCH monitoring occasions in the slot. |
| [6] | Also, it was discussed whether to add a new component/note related to CCE/BD counting in spans in a slot at the last meeting. The following was proposed but no conclusion was achieved. Proposed conclusion: - UE does not expect to do the CCE/BD counting in spans except the first one within a slot for Rel-16 PDCCH monitoring capability on the primary cell, as defined by the following two paragraphs in TS 38.213. View - We are open to introduce the FG with the span pattern restriction to alleviate UE complexity. As the restriction also exists for FG3-5b, it would not be a big issue for gNB scheduling. Regarding the proposed conclusion, we acknowledge the issue but it should be clarified in the spec rather than the UE feature. |
| [7] | Given that the (X,Y) constraints should be satisfied across the slots of one carrier, but the actual spans are not necessarily identical, if the working assumption is not confirmed: - The span pattern across slots may change from “aligned” to “non-aligned” and vice versa. - A UE needs to know whether the spans should be considered as aligned or not from the beginning since it impacts the maximum number of CCEs/BDs a UE is expected to monitor. Proposal 1: Confirm the working assumption and introduce a new FG as follows: “Rel-16 PDCCH monitoring capability with restriction that the same span pattern repeats in every slot for a given CC”. |
| [10] | One problem with the working assumption is that if confirmed it creates an NBC change, as it is essentially a restriction on top of 11-2. A gNB built according to September version of the specifications would not be aware of such limitation from the UE side. Hence, we do not support confirming the working assumption as formulated above. Proposal 8: Do not confirm working assumption on introducing FG11-2d. |

Discussion point #1

- Whether or not to confirm Working assumption on new FG for Rel-16 PDCCH monitoring capability with restriction that the same span pattern repeats in every slot for a given CC

  - A new FG for Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission

Following conclusion was made at RAN#89-e.

Conclusion: Introduce a new FG "Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission". Details are to be finalized by RAN1 and RAN2.

Following proposals are made in contributions.

| [3] | Following the decision in RAN #89E meeting, the proposed FG as discussed during RAN #89E, that is based on Option 1a (discussed as part of eURLLC maintenance during RAN1 #102E) can be adopted. This would be the simplest, and least “intrusive” option to follow considering the late stage beyond regular maintenance phase for Rel-16. Further, it is understood that the introduction of FG 11-12 does change the expectation for a Rel-16 UE indicating support of FG 5-25. 11. NR_L1enh_URLLC; 11-12; Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission; Support CBG-based re-transmission(s) of a TB in case the initial PUSCH transmission was cancelled and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #N-1 has not been transmitted before and it is not scheduled in the same UL grant that includes CBG#N.; 5-25; Yes; N/A; Per UE; N/A; N/A; A UE supporting 5-25 shall support CBG-based retransmission(s) with cancelled initial PUSCH transmission if the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB only if CBG #N-1 has been transmitted before or it is scheduled in the same UL grant that includes CBG#N.; Optional with capability signaling. Proposal 2: - Add the following FG #11-2 to the Rel-16 UE features list 11. NR_L1enh_URLLC; 11-12; Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission; Support CBG-based re-transmission(s) of a TB in case the initial PUSCH transmission was cancelled and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #N-1 has not been transmitted before and it is not scheduled in the same UL grant that includes CBG#N.; 5-25; Yes; N/A; Per UE; N/A; N/A; A UE supporting 5-25 shall support CBG-based retransmission(s) with cancelled initial PUSCH transmission if the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB only if CBG #N-1 has been transmitted before or it is scheduled in the same UL grant that includes CBG#N.; Optional with capability signaling. |
| --- | --- |
| [4] | The problem with CBG based re-transmission is the generation of the TB-CRC in case the initial transmission gets partially cancelled and that these partially cancelled CBGs are scheduled out-of-order in the re-transmissions, for example as illustrated in Figure eURLLC-1 below. The TB-CRC shall be sent in the last CBG (CBG#4 in the example of Figure eURLLC 1). For the calculation of the TB-CRC it is needed that the source bits are supplied in the correct order. If CBGs #3 and #4 are cancelled in the initial transmission, the TB-CRC calculation would stop after CBG #2. The UE would then firstly need CBG #3 to correctly calculate the TB-CRC. But according to the current specification the gNB could schedule any CBG in the first re-TX. For the UE implementation it would be very difficult to prepare the TB-CRC when previously cancelled CBGs are re-scheduled in out-of-order fashion and this situation should therefore be avoided. Figure eURLLC- 1 Out of order scheduling of CBGs During the previous RAN1 meetings, multiple options have been discussed and the following two candidate solutions, Option 1 and Option 1a, were left at the end: - Option 1: the UE is not expected to be scheduled for a re-transmission of the TB including the last CBG if each of the other CBGs (except for the last one) have either not been transmitted at least once before or are not scheduled for a re-transmission in the same UL grant as the last CBG. - Option 1a: The UE is not expected to be scheduled for a re-transmission of a CBG #N in a given TB unless CBG #N-1 has been transmitted before or is scheduled in the same UL grant that includes CBG#N. Option 1 and Option 1a intend to ensure that the cancelled CBGs are re-scheduled in in-order fashion. However, Option 1 is not complete and can result into problems for some cases because it does not in general solve the issue that is supposed to be addressed. Consider the example in Figure eURLLC-2 below, where CBGs #2, #3 and #4 are cancelled in the initial transmission. In the first re-TX, only CBG3 is scheduled. The generation of the TB-CRC, on the other hand, would require the source bits from CBG#2 are taken into account before CBG#3. Thus, scheduling according to Option 1 as shown in the example below result into problems for the UE implementation. Figure eURLLC-2 CBG re-transmission according to Option 1 The above described problem is avoided with Option 1a, where the scheduling of each CBG requires that all the previous CBGs have either already been transmitted or are scheduled by the same grant. Therefore, Option 1a should be supported. The issue was discussed in RAN#89-e and the following conclusion was achieved: Conclusion: Introduce a new FG "Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission". Details are to be finalised by RAN1 and RAN2.. Based on the above analysis, we have the following proposal for the details of the new FG: Proposal eURLLC-2: Add the new FG “Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission” for eURLLC WI with details as below (based on Option 1a above): 11. NR_L1enh_URLLC; 11-12; Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission; Support CBG-based re-transmission(s) of a TB in case the initial PUSCH transmission was cancelled and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #N-1 has not been transmitted before and it is not scheduled in the same UL grant that includes CBG#N.; 5-25; Yes; N/A; Per UE; N/A; N/A; A UE supporting 5-25 but not 11-12 shall support CBG-based retransmission(s) with cancelled initial PUSCH transmission if the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB only if CBG #N-1 has been transmitted before or it is scheduled in the same UL grant that includes CBG#N.; Optional with capability signaling. |
| [5] | The issue of TB CRC generation for CBG-based PUSCH retransmission in case the initial transmission was cancelled has been discussed in RAN1 for a few meetings without any conclusion. It was further discussed in RAN#89, and the following conclusion was made: Conclusion: Introduce a new FG "Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission". Details are to be finalized by RAN1 and RAN2. The latest summary of the RAN1#102-e email discussion on this issue in the maintenance session is provided in [2]. As summarized, a few options had been identified to resolve the issue: - Re-transmission of the CBG-based PUSCH with cancellation - Option 1: the UE is not expected to be scheduled for a re-transmission of the TB including the last CBG if each of the other CBGs (except for the last one) have either not been transmitted at least once before or are not scheduled for a re-transmission in the same UL grant as the last CBG. - Option 1a: The UE is not expected to be scheduled for a re-transmission of a CBG #N in a given TB unless CBG #N-1 has been transmitted before or is scheduled in the same UL grant that includes CBG#N. - Option 2: the TB CRC for the retransmission of the same TB is set to all zeros. - Option 3: It is up to UE implementation to determine which values to use as the TB CRC (which may not be the actual TB CRC) for the retransmission of the same TB. - Option 4: the minimum processing time for PUSCH scheduled for re-transmission is extended by D symbols. - Option 5: The UE is not expected to be scheduled with partial TB for the retransmission.. Among these options, Option 1a received most support, because it resolves the UE implementation issue without adding additional restriction on gNB’s scheduling in any practical sense. Therefore, we propose the new FG to be defined based on Option 1a. Proposal 2-1: Introduce a new FG for out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission: 11. NR_L1enh_URLLC; 11-12; Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission; Support CBG-based re-transmission(s) of a TB in case the initial PUSCH transmission was cancelled and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #N-1 has not been transmitted before and it is not scheduled in the same UL grant that includes CBG#N.; 5-25; Yes; N/A; Per UE; N/A; N/A; N/A; A UE supporting 5-25 shall support CBG-based retransmission(s) with cancelled initial PUSCH transmission if the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB only if CBG #N-1 has been transmitted before or it is scheduled in the same UL grant that includes CBG#N.; Optional with capability signaling. |
| [6] | View - We are open to support the proposed FG11-12 in [3]. However, it should be discussed whether to change FG 5-25 as described in the note. If it leads to NBC issue for Rel-15 UE, FG 5-25 should be kept as it is so that it supports in-order/out-of-order CBG based re-transmission and the new FG is introduced to support in-order CBG based re-transmission only. Othewise, the current proposed FG 11-12 is fine. |
| [7] | In RANP #89e, it was agreed to to introduce a new FG for handling CBG-based PUSCH with cancellation. Hence, we propose the following: Proposal 2: Introduce the following FG for handling CBG-based PUSCH with cancellation: 11. NR_L1enh_URLLC; 11-12; Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission; Support CBG-based re-transmission(s) of a TB in case the initial PUSCH transmission was cancelled and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #N-1 has not been transmitted before and it is not scheduled in the same UL grant that includes CBG#N.; 5-25; Yes; N/A; Per UE; N/A; N/A; Optional with capability signaling. |
| [8] | In RAN#89, it was concluded that a new FG "Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission" is introduced. Details are to be finalized by RAN1 and RAN2. In the following, we discuss the options for introducing the new FG. Firstly, CBG-based re-transmission(s) can already be indicated by the below Rel-15 capability. If UE indicates this, it supports both in-order and out-of-order CBG transmission. Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12].; UE; No; No; No. The proposal in [2] is to introduce an out-of-order indication for CBG-based re-transmission in Rel-16. cbg-TransCancelledPUSCH-UL Indicates whether the UE supports CBG-based retransmission for UL using CBG transmission information (CBGTI) in case the initial PUSCH transmission was cancelled and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #N-1 has not been transmitted before and it is not scheduled in the same UL grant that includes CBG#N. The UE indicating support of this feature shall indicate the support of CBG-based (re)transmission for UL with cbg-TransIndication-UL. The new FG proposed for Rel-16 [2] means that UE is not able to indicate that it ONLY supports in-order. - If UE supports both cbg-TransIndication-UL and cbg-TransCancelledPUSCH-UL, then it supports both in-order and out-of-order. - If UE supports cbg-TransIndication-UL but not cbg-TransCancelledPUSCH-UL, then it supports both in-order and out-of-order - The case that UE supports cbg-TransCancelledPUSCH-UL but not cbg-TransIndication-UL is precluded by the green text above. It is our understanding that in-order operation is simpler to implement than out-of-order operation, and this is the reason for introducing a new FG. Thus, the UE capability signalling should be able to signal only in-order. In the following, we provide several potential approaches for introducing the new FG for indicating that a UE supports only in-order CBG-based retransmissions. Approach 1: Approach 1 for Rel-15: Slim down the meaning of the legacy bit to such that it only applies for in-order scheduling of CBG: cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12]. In case a retransmission is scheduled, the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #(N-1) has been scheduled before or CBG #(N-1) is scheduled in the same UL grant that includes CBG#N.; UE; No; No; No. Approach 1 for Rel-16: Add a new bit for the out-of-order scheduling of CBG: cbg-TransCancelledPUSCH-UL Indicates whether the UE supports CBG-based retransmission for UL using CBG transmission information (CBGTI) in case the initial PUSCH transmission was cancelled and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #(N-1) has not been scheduled before and it is not scheduled in the same UL grant that includes CBG#N. The UE indicating support of this feature shall indicate the support of CBG-based (re)transmission for UL with cbg-TransIndication-UL; UE; No; No; No. cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12]. In case a retransmission is scheduled, the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #(N-1) has been scheduled before or CBG #(N-1) is scheduled in the same UL grant that includes CBG#N.; UE; No; No; No. UE signalling: - A Rel-15 UE can indicate that it supports in-order CBG retransmission, and does not support out-of-order CBG retransmission, by indicating that it supports cbg-TransIndication-UL; - A Rel-16 can indicate that - it supports in-order CBG retransmission, and does not support out-of-order CBG retransmission, by indicating that it supports cbg-TransIndication-UL and does not support cbg-TransCancelledPUSCH-UL. - it supports both in-order and out-of-order CBG retransmission indicating that it supports both cbg-TransIndication-UL and cbg-TransCancelledPUSCH-UL. This approach has the drawback that it is non-backwards-compatible (NBC). If a Rel-15 UE implements this CR and sets cbg-TransIndication-UL, a NW not implementing the CR may attempt scheduling out-of-order CBG retransmission for this UE and it would break. It would be acceptable though if 3GPP knows that there is no NWs out there that would ever attempt out-of-order CBG retransmission for Rel-15 UE. It is noted that in Rel-15, no PUSCH cancellation (intra-UE or inter-UE) was defined. Hence for Rel-15 FG description, it cannot mention PUSCH cancellation. For Rel-16 FG, “the initial PUSCH transmission was cancelled” due to either intra-UE prioritization or inter-UE cancellation indicator. Approach 2: Approach 2 for Rel-15: No change to Rel-15. Rel-15 FG would hence only have the legacy bit with the legacy meaning, i.e. it would indicate that the UE supports both in-order and out-of-order CBG retransmissions. cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12].; UE; No; No; No. Approach 2 for Rel-16: Add a new bit for the in-order scheduling alternative. Basically a slimmed down version of the legacy bit. cbg-TransCancelledPUSCH-UL Indicates whether the UE supports CBG-based retransmission for UL using CBG transmission information (CBGTI) in case the initial PUSCH transmission was cancelled and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #(N-1) has been scheduled before or CBG #(N-1) is scheduled in the same UL grant that includes CBG#N. The UE indicating support of cbg-TransIndication-UL also supports this feature.; UE; No; No; No. cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12].; UE; No; No; No. UE signalling: - A Rel-15 UE can indicate that it supports CBG retransmission by indicating that it supports cbg-TransIndication-UL. Both in-order and out-of-order CBG retransmission are supported. - A Rel-16 can indicate that - it supports in-order CBG retransmission, and does not support out-of-order CBG retransmission, when initial PUSCH transmission is cancelled, by indicating that it does not support cbg-TransIndication-UL and support cbg-TransCancelledPUSCH-UL. - it supports both in-order and out-of-order CBG retransmission regardless of cancellation of initial PUSCH transmission, by indicating that it supports cbg-TransIndication-UL. In this case, the UE has to indicate support of cbg-TransCancelledPUSCH-UL. This approach has the advantage that it is backwards compatible. A drawback is, the entire feature of CBG-based retransmissions cannot be enabled even if only in-order CBG retransmission is scheduled by gNB. The Rel-15 UE does report cbg-TransIndication-UL if it supports of in-order, but does not support of out-of-order CBG retransmission, regardless of how gNB schedules. Approach 3: Approach 3 for Rel-15: Add a new FG to Rel-15 for only scheduling the in-order CBG retransmission. Basically a slimmed down version of the legacy bit is additionally provided. The legacy bit would hence have the legacy meaning, i.e. it would indicate that the UE supports both in-sequence and out-of-sequence retransmissions. cbg-TransCancelledPUSCH-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12]. In case a retransmission is scheduled, the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #(N-1) has been scheduled before or CBG #(N-1) is scheduled in the same UL grant that includes CBG#N.; UE; No; No; No. cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12].; UE; No; No; No. Approach 3 for Rel-16: Reuse of the two Rel-15 FGs. No change is needed compare to the revised Rel-15. cbg-TransCancelledPUSCH-UL Indicates whether the UE supports CBG-based retransmission for UL using CBG transmission information (CBGTI) in case a retransmission is scheduled, the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #(N-1) has been scheduled before or CBG #(N-1) is scheduled in the same UL grant that includes CBG#N.; UE; No; No; No. cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12].; UE; No; No; No. UE signalling: - A Rel-15 UE can indicate that - it supports CBG retransmission with in-order retransmission only, by indicating that it does not support cbg-TransIndication-UL, but supports cbg-TransCancelledPUSCH-UL. - it supports CBG retransmission, both in-order and out-of-order CBG, by indicating that it supports cbg-TransIndication-UL. In this case, the UE has to indicate support of cbg-TransCancelledPUSCH-UL also. - A Rel-16 can indicate similar to Rel-15 UE. This approach has the benefit that it is backwards compatible. For both Rel-15 and Rel-16 UEs, it allows CBG-based retransmission even if a UE only supports in-order CBG retransmission. Comparing Approach 1, 2, 3, Approach 1 is the cleanest and preferred, if non-backwards-compatibility is not a concern. Otherwise, Approach 3 can be adopted. Approach 2 is not favorable. - For introducing the new FG "Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission", adopt approach 1 if no concern of non-backwards-compatibility; otherwise, adopt Approach 3. |
| [9] | One major remaining issue for URLLC/IIOT UE feature is the introduction of New UE FG for CBG-based PUSCH retransmission with cancelled initial transmission. In RAN#89e, it was agreed in principle to introduce such new UE FG [1], with details to be decided in RAN1/2. In [2], the latest proposal of this new FG is captured as the following, which can be used as the starting point for this FG. However, it makes more sense to make it “per UE” rather than “per band”. It is also proposed to revise the note column. TB CRC for cancelled initial PUSCH with CBG based re-transmission; PUSCH TB CRC calculated according to section 6.2.1 of TS38.212 for a re-transmission of a TB in case the initial transmission was cancelled and CBG-based re-transmission is configured and the following condition is not satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB unless CBG #N-1 has been transmitted before or is scheduled in the same UL grant that includes CBG#N.; 5-25; Yes; N/A; Per band; N/A; N/A; The cancellation could be due to support of ULCI [and/or intra-UE prioritization and/or dynamic SFI] If the UE does not report the support of this FG, the UE behaviour is undefined.; Optional with capability signaling. Proposal 2-1: To introduce a New UE FG for CBG-based PUSCH retransmission with cancelled initial transmission as the following. No RAN1 specification change is required. TB CRC for cancelled initial PUSCH with CBG based re-transmission; PUSCH TB CRC calculated according to section 6.2.1 of TS38.212 for a re-transmission of a TB in case the initial transmission was cancelled and CBG-based re-transmission is configured and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB only if CBG #N-1 has been transmitted before or is scheduled in the same UL grant that includes CBG#N.; 5-25; Yes; N/A; Per UE; N/A; N/A; The cancellation could be due to support of UL CI and/or intra-UE prioritization and/or dynamic SFI A UE supporting 5-25 shall support CBG-based retransmission(s) with cancelled initial PUSCH transmission if the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB only if CBG #N-1 has been transmitted before or it is scheduled in the same UL grant that includes CBG#N. If the condition is not satisfied, the UE behavior is undefined.; Optional with capability signaling. |

Discussion point #2

- How to define a new FG for Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission according to conclusion made at RAN#89-e

  - Interpretation of UE capabilities in case of cross-carrier operation

Following proposals are made in contributions.

| [2] | For all the UE features defined under URLLC, FG 11-7a, 11-9 and 11-9a may need further clarification in case of cross-carrier operation. To support FG 11-7a, UE needs to receive DCI format 2-4 in one carrier and cancel the PUSCH in another carrier. This UE feature is similar to dynamicSFI, in which case UE needs to receive SFI in one carrier and update the slot format for another carrier. Similar to the discussion we had in Rel-15 for dynamicSFI (See section A.1 of TS38.306), Interpretation#3 is more appropriate for FG 11-7a. 11. NR_L1enh_URLLC; 11-7a; UL cancelation scheme for cross-carrier; - Supports group common DCI (i.e. DCI format 2_4) for cancelation indication on a different DL CC than that scheduling PUSCH or SRS - UL cancelation for PUSCH - Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions - UL cancelation for SRS symbols that overlap with the cancelled symbols. For FG11-9 and FG11-9a, UE may need to receive regular DCI format with some repurposed fields in one carrier and activate/release configured grant in another carrier. These two FGs are more related to configured grant transmission/activation/deactivation. In this case, Interpretation#1 would make more sense for them. Furthermore, if we adopt Interpretation#3 for FG11-9 and FG11-9a, in order to support cross-carrier activate/deactivate configure grant, both the scheduling cell and the scheduled cell need to support more than 1 configured grant, which is too restrictive. 11. NR_L1enh_URLLC; 11-9; Multiple active configured grant configurations for a BWP of a serving cell; - Supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations - Supported maximum number of configured/active configured grant configurations in a BWP of a serving cell Candidate values for component 2: {1, 2, 4, 8, 12} - Supported maximum number of configured/active configured grant configurations across all serving cells Candidate values for component 3: {2, …, 32}. 11. NR_L1enh_URLLC; 11-9a; Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell; - M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication. Proposal 1 (URLLC): - Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on the support of this capability for the band of the scheduled/triggered/indicated cell only. - FG11-9 and FG11-9a - Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on both the support of this capability for the band of the scheduled/triggered/indicated cell and the support of this capability for the band of the scheduling/triggering/indicating cell. - FG11-7a |
| --- | --- |
| [8] | For the URLLC/IIoT features, they are about (a) inter-UE cancellation; (b) joint release of multiple DL-SPS or multiple UL-CG configurations. The procedure is: (1) receive DCI in the scheduling/triggering/indicating cell, (2) perform the action ((a) or (b)) in the scheduled/triggered/indicated cell. Thus interpretation 1 seems suitable. That is, no requirement to perform action ((a) or (b)) in scheduling cell; UE only needs to receive the DCI. eURLLC/IIoT; 11-7a UL cancelation scheme for cross-carrier 11-7b Independent cancellation of the overlapping PUSCHs in an intra-band UL CA 11-9a Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell 12-2a Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell; Interpretation 1. |

Discussion point #3

- Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG 11-7a/7b/9/9a and 12-2a

  - New components for the restriction on the number of different start symbol indices of PDCCH monitoring occasions per slot/half-slot in FG11-2

| 11. NR_L1enh_URLLC | 11-2 | Rel-16 PDCCH monitoring capability | - Supported combination(s) of (X, Y, ). For each reported combination, the UE supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span - Maximum number of DL and UL unicast DCI formats in a span For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD |  | Yes | N/A |  | Per FS for component 1 Note: Indicating support of this capability in a band in a BC implies that only rel-16 monitoring can be configured in a CA configuration for the BC if the CA configuration includes the band and if rel-16 monitoring is configured for the band | N/A | N/A | N/A | This capability is signaled for SCS 15 kHz and 30 kHz. For =0 and 1, candidate value set for (X, Y, ): {(7, 3, ), (4, 3, ), (2, 2, )} For component 1, a list of separate UE capabilities (X, Y, )for processing capability #1; For component 1, a list of separate UE capabilities (X, Y, )for processing capability #2; | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is made in contributions.

| [4] | In FG 3-5b, the following two sub-bullets to define the restriction on the number of different start symbol indices of PDCCH monitoring occasions per slot and per half-slot is given from UE implementation complexity perspective: - The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7. - The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell. FG 11-2 is designed based on the framework of FG 3-5b, therefore similar restriction should be included also. One issue discussed in RAN1#102-e is that whether the number of different start symbol indices of PDCCH monitoring occasions per slot will have impact on the UE complexity. In our understanding, it will have impact on the UE complexity at least from PDCCH overbooking perspective, unless it is clarified that UE does not expect to do the CCE/BD counting in spans except the first one within a slot for Rel-16 PDCCH monitoring capability on the primary cell, where CCE/BD counting is as defined by the following two paragraphs in TS 38.213: A PDCCH candidate with index for a search space set using a set of CCEs in a CORESET on the active DL BWP for serving cell is not counted for monitoring if there is a PDCCH candidate with index for a search space set , or if there is a PDCCH candidate with index and , in the CORESET on the active DL BWP for serving cell using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index is counted for monitoring. … CCEs for PDCCH candidates are non-overlapped if they correspond to - different CORESET indexes, or - different first symbols for the reception of the respective PDCCH candidates.. If UE needs to do CCE/BD counting in all the spans in a slot, the UE complexity would be too high, since the number of PDCCH monitoring occasions with different starting symbol index will have impact on UE complexity at least from CCE/BD counting perspective, as you can tell from the two paragraphs in the spec above. Based on the discussion in RAN1#102-e, some other UE vendors expressed that even without CCE/BD counting, it will increase the UE complexity on some other aspects also, therefore seems safer to include the two bullets directly in FG 11-2. Proposal eURLLC-1: Add the components for the restriction on the number of different start symbol indices of PDCCH monitoring occasions per slot/half-slot in FG 11-2. 11. NR_L1enh_URLLC; 11-2; Rel-16 PDCCH monitoring capability; - Supported combination(s) of (X, Y, ). For each reported combination, the UE supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span - Maximum number of DL and UL unicast DCI formats in a span For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7. - The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell.. In addition, another aspect that may have impact on FG 11-2 is whether to add one component for the restriction that same span pattern repeats in every slot for a given CC, which may depend on the discussion in maintenance. |
| --- | --- |

Discussion point #4

- Whether or not to add components for the restriction on the number of different start symbol indices of PDCCH monitoring occasions per slot/half-slot in FG11-2

  - New component on maximum number of PUCCHs for HARQ-ACK for FG 11-3

| 11. NR_L1enh_URLLC | 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | - Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. - Supported sub-slot configuration |  | Yes | N/A |  | Per FS Per FS is selected because in bands or BCs with large number of carriers or large BW, the UE’s processing power is spent on PDCCH/PDSCH decoding, and hence in some cases the support of the new codebook or some codebook configurations may not be possible | N/A | N/A | N/A | Candidate value set for component 2: { 7-symbol*2, 2-symbol*7 and 7-symbol*2} for NCP or { 6-symbol*2, 2-symbol*6 and 6-symbol*2} for ECP The number of PUCCHs for CSI reporting per slot is not impacted compared with Rel-15 by introducing the new HARQ-ACK CBs A UE supporting 11-3 is also expected to support FGs 4-1, 4-3, 4-4, 4-5, and 4-19 with a “slot” being replaced by a sub-slot of length 2 or 7 symbols for NCP and (2 and 6 symbols for ECP) for the PUCCH formats that can be accommodated in the corresponding sub-slot durations | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is made in contributions.

| [5] | Currently for FG 11-3 (More than one PUCCH for HARQ-ACK transmission within a slot, a.k.a. sub-slot based HARQ-ACK feedback), there is no limit on how many PUCCHs for HARQ-ACK is supported in a slot, even for the case of 2-symbol sub-slot. However, for FG 11-4/4a, we agreed on a component for UE to report such a limit. E.g. for FG 11-4 with one slot-based and one sub-slot-based HARQ-ACK feedback, we have component 6 as: Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot Candidate values for the component 6 of FG11-4 is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration. This leads to a strange situation where the UE may report 4 PUCCHs for HARQ-ACK within a slot for the sub-slot-based codebook when there are two HARQ-ACK codebooks, one slot-based and one-sub-slot-based. But if it is configured with a single sub-slot-based HARQ-ACK codebook, it has to support 7 PUCCHs for HARQ-ACK within a slot. This makes the relaxation from component 6 in FG 11-4 somewhat useless, because the UE still has to dimension for 7 PUCCHs for HARQ-ACK in a slot. To allow meaningful relaxation for UE implementation, we propose to add a similar component for 11-3. Proposal 2-2: Add a new component for FG 11-3 as follows: 11. NR_L1enh_URLLC; 11-3; More than one PUCCH for HARQ-ACK transmission within a slot; - Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. - Supported sub-slot configuration - Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot; Yes; N/A; Per FS Per FS is selected because in bands or BCs with large number of carriers or large BW, the UE’s processing power is spent on PDCCH/PDSCH decoding, and hence in some cases the support of the new codebook or some codebook configurations may not be possible; N/A; N/A; N/A; Candidate value set for component 2: { 7-symbol*2, 2-symbol*7 and 7-symbol*2} for NCP or { 6-symbol*2, 2-symbol*6 and 6-symbol*2} for ECP The number of PUCCHs for CSI reporting per slot is not impacted compared with Rel-15 by introducing the new HARQ-ACK CBs A UE supporting 11-3 is also expected to support FGs 4-1, 4-3, 4-4, 4-5, and 4-19 with a “slot” being replaced by a sub-slot of length 2 or 7 symbols for NCP and (2 and 6 symbols for ECP) for the PUCCH formats that can be accommodated in the corresponding sub-slot durations Component 3 is only applicable to the following sub-slot configurations: - 2-symbol*7 for NCP, with the candidate value of {4, 5, 6, 7} - 2-symbol*6 for ECP, with the candidate value of {4, 5, 6}; Optional with capability signalling. |
| --- | --- |

Discussion point #5

- Whether or not to add component for the supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot in FG11-3

  - Relationship between FG12-1 and the feature of up to two HARQ-ACK codebooks of 11-4 and 11-4x

| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of overlapping PUCCH/PUCCH and PUCCH/PUSCH with two priority levels in physical layer (PHY) - Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format - Multiplexing/prioritization between UL channels/signals with the same PHY priority level - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission |  | Yes | N/A |  | Per FS Per FS is selected because this FG involves various kinds of prioritization/cancellation/multiplexing, it is very processing intensive, and hence it is important to have finer granularity so that the UE does not have to under-report based on the worst band/band combination | N/A | N/A | N/A | Candidate value set for component 4: {0, 1, 2} Candidate value set for component 5: {0, 1, 2} The relationship between this feature and the feature of up to two HARQ-ACK codebooks of 11-4 and 11-4xshould be further discussed. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is made in contributions.

| [6] | View - Regarding the note highlighted by yellow, as the prerequisite FGs of FG11-4 has been removed from FG12-1, UE may report these FGs independently. In that sense, the note should be removed. |
| --- | --- |

Discussion point #6

- Whether or not to remove the note regarding relationship between FG12-1 and the feature of up to two HARQ-ACK codebooks of 11-4 and 11-4x

- Conclusion

Based on the email discussion in preparation phase, it was agreed to have following email discussion in RAN1#103-e.

[103-e-NR-UEFeature-URLLCIIoT-01] Email discussion/approval on UE features for URLLC/IIoT (26th Oct – 3rd Nov) – Hiroki (DCM)

- Whether or not to confirm Working assumption on new FG for Rel-16 PDCCH monitoring capability with restriction that the same span pattern repeats in every slot for a given CC

- How to define a new FG for Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission according to conclusion made at RAN#89-e

- Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG 11-7a/7b/9/9a and 12-2a

- Whether or not to add components for the restriction on the number of different start symbol indices of PDCCH monitoring occasions per slot/half-slot in FG11-2

- Whether or not to add component for the supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot in FG11-3

- Whether or not to remove the note regarding relationship between FG12-1 and the feature of up to two HARQ-ACK codebooks of 11-4 and 11-4x

# Reference

[1]	R1-2007326	Updated RAN1 UE features list for Rel-16 NR including remaining RAN1 issues				Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2007739	Discussion on NR Rel-16 UE Features	ZTE

[3]	R1-2007939	Rel-16 UE features	Intel Corporation

[4]	R1-2008333	Remaining details of Rel-16 NR UE features	Huawei, HiSilicon

[5]	R1-2008427	Discussions on NR Rel-16 UE features	Apple

[6]	R1-2008537	Discussion on NR Rel-16 UE features	NTT DOCOMO, INC.

[7]	R1-2008614	Discussion on NR Rel-16 UE features	Qualcomm Incorporated

[8]	R1-2008639	Remaining details of Rel-16 NR UE features	Ericsson

[9]	R1-2008681	Remaining issues on NR Rel-16 UE features	vivo

[10]	R1-2008737	Remaining issues on NR UE features	Nokia, Nokia Shanghai Bell

# Appendix: UE features list for URLLC/IIoT in [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 | - Supports monitoring DCI format 1_2 for DL scheduling - Supports monitoring DCI format 0_2 for UL scheduling |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-1a | Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | - Supports monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-1b | Type 1 HARQ-ACK codebook support for relative TDRA for DL | - Support Type 1 HARQ-ACK codebook for TDRA using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV | 11-1 | Yes | N/A |  | Per UE | No | Yes Note: Differentiation is from the perspective of the scheduled carrier | N/A |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2 | Rel-16 PDCCH monitoring capability | - Supported combination(s) of (X, Y, ). For each reported combination, the UE supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span - Maximum number of DL and UL unicast DCI formats in a span For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD |  | Yes | N/A |  | Per FS for component 1 Note: Indicating support of this capability in a band in a BC implies that only rel-16 monitoring can be configured in a CA configuration for the BC if the CA configuration includes the band and if rel-16 monitoring is configured for the band | N/A | N/A | N/A | This capability is signaled for SCS 15 kHz and 30 kHz. For =0 and 1, candidate value set for (X, Y, ): {(7, 3, ), (4, 3, ), (2, 2, )} For component 1, a list of separate UE capabilities (X, Y, )for processing capability #1; For component 1, a list of separate UE capabilities (X, Y, )for processing capability #2; | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2a | Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells | - Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells - Candidate value for the component: {2, 3, …, 16} - Supported span arrangement for CA - Candidate value for the component: {aligned spans only, aligned spans and non-aligned spans} | 11-2 | Yes | N/A |  | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2b | Mix of Rel. 16 PDCCH monitoring capability and Rel. 15 PDCCH monitoring capability on different carriers | - Support Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells | 11-2 | Yes | N/A |  | Per FS Note: Per FS is selected because same type with 3-5b is preferred | N/A | N/A | N/A |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2c | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers | - Supported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) - Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 - Supported span arrangement for CA - Candidate value for the component: {aligned spans only, aligned spans and non-aligned spans} | 11-2b | Yes | N/A |  | Per BC | N/A | N/A | N/A | The minimum of the summation of capability on the number of CCs with Rel-15 PDCCH monitoring capability and the capability on the number of CCs with Rel-16 PDCCH monitoring capability is 3 | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2d | Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span for MCG and for SCG when configured for NR-DC operation with Rel-16 PDCCH monitoring capability on all the serving cells | - Supported combination of (pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) | 11-2 | Yes | N/A |  | Per BC | N/A | N/A | N/A | If the UE reports pdcch-BlindDetectionCA-r16, - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is 1 to pdcch-BlindDetectionCA-r16-1 - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is 1 to pdcch-BlindDetectionCA-r16-1 - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= pdcch-BlindDetectionCA-r16 Otherwise, if N_(NR-DC,max,r16)^(DL,cells) is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r16monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability - the value of pdcch-BlindDetectionMCG-UE-r16 or of pdcch-BlindDetectionSCG-UE-r16 is 1, - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= N_(NR-DC,max,r16)^(DL,cells). | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2e | Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers | - Supported combination(s) of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) | 11-2b | Yes | N/A |  | Per BC | N/A | N/A | N/A | One combination of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) corresponds to one combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16) If the UE reports pdcch-BlindDetectionCA-r15, - Candidate values for pdcch-BlindDetectionMCG-UE-r15 is 0 to pdcch-BlindDetectionCA-r15 - Candidate values for pdcch-BlindDetectionSCG-UE-r15 is 0 to pdcch-BlindDetectionCA-r15 - pdcch-BlindDetectionMCG-UE-r15 + pdcch-BlindDetectionSCG-UE-r15>= pdcch-BlindDetectionCA-r15 Otherwise, if N_(NR-DC,max,r15)^(DL,cells) is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r15monitoringcapability - Candidate values for pdcch-BlindDetectionMCG-UE-r15 is [0, 1, 2] - Candidate values for pdcch-BlindDetectionSCG-UE-r15 is [0, 1, 2] - pdcch-BlindDetectionMCG-UE-r15 + pdcch-BlindDetectionSCG-UE-r15 >= N_(NR-DC,max,r15)^(DL,cells) If the UE reports pdcch-BlindDetectionCA-r16, - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is 0 to pdcch-BlindDetectionCA-r16 - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is 0 to pdcch-BlindDetectionCA-r16 - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16>= pdcch-BlindDetectionCA-r16 Otherwise, if N_(NR-DC,max,r16)^(DL,cells) is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r16monitoringcapability - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is [0, 1] - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is [0, 1] - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= N_(NR-DC,max,r16)^(DL,cells) | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | - Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. - Supported sub-slot configuration |  | Yes | N/A |  | Per FS Per FS is selected because in bands or BCs with large number of carriers or large BW, the UE’s processing power is spent on PDCCH/PDSCH decoding, and hence in some cases the support of the new codebook or some codebook configurations may not be possible | N/A | N/A | N/A | Candidate value set for component 2: { 7-symbol*2, 2-symbol*7 and 7-symbol*2} for NCP or { 6-symbol*2, 2-symbol*6 and 6-symbol*2} for ECP The number of PUCCHs for CSI reporting per slot is not impacted compared with Rel-15 by introducing the new HARQ-ACK CBs A UE supporting 11-3 is also expected to support FGs 4-1, 4-3, 4-4, 4-5, and 4-19 with a “slot” being replaced by a sub-slot of length 2 or 7 symbols for NCP and (2 and 6 symbols for ECP) for the PUCCH formats that can be accommodated in the corresponding sub-slot durations | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-3c | 2 PUCCH of format 0 or 2 for a single 7*2-symbol subslot based HARQ-ACK codebook | 1) 2 PUCCH format 0/2 in different symbols and once per subslot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot for SR | 11-3 | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For ECP, “7” is replaced by “6” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-3d | 2 PUCCH of format 0 or 2 for a single 2*7-symbol subslot based HARQ-ACK codebook | 1) 2 PUCCH format 0/2 in different symbols and once per subslot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot for SR | 11-3 | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For ECP, “7 symbols” is replaced by “6 symbols” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-3e | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for a single 2*7-symbol HARQ-ACK codebooks | If the UE supports a 2*7-symbol subslot HARQ-ACK codebook, the UE also supports: 1) 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 and 4 in the same subslot | 11-3 | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For ECP, “7 symbols” is replaced by “6 symbols” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-3f | 2 PUCCH transmissions in the same subslot for a single 2*7-symbol HARQ-ACK codebooks which are not covered by 11-3d and 11-3e | If the UE supports a 2*7-symbol subslot HARQ-ACK codebook, the UE also supports: 2 PUCCH transmissions in the same subslot for a single 2*7-symbol HARQ-ACK codebooks which are not covered by 11-3d and 11-3e | 11-3 | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For ECP, “7 symbols” is replaced by “6 symbols” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-3g | SR/HARQ-ACK multiplexing once per subslot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent with different starting symbols in a subslot | If a UE supports a subslot based HARQ-ACK codebook, the UE also supports: Overlapping PUCCH resources with different starting symbols in a subslot | 11-3 | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4 | Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook (i.e. slot-based + slot-based, or slot-based + sub-slot based) simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE | - Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed with the restriction up to one sub-slot based HARQ-ACK codebook. - Supports separate PUCCH configuration for different HARQ-ACK codebooks - Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. - Supports a DCI format (from the formats 1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured per BWP - Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. - Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot Candidate values for the component 6 of FG11-4 is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration. - Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels for HARQ-ACK |  | Yes | N/A |  | Per FS Per FS is selected because in bands or BCs with large number of carriers or large BW, the UE’s procesing power is spent on PDCCH/PDSCH decoding, and hence in some cases the support of the new codebook or some codebook configurations may not be possible | N/A | N/A | N/A | If a UE reports both 11-3 and 11-4, it can support two slot-based HARQ-ACK codebooks, and one slot-based and one-sub-slot-based HARQ-ACK codebooks. If a UE reports 11-4 but not 11-3, it can only support two slot-based HARQ-ACK codebooks. The number of PUCCHs for CSI reporting per slot is not impacted compared with Rel-15 by introducing the new HARQ-ACK CBs Component 6 is applied to the sub-slot HARQ-ACK codebook. It is assumed that only 1 actual PUCCH transmission for HARQ-ACK within a slot for slot-based HARQ-ACK codebook. - Component 6 is reported for 2-symbol*7 sub-slot configuration. For 7-symbol*2 sub-slot configuration, the value of component 6 is {2} for both NCP and ECP cases. - For component 6, maximum of 1 actual PUCCH transmission for HARQ-ACK within a slot for slot-based HARQ-ACK codebook. Thus value reported for component 6 has no meaning for “slot-based + slot based”. | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4a | Two subslot based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE | - Supports two subslot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. - Supports separate PUCCH configuration for different HARQ-ACK codebooks - Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. - Supports a DCI format (from the formats /1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP - Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. - Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot Candidate values for the component 6 of FG11-4a is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration. | 11-3 and 11-4 | Yes | N/A |  | Per FS Per FS is selected because in bands or BCs with large number of carriers or large BW, the UE’s procesing power is spent on PDCCH/PDSCH decoding, and hence in some cases the support of the new codebook or some codebook configurations may not be possible | N/A | N/A | N/A | The number of PUCCHs for CSI reporting per slot is not impacted compared with Rel-15 by introducing the new HARQ-ACK CBs Component 6 is applied to the two sub-slot HARQ-ACK codebooks, respectively. - Component 6 is reported for 2-symbol*7 sub-slot configuration. For 7-symbol*2 sub-slot configuration, the value of component 6 is {2} for both NCP and ECP cases. | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4b | DL priority indication in DCI with mixed DCI formats | - Support of priority indicator field configured in DCI formats 1_1 and 1_2 in a BWP when configured to monitor both DCI formats 1_1 and 1_2 in the BWP | 11-1, 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4c | 2 PUCCH of format 0 or 2 for two HARQ-ACK codebooks with one 7*2-symbol sub-slot based HARQ-ACK codebook | If the UE supports a 7*2-symbol subslot HARQ codebook, the UE also supports: 1) 2 PUCCH format 0/2 in different symbols and once per subslot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot for SR | 11-4 | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For slot based + slot based case, the capability for each HARQ-ACK codebook is subjected to the capability reported by FG 4-2 For ECP, “7” is replaced by “6” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4d | 2 PUCCH of format 0 or 2 in consecutive symbols for two HARQ-ACK codebooks with one 2*7-symbol sub-slot based HARQ-ACK codebook | If the UE supports a 2*7-symbol subslot HARQ codebook, the UE also supports: 1) 2 PUCCH format 0/2 in different symbols and once per subslot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot for SR | 11-4 | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For slot based + slot based case, the capability for each HARQ-ACK codebook is subjected to the capability reported by FG 4-2 For ECP, “7 symbols” is replaced by “6 symbols” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4e | 2 PUCCH of format 0 or 2 for two subslot based HARQ-ACK codebooks | If the UE supports two subslot HARQ codebooks, the UE also supports: 1) 2 PUCCH format 0/2 in different symbols and once per subslot per codebook for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per subslot per priority for SR | 11-4a | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4f | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for HARQ-ACK codebooks with one 2*7-symbol subslot based HARQ-ACK codebook | If the UE supports a 2*7-symbol subslot HARQ-ACK codebook, the UE also supports: 1) 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 and 4 in the same subslot of the codebook | 11-4 | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For slot based + slot based case, the capability for each HARQ-ACK codebook is subjected to the capability reported by FG 4-22 For ECP, “7 symbols” is replaced by “6 symbols” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4g | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for two subslot based HARQ-ACK codebooks | If the UE supports two subslot HARQ-ACK codebooks both configured with 2*7-symbols, the UE also supports: 1) 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 and 4 in the same subslot of a codebook | 11-4a | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For ECP, “7 symbols” is replaced by “6 symbols” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4h | 2 PUCCH transmissions in the same subslot for two HARQ-ACK codebooks with one 2*7-symbol subslot which are not covered by 11-4c and 11-4e | If the UE supports two HARQ-ACK codebooks with one subslot based codebook with 2*7-symbol configuration, the UE also supports: 1) 2PUCCH transmissions in the same subslot of the codebook which are not covered by 11-4c and 11-4e | 11-4 | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For slot based + slot based case, the capability for each HARQ-ACK codebook is subjected to the capability reported by FG 4-22a For ECP, “7 symbols” is replaced by “6 symbols” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4i | 2 PUCCH transmissions in the same subslot for two subslot based HARQ-ACK codebooks which are not covered by 11-4d and 11-4f | If the UE supports two HARQ-ACK codebooks both with 2*7-symbol configuration, the UE also supports: 1) 2PUCCH transmissions in the same subslot of a codebook which are not covered by 11-4d and 11-4f | 11-4a | Yes | N/A |  | Per FS Per FS is selected because the processing power the UE has to spend on preparing PUCCH has a relation with PDSCH processing power and that is related to number of carriers on which the UE has to process PDSCH | N/A | N/A | N/A | This FG covers any PUCCH transmission and not only those for HARQ-ACK reporting. For ECP, “7 symbols” is replaced by “6 symbols” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-5 | PUSCH repetition Type B | - For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots. - Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. - The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. - PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). - S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. - Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured - Supported maximum number of PUSCH transmissions within a slot for all TB(s), where each actual repetition for PUSCH repetition type B is counted as 1 PUSCH transmission, separately reported for UE processing capability 1 and for UE processing capability 2 if UE supports both processing capabilities Note: Number of TBs are based on reported Rel-15 capability on number of TBs, and reported value for component 7 cannot be smaller than the reported value of the number of TBs - Supported PUSCH hopping scheme |  | Yes | N/A |  | Per FS Note: Per FS is selected to follow Rel-15 reporting type for number of TBs to be supported | N/A | N/A | N/A | Candidate value for component 7: {2, 3, 4, 7, 8, 12} Candidate value for component 8: {Inter-slot hopping, Inter-repetition hopping, both Inter-slot hopping and Inter-repetition hopping} PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20 The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6 | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-6 | PUSCH repetition Type A | - PUSCH transmission with Rel-15 behavior with or without slot aggregation. • With slot aggregation, the number of repetitions can be dynamically indicated (as agreed for Rel-16). • When dynamically indicated, the number of repetitions is jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. | One of {5-16, 5-17] | Yes | N/A |  | Per UE | No | No | N/A | Note: RAN1 agreed it should be possible to separately indicate support of this FG based on whether the UE is operated with or without shared spectrum access. It is left to RAN2 how to implement this while leaving the type as “per UE” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-7 | UL cancelation scheme for self-carrier | - Supports group common DCI (i.e. DCI format 2_4) for cancelation indication on the same DL CC as that scheduling PUSCH or SRS - UL cancelation for PUSCH - Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions - UL cancelation for SRS symbols that overlap with the cancelled symbols |  | Yes | N/A |  | Per FS Per FS is selected because the FG is very demanding in UE processing, considering that this can be a UE with processing capability 1 but required to be able to cancel according to processing capability 2, and hence it is important to take into account the BC information for dimensioning purpose | N/A | N/A | N/A | More than one monitoring occasion for DCI format 2_4 per slot is applied only if the UE reports to support FG 3-5 or FG 3-5a or FG 3-5b or 11-2 or 11-2a | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-7a | UL cancelation scheme for cross-carrier | - Supports group common DCI (i.e. DCI format 2_4) for cancelation indication on a different DL CC than that scheduling PUSCH or SRS - UL cancelation for PUSCH - Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions - UL cancelation for SRS symbols that overlap with the cancelled symbols |  | Yes | N/A |  | Per FS Per FS is selected because the FG is very demanding in UE processing, considering that this can be a UE with processing capability 1 but required to be able to cancel according to processing capability 2, and hence it is important to take into account the BC information for dimensioning purpose | N/A | N/A | N/A | More than one monitoring occasion for DCI format 2_4 per slot is applied only if the UE reports to support FG 3-5 or FG 3-5a or FG 3-5b or 11-2 or 11-2a | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-7b | Independent cancellation of the overlapping PUSCHs in an intra-band UL CA | - For a UE indicating the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE may cancel the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to. | 6-23, 11-7 | Yes | N/A |  | Per band | N/A | N/A | N/A | If UE indicates 6-23 but does not support this FG, UE is not expected to be scheduled simultaneous PUSCHs on multiple carriers but receiving UL CI only for subset of carriers in intra-band carriers | Optional with capability signaling |
| 11. NR_L1enh_URLLC | 11-8 | Enhanced UL power control scheme | - For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the P0 value if SRI is present in the UL grant, and 1 or 2 bits is used to indicate the P0 value if SRI is not present in the UL grant |  | Yes | N/A |  | Per UE | No | Yes Note: Differentiation is from the perspective of the scheduled carrier | N/A |  | Optional with capability signaling |
| 11. NR_L1enh_URLLC | 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | - Supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations - Supported maximum number of configured/active configured grant configurations in a BWP of a serving cell Candidate values for component 2: {1, 2, 4, 8, 12} - Supported maximum number of configured/active configured grant configurations across all serving cells Candidate values for component 3: {2, …, 32} | One of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | For component 3: Total number in FR1 is not greater than X value reported for FR1. Total number in FR2 is not greater than X value reported for FR2.Total number across FR1 and FR2 is not greater than the larger of the FR1 and FR2 values | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-9a | Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell | - M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication | 11-9 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-10 | Type 2 configured grant release by DCI format 0_1 | - Support of type 2 configured grant release by DCI format 0_1 | 5-20 | Yes | N/A |  | Per UE | No | No | N/A | A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 11-11 (Type 2 configured grant release by DCI format 0_2). | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-11 | Type 2 configured grant release by DCI format 0_2 | - Support of type 2 configured grant release by DCI format 0_2 | 5-20, 11-1 | Yes | N/A |  | Per UE | No | No | N/A | A UE supporting this feature shall also support 11-10 (Type 2 configured grant release by DCI format 0_1). | Optional with capability signalling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of overlapping PUCCH/PUCCH and PUCCH/PUSCH with two priority levels in physical layer (PHY) - Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format - Multiplexing/prioritization between UL channels/signals with the same PHY priority level - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission |  | Yes | N/A |  | Per FS Per FS is selected because this FG involves various kinds of prioritization/cancellation/multiplexing, it is very processing intensive, and hence it is important to have finer granularity so that the UE does not have to under-report based on the worst band/band combination | N/A | N/A | N/A | Candidate value set for component 4: {0, 1, 2} Candidate value set for component 5: {0, 1, 2} The relationship between this feature and the feature of up to two HARQ-ACK codebooks of 11-4 and 11-4xshould be further discussed. | Optional with capability signaling |
| 12. NR_IIOT | 12-1a | UL priority indication in DCI with mixed DCI formats | Support of priority indicator field configured in DCI formats 0_1 and 0_2 in a BWP when configured to monitor both DCI formats 0_1 and 0_2 in the BWP | 12-1 and 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signalling |
| 12. NR_IIOT | 12-2 | Multiple SPS configurations | - Support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group, including separate RRC parameters and separate activation/release for different SPS configurations - The max number of active SPS configurations in a BWP of a serving cell - The max number of active SPS configurations across all serving cells - The related HARQ-ACK enhancements to support multiple active SPS configurations | 5-18 DL SPS | Yes | N/A |  | Per band | N/A | N/A | N/A | Component-2, candidate value set is {1, 2, …, 8} Component-3, candidate value set is {2, …, 32} | Optional with capability signaling |
| 12. NR_IIOT | 12-2a | Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell | - M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, where the association between each state indicated by the indication and the SPS configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple SPS configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the SPS configuration index indicated by the indication - The related HARQ-ACK enhancements to support joint release | 12-2 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 12. NR_IIOT | 12-3 | SPS release by DCI format 1_1 | Support of SPS release by DCI format 1_1 | 5-18 DL SPS | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 12. NR_IIOT | 12-3a | SPS release by DCI format 1_2 | Support of SPS release by DCI format 1_2 | 5-18 DL SPS and 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 12. NR_IIOT | 12-5 | Configuration of aggregation factor per SPS configuration | Support of configurable PDSCH aggregation factor ({1, 2, 4, 8}) per DL SPS configuration | 5-18 DL SPS | Yes | N/A |  | Per UE | No | Yes | N/A |  | Optional with capability signaling |
| 12. NR_IIOT | 12-6 | Support of SPS periodicity shorter than 10 ms | Support of SPS periodicity shorter than 10 ms | 5-18 DL SPS | Yes | N/A |  | Per UE | No | Yes | N/A |  | Optional with capability signalling |