3GPP TSG RAN WG1 #103-e		R1-2008565

e-Meeting, October 26th – November 13th, 2020

Source:	Ericsson

Title:	Maintenance for UE power saving

Agenda Item:	7.2.7

Document for:	Discussion and Decision

# Introduction

This document identifies some maintenance issues related to UE power saving.

# Discussion

# 2.1 Correction to 38.213

Following TP is proposed to align the RAN1 specification related to the minimum time gap with the corresponding minimum time gap capability parameter described in RAN2 specifications.

TP for 38.213-g30, subclause 10.3

## 10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

<omitted unchanged text>

If a UE reports for an active DL BWP a MinTimeGap-r16 value requirement of X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, where X corresponds to the reported MinTimeGap-r16 value for the requirement of the SCS of the active DL BWP in Table 10.3-1.

Table 10.3-1 Minimum time gap value X

| SCS (kHz) | Minimum Time Gap X (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

<omitted unchanged text>

# 2.2 RRC parameter alignment

Following RRC parameter name alignment/corrections are needed in some places. These are mentioned here for editors’ convenience.

38.213 subclause 10.3

- sizeDCI_2-6  → sizeDCI-2-6

- psPositionDCI-2-6  → ps-PositionDCI-2-6

- drx-onDuarationTimer → drx-onDurationTimer

38.214 subclauses 5.1.6.1 and 5.2.2.5

- ps-TransmitOtherPeriodicCSI → ps-TransmitPeriodicCSI