3GPP TSG RAN WG1 #103-e	   	 	  	                	      R1-2008578

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.1.3

Source:	LG Electronics

Title:	Enhancements on SRS flexibility, coverage and capacity

Document for:	Discussion and Decision

Introduction

The approved WID [1] includes enhancements on MIMO for NR, and main target of the enhancements is to enhance multi-beam operation, multi-TRP transmission and reception, SRS and CSI measurement and reporting. Specifically for SRS enhancements, there are three sub-topics regarding aperiodic SRS, SRS antenna switching, SRS capacity and coverage, respectively.

- Enhancement on SRS, targeting both FR1 and FR2: - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}) - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

In this contribution, we discuss the enhancements on SRS including triggering flexibility, antenna switching, coverage and capacity.

Discussion

Enhancements on aperiodic SRS

In Rel-15 NR MIMO, aperiodic SRS can be triggered by DCI per SRS resource set level except for 1T4R antenna switching. Also, each aperiodic SRS resource set has own slotOffset configured by RRC to be sounded from a slot of the triggering DCI reception. In that regard, there are several potential enhancement points on aperiodic SRS triggering. Firstly, as discussed in Rel-16 TEI [2], aperiodic SRS triggering DCI can be overloaded in a certain slot, since the slotOffset can only be configured by RRC signaling per SRS resource set. Regarding the triggering offset of aperiodic SRS, the following alternatives were discussed and agreed in RAN1#102-e meeting.

Agreement Enhance the determination of aperiodic SRS triggering offset, with at least one of the following alternatives - Alt 1: Delay the SRS transmission to an available slot later than the triggering offset defined in current specification, including possible re-definition of the triggering offset - Alt 2: Indicate triggering offset in DCI explicitly or implicitly - Alt 3: Update triggering offset in MAC CE - Further consideration aspects may include the cost v.s. the total combinations PDCCH and SRS locations for gNB to choose, DCI overhead, multi-UE SRS multiplexing, CA aspect, whether to have multiple opportunities to transmit SRS, etc.

We think the problem can be easily resolved by enabling update of slotOffset by MAC CE signaling or dynamically indicating SRS timing offset by DCI like DCI format 1A/2B/2C/2D in LTE. Furthermore, since SRS symbol-level position can be anywhere in a slot as enhanced in Rel-16 NR-U which is also applicable to NR licensed band [3], the update of SRS symbol-level position by MAC CE can be a benefit for multi-UE SRS multiplexing aspect. It can also help for increased repetition of SRS (being discussed in this agenda) to avoid multi-UE SRS collision in a busy UL slot.

Proposal #1: Support update of triggering offset and symbol-level starting position for SRS by a MAC CE.

Secondly, only one SRS resource set can be triggered by single DCI triggering, because only a single SRS resource set can be mapped to each codepoint of SRS request field except for antenna switching case (i.e., especially in case of 1T4R). To reduce DCI overhead, one potential enhancement is to support multiple SRS resource sets and/or multi-shot SRS to be triggered by a single DCI. Lastly, in the current specification, sounding bandwidth can only be configured by RRC level signaling per SRS resource so multiple SRS resource configuration is required to support dynamic switching of sounding band. This can also be a potential enhancement point for aperiodic SRS.

Proposal #2: Support triggering multiple SRS resource sets and/or triggering multi-shot SRS by a single DCI for DCI overhead reduction.

SRS antenna switching up to 8 antennas

In Rel-15 NR MIMO, it is supported up to 8 MIMO layers for DL data transmission. However, in the current specification, up to 4 Rx SRS antenna switching is supported for DL CSI acquisition. For that reason, introducing SRS antenna switching up to 8 Rx antennas can be beneficial in eMBB scenario. For SRS antenna switching up to 8 Rx antennas, legacy NR SRS antenna switching mechanism should be inherited as much as possible. In the last meeting, it had been discussed regarding supportable configurations of SRS antenna switching up to 8 antennas as below.

Agreement For SRS antenna switching up to 8Rx, study the configuration of {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R}. - Study points may include CSI latency, performance considering aspects like insertion loss, use cases, antenna structure, UE power saving, SRS resource configuration, etc..

Basically, configuration of 2T6R/2T8R/4T8R can be supported with less SRS resources configured in a SRS resource set. Additionally, for the configuration that y cannot be divided by x in xTyR, e.g., 4T6R, UE behavior should be specified for different SRS transmission occasions by multiple SRS resources.

On the other hand, maximum 2 Tx antennas for a UE are considered in RAN4 test case. Thus, in order to support 2T8R, the same issue as when designing 1T4R antenna switching occurs, i.e., 4 SRS resources which contains 2 different Rx antenna ports cannot be sounded within a slot in case of insufficient space of UE specific SRS symbol-level region, considering 3 gap symbols between 4 SRS resources. This issue becomes more severe if 1T8R antenna switching is considered, since 7 gap symbols are needed in total with 8 SRS resources due to the transient time for RF switching. It may require two or more SRS resource sets which have different values of slotOffset in order to enable 1T8R antenna switching. It means that a single antenna switching behavior can be spread across 3 different slots, which may lead to time varying phase rotation of each Rx/Tx antenna port.

Proposal #3: Support configuration of 2T6R/2T8R/4T8R in Rel-17 SRS antenna switching enhancement and further consider supporting configuration of 4T6R.

Proposal #4: For antenna switching across multiple slots (e.g., 3 slots) or with large number of switching, the impact of time varying channel and phase rotation should be taken into account.

From UE implementation perspective, 6 or 8 Rx antenna ports are likely to be implemented with multiple panels in both FR1 and FR2. In this regard, the design of SRS antenna switching should take multiple UE panels into account with consideration of Rel-17 multi-panel UE related enhancements. As a starting point, the discussion outcome of Rel-16 multi-panel UE can be considered, which are captured in Table 1.

Proposal #5: Antenna switching across UE multiple panels should be considered.

Table 1. Multi-panel UE related agreements in Rel-16 NR MIMO multi-beam enhancement

Agreement@RAN1#95 In Rel-16, an identifier (ID) that can be used at least for indicating panel-specific UL transmission is supported, where detailed usages for the panel-specific UL transmission are FFS - The ID should be defined considering the possibility to reuse/modification of Rel-15 specification support or introducing new ID - Note: RAN1 to avoid unnecessary specification support requiring UE to explicitly disclose its UL antenna panel implementation - FFS: Whether UE capability signalling is introduced for panel-specific UL transmission Agreement@RAN1#AH1901 An identifier (ID), agreed in RAN1#95, that can be used at least for indicating panel-specific UL transmission is to be down-selected or merged from the following alternatives in next RAN1 meeting: - Alt.1: an SRS resource set ID, where FFS on further association to other RS (if needed) - Alt.2: an ID, which is directly associated to a reference RS resource and/or resource set - Alt.3: an ID, which can be assigned for a target RS resource or resource set - Alt.4: an ID which is additionally configured in spatial relation info For purpose of further discussion on this topic for RAN1#96 and future meetings Following multi-panel UE (MPUE) categories can be used for discussions on possible enhancements over Rel-15, if needed. - MPUE-Assumption1: Multiple panels are implemented on a UE and only one panel can be activated at a time, with panel switching/activation delay of [X] ms - MPUE-Assumption2: Multiple panels are implemented on a UE and multiple panels can be activated at a time and one or more panels can be used for transmission - MPUE-Assumption3: Multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission Note: Above does not imply the support of either one or both of the categories but is only for efficient discussions at least for this meeting, which may also be updated further. Whether to support either one or both categories will depend on subsequent discussions Note: There is no consensus among the companies in RAN1 whether MPUE-Assumption2 is in the work scope of Rel-16 WI Working Assumption@RAN1#96bis The agreed ID (not excluding to reuse existing ID) for a panel can be used for panel-selection-based transmission of PUSCH, PUCCH and SRS, among multiple activated panels. - FFS details, including an explicit/implicit indication of the panel, also considering beam correspondence at UE. FFS on whether the ID can be used for panel-specific PRACH transmission, if supported. Conclusion@RAN1#98 - From RAN1 point of view, a “UE panel” would be a logical entity and how to map physical UE antennas to the logical entity is up to UE implementation. - (Informative) For certain condition(s), gNB can assume the mapping between UE’s physical antennas to the logical entity “UE panel” activated for transmission will not be changed - FFS: Whether “UE panel” is transparent to gNB - FFS: UE capability includes at least the number of “UE panels”. - FFS: Whether/how to define the certain condition(s) - For example: The duration of time over which the gNB assumes there will be no change - For example: Until next update or report from UE - (Informative) Depending on UE’s own implementation, a “UE panel” can have at least the following functionality as an operational role of - Unit of antenna group to control its Tx beam independently

Enhancements on SRS capacity and/or coverage

In RAN1#102-e meeting, three categories were agreed for enhancement on SRS capacity and/or coverage as follows.

Agreement For SRS coverage/capacity enhancements, evaluate and, if needed, specify one or more from three categories based on the following definition. - Class 1 (Time bundling): Utilize relationship among two or more occasions of one or more SRS resources in one or more slots to enable joint processing within time domain. - Study aspects include the issue of phase discontinuity, interruption of SRS transmission by other UL signals, etc.. - Class 2 (Increase repetition): Change the legacy SRS pattern in one resource and one occasion from time domain by increasing SRS symbols for repetition. - Study aspects include to use TD-OCC to compensate the negative impact on SRS capacity, inter-cell interference randomization, whether these SRS symbols are in one slot or consecutive slots, etc.. - Class 3 (Partial frequency sounding): Support more flexibility on SRS frequency resources to allow SRS transmission on partial frequency resources within the legacy SRS frequency resources. - Study aspects include the partial frequency resources are with RB level or subcarrier level (e.g., larger comb, partial bandwidth), PAPR issue, etc..

Among three categories, we think class 2 (increase repetition) should be handled first, since it was already supported in the other WI in Rel-16. In Rel-15 specification, SRS can only be transmitted on the last 6 symbols within a slot, and the maximum configurable number of symbols for an SRS resource is 4. In Rel-16 NR-U, RRC configurable symbol-level starting position of an SRS resource was enhanced to be all of symbols within a slot. Furthermore, in Rel-16 NR positioning, the maximum configurable number of symbols for an SRS resource was increased to 12, i.e., NsymbSRS∈{1,2,4,8,12}. These enhancements in Rel-16 can be a starting point for the discussion on Rel-17 SRS capacity and/or coverage enhancements.

Proposal #6: For class 2, Rel-16 SRS enhancements on NR-U and NR positioning can be a starting point for the discussion on increased SRS repetition.

Additionally, class 3(Partial frequency sounding) can be discussed with second priority. if symbol-level starting position of an SRS resource other than the last 6 symbols within a slot is allowed and the maximum configurable number of symbols for an SRS resource is increased to 12 or more in Rel-17, it should be carefully investigated regarding the potential shortage of UL resource (slot) for PUCCH/PUSCH, including whether existing collision handling specified in Rel-15/16 specification is sufficient to handle different cases. Additionally, multi-UE SRS scheduling will be more complicated when the SRS duration is increased from gNB perspective. In order to enable the multi-UE SRS scheduling on the same symbols, dynamic frequency domain resource allocation for aperiodic SRS can be considered.

Proposal #7: Dynamic frequency domain resource allocation for aperiodic SRS can be considered for delicate control on avoiding collision of UL signals from multiple UEs at gNB side by considering expanded SRS duration and a large variation of SRS hopping behavior.

Conclusion

In this contribution, we discuss our views for Rel-17 enhancements on SRS, and propose the followings based on the discussion.

Proposal #1: Support update of triggering offset and symbol-level starting position for SRS by a MAC CE.

Proposal #2: Support triggering multiple SRS resource sets and/or triggering multi-shot SRS by a single DCI for DCI overhead reduction.

Proposal #3: Support configuration of 2T6R/2T8R/4T8R in Rel-17 SRS antenna switching enhancement and further consider supporting configuration of 4T6R.

Proposal #4: For antenna switching across multiple slots (e.g., 3 slots) or with large number of switching, the impact of time varying channel and phase rotation should be taken into account.

Proposal #5: Antenna switching across UE multiple panels should be considered.

Proposal #6: For class 2, Rel-16 SRS enhancements on NR-U and NR positioning can be a starting point for the discussion on increased SRS repetition.

Proposal #7: Dynamic frequency domain resource allocation for aperiodic SRS can be considered for delicate control on avoiding collision of UL signals from multiple UEs at gNB side by considering expanded SRS duration and a large variation of SRS hopping behavior.

References

[1] RP-193133, WID proposal for Rel.17 enhancements on MIMO for NR

[2] R1-1910289, TEI proposal on aperiodic RS triggering offset, ZTE

[3] R1-2007326, Updated RAN1 UE features list for Rel-16 NR including remaining RAN1 issues, Moderators (AT&T, NTT DOCOMO, INC.)