3GPP TSG RAN WG1 Meeting #103-e	<TDoc#>

# E-meeting,  October 26th – November 13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | <CR#> | rev | - | Current version: | v16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the subcarrier allocation for sub-PRB in CE Mode B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For sub-PRB in both CE Mode A and CE Mode B, the allocated number of subcarriers is obtained using Table 8.1.6-1, which applies a modulo operation on the variable “”. The variable “” consists of 6-bits for CE Mode A and 4-bits for CE Mode B and is obtained respectively from the 'resource allocation' field in the scheduling grant. However, for CE Mode B, the variable “” is currently undefined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 8.1.6-1 of TS 36.213 where the variable “” is cited and refers to the “resource allocation field”, the corresponding clauses for CE Mode A and CE Mode have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If “” remains undefined for CE Mode B, it won’t be possible to apply the modulo operation in Table 8.1.6-1, hence it won’t be possible to allocate subcarriers in CE Mode B. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------------------- Text Starts -----------------------------------------------------------------

Table 8.1.6-1: Subcarriers allocation for BL/CE UE.

| = value of resource allocation field in clause 5.3.3.1.10 for CE Mode A and in clause 5.3.3.1.11 for CE Mode B [4] or indicated by higher layer parameter prb-AllocationInfo in PUR-Config | Modulation | Set of Allocated subcarriers |
| --- | --- | --- |
| 0 | π/2-BPSK |  |
| 1 | π/2-BPSK |  |
| 2 | π/2-BPSK |  |
| 3 | π/2-BPSK |  |
| 4 | QPSK | 0,1,2 |
| 5 | QPSK | 3,4,5 |
| 6 | QPSK | 6,7,8 |
| 7 | QPSK | 9,10,11 |
| 8 | QPSK | 0,1,2,3,4,5 |
| 9 | QPSK | 6,7,8,9,10,11 |

----------------------------------------------------------------- Text Ends -----------------------------------------------------------------