3GPP TSG RAN WG1 Meeting #103-e	R1- 2008591

October 26th – November 13th, 2020

Agenda item:	5

Source: 	Qualcomm Incorporated

Title: 	Draft reply LS on RAN 2 LS on beamSwitchTiming

Document for:	Discussion/Decision

- Draft Reply LS

1. Introduction:

RAN1 would like to thank RAN2 for the LS R2-2008318 on beamSwitchTiming. We provided answers to the questions in the following.

Q1: RAN2 kindly requests RAN1 to confirm the following understanding

- For Rel-15 UEs, - if the UE indicates a value from the set {14, 28, 48} in Rel-15 capability signaling, the reported value is used for all cases, including CSI-RS with repetition is configured and set to “ON” or “OFF”. - if the UE indicates a value from the set {224, 336} in Rel-15 capability signaling, there is no specified beahviour for value of 224 and 336. - If the Rel-15 capability signaling (beamSwitchTiming) is not included, the beam switch timing is up to 48 OFDM symbols for each supported sub-carrier spacing.

A1: RAN1 confirms the above understanding.

Q2: RAN2 kindly requests RAN1 to confirm the following understanding

- For Rel-16 UEs, the UE signals beamSwitchTiming and beamSwitchTiming-r16. The first one, i.e., the Rel-15 capability signaling from the set {14, 28, 48}, is always signaled (if this capability needs to be reported). The second one, i.e., the newly introduced Rel-16 capability signaling from the set {224, 336}, is optional. - If the UE doesn’t include the Rel-16 capability, it behaves in all respects as the Rel-15 UEs above. - If the UE does include the Rel-16 capability, then, the UE may or may not receive configured parameter enableBeamSwitchTiming-r16 in the RRC configuration: - If the UE doesn’t receive parameter enableBeamSwitchTiming-r16, it behaves in all respect as the Rel-15 UE above. - If the UE receives parameter enableBeamSwitchTiming-r16, then, the UE does the following: - Question A: For CSI-RS configured with repetition and set to “ON”, whether the UE indicates switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signaling or the default value of 48? - Question B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling? - Question C: For CSI-RS configured without repetition, whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling?

A2: RAN1 has the following understanding

- Both beamSwitchTiming and beamSwitchTiming-r16 are optional to report

- If the UE doesn’t include the beamSwitchTiming-r16, its behaviour is same as Rel-15

- If the UE does include the Rel-16 capability, UE may or may not receive enableBeamSwitchTiming-r16

- If the UE does include the Rel-16 capability but doesn’t receive enableBeamSwitchTiming-r16, its behaviour is same as Rel-15

- If the UE receives enableBeamSwitchTiming-r16

  - For CSI-RS configured with repetition “ON”, the UE indicates switch time that is the same as the signalled value from the set {224, 336}

  - For CSI-RS configured with repetition “OFF”, the UE indicates switch time of 48 if beamSwitchTiming-r16 is reported

  - For CSI-RS configured without repetition, the UE indicates switch time of 48 if beamSwitchTiming-r16 is reported

2. Actions:

To RAN2 group

RAN1 respectfully requests RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #104-e	25 January – 05 February 2021, Online

TSG RAN WG1 Meeting #104-bis-e	12 – 20 April 2021, Online

- Discussions

The spec related to Q1 is summarized below

- The 1st bullet in Q1 is indicated by the following text in [1]

  - If reported value is in {14, 28, 48}, it is used for all cases including CSI-RS for CSI, TRS, and BM

- The 2nd bullet is also indicated in [1]

  - {224, 336} is not mentioned as well as the corresponding behaviour

- The 3rd bullet is indicated in [2]

##### 38.214 v15.11.0 ##### 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS […] - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, periodic CSI-RS, semi-persistent CSI-RS; - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. […]

The spec/agreement related to Q2 is summarized below

- Both beamSwitchTiming and beamSwitchTiming-r16 are optional to report

  - This is indicated in [3]

- If the UE doesn’t include the beamSwitchTiming-r16, its behaviour is same as Rel-15

  - This is indicated in [4]

- If the UE does include the Rel-16 capability, UE may or may not receive enableBeamSwitchTiming-r16

  - This is based on [5]

- If the UE does include the Rel-16 capability but doesn’t receive enableBeamSwitchTiming-r16, its behaviour is same as Rel-15

- If the UE receives parameter enableBeamSwitchTiming-r16

  - For CSI-RS configured with repetition “ON”, the UE indicates switch time that is the same as the signalled value from the set {224, 336}

    - This is based on the agreement in RAN1 #98 below that 224 or 336 is only used for CSI-RS with repetition “ON”.

      - The [4] is not aligned with the #98 agreement on this point. Specifically, this part should be updated as “without higher layer parameter trs-Info and without higher layer parameter repetition configured as ON”.

  - For CSI-RS configured with repetition “OFF”, the UE indicates switch time of 48 if beamSwitchTiming-r16 is reported

    - This is indicated in [4]

  - For CSI-RS configured without repetition, the UE indicates switch time of 48 if beamSwitchTiming-r16 is reported

    - This is indicated in [4]

##### 38.214 v16.3.0 ##### 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS […] - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS; - else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored; - else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or is equal to or greater than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. […]

##### RAN1 #98 Agreements: Regarding TEI “Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336” - 48 is used as the beam switching threshold for UEs reporting 224 or 336 - When using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI.

- References

- TS 38.214 v15.11.0

- TS 38.306 v15.11.0

- TS 38.306 v16.2.0

- TS 38.214 v16.3.0

- TS 38.331 v16.1.0