3GPP TSG RAN WG1 #102e 	R1- 2008608

October 26th – November 13th, 2020

Agenda item:	7.2.5

Source: 	Qualcomm

Title: 	Remaining Issues on UCI Enhancements for URLLC

Document for:	Discussion and Decision

# 1	Introduction

In this document, the text proposals for the following topics are presented. Each corresponding section also includes a reason for the proposed addition or modification.

| Section 2 | Intra-UE prioritization for PUCCH repetition |
| --- | --- |
| Section 3 | Clarification of sub-slot based Type 1 HARQ-ACK codebook |

# 2	Prioritization for PUCCH repetition

In the current NR standard TS 38.213 v16.2.0, the UE behavior for handling overlapping PUCCH/PUSCH transmissions with different priorities are specified. However, if one of the PUCCH is with repetitions, then the UE behavior is missing. This issue was identified and discussed in 3GPP RAN1 #102_e meeting. A reasonable UE behavior is to drop the low priority channel in the overlapping slots, and the UE shall drop the overlapping low priority channels per slot.

| Reason for change: | In TS 38.213 v16.3.0, the UE behavior for handling overlapping PUCCH/PUSCH transmissions with repetitions and with different priorities are missing. The TP in this section specifies the UE behavior to resolve overlapping PUCCH/PUSCH transmissions with repetitions and with different priorities. |
| --- | --- |
| Summary of change: | The current UE behavior to cancel low priority channel due to collision with high priority channel are extended to cover the case of at least one of the overlapping channels are scheduled with repetitions. |
| Consequences if not approved: | PUCCH repetition is broken whenever the UE is configured with two priorities at the physical layer. |

Modified clause (Section 9 of TS 38.213)

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clause 9.2.5 and 9.2.6. Then,

-	if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time in a slot with a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission in the slot

-	if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time in a slot with a transmission of a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission in the slot

where

-	the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5 and 9.2.6

-	the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability

------------------------------------ Omitted parts --------------

If a UE would transmit the following channels, including repetitions if any, that would overlap in time

-	a first PUCCH of larger priority index with SR and a second PUCCH or PUSCH of smaller priority index, or

-	a configured grant PUSCH of larger priority index and a PUCCH of smaller priority index, or

-	a first PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH and a second PUCCH of smaller priority index with SR and/or CSI, or a configured grant PUSCH with smaller priority index, or a PUSCH of smaller priority index with SP-CSI report(s) without a corresponding PDCCH, or

-	a PUSCH of larger priority index with SP-CSI reports(s) without a corresponding PDCCH and a PUCCH of smaller priority index with SR, or CSI, or HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH, or

-	a configured grant PUSCH of larger priority index and a configured PUSCH of lower priority index on a same serving cell

the UE is expected to cancel the PUCCH/PUSCH transmissions of smaller priority index before the first symbol overlapping with the PUCCH/PUSCH transmission of larger priority index in the overlapping slot(s).

End

# 3 Clarification of sub-slot based Type 1 HARQ-ACK codebook

In previous 3GPP RAN1 meetings, it was agreed that R15 HARQ-codebook construction is applied in unit of sub-slot at least for Type-2 HARQ-ACK codebook in Rel-16. For Type-1 codebook, it is our understanding that sub-slot based construction was also supported in the specification by interpreting “slot” as “sub-slot” if UE is configured with subslotLength-ForPUCCH. However, the pseudo code in TS 38.213 is broken when UE is configured with subslotLength-ForPUCCH. More specifically, according to the spec,  nU in the pseudo code shall be interpreted as the subslot index, but the corresponding pseudo code is problematic for TDD system, since we need to first convert nU to an uplink slot index, and then map the slot index nU to a downlink slot. Furthermore, for Type-1 HARQ-ACK codebook construction has large redundancies as analyzed in [1].

In this section, we propose some TP to fix the issues analyzed above in TS 38.213.

| Reason for change: | 1. In TS 38.213 v16.3.0, the Type-1 HARQ-ACK codebook design as provided in Section 9.1.2.1 is broken for TDD and when the UE is provided by subslotLength-ForPUCCH. 2. The Type-1 HARQ-ACK codebook design as provided in Section 9.1.2.1 results in a large redundancy in the codebook size, when the UE is provided by subslotLength-ForPUCCH. |
| --- | --- |
| Summary of change: | The pesudo code in Section 9.1.2.1 was modified to account for - The slot nU denotes subslot instead of slot - To determine the PDSCH occasions for the Type 1 HARQ-ACK CB, SLIV entries that does not end in the subslot is removed from counting. |
| Consequences if not approved: | The Type 1 HARQ-ACK codebook as provided in Section 9.1.2.1 is broken for TDD and when the UE is provided by subslotLength-ForPUCCH. |

Modified clause (Section 9 of 38.212)

For the set of slot timing values , the UE determines a set of  occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases.

Set  - index of occasion for candidate PDSCH reception or SPS PDSCH release

Set

Set

Set  to the cardinality of set

Set k =0 – index of slot timing values , in descending order of the slot timing values, in set  for serving cell

If the UE is provided subslotLength-ForPUCCH

Set Nslotsubslot to be the number of subslots contained in an uplink slot on the PUCCH cell

else

Set Nslotsubslot=1

end if

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while

if modnU -K1,kNslotsubslot+1,max2μUL-μDL,1=0 or if the UE is provided subslotLength-ForPUCCH

Set  – index of a DL slot within an UL slot

while nD<max2μDL-μULNslotsubslot,  1

Set  to the set of rows

Set  to the cardinality of

Set  – index of row in set

if slot  starts at a same time as or after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot (nU-K1,k) ⋅2μDL-μUL/Nslotsubslot+nD is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell

nD=nD+1;

else

while

if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot (nU-K1,k) ⋅2μDL-μUL/NslotsubslotnU-K1,k⋅2μDL-μUL+nD-NPDSCHrepeat,max+1 to slot (nU-K1,k) ⋅2μDL-μUL/NslotsubslotnU-K1,k⋅2μDL-μUL+nD, at least one symbol of the PDSCH time resource derived by row  is configured as UL where  is the k-th slot timing value in set ,

;

else if the UE is provided subslotLength-ForPUCCH and the end of the PDSCH time resource derived by row r in slot (nU-K1,k) ⋅2μDL-μUL/Nslotsubslot+nD does not overlap with the PUCCH slot nU-K1,k

;

else

;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

;

;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row

;

;

else

;

end if

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

nD=nD+1;

end if

end while

end if

;

end while

else

while

if  modnU-K1,kNslotsubslot+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0 or if the UE is provided subslotLength-ForPUCCH

Set  – index of a DL slot within an UL slot

while

Set  to the set of rows

Set  to the cardinality of

Set  – index of row in set

if slot  starts at a same time as or after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot (nU-K1,k) ⋅2μDL-μUL/NslotsubslotnU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD

is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell

nD=nD+1;

else

while

if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot (nU-K1,k) ⋅2μDL-μUL/NslotsubslotnU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD-NPDSCHrepeat,max+1 to slot (nU-K1,k) ⋅2μDL-μUL/NslotsubslotnU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD, at least one symbol of the PDSCH time resource derived by row  is configured as UL where  is the k-th slot timing value in set ,

;

else if the UE is provided subslotLength-ForPUCCH and the end of the PDSCH time resource derived by row r in slot (nU-K1,k) ⋅2μDL-μUL/Nslotsubslot+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD does not overlap with the slot nU-K1,k

;

else

;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

;

;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row

;

;

else

;

end if

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

nD=nD+1;

end if

end while

end if

;

end while

end if

End

# References

  - R1-2005701, UE feedback enhancement for HARQ-ACK, 3GPP TSG RAN WG1#102_e, August 17-28, 2020.