# 3GPP TSG RAN WG1 #103-e		                                    R1-2008633

# e-Meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.4

Source: 	ASUSTeK

Title:  	Remaining issues for Mode 2 resource allocation in NR V2X

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss remaining issues for mode 2 resource allocation in NR V2X.

- Discussion

Short reserved period

RAN1 #99 Agreements: - On a per resource pool basis, when reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is enabled: - A period is additionally signalled in SCI and the same reservation is applied with respect to resources indicated within NMAX within window W at subsequent periods - A set of possible period values is the following: 0, [1:99], 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000 ms - <= 4 bits are used in SCI to indicate a period - An actual set of values is (pre-)configured RAN1 #100e Agreements: - On a per resource pool basis, when reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is enabled: - A set of possible period values additionally includes all integer values from 1 to 99 ms

In LTE sidelink V2X, short reserved period only supports 20ms, 50ms. When a RX UE receives a SCI indicative of reserved period Prsvp_RX=20 ms or 50 ms in a slot tmSL, the RX UE would consider same frequency assignment of the SCI for reserving resources in slots tm+q*Prsvp_RX'SL, q=1, 2, … 10020=5 for Prsvp_RX=20 ms, or q=1, 10050=2 for Prsvp_RX=50 ms. In our view, such mechanism for short reserved period could avoid collision when the RX UE is triggered for resource selection since the RX UE does not know whether tm+q*Prsvp_RX'SL, q=1 reserves tm+q*Prsvp_RX'SL q=2 or not. Hence, when the RX UE performs resource identification, the RX UE excludes more than one reserved slots associated to the SCI indicative of the short reserved period. Figure 1 is an example for illustrating when UE excludes multiple slots in a periodic manner based on the received SCI.

NR sidelink V2X supports short reserved period Prsvp_RX=1 to 99 ms. If applying similar mechanism for short reserved period as LTE sidelink V2X, it may have impact on multiple reserved slots for some short reserved periods. For example, like short reserved period Prsvp_RX=1, 2, …9 ms, it may cause dense resource exclusion which may be hard for the RX UE to identify and select a resource for sidelink transmission. For example, in Figure. 2, a UE receives a SCI in slot tmSL, and the SCI indicates a reserved period Prsvp_RX=1 ms. The UE, when identifying resource in step 1, would exclude same frequency assignment of the SCI every slots due to SCI reserving resource via reserved period Prsvp_RX=1 ms. Thus, it may be hard for UE to identify qualified resource. Consequently, if the UE cannot identify 20% resources among the selection window, the UE may relax RSRP threshold which may cause the selected resource with higher interference. In addition, it’s unfair for a UE to reserve resource via indicating very short reserved period. Therefore, this issue needs to solve.

Observation: 	For short reserved period (e.g., 1~9ms), it may cause dense resource reservation which may make other UE hard for identify and select resource.

Figure. 1

Figure. 2

In our view, one possible solution is via relaxing the number of excluded reserved slots associated to short reserved period rather than all reserved slots associated to the short period in selection window. Since a UE which reserves resource via very short period may not use in every reserved slots, some of reserved slots could be used by other UE. For example, in Figure. 3, a UE, when identifying resource, only excludes a few number of reserved slots associated to the short reserved period in a short duration (i.e., Tscal_short) rather than in a legacy duration (i.e., Tscal). Another solution is specifying a few number of reserved slots based on an upper reserved period (e.g., 10ms or 20ms) instead of short reserved period (e.g., the few number Q=Tscalmax(Prsvp_RX,10ms) instead of Q=TscalPrsvp_RX).

Figure. 3

Proposal 1: 	For a short reserved period, Prsvp_RX indicated by SCI received in slot tmSL, UE excludes a candidate resource if the condition in TS 38.214, section 8.1.4 bullet (6) are all met with update that Q is determined in the following

    - Alt 1: Q=Tscal_shortPrsvp_RX, where Tscal_short<Tscal and Tscal_short is 20ms specified in specification

    - Alt 2 :  Q=Tscalmax(Prsvp_RX,Pupper), where Prsvp_RX≤Pupper and Pupper is 20ms specified in specification

Proposal 2: 	Adopt Text proposal 1 or Text proposal 2 in updating of TS 38.214 section 8.1.4

Text proposal 1

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <…> 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms if Prsvp_RX≥20ms; otherwise Tscal is set to 20ms.

Text proposal 2

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <…> 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=Tscalmax (Prsvp_RX, 20ms) if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

- Conclusion

In this contribution, we propose the following:

Observation: 	For short reserved period (e.g., 1~9ms), it may cause dense resource reservation which may make other UE hard for identify and select resource.

Proposal 1: 	For a short reserved period, Prsvp_RX indicated by SCI received in slot tmSL, UE excludes a candidate resource if the condition in TS 38.214, section 8.1.4 bullet (6) are all met with update that Q is determined in the following

    - Alt 1: Q=Tscal_shortPrsvp_RX, where Tscal_short<Tscal and Tscal_short is 20ms specified in specification

    - Alt 2 :  Q=Tscalmax(Prsvp_RX,Pupper), where Prsvp_RX≤Pupper and Pupper is 20ms specified in specification

Proposal 2: 	Adopt Text proposal 1 or Text proposal 2 in updating of TS 38.214 section 8.1.4

Text proposal 1

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <…> 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms if Prsvp_RX≥20ms; otherwise Tscal is set to 20ms.

Text proposal 2

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <…> 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=Tscalmax (Prsvp_RX, 20ms) if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

- Reference

- TS 38.214, v16.2.0