3GPP TSG-RAN WG1 Meeting #103e	R1-2008635

e-Meeting, October 26 – November 13, 2020

| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.214 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TCI states for Aperiodic CSI-RS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#102-e, agreements were made on default QCL for aperiodic CSI-RS for cases involving both multi-DCI based multi-TRP and single-DCI based multi-TRP. However, these agreements are not reflected in the specifications. This CR proposes to cature these agreements in 38.214. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the texts related to agreements made on default QCL for aperiodic CSI-RS from RAN1#102-e for both multi-DCI based multi-TRP and single-DCI based multi-TRP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreements from RAN1#102-e on default QCL for aperiodic CSI-RS are not captured in specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

---- Unchanged texts omitted ----

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided.

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], a second aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, a second aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS;

- if the UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint is mapped with two TCI states, when the other signal is a PDSCH scheduled with two TCI states, the UE applies the QCL assumption corresponding to the first TCI state of those two TCI states for receiving the aperiodic CSI-RS;

- else if the UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the other DL signal refers to a PDSCH associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS, the second aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS, periodic CSI-RS, or semi-persistent CSI-RS;

-	else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS,

- if the UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint is mapped with two TCI states, the UE applies the QCL assumption corresponding to the first one of two TCI states corresponding to the lowest DCI codepoint among those mapped to two TCI states;

- else if UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs , which are configured with the same value of CORESETPoolIndex as the CORESET that carries the PDCCH triggering that AP CSI-RS, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the CORESET that carries the PDCCH triggering that AP CSI-RS;

- else, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored;

-	else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

---- Unchanged texts omitted ----