3GPP TSG-RAN WG1 Meeting #103e	R1-2008638

e-Meeting, October 26 – November 13, 2020

| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.214 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TCI state codepoint mapping for DCI format 1_2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In an LS sent to RAN1 (R2-2005811), RAN2 asked the following question: Question 1: Could RAN1 specify how the TCI state codepoints are mapped when the number of codepoints in the TCI field of DCI format 1_2 is less than the maximum number of codepoints in the TCI field of DCI format 1_1 taking into account that the MAC CE may signal TCI states corresponding to maximum amount of DCI codepoints.? In the Reply LS (R1-2007197), RAN1 agreed to reply “RAN1 will specify the mapping for TCI codepoints of DCI format 1_2, of which details are under discussion.” However, the mapping for TCI codepoints of DCI format 1_2 are yet to be specified in RAN1 specifications. This CR addresses the clarification needed to specify the TCI codepoint mapping of DCI format 1_2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TCI codepoint mapping for DCI format 1_2 is specified when the number of codepoints in the TCI field of DCI format 1_2 is less than the maximum number of codepoints in the TCI field of DCI format 1_1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear TCI codepoint mapping for DCI format 1_2 when the number of codepoints in the TCI field of DCI format 1_2 is less than the maximum number of codepoints in the TCI field of DCI format 1_1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.1.5	Antenna ports quasi co-location

---- Unchanged texts omitted ----

The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP or in a set of CCs/DL BWPs, respectively. When a set of TCI state IDs are activated for a set of CCs/DL BWPs, where the applicable list of CCs is determined by indicated CC in the activation command, the same set of TCI state IDs are applied for all DL BWPs in the indicated CCs.

When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command.

When the DCI field 'Transmission Configuration Indication' is present in both DCI formats 1_1 and 1_2 and when the number of codepoints S in the 'Transmission Configuration Indication' field of DCI format 1_2 is less than the number of codepoints in the 'Transmission Configuration Indication' field of DCI format 1_1, the activated TCI states mapped to the first S 'Transmission Configuration Indication' field codepoints in DCI format 1_1 are mapped to the 'Transmission Configuration Indication' field codepoints in DCI format 1_2.

---- Unchanged texts omitted ----