3GPP TSG RAN WG1 Meeting #103-e	R1-2008643

e-Meeting, October 26th – November 13th, 2020

Title:	[Draft] Reply LS on cell-grouping UE capability for synchronous NR-DC

Response to:              R2-2008662

Release:	Release 16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	vivo [To be TSG RAN WG1]

To:	TSG RAN WG2, TSG RAN WG4

Cc:

Contact Person:

Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 would like to thank RAN2 for the LS R2-2008662.

RAN1 discussed the cell grouping signaling including the differentiation between MCG and SCG for synchronous NR-DC, and concluded the following reasons on the necessity are valid.

- NR supports FR2 (as a key difference from LTE)

  - The UE may not support FR2 MCG and thus there might be need to indicate whether FR2 MCG is supported in NR-DC if RAN4 has such potential band combinations.

  - The UE may not support FR1-FR2 CA and thus there might be need to differentiate whether it is a FR1-FR2 CA or a FR1-FR2 DC.

- The UE may not support inter-CG power sharing for a given frequency range.

- The UE may not support PUCCH group across serving cells with different numerologies.

2. Actions:

To RAN2 group.

ACTION: RAN1 see the need and benefits to introduce the cell grouping UE capability signalling, including the differentiation between MCG and SCG for synchronous NR-DC.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #104-e 	Jan. 25 – Feb.05, 2021			e-Meeting

TSG-RAN WG1 Meeting #104bis-e 	April 12 – 20, 2021			e-Meeting