3GPP TSG-RAN WG1 Meeting # 103-e	R1-2008656

# e-Meeting,  October 26th - November 13th

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on correction for Msg3 retransmission power control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN1#102e, R1-2007170 was approved to specify the PO_UE_PUSCH for msg3 retransmission, by setting the same PO_UE_PUSCH value between the initial msg3 transmission and msg3 retransmission to avoid the impact on the performance of the msg3 retransmission. However, similar issue also exists for determining the pathloss RS for msg3 retransmission in TS38.213, i.e., the msg3 retransmission may base on a RS for pathloss calculation different from that for the initial msg3 transmission according to the current spec. More specifically, msg3 retransmission would use the same RS resource as for PUCCH transmission in the PUCCH resource with the lowest index, or the RS with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero to determine , while msg3 initial transmission uses the RS selected for the corresponding PRACH transmission. If reference RS for determining is not the same between the initial msg3 transmission and msg3 retransmission, the difference between transmission power of initial msg3 transmission and msg3 retransmission may be large, thus causing frequent random-access failures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Using the RS for the corresponding PRACH transmission as the RS for pathloss for msg3 retransmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will cause frequent random-access failure in the system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: It is expected UEs and networks are implemented with the same understanding that initial msg3 transmission and msg3 retransmission use the same RS for pathloss determination, thus no change is required. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

7.1.1 UE behaviour

<Unchanged part omitted>

-	is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index  for the active DL BWP, as described in Clause 12, of carrier  of serving cell

-	If the UE is not provided PUSCH-PathlossReferenceRS or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource from the SS/PBCH block that the UE uses to obtain MIB

-	If the UE is configured with a number of RS resource indexes, up to the value of maxNrofPUSCH-PathlossReferenceRSs, and a respective set of RS configurations for the number of RS resource indexes by PUSCH-PathlossReferenceRS, the set of RS resource indexes can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource index  in the set of RS resource indexes to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pusch-PathlossReferenceRS-Id in PUSCH-PathlossReferenceRS

-	For a PUSCH (re)transmission corresponding to a RAR UL grant as described in Clause 8.3, the UE uses the same RS resource index  as for a corresponding PRACH transmission

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of PUSCH-PathlossReferenceRS-Id, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 and a set of PUSCH-PathlossReferenceRS-Id values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the RS resource index  from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the SRI field value where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format 0_0, and if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP  of each carrier  and serving cell , as described in Clause 9.2.2, the UE uses the same RS resource index  as for a PUCCH transmission in the PUCCH resource with the lowest index

-	If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format 0_0 and if the UE is not provided a spatial setting for a PUCCH transmission, or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, the UE determines a RS resource index  with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig , a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	For a PUSCH transmission configured by ConfiguredGrantConfig that does not include rrc-ConfiguredUplinkGrant, the UE determines a RS resource index  from a value of PUSCH-PathlossReferenceRS-Id that is mapped to a SRI field value in a DCI format activating the PUSCH transmission. If the DCI format activating the PUSCH transmission does not include a SRI field, the UE determines a RS resource index  with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

<Unchanged part omitted>