3GPP TSG RAN WG1 #103-e		R1-2008660

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Miscellaneous corrections for 2-step RACH

Agenda Item:	7.2.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss the text proposals for miscellaneous corrections for 2-step RACH.

- Discussion and text proposal

  - Text proposal in 38.213

In 38.213 [2], msgADeltaPreamble indicates a power offset between msgA PUSCH and preamble. However, in 38.331 [4], the name for this parameter is msgA-DeltaPreamble, which is shown in the following table. The name of this parameter is not aligned.

MsgA-PUSCH-Config field descriptions

……

msgA-DeltaPreamble Power offset of msgA PUSCH relative to the preamble received target power (see TS 38.213 [13], clause 7.1).

……

Hence, we have the following text proposal #1 for Section 7.1.1 of TS 38.213:

-----------------start of TP #1 for 38.213-------------------------

#### 7.1.1	UE behaviour

<Unchanged part omitted>

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by preambleReceivedTargetPower and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

----------------end-------------------------

Proposal 1: Adopt the text proposal #1 for 38.213.

  - Text proposal in 38.211

In 38.211 [1], msgA-dataScramblingIdentity indicates the identifier used to initiate data scrambling (c_init) for msgA PUSCH. However, in 38.331 [4], the name for this parameter is msgA-DataScramblingIndex, which is shown in the following table. The name of this parameter is not aligned.

MsgA-PUSCH-Config field descriptions

msgA-DataScramblingIndex Identifier used to initiate data scrambling (c_init) for msgA PUSCH. If the field is absent the UE applies the value Physical cell ID (physCellID).

Hence, we have the following text proposal #2 for Section 6.3.1.1 of TS 38.211:

-----------------start of TP #2 for 38.211-------------------------

#### 6.3.1.1	Scrambling

<Unchanged part omitted>

cinit=nRNTI∙216+nRAPID∙210+nIDfor msgA on PUSCHnRNTI∙215+nIDotherwise

where

-	 equals the higher-layer parameter dataScramblingIdentityPUSCH if configured and the RNTI equals the C-RNTI, MCS-C-RNTI, SP-CSI-RNTI or CS-RNTI, and the transmission is not scheduled using DCI format 0_0 in a common search space;

-	nID∈0,1,…,1023 equals the higher-layer parameter msgA-DataScramblingIndex if configured and the PUSCH transmission is triggered by a Type-2 random access procedure as described in clause 8.1A of [5, TS 38.213];

-	 otherwise

----------------end-------------------------

Proposal 2: Adopt the text proposal #2 for 38.211.

In 38.211 [1], msgA-PRACH-ConfigurationIndex indicates the PRACH configuration index for 2-step RA. However, the suffix “-r16” is not needed.

Hence, we have the following text proposal #3 for Section 6.3.3.2 of TS 38.211:

-----------------start of TP #3 for 38.211-------------------------

#### 6.3.3.2	Mapping to physical resources

<Unchanged part omitted>

Random access preambles can only be transmitted in the time resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104]. The PRACH configuration index in Tables 6.3.3.2-2 to 6.3.3.2-4 is

-	for Table 6.3.3.2-3 given by the higher-layer parameter prach-ConfigurationIndex-v1610 if configured, otherwise by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex if configured; and

-	for Tables 6.3.3.2-2 and 6.3.3.2-4 given by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex if configured.

----------------end-------------------------

Proposal 3: Adopt the text proposal #3 for 38.211.

  - Text proposal in 38.214

In 38.214 [3], MsgA-MCS indicates the the MCS index for msgA PUSCH. However, in 38.331 [4], the name for this parameter is msgA-MCS, which is shown in the following table. The name of this parameter is not aligned.

MsgA-PUSCH-Resource field descriptions

msgA-MCS Indicates the MCS index for msgA PUSCH from the Table 6.1.4.1-1 for DFT-s-OFDM and Table 5.1.3.1-1 for CP-OFDM in 38.214 [19].

Hence, we have the following text proposal #4 for Section 6.1.4.1 of TS 38.214:

-----------------start of TP #4 for 38.214-------------------------

#### 6.1.4.1	Modulation order and target code rate determination

<Unchanged part omitted>

-	elseif for a MsgA PUSCH transmission,

-	the UE shall use higher layer parameter msgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel.

----------------end-------------------------

Proposal 4: Adopt the text proposal #4 for 38.214.

- Conclusion

In this contribution, we discuss the text proposals for miscellaneous corrections for 2-step RACH. The proposals are summarized below.

Proposal 1: Adopt the text proposal #1 for 38.213.

-----------------start of TP #1 for 38.213-------------------------

#### 7.1.1	UE behaviour

<Unchanged part omitted>

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by preambleReceivedTargetPower and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

----------------end-------------------------

Proposal 2: Adopt the text proposal #2 for 38.211.

-----------------start of TP #2 for 38.211-------------------------

#### 6.3.1.1	Scrambling

<Unchanged part omitted>

cinit=nRNTI∙216+nRAPID∙210+nIDfor msgA on PUSCHnRNTI∙215+nIDotherwise

where

-	 equals the higher-layer parameter dataScramblingIdentityPUSCH if configured and the RNTI equals the C-RNTI, MCS-C-RNTI, SP-CSI-RNTI or CS-RNTI, and the transmission is not scheduled using DCI format 0_0 in a common search space;

-	nID∈0,1,…,1023 equals the higher-layer parameter msgA-DataScramblingIndex if configured and the PUSCH transmission is triggered by a Type-2 random access procedure as described in clause 8.1A of [5, TS 38.213];

-	 otherwise

----------------end-------------------------

Proposal 3: Adopt the text proposal #3 for 38.211.

-----------------start of TP #3 for 38.211-------------------------

#### 6.3.3.2	Mapping to physical resources

<Unchanged part omitted>

Random access preambles can only be transmitted in the time resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104]. The PRACH configuration index in Tables 6.3.3.2-2 to 6.3.3.2-4 is

-	for Table 6.3.3.2-3 given by the higher-layer parameter prach-ConfigurationIndex-v1610 if configured, otherwise by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex if configured; and

-	for Tables 6.3.3.2-2 and 6.3.3.2-4 given by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex if configured.

----------------end-------------------------

Proposal 4: Adopt the text proposal #4 for 38.214.

-----------------start of TP #4 for 38.214-------------------------

#### 6.1.4.1	Modulation order and target code rate determination

<Unchanged part omitted>

-	elseif for a MsgA PUSCH transmission,

-	the UE shall use higher layer parameter msgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel.

----------------end-------------------------

- References

- 3GPP TS 38.211 “Physical channels and modulation”, V16.3.0.

- 3GPP TS 38.213 “Physical layer procedures for control”, V16.3.0.

- 3GPP TS 38.214 “Physical layer procedures for control”, V16.3.0.

- 3GPP TS 38.331 “Radio Resource Control (RRC) protocol specification”, V16.2.0.