3GPP TSG RAN WG1 #103-e		R1-2008663

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	TPs on HARQ feedback in CG-DFI for CBG-based PUSCH transmission

Agenda Item:	7.2.2

Document for:	Discussion and Decision

- Introduction

This contribution discusses TPs related with CG-DFI for CBG-based PUSCH scheduled by a DCI format.

- Discussion

In RAN1 #101-e [1], the following agreement was made on CG-DFI values determination for CBG-based CG-PUSCH:

Agreement: =========================== Start of TP#1 for TS 38.213 ======================= For an initial PUSCH transmission configured by ConfiguredGrantConfig, if a UE is provided PUSCH-CodeBlockGroupTransmission for a serving cell, a value of HARQ-ACK information for a transport block of a corresponding HARQ process number in CG-DFI is ACK if HARQ-ACK for the transport block all of CBGs for the PUSCH are is ACK; otherwise, a value of HARQ-ACK information is NACK. For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission or, if the PUSCH transmission is over multiple slots, - after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is ACK. - after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is NACK. < Unchanged Texts Omitted > ======================== End of TP#1 for TS 38.213 ==============================

However, the CG-DFI value determination for CBG-based PUSCH scheduled by a DCI format is yet to be agreed. According to the discussion in RAN1 #101-e meeting, there are the following two options:

- Option 1: HARQ-ACK value in CG-DFI is ACK if at least one of the CBGs of the PUSCH is ACK

- Option 2: HARQ-ACK value in CG-DFI is ACK if the whole TB of the PUSCH is ACK

TB-level HARQ-ACK feedback for DG-PUSCH in CG-DFI is only used for CW adjustment, if option 1 is adopted, the HARQ feedback for CBG-based PUSCH which is not overlapping with LBT sub-band may be used for its CW adjustment, which is not aligned with the previous agreement as shown below. Moreover, to align the same rule with that of CG-DFI value determination for CG-PUSCH, we support option 2.

Agreement: For CWS adjustment for an LBT sub-band when a single contention window is maintained per LBT subband, all CBGs (if any are present) and TBs that partially or fully overlap with that LBT sub-band are taken into account. - CW is reset if “ACK” is received for at least 10 % of the CBGs or for at least one TB in the reference duration - Note: Other procedures for contention window adjustment within an LBT subband are also applicable - A UE can choose to apply feedback only based on TBs for CW adjustment

We propose the HARQ-ACK value in CG-DFI is ACK if the whole TB of the PUSCH is ACK, and the TP#1 for TS 38.213 is proposed as follows:

=========================== Start of TP #1 for TS 38.213 ======================= For an initial transmission by a UE of a transport block in a PUSCH configured by ConfiguredGrantConfig, if the UE receives a CG-DFI that provides HARQ-ACK information for the transport block, the UE assumes that the transport block was correctly decoded if the HARQ-ACK information value is ACK; otherwise, the UE assumes that the transport block was not correctly decoded. For a PUSCH transmission scheduled by a DCI format, if the UE receives a CG-DFI that provides HARQ-ACK information for the transport block, the UE assumes that the transport block was correctly decoded if the HARQ-ACK information value is ACK; otherwise, the UE assumes that the transport block was not correctly decoded. For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission by a number of symbols provided by cg-minDFIDelay-r16 or, if the PUSCH transmission is over multiple slots, - after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is ACK. - after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is NACK. < Unchanged Texts Omitted > ======================== End of TP #1 for TS 38.213 ==============================

Proposal 1: RAN1 adopts TP1 for TS 38.213.

- Conclusion

In this contribution, we discussed the remaining issue of CG-DFI value determination, and have the following proposal:

Proposal 1: RAN1 adopts TP1 for TS 38.213.

=========================== Start of TP #1 for TS 38.213 ======================= For an initial transmission by a UE of a transport block in a PUSCH configured by ConfiguredGrantConfig, if the UE receives a CG-DFI that provides HARQ-ACK information for the transport block, the UE assumes that the transport block was correctly decoded if the HARQ-ACK information value is ACK; otherwise, the UE assumes that the transport block was not correctly decoded. For a PUSCH transmission scheduled by a DCI format, if the UE receives a CG-DFI that provides HARQ-ACK information for the transport block, the UE assumes that the transport block was correctly decoded if the HARQ-ACK information value is ACK; otherwise, the UE assumes that the transport block was not correctly decoded. For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission by a number of symbols provided by cg-minDFIDelay-r16 or, if the PUSCH transmission is over multiple slots, - after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is ACK. - after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is NACK. < Unchanged Texts Omitted > ======================== End of TP #1 for TS 38.213 ==============================

- References

- RAN1 chairman notes, RAN1 #101-e, e-Meeting, 25th May – 5th June 2020.