3GPP TSG RAN WG1 #103-e		R1-2008671

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	UCI enhancements for URLLC

Agenda Item:	7.2.5

Document for:  Discussion and Decision

Introduction

In the last e-meeting, significant progress was achieved regarding UCI enhancements for URLLC. In this contribution, we share our views on remaining issues for UCI enhancements for URLLC.

Discussion

Ambiguous timing for MAC-CE command

Based on the summary [1] and email discussion in the last meeting, there is one remaining issue on ambiguous timing for SCell activation or deactivation. According to the current specification, when a UE receives in a PDSCH an activation/deactivation command for a secondary cell ending in slot n, the UE applies a number of actions corresponding to SCell activation/deactivation in slot n+k, where k =, where k1 is indicated by PDSCH-to-HARQ_feedback timing indicator field in the scheduling DCI, and is a number of slots per subframe. Here, it is notable that the indication provided by the PDSCH-to-HARQ_feedback timing indicator field is in the unit of slot.

In Rel-16, sub-slot based HARQ-ACK feedback is introduced, the indication of k1 provided by PDSCH-to-HARQ_feedback timing indicator field can be either in unit of slot or in unit of sub-slot. Then, the description of current specification would be questionable if the indication of k1 is in the unit of sub-slot.

In addition, the current specification for sub-slot PUCCH was captured in TS 38.213 section 9. For the timing of applying spatial setting for a PUCCH transmission defined in section 9.2.2, it is unclear whether slot is replaced by sub-slot based on the current specification quoted in the following [2].

- 9 UE procedure for reporting control information … In the remaining of this Clause, if a UE is provided subslotLengthForPUCCH-r16, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH-r16. … 9.2.2 PUCCH Formats for UCI transmission … A spatial setting for a PUCCH transmission is provided by PUCCH-SpatialRelationInfo if the UE is configured with a single value for pucch-SpatialRelationInfoId; otherwise, if the UE is provided multiple values for PUCCH-SpatialRelationInfo, the UE determines a spatial setting for the PUCCH transmission as described in [11, TS 38.321]. The UE applies corresponding actions in [11, TS 38.321] and a corresponding setting for a spatial domain filter to transmit PUCCH in the first slot that is after slot where is the slot where the UE would transmit a PUCCH with HARQ-ACK information with ACK value corresponding to a PDSCH reception providing the PUCCH-SpatialRelationInfo and is the SCS configuration for the PUCCH …

In our view, there are multiple places to define the effective time of MAC-CE in specification, such as TS 38.213 and TS 38.214, in order to ensure that the effective time of MAC-CE is consistent under different situations, slot should be used for spatial setting and SCell activation/deactivation.

Proposal 1: Slot should be used when defining the effective time of MAC-CE when a UE is provided subslotLength-ForPUCCH.

Proposal 2: Adopt the following text proposal for timing for SCell activation/deactivation in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

4.3	Timing for secondary cell activation / deactivation

With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell and transmits corresponding HARQ-ACK in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is 3∙Nslotsubframe,μ+1  where  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission.

With reference to slots for PUCCH transmissions, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell and transmits corresponding HARQ-ACK in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot .

If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot  where  is the SCS configuration for PDSCH reception on the secondary cell.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

Text proposal for correction for timing of applying spatial setting for a PUCCH transmission is provided in the following section 2.2.

Corrections for sub-slot PUCCH

In the current specification, for sub-slot PUCCH, it is captured that in the remaining of this Clause, if a UE is provided subslotLengthForPUCCH-r16, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH-r16.

However, not all the slots in section 9 should be replaced by sub-slot when a UE is provided subslotLength-ForPUCCH. The following aspects should be considered:

- For the timing of applying spatial setting for a PUCCH transmission defined in section 9.2.2 as mentioned as above, slot should be used.

- For SR reporting in section 9.2.4, slot should not be replaced by sub-slot since slot is used as the unit for SR periodicity and offset determination;

- For handling multiple PUCCH resources in a slot to transmit CSI reports in the beginning of section 9.2.5, here, slot should not be replaced by sub-slot since sub-slot based PUCCH was introduced for HARQ-ACK and there is no enhancement for CSI report.

- For PUCCH repetition in section 9.2.6, slot should not be replaced by sub-slot according to the conclusion of not supporting sub-slot based PUCCH repetition in Rel-16 in the last meeting

Proposal 3: Adopt the following text proposal for sub-slot based PUCCH transmission in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this Clause except for section 9.2.2, 9.2.4, 9.2.5 for multiple PUCCH resources in a slot to transmit CSI reports and 9.2.6, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

Conclusion

In this contribution, we discuss the remaining issues about enhancements to UCI, and the following proposals are made.

Proposal 1: Slot should be used when defining the effective time of MAC-CE when a UE is provided subslotLength-ForPUCCH.

Proposal 2: Adopt the following text proposal for timing for SCell activation/deactivation in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

4.3	Timing for secondary cell activation / deactivation

With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell and transmits corresponding HARQ-ACK in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is 3∙Nslotsubframe,μ+1  where  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission.

With reference to slots for PUCCH transmissions, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell and transmits corresponding HARQ-ACK in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot .

If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot  where  is the SCS configuration for PDSCH reception on the secondary cell.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

Proposal 3: Adopt the following text proposal for sub-slot based PUCCH transmission in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this Clause except for section 9.2.2, 9.2.4, 9.2.5 for multiple PUCCH resources in a slot to transmit CSI reports and 9.2.6, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

References

- R1-2002695, Summary#1 on UCI enhancements for R16 URLLC, Moderator (OPPO)

TS 38.213, Physical layer procedures for control, v16.3.0, September 2020