3GPP TSG RAN WG1 #103-e		R1-2008675

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Corrections on multi TRP related issues

Agenda Item:	7.2.6

Document for:	Discussion and Decision

Introductions

In previous meetings, some issues were resolved while other issues were either discussed but still with no consensus or deprioritized. This paper discusses some remaining issues and suggests text proposals on Rel-16 M-TRP.

Discussions and corrections on TCI States

During RAN1 #101-e [1], the default QCLs of PDSCH for Scheme 3 and Scheme 4 for single-PDCCH based multi-TRP system when the scheduling offset is less than timeDurationForQCL were agreed. Similarly, there are some other cases where more details of the default QCL(s) for PDSCH need to be specified depending on DCI format and the transmission schemes. Many companies have recognized these issues as important ones and the specification would not be complete if there were no further agreements.

Besides, in RAN1 #102-e [4] there was agreement on default QCL for AP CSI-RS when the scheduling offset of AP CSI-RS is less than beamSwitchTiming for both single-DCI-based and multi-DCI-based M-TRP. However, the latest specification has not captured the agreement.

In this section we would like to elaborate our views and provide our proposals on these issues.

Default QCL for PDSCH

As depicted in [2], “Independent of the configuration of tci-PresentInDCI and tci-PresentForDCI-Format1-2-r16 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains the 'QCL-TypeD',

-	If a UE is configured with enableTwoDefaultTCIStates-r16, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme-r16 set to 'TDMSchemeA' or is configured with higher layer parameter repetitionNumber-r16, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion.”

The common understanding is that when at least one TCI codepoint indicates two TCI states, the UE may buffer signals with two default TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. For Scheme 3 and Scheme 4, the mapping of the two default TCI states applied to the PDSCH reception by the UE shall depend on the decoding of the scheduling DCI because only after the successful decoding the UE is able to identify the transmission scheme. Still, the applying default TCI state(s) of following cases should also be discussed:

tci-PresentInDCI is not configured

one TCI-state is indicated by the DCI

PDSCH of Scheme 2a/2b

Default QCL when TCI field in DCI is not present

The issue of default QCL assumption for PDSCH in single-PDCCH-based multi-TRP when tci-PresentInDCI is not configured, i.e., TCI field not present in the DCI, was addressed in [3] but no consensus was reached. Some companies think it can be up to UE implementation to select one of the two default TCI states which are from the TCI states of the lowest codepoint among the TCI codepoints containing two different TCI states. However, other companies have concerns on the UE complexity and performance if it is up to UE implementation because it will impact channel estimation by DMRS and PDSCH demodulation and decoding. To simplify the UE implementation and let both network and UE have same understanding of default QCL assumption when TCI field is not present in DCI, we have the following proposal.

When at least one TCI codepoint indicates two TCI states, if the PDSCH is scheduled by a DCI format not having the TCI field present, and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE assumes that the default TCI state of the PDSCH is the first one of the two TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states.

Default QCL when indicating one TCI state

It was discussed in [3] that default QCL assumption for PDSCH in single-PDCCH-based multi-TRP when the PDSCH is indicated with one TCI state by the scheduling DCI but no consensus was reached either. Same arguments as those in the case of TCI field not present in the DCI were raised. In our view, when at least one TCI codepoint indicates two TCI states, the UE may buffer signals with two default TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, so that the UE can select either one of the two buffered signals. If the indicated TCI state of the PDSCH happens to be one of the two default TCI states, UE select the buffered signal with the TCI state identical to the indicated TCI states would require no cost or complexity while following the desired TCI state as the indication. Therefore, we have following proposal for this case.

When at least one TCI codepoint indicates two TCI states, if the PDSCH is indicated with one TCI state by the scheduling DCI, and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL if applicable,

If the indicated TCI state of the PDSCH is one of the two TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, the UE assumes that the default TCI state of PDSCH or PDSCH transmission occasions is the indicated TCI state.

Otherwise, the UE assumes that the default TCI state of PDSCH or PDSCH transmission occasions is the first one of the two TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states.

Default QCL for Scheme 2a/2b

In Scheme 2a/2b, comb-like frequency resource allocation between two TRPs. For wideband PRG, first ⌈N_RB/2⌉ RBs are assigned to TCI state 1 and the remaining ⌊N_RB/2⌋ RBs are assigned to TCI state 2. For PRG size=2 or 4, even PRGs within the allocated FDRA are assigned to TCI state 1 and odd PRGs within the allocated FDRA are assigned to TCI state 2. The abovementioned TCI state 1 and 2 are the two TCI states indicated by DCI. If the offset between the reception of the DL DCI and scheduled PDSCH of Scheme 2a/2b are less than the threshold timeDurationForQCL, although the UE has buffered signals with two default TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, it is desired to specify the mapping between the two default TCI states and the allocated frequency resources to different TRPs for the sake of low UE implementation complexity and aligned network and UE assumption on the default TCI state, just like Scheme 3 and Scheme 4. Therefore, we have following proposal:

For default TCI-states for PDSCH of Scheme 2a/2b, when the time offset between the DCI and the PDSCH is less than the threshold, the mapping between two default TCI states and frequency resources of PDSCH follows the mapping specified for indicated TCI states in Section 5.1.2.3 in TS 38.214.

PDSCH default QCL for cross-carrier scheduling

For non-CA case, the default QCL(s) of a scheduled PDSCH depends on the maximum number of TCI states mapped to a codepoint in the MAC CE activation command, or the configured number of values of CORESETPoolIndex, supporting both single-TRP and multi-TRP cases. However, for cross-carrier scheduling, the default QCL for a PDSCH follows the activated TCI state with the lowest ID in the active BWP which was obviously specified for single TRP. To align with non-CA, following default QCL determination can be applied to the cross-carrier scheduling for both single-PDCCH-based and multi-PDCCH-based M-TRP cases for cross carrier scheduling.

For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present,

If two different values of CORESETPoolIndex are configured, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell.

If at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell.

For M-TRP in cross carrier scheduling, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present,

If two different values of CORESETPoolIndex are configured, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell.

If at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell.

In general, we have following text proposal on clause 5.1.5 of TS38.214.

Text proposal on clause 5.1.5 of TS38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > Independent of the configuration of tci-PresentInDCI and tci-PresentForDCI-Format1-2-r16 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', - the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). - If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. - If a UE is configured with enableTwoDefaultTCIStates-r16, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. - if the PDSCH is scheduled by a DCI format not having the TCI field present, the UE maps the first TCI state corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states to PDSCH or PDSCH transmission occasions. - when the PDSCH is scheduled by a DCI format having the TCI field present, and one TCI state is indicated in DCI with 'Transmission Configuration Indication' field, if the indicated TCI state is one of the two TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, the UE maps the indicated TCI state to PDSCH or PDSCH transmission occasions; otherwise, the UE maps the first TCI state corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states to PDSCH or PDSCH transmission occasions. - When the UE is configured by higher layer parameter repetitionScheme-r16 set to 'TDMSchemeA', 'FDMSchemeA', 'FDMSchemeB' or is configured with higher layer parameter repetitionNumber-r16, and two TCI states are indicated in a DCI with 'Transmission Configuration Indication' field, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 or 5.1.2.3 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. - In all cases above, if none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enableDefaultBeamForCCS]: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell; if a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell; if a UE is configured with enableTwoDefaultTCIStates-r16, and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Default QCL for aperiodic CSI-RS

In RAN1 #102-e meeting [4], following agreements on default QCL for AP CSI-RS were achieved:

Agreement In multi-DCI based multi-TRP system, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is less than threshold beamSwitchTiming, the UE determines the QCL assumption for AP CSI-RS resource as follows: - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. - When a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet , the other DL signal refers to PDSCH associated with the same CORESETPoolIndex as the PDCCH triggering the AP CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the AP CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the AP CSI-RS and scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336} and enableBeamSwitchTiming-r16 is not provided, periodic CSI-RS, semi-persistent CSI-RS; - If there is no other DL signal with an indicated TCI state in the same symbols as the CSI-RS, - When a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet , the UE applies the QCL parameter(s) of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs , which are configured with the same value of CORESETPoolIndex as the PDCCH triggering that AP CSI-RS, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH triggering that AP CSI-RS. - UE is not expected to receive AP CSI -RS and PDSCH /AP-CSI-RS associated with different CORESETPoolIndex in overlapped symbol(s). - UE is not expected to receive AP CSI-RS and SP/P CSI-RS with different QCL type D in overlapped symbol(s) Note: The above behavior is applied at least for the same carrier scheduling case. Agreement In single-DCI based multi-TRP system if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is less than threshold beamSwitchTiming , the UE determines the QCL assumption for AP CSI-RS resource as follows - If there is any other DL signal with indicated TCI state in the same symbols as AP CSI-RS, The UE applies QCL assumption of the other signal. - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, when the other signal is a PDSCH indicated with two TCI states, the UE applies the first TCI state of those two TCI states. - Otherwise, - If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, the UE applies the first one of two TCI states corresponding to the lowest DCI codepoint among those mapped to two TCI states - UE is not expected to receive AP CSI-RS and SP/P CSI-RS with different QCL type D in overlapped symbol(s) Note: The above behavior is applied at least for the same carrier scheduling case.

Then we have following text proposal to capture the above agreements in TS38.214.

Text proposal on subclause 5.2.1.5.1 of TS38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology < Unchanged parts are omitted > - For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a TCI-State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS; when a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the other DL signal refers to PDSCH associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336} and enableBeamSwitchTiming-r16 is not provided, periodic CSI-RS, semi-persistent CSI-RS; If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, and when the other signal is a PDSCH indicated with two TCI states, the UE applies the first TCI state of the two indicated TCI states; - else if a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, the UE applies the first TCI state of the two TCI states corresponding to the lowest DCI codepoint among those mapped to two TCI states; - else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored; when a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE applies the QCL parameter(s) of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH triggering that aperiodic CSI-RS, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH triggering that aperiodic CSI-RS; - else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or is equal to or greater than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.- - A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position. A UE is not expected to receive aperiodic CSI-RS and PDSCH/aperiodic CSI-RS associated with different values of CORESETPoolIndex in overlapped symbol(s). The UE is not expected to receive aperiodic CSI-RS and semi-persistent/periodic CSI-RS with different QCL type D in overlapped symbol(s). For a UE configured with the higher layer parameter CSI-AperiodicTriggerStateList, if a Resource Setting linked to a CSI-ReportConfig has multiple aperiodic resource sets, only one of the aperiodic CSI-RS resource sets from the Resource Setting is associated with the trigger state, and the UE is higher layer configured per trigger state per Resource Setting to select the one CSI-IM/NZP CSI-RS resource set from the Resource Setting. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Miscellaneous

PUCCH for HARQ-ACK feedback and other UCIs

In RAN1 #98bis [5], PUCCH/PUSCH collision between different TRPs is avoided by implementation when separate HARQ-ACK feedback is enabled.

Agreement For multi-PDCCH based multi-TRP/panel transmission, when separated ACK/NACK feedback is enabled, - PUCCH/PUSCH collision between different TRPs can be avoided by implementation and UE doesn’t expect overlapping PUCCHs/PUSCHs transmission toward different TRPs. For PUCCH/PUSCH transmission toward the same TRP, Rel-15 multiplexing rules apply. - Note that PUCCH resources can be associated with values of higher layer index per CORESET so that indices may be used to differentiate TRP to determine whether there is overlapping among TRPs. PUSCH can be differentiated by scheduling CORESET in terms of targeted TRP.

As stated in [6], “HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE”. And a UE configured with separate HARQ-ACK feedback does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs to overlap with a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs.

For M-TRP operation, it may be required to transmit PUCCH for CSI, SR or LRR. However, it is not clear the association between the CORESETPoolIndex and a PUCCH for P/SP-CSI, SR or LRR. Thus, we prefer to clarify the corresponding behavior when a PUCCH for CSI, SR or LRR collides with PUCCH for HARQ-ACK. Considering the above agreements, some restriction can be addressed as follows:

A UE doesn’t expect a PUCCH for CSI/SR/LRR overlap with more than one PUCCH for HARQ-ACK.

If a PUCCH with HARQ-ACK overlaps with a PUCCH with CSI/SR/LRR information, the UE shall apply the procedures specified in Rel-15.

Adopt following text proposal in section 9 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = SeparateFeedback the UE shall separately apply the procedures described in Clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE. If a PUCCH with HARQ-ACK overlaps with a PUCCH with CSI/SR/LRR information, the UE shall apply the procedures described in Clause 9.2. A UE doesn’t expect a PUCCH with CSI/SR/LRR information overlap with more than one PUCCH with HARQ-ACK information. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Conflicts in HARQ-ACK feedback in one slot

In clause 9.2 of TS38.213 [6], “If a UE is provided ackNackFeedbackMode-r16 = separate, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot”. However, the first sentence in clause 9.2.3 of [6] “A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot” is obviously conflict with the former sentence and it is not the case for a UE configured with separate HARQ-ACK feedback in multi-PDCCH-based M-TRP. To reflect the agreement in M-TRP session, we have following proposal in which no more than one PUCCH with HARQ-ACK are expected in the cases of “otherwise”, including the case that the UE is either not configured with separate HARQ-ACK feedback or sub-slot.

Adopt following text proposal in clause 9.2 and 9.2.3 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9.2 UCI reporting in physical uplink control channel UCI types reported in a PUCCH include HARQ-ACK information, SR, LRR, and CSI. UCI bits include HARQ-ACK information bits, if any, SR information bits, if any, LRR information bit, if any, and CSI bits, if any. The HARQ-ACK information bits correspond to a HARQ-ACK codebook as described in Clause 9.1. For the remaining of this clause, any reference to SR is applicable for SR and/or for LRR. A UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot. When the UE transmits two PUCCHs in a slot and the UE is not provided ackNackFeedbackMode-r16 = separate, at least one of the two PUCCHs uses PUCCH format 0 or PUCCH format 2. In Clauses 9.2.3, 9.2.5.1 and 9.2.5.2, a UE assumes 11 CRC bits if a number of respective UCI bits is larger than or equal to 360; otherwise, the UE determines a number of CRC bits based on the number of respective UCI bits as described in [5, TS 38.212]. < Unchanged parts are omitted > 9.2.3 UE procedure for reporting HARQ-ACK If a UE is provided ACKNACKFeedbackMode = separate and it is not provided subslotLength-ForPUCCH, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot. Otherwise, a UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

PDSCH overlapping with PDCCH from different TRPs

In current spec [2], it is clearly stated the UE behaviour when the PDSCH overlaps with its scheduling PDCCH: “If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH.”

However, “when PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex, the following operations are allowed:

-	For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i.”

It implies that the PDSCH scheduled by a PDCCH from one TRP may overlap with another PDCCH from different TRP. Meanwhile, “a UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH”, which includes the PDCCH associated with any CORESETPoolIndex in our understanding. To avoid the unclear UE behavior to deal with such cases, we propose to limit the

Conclude in RAN1 #103e that UE does not expect to be scheduled a PDSCH overlapping with a PDCCH associated to CORESET having different CORESETPoolIndex from the scheduling PDCCH.

Conclusion

We have following proposals regarding to Rel-16 M-TRP.

- When at least one TCI codepoint indicates two TCI states, if the PDSCH is scheduled by a DCI format not having the TCI field present, and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE assumes that the default TCI state of the PDSCH is the first one of the two TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states.

When at least one TCI codepoint indicates two TCI states, if the PDSCH is indicated with one TCI state by the scheduling DCI, and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL if applicable,

If the indicated TCI state of the PDSCH is one of the two TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, the UE assumes that the default TCI state of PDSCH or PDSCH transmission occasions is the indicated TCI state.

Otherwise, the UE assumes that the default TCI state of PDSCH or PDSCH transmission occasions is the first one of the two TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states.

For default TCI-states for PDSCH of Scheme 2a/2b, when the time offset between the DCI and the PDSCH is less than the threshold, the mapping between two default TCI states and frequency resources of PDSCH follows the mapping specified for indicated TCI states in Section 5.1.2.3 in TS 38.214.

For M-TRP in cross carrier scheduling, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present,

If two different values of CORESETPoolIndex are configured, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell.

If at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell.

Text proposal on clause 5.1.5 of TS38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > Independent of the configuration of tci-PresentInDCI and tci-PresentForDCI-Format1-2-r16 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', - the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). - If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. - If a UE is configured with enableTwoDefaultTCIStates-r16, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. - if the PDSCH is scheduled by a DCI format not having the TCI field present, the UE maps the first TCI state corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states to PDSCH or PDSCH transmission occasions. - when the PDSCH is scheduled by a DCI format having the TCI field present, and one TCI state is indicated in DCI with 'Transmission Configuration Indication' field, if the indicated TCI state is one of the two TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, the UE maps the indicated TCI state to PDSCH or PDSCH transmission occasions; otherwise, the UE maps the first TCI state corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states to PDSCH or PDSCH transmission occasions. - When the UE is configured by higher layer parameter repetitionScheme-r16 set to 'TDMSchemeA', 'FDMSchemeA', 'FDMSchemeB' or is configured with higher layer parameter repetitionNumber-r16, and two TCI states are indicated in a DCI with 'Transmission Configuration Indication' field, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 or 5.1.2.3 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. - In all cases above, if none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enableDefaultBeamForCCS]: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell; if a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell; if a UE is configured with enableTwoDefaultTCIStates-r16, and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Text proposal on subclause 5.2.1.5.1 of TS38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology < Unchanged parts are omitted > - For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a TCI-State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS; when a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the other DL signal refers to PDSCH associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS associated with the same CORESETPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336} and enableBeamSwitchTiming-r16 is not provided, periodic CSI-RS, semi-persistent CSI-RS; If a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, and when the other signal is a PDSCH indicated with two TCI states, the UE applies the first TCI state of the two indicated TCI states; - else if a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint indicates two TCI states, the UE applies the first TCI state of the two TCI states corresponding to the lowest DCI codepoint among those mapped to two TCI states; - else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored; when a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE applies the QCL parameter(s) of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH triggering that aperiodic CSI-RS, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH triggering that aperiodic CSI-RS; - else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or is equal to or greater than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.- - A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position. A UE is not expected to receive aperiodic CSI-RS and PDSCH/aperiodic CSI-RS associated with different values of CORESETPoolIndex in overlapped symbol(s). The UE is not expected to receive aperiodic CSI-RS and semi-persistent/periodic CSI-RS with different QCL type D in overlapped symbol(s). For a UE configured with the higher layer parameter CSI-AperiodicTriggerStateList, if a Resource Setting linked to a CSI-ReportConfig has multiple aperiodic resource sets, only one of the aperiodic CSI-RS resource sets from the Resource Setting is associated with the trigger state, and the UE is higher layer configured per trigger state per Resource Setting to select the one CSI-IM/NZP CSI-RS resource set from the Resource Setting. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Adopt following text proposal in section 9 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = SeparateFeedback the UE shall separately apply the procedures described in Clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE. If a PUCCH with HARQ-ACK overlaps with a PUCCH with CSI/SR/LRR information, the UE shall apply the procedures described in Clause 9.2. A UE doesn’t expect a PUCCH with CSI/SR/LRR information overlap with more than one PUCCH with HARQ-ACK information. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Adopt following text proposal in clause 9.2 and 9.2.3 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9.2 UCI reporting in physical uplink control channel UCI types reported in a PUCCH include HARQ-ACK information, SR, LRR, and CSI. UCI bits include HARQ-ACK information bits, if any, SR information bits, if any, LRR information bit, if any, and CSI bits, if any. The HARQ-ACK information bits correspond to a HARQ-ACK codebook as described in Clause 9.1. For the remaining of this clause, any reference to SR is applicable for SR and/or for LRR. A UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot. When the UE transmits two PUCCHs in a slot and the UE is not provided ackNackFeedbackMode-r16 = separate, at least one of the two PUCCHs uses PUCCH format 0 or PUCCH format 2. In Clauses 9.2.3, 9.2.5.1 and 9.2.5.2, a UE assumes 11 CRC bits if a number of respective UCI bits is larger than or equal to 360; otherwise, the UE determines a number of CRC bits based on the number of respective UCI bits as described in [5, TS 38.212]. < Unchanged parts are omitted > 9.2.3 UE procedure for reporting HARQ-ACK If a UE is provided ACKNACKFeedbackMode = separate and it is not provided subslotLength-ForPUCCH, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot. Otherwise, a UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Conclude in RAN1 #103e that UE does not expect to be scheduled a PDSCH overlapping with a PDCCH associated to CORESET having different CORESETPoolIndex from the scheduling PDCCH.

References

- Chairman's Notes, RAN1 #101-e.

- 3GPP TS 38.214 V16.3.0.

- R1-2001377, Outcome of email thread [100e-NR-eMIMO-multiTRP-01], RAN1 #100-e.

- Chairman's Notes, RAN1 #102-e.

- Chairman's Notes, RAN1 #98bis.

- 3GPP TS 38.213 V16.3.0.

- Chairman's Notes, RAN1 #96bis.

- Chairman's Notes, RAN1 #98.