3GPP TSG-RAN WG1 Meeting #103-e	R1-2008682

# e-Meeting, October 26th – November 13th, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on increased number of CSI-RS for mobility per MO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2020-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently, there is restriction of 96 on the number of CSI-RS resources per MO. Based on latest input from RAN4, it was agreed to increase the number of configurable CSI-RS resources per MO from 96 (as indicated in R1-1905909 and captured in current RAN1/2 specifications) to 192, only for the case when CSI-RS is configured with associated SSB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The number of 96 is further increased to 192 for UE supporting increased number of CSI-RS resources per MO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is restriction of 96 CSI-RS resources per MO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is Non-BC change. For UE not supporting larger than 96 CSI-RS resources per MO, the network can not configure CSI-RS resources per MO larger than 96. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.6.1.3	CSI-RS for mobility

If a UE is configured with the higher layer parameter CSI-RS-Resource-Mobility and the higher layer parameter associatedSSB is not configured, the UE shall perform measurements based on CSI-RS-Resource-Mobility and the UE may base the timing of the CSI-RS resource on the timing of the serving cell.

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the UE may base the timing of the CSI-RS resource on the timing of the cell given by the cellId of the CSI-RS resource configuration. Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, the UE is not required to monitor the corresponding CSI-RS resource. The higher layer parameter isQuasiColocated indicates whether the associated SS/PBCH block given by the associatedSSB and the CSI-RS resource(s) are quasi co-located with respect to 'QCL-TypeD', when applicable.

If a UE is configured with the higher layer parameter CSI-RS-Resource-Mobility and with periodicity greater than 10 ms in paired spectrum, the UE may assume the absolute value of the time difference between radio frame i between any two cells, listed in the configuration with the higher layer parameter CSI-RS-CellMobility and with same refFreqCSI-RS, is less than 153600 Ts.

If the UE is configured with DRX, the UE is not required to perform measurement of CSI-RS resources other than during the active time for measurements based on CSI-RS-Resource-Mobility. When the UE is configured to monitor DCI format 2_6, the UE is not required to perform measurements other than during the active time and during the timer duration indicated by drx-onDurationTimer also outside active time based on CSI-RS-Resource-Mobility.

If the UE is configured with DRX and DRX cycle in use is larger than 80 ms, the UE may not expect CSI-RS resources are available other than during the active time for measurements based on CSI-RS-Resource-Mobility. If the UE is configured with DRX and configured to monitor DCI format 2_6 and DRX cycle in use is larger than 80ms, the UE may not expect that the CSI-RS resources are available other than during the active time and during the time duration indicated by drx-onDurationTimer also outside active time for measurements based on CSI-RS-Resource-Mobility. Otherwise, the UE may assume CSI-RS are available for measurements based on CSI-RS-Resource-Mobility.

A UE configured with the higher layer parameters CSI-RS-Resource-Mobility may expect to be configured

-	with no more than 96 CSI-RS resources per higher layer parameter MeasObjectNR for UEs not supporting [increasedNumberofCSIRSPerMO] when all CSI-RS resources configured by the same higher layer parameter MeasObjectNR have been configured with associatedSSB, or,

-	with no more than 192 CSI-RS resources per higher layer parameter MeasObjectNR for UEs supporting [increasedNumberofCSIRSPerMO] when all CSI-RS resources configured by the same higher layer parameter MeasObjectNR have been configured with associatedSSB, or,

-	with no more than 64 CSI-RS resources per higher layer parameter MeasObjectNR when all CSI-RS resources have been configured without associatedSSB or when only some of the CSI-RS resources have been configured with associatedSSB by the same higher layer parameter MeasObjectNR

-	For frequency range 1 the associatedSSB is optionally present for each CSI-RS resource

-	For frequency range 2 the associatedSSB is either present for all configured CSI-RS resources or not present for any configured CSI-RS resource per higher layer parameter MeasObjectNR.

For any CSI-RS resource configuration, the UE shall assume that the value for parameter cdm-Type is 'noCDM', and there is only one antenna port.