3GPP TSG RAN WG1 #103	R1-2008706

e-Meeting, October 26th – November 13th, 2020

Agenda item:       8.8.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation assumptions for NR coverage enhancement evaluation

Document for:     Discussion and Decision

# 1	Introduction

In this contribution, we provide in detail the evaluation assumptions that have been used for link budget evaluation in our companion contributions [1][2]. These assumptions follow the agreements in RAN1#101-e and RAN1#102-e regarding the baseline LLS parameters.

# 2	General assumptions for link budget evaluation

## 2.1	FR1 scenarios

This subsection summarizes general assumptions for the link budget evaluation in our companion contribution [1] for the Rural and Urban scenarios in FR1. Parameters are presented in Table 1.

| Scenario parameters |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Duplexing |  | TDD |  |  |  |  | FDD |  |
| Scenario |  | Urban/Rural |  |  |  |  | Rural |  |
| Carrier frequency |  | 4 GHz |  |  |  |  | 700MHz |  |
| SCS |  | 30 kHz |  |  |  |  | 15 kHz |  |
| BW |  | 100 MHz |  |  |  |  | 20 MHz |  |
| Channel parameters |  |  |  |  |  |  |  |  |
| Model |  | TDL-C for NLOS and TDL-D for LOS |  |  |  |  |  |  |
| Delay spread |  | TDD |  |  |  |  | FDD |  |
|  |  | Urban |  | Rural |  |  | Rural |  |
|  |  | 300 ns |  | 300 ns |  |  | 300 ns |  |
| UE speed | Low speed | 3 Km/h |  | 3 Km/h |  |  | 3 Km/h |  |
|  | High speed | 30 Km/h |  | 120 Km/h |  |  | 120 Km/h |  |
| Propagation |  | NLOS |  | NLOS/LOS |  |  |  |  |
| Power allocation parameters |  |  |  |  |  |  |  |  |
| Tx power gNB |  | TDD |  |  |  |  | FDD |  |
|  |  | 53 dBm |  |  |  |  | 49 dBm |  |
| Tx power UE |  | 23 dBm |  |  |  |  |  |  |
| Power allocation |  | DL | Uniformly allocated to CBW |  |  |  |  |  |
|  |  | UL | Concentrated on occupied BW |  |  |  |  |  |
| Antenna array configuration parameters |  |  |  |  |  |  |  |  |
| Antennas |  | Elements [#E] |  |  |  |  | Ports [#P] |  |
|  |  | gNB | Urban |  | TDD: 192 |  | gNB | 2DL 2UL |
|  |  |  | Rural |  | TDD: 64 FDD: 32 (2GHz) /16 (700MHz) |  |  |  |
|  |  | UE | FDD |  | 1UL / 2 DL |  | UE | 2 (FDD)/4 (TDD) DL 1UL |
|  |  |  | TDD |  | 1 UL / 4 DL |  |  |  |
| Frame configuration parameters |  |  |  |  |  |  |  |  |
| Frame structure for TDD |  | DDDSUDDSUU (D:U=10:2, 5 ms periodicity) for 4GHz |  |  |  |  |  |  |
| Symbols per slot |  | 14 |  |  |  |  |  |  |
| DMRS |  | PxSCH |  | Urban/Rural |  | DL/UL |  |  |
|  |  |  |  |  |  | 2 symbols Type 1 - 1 layer no multiplexing with data 3dB power boost |  |  |
|  |  | PxCCH |  | Urban/Rural |  | DL/UL |  |  |
|  |  |  |  |  |  | According to NR specification |  |  |
| HARQ eMBB |  | No |  |  |  |  |  |  |
| HARQ VoIP |  | Yes [Up to 4 HARQ transmissions] |  |  |  |  |  |  |

Table 1. General assumptions for link budget evaluation in FR1.

## 2.2	FR2 scenarios

This subsection summarizes general assumptions for the link budget evaluation in our companion contribution [2] for the Urban, Suburban and Indoor scenarios in FR2. Parameters are presented in Table 2.

| Scenario parameters |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Duplexing |  | TDD |  |  |  |  |  |  |
| Scenario |  | Urban |  |  |  |  |  |  |
| Carrier frequency |  | 28 GHz |  |  |  |  |  |  |
| SCS |  | 120 kHz |  |  |  |  |  |  |
| BW |  | 100 MHz |  |  |  |  |  |  |
| Channel parameters |  |  |  |  |  |  |  |  |
| Model |  | CDL-A / TDL-A |  |  |  |  |  |  |
| Delay spread |  | TDD |  |  |  |  |  |  |
|  |  | 100 ns |  |  |  |  |  |  |
| UE speed | Low speed | 3 Km/h |  |  |  |  |  |  |
|  | High speed | 30 Km/h |  |  |  |  |  |  |
| Propagation |  | NLOS |  |  |  |  |  |  |
| Power allocation parameters |  |  |  |  |  |  |  |  |
| Tx power gNB |  | Urban |  |  |  |  |  |  |
|  |  | 40 dBm |  |  |  |  |  |  |
| Tx power UE |  | 23 dBm or 12 dBm |  |  |  |  |  |  |
| Power allocation |  | DL | Uniformly allocated to CBW |  |  |  |  |  |
|  |  | UL | Concentrated on occupied BW |  |  |  |  |  |
| Antenna array configuration parameters |  |  |  |  |  |  |  |  |
| Antennas |  | Elements [#E] |  |  |  |  | Ports [#P] |  |
|  |  | gNB | Urban |  | 256 |  | gNB | 2DL 2UL |
|  |  | UE | Urban |  | 8 |  | UE | 2DL 1UL |
| Frame configuration parameters |  |  |  |  |  |  |  |  |
| Frame structure for TDD |  | DDSU (D:U=11:0, 0.5 ms periodicity) |  |  |  |  |  |  |
| Symbols per slot |  | 14 |  |  |  |  |  |  |
| DMRS |  | PxSCH |  | Urban |  | DL/UL |  |  |
|  |  |  |  |  |  | 2 symbols Type 1 - 1 layer No multiplexing with data 3dB power boost |  |  |
|  |  | PxCCH |  | Urban |  | DL/UL |  |  |
|  |  |  |  |  |  | According to NR specification |  |  |
| HARQ eMBB |  | No |  |  |  |  |  |  |
| HARQ VoIP |  | Yes [Up to 4 HARQ transmissions] |  |  |  |  |  |  |

Table 2. General assumptions for link budget evaluation in FR2.

# 3	Detailed assumptions for each physical channel

## 3.1	FR1 shared channels

Table 3 shows the number of PRBs and MCS indices that are used for link budget evaluation in our companion contribution [1] for PDSCH in FR1. Similarly, Table 4 shows the number of PRBs and MCS indices that are used for link budget evaluation in [1] for PUSCH in FR1. It is worth mentioning that intra-slot frequency hopping is enabled for PUSCH. Given the number of PRBs, the lowest MCS index that satisfies target throughput is used for both PDSCH and PUSCH.

| Scenario | Carrier Freq./Frame structure | Target throughput | Number of PRBs | MCS index |
| --- | --- | --- | --- | --- |
| Rural FDD | 2GHz and 700MHz/- | 1Mbps | 106 | 1 |
| Rural TDD | 4GHz/ DDDSUDDSUU (10D:2G:2U) | 1Mbps | 273 | 0 |
| Urban TDD | 4GHz/ DDDSUDDSUU (10D:2G:2U) | 10Mbps | 273 | 8 |

Table 3. The considered PRBs and MCS indices that satisfy target throughput for PDSCH in FR1 scenarios.

| Scenario | Carrier Freq./Frame structure | Target throughput | Number of PRBs | MCS index |
| --- | --- | --- | --- | --- |
| Rural FDD | 2GHz and 700MHz/- | 100kbps | 4 | 5 |
| Rural TDD | 4GHz/ DDDSUDDSUU (10D:2G:2U) | 100kbps | 4 | 7 |
| Urban TDD | 4GHz/ DDDSUDDSUU (10D:2G:2U) | 1Mbps | 30 | 8 |

Table 4. The considered PRBs and MCS indices that satisfy target throughput for PUSCH in FR1 scenarios.

VoIP is also evaluated in FR1. We consider 4 PRBs as a baseline for both PDSCH and PUSCH, with TBS = 320 bits. The valid MCSs for 4 PRBs and 320 bits TBS are 11 and 10 for PDSCH and PUSCH, respectively.

## 3.2	FR2 shared channels

Table 5 shows the number of PRBs and MCS indices that are used for link budget evaluation in our companion contribution [2] for PDSCH in FR2. Similarly, Table 6 shows the number of PRBs and MCS indices that are used for link budget evaluation in [2] for PUSCH in FR2. As mentioned above for FR1, intra-slot frequency hopping is enabled for PUSCH in FR2 and, given the number of PRBs, the lowest MCS index that satisfies target throughput is used for both PDSCH and PUSCH. Since VoIP is considered as second priority in FR2 as stated in the SID [2], we do not present any simulation assumption for VoIP in FR2, for the sake of compactness.

| Scenario | Frame structure | Target throughput | Number of PRBs | MCS index |
| --- | --- | --- | --- | --- |
| Urban | DDSU (11D:3G:0U) | 25Mbps | 66 | 12 |

Table 5. The considered PRBs and MCS indices that satisfy target throughput for PDSCH in FR2 scenarios.

| Scenario | Frame structure | Target throughput | Number of PRBs | MCS index |
| --- | --- | --- | --- | --- |
| Urban | DDSU (11D:3G:0U) | 5Mbps | 30 | 10 |

Table 6. The considered PRBs and MCS indices that satisfy target throughput for PUSCH in FR2 scenarios.

## 3.3	Control channels and DMRS

For PDCCH, we consider aggregation level 16 with 40 bits DCI and 24 bits CRC. A CORESET bandwidth of 48 PRBs and two OFDM symbols are used. The channel bandwidth for PDCCH is the same as for PDSCH.

For PUCCH, we consider both Format 1 with 2 bits UCI and Format 3 with 22 bits UCI. One PRB is considered for Format 1 and Format 3, respectively. Intra-slot frequency hopping is enabled for both PUCCH formats.

Concerning DMRS, we consider configuration type 1 with 1 layer and 3.0 dB power boost for both DL and UL. Two OFDM symbols for DMRS are used in  both DL and UL for all scenarios. Finally, for both PDCCH and PUCCH, DMRS configuration follows NR specification and no additional DMRS is considered for PUCCH.

# 4	Conclusion

In this contribution, we have presented the evaluation assumptions for the link budget study performed in [1][2]. Firstly, general assumptions for scenarios in FR1 and FR2 were summarized in two tables. Secondly, detailed channel configuration assumptions are provided for each physical channel for both FR1 and FR2.

# 5	References

- R1-2008701 Baseline coverage evaluation of UL and DL channels – FR1, Nokia/NSB.

- R1-2008702 Baseline coverage evaluation of UL and DL channels – FR2, Nokia/NSB.

- Report ITU-R M.2412-0, Guidelines for evaluation of radio interface technologies for IMT-2020, ITU, Oct. 2017.

- RP-193240, New SID on NR coverage enhancement, China Telecom.