3GPP TSG RAN WG1 Meeting #103-e	R1-2008709

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.5.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Evaluation of positioning enhancements

Document for:	Discussion & Decision

# Introduction

In this contribution, we are evaluating the positioning performance for UL-TDOA in InF scenarios. The evaluations are based on the agreements in RAN1#101-e and RAN1#102-e for the baseline and optional complementary parameters. We provide evaluation results on the achievable positioning accuracy and focus on the impact of interference, antenna configuration and UL transmit power.

# Simulation setup overview

The simulation setup features are summarized in the following table:

| Parameter | Description | Used for R1-2008709 |
| --- | --- | --- |
| Network | The setup supports different deployment scenarios (ISD, number of BS, hexagon or other grid, ….) | InF-grid, 20m, 18BS |
| Channel | All channels as defined by TR38.901 | InF_LOS InF_NLOS_DH |
| Noise | Noise is inserted after RX-antenna beamforming (assuming the beam forming losses are already included in the antenna virtualization) |  |
| LOS/NLOS model | Random selection of LOS/NLOS receive scenario. The model as defined by 3GPP supports distance dependent LOS probability. | Inline with the agreements in RAN1#101-e : r = 0.4 hc = 2m dClutter = 2m |
| gNB antenna | Different antennas are supported - Omni (for comparison) - 3 panels per BS, 1 element per panel is used, antenna pattern per element (inline with the agreements) - 3 panels per BS, several beams per panel, beams are generated assuming using the “antenna virtualization” method. | The used antenna config is defined by the config number Xxx; where 2xx omni antennas for BS 3xx Three sectors, 1 element per sector 4xx Three sectors, FR1 antenna panel config, several beams 5xx Three sectors, FR2 antenna panel config, several beams |
| UE antenna | FR1: baseline one Omni Optional: several antenna elements at different positions of the UE, “ideal antenna element”, “non-ideal antenna element with 5dB gain variation”, 3GPP antenna element FR2: 4 panels with 4 elements each - One element/one polarization per panel is used - Several elements (and beam forming) per panel | For FR1: - one Omni - optional: several antennas For FR2: - 4 panels (4 elements each), one element is used - 4 panels, 3 beams per panels (12 beams total) |
| UE dropping | Different dropping modes including - “random dropping” - Random dropping with short tracks to emulate the movement of the UE in case of sequential measurements. - Long tracks (emulating the move movement of the UE and changes of the receive scenario along the track. This mode is especially foreseen for enhanced positioning algorithms using Kalman-Filter - “group dropping” (UEs sharing the same resources are in the same area | Random dropping |
| Frequency range | carrier frequency | FR1 (4GHz) and FR2 (28GHz) |
| Signal bandwidth and subcarrier spacing | According to Rel-16 | Default configuration is 100MHz, SCS=30kHz |
| SRS pattern | The preferred configuration depends on the network (ISD), the UE capabilities (available TX-power) and BS antenna gain. The COMB factor, number of symbols, staggering on/off, etc. depends on the link budget, the expected delay spread and the level difference of the signals transmitted from different UEs. We assume “cyclic shift multiplex” (several UEs share the same REs and are separated by different cyclic shifts only). KTC= 4 supports 12 cyclic shift steps. This allows that up to 12 UEs can share the same REs. KTC= 8 supports 6 cyclic shift steps | For InF scenarios there are no/minor link budget issues. Hence 1 OFDM symbol per SRS is sufficient. The delay spread is low. Therefore, high COMB factors can be used. Default configuration is KTC = 4, 6 UEs share the same REs (Interference included in all results) |
| UE antenna ports | Inline, with Rel-16 we assume for SRS for positioning only one TX-port can be configured. The setup supports also UEs with several antennas. Two modes are supported - Several SRS resources are configured targeting different UE spatial TX filter. If only one TX-port is allowed, the SRS are transmitted sequential (no interference between SRS of the same UE) - The UE supports also parallel transmission over several antenna ports (multi-Port SRS transmissions) | One TXRU is active per OFDM symbol. If different SRS resources are configured as sequential transmission is assumed. |
| Beam management | The simulation setup supports beam management. For positioning purpose we used: - The gNB transmits a DL-RS (SSB or DL-PRS) - The UE reports the signal strength for each received RS - The spatial relationship for the SRS transmissions is configured accordingly - The gNB receives the SRS of this beam only. | DL-RS signal power based beam selection. |
| Timing advance | Ideal timing advance setting according to the serving gNB (s-gNB). The nearest gNB is assumed as s-gNB. Optional: non-ideal timing advance | Ideal timing advance |
| TX power control | In case of omni-directional a simplified power control can be used. The configured TX power is a trade-off between the - Generated interference - Probability that a faraway gNB can detect the signal To study this tradeoff we use a fixed setting of the TX power per UE and assume 3 power level - +23dBm - 0dBm - -20dBm Other TX power control methods (especially methods for UEs using non-omni-antennas) are FFS. | Constant TX power setting |
| ToA- Measurements | Cyclic correlation, selection of the time window according the configured cyclic shift, evaluation of the correlation function including threshold based “first arrival path detection” (FAP) and evaluation of the first rising edge of the correlation function (“IFP”). Other ToA-Estimators: MUSIC, Machine Learning | FAP+IFP |
| ToA-Set analysis | Especially in case of UE-TX antennas with high directivity the signal quality of the different SRS transmissions may be significantly different. It may happen that the FAP can’t be detected (depending on noise and interference). If several SRS are transmitted per UE the gNB used a signal power based selection of the ToA measurement further used by the positioning algorithm. | Quality selection: Signal power |
| LOS-Detection | Taking into account the high ToA-error introduced by the ATOA model of TR38.901 it is essential to detect the LOS links. Different concept are currently FFS | The LOS detection algorithm is under development and not yet used for the simulation. Ideal LOS detection is assumed |
| Positioning algorithm | We distinguish “single shot positioning” (= one SRS resource set is used per positioning event) and “positioning with tracking” (e.g. Kalman filter based positioning algorithms. As single shot positioning algorithm we use - Bancroft - Levenberg-Marquardt | Levenberg-Marquardt |

# Positioning evaluation

We compared different configurations to identify relevant enhancements in Rel-17. The methods, signaling and procedures, if needed, are subject to the discussion in agenda item. 8.5.3.

## Impact of gNB antenna configuration

In the SLS-2D- and 3D-results (Figure 1 and Figure 2) we characterize the performance w.r.t. to the gNB antenna configuration. Three different gNB antenna configurations are considered:

- Config 220: Omni antenna

- Config 320: 3 sectors (3 panels, one element per panel is used)

- Config 420: 3 panels, 5 beams per panel

The UEs assumed for these results are characterized by an omni antenna and a high transmit power (23dBm). An omni-antenna may receive more multipath components, hence Config 420 outperforms the other configurations. The evaluations in the following sections use configuration gNB antenna configuration Config-4xx for the further detailed analysis on the uplink transmit power and the UE antenna assumptions.

Observation 1: 	Using antennas with higher directivity increase the performance.

Figure 1-CDF 2D (horizontal) error for UTDOA, comparison of different gNB antenna configurations

Figure 2 - CDF 3D error for UTDOA, comparison of different gNB antenna configurations

## Impact UE Tx Power in the presence of interference

Due to the low ISD and the high BS antenna gain of the config-4xx configuration a low UE TX-power may be sufficient. We compared for the UE power analysis the following configurations:

- Config 420: +23dBm (applied configuration in Figure 1and Figure 2)

- Config 422: 0dBm

- Config 423: -20dBm

It can be observed from Figure 3 and Figure 4 that a low UE-TX power is sufficient in the InF scenario. In the example we see that for 0dBm shows almost no performance degradation compared to +23dBm.

Observation 2: 	A low UE-TX power is sufficient for gNBs with beam forming in InF scenarios.

Figure 3 - CDF horizontal error for UTDOA, comparison of different UE TX Power levels

Figure 4 - CDF 3D error for UTDOA, comparison of different UE Tx Power levels.

For this analysis, we assumed interference between UEs sharing the same RE (“cyclic shift multiplex”). The reported high accuracy performances (with low transmission power) in the presence of interference is not feasible when other interference sources are applied (for example: interference from UEs using different SRS root sequences or data).

Observation 3:	When considering interference, the performance with low TX power is only feasible if UEs sharing the same REs use different cyclic shifts for the same SRS root sequence (“cyclic shift multiplex”)

Proposal 1:	For “low power operation” in combination with interference between positioning-SRS resources, cyclic shift enhancements shall be considered in Rel-17.

## Supporting “TX antenna diversity”

The baseline simulation in FR1 assumes an omni-directional antenna. If a low TX power is used the gain variation of a non-ideal UE antenna becomes critical. To study this effect we compare

- Config 423: 	Low power (-20dBm) operation (as above)

- Config 422: 	Medium power (0dBm) operation (as above)

- Config 444:	Low power operation, 4 identical UE antennas, the 4 antennas are at different position at the UE, sequential transmission of the 4 SRS resources.

- Config 443: 	Low power operation, 4 non-ideal UE antennas (app. 5dB gain variation), the 4 antennas are at different position at the UE, sequential transmission of the 4 SRS resources

Figure 5 - CDF horizontal error (with LOS detection),

Figure 6 - CDF 3D error for UTDOA (with LOS detection),

Observation 4: 	Using TX-diversity (4 transmissions at -20dBm from different antennas) shows a similar performance as single port transmission with 0dBm.

Multi-port SRS transmission reduces the number of OFDM symbols needed compared to sequential transmission.

Proposal 2: 	Support multi-port transmission in Rel-17.

## Uplink beam management evaluation

### Tx-Rx Beam pair selection criteria

One important criteria for uplink based methods (UL-TDOA, multi-RTT) using beam based operation is the choice of the Tx-Rx beam pair for timing. We compared:

- Config 420: CIR analysis is used for the beam selection/spatial relationship establishment

- Config 421: only power (LOS power and multipath power) is used for selection of the spatial relationship (a beam with low gain for the LOS  path was selected)

Figure 9 and Figure 10 show that an efficient beams management needs more than the analysis of total power as a base for selection. An approach for selecting the links based on RSRP reports is not reliable for positioning (Config 421). For better establishing the spatial relationship several methods can be considered. In Config 420 we implemented an enhanced method (not supported in Rel-16) evaluating the CIR and using the power of the first arriving taps only.

Figure 7 - CDF horizontal error UTDOA, Beam pair selection

Figure 8 - CDF 3D error for UTDOA, Beam pair selection

Observation 5: 	The beam selection and spatial relationship establishment criteria have major impact on the performance.

### “Narrow” TX beams

For FR2, the UE may use antenna panels with narrow TX beams [2]. For positioning, each transmit direction may be required. This may demand many SRS resources and advanced ToA analysis at the TRP. To study the performance of a UE with narrow beams compared to a UE with “omni like pattern” we implemented FR2 like antenna panels also at FR1 to evaluate the beam management and spatial relation configuration.

We compared:

- Config 447: 4 antenna panels (similar to FR2) at FR1, one element per panel is used (i.e.. the element has a beam width of app. 120deg)

- Config 552: Same config at FR2; for FR2 a higher TX power is assumed to compensate the higher pathloss at FR2

- Config 423: For reference, a FR1 configuration with low TX power

With an ideal beam management and ToA analysis a configuration with (many) narrow beams should perform equal or better than a configuration with wide beams (and low TX power).

Figure 9-CDF horizontal error UTDOA (with LOS detection), FR2 Tx Beams

Figure 10 - CDF 3D error for UTDOA, FR2 Tx Beams

Observation 6: 	When applying narrow beam antenna configuration in FR2 over multiple SRS resources, and in the presence of interference, a degradation in the positioning performance is observed. In case of several TX beams a RTOA selection/combining strategy is required. This algorithm may need further enhancements.

# Conclusions

We conclude with the following observations and proposals:

Observation 1: 	Using antennas with higher directivity increase the performance.

Observation 2: 	A low UE-TX power is sufficient for gNBs with beam forming in InF scenarios.

Observation 3:	When considering interference, the performance with low TX power is only feasible if UEs sharing the same REs use different cyclic shifts for the same SRS root sequence (“Cyclic shift multiplex”)

Observation 4: 	Using TX-diversity (4 transmissions at -20dBm from different antennas) shows a similar performance as single port transmission with 0dBm.

Observation 5: 	The beam selection and spatial relationship establishment criteria have major impact on the performance.

Observation 6: 	When applying narrow beam antenna configuration in FR2 over multiple SRS resources, and in the presence of interference, a degradation in the positioning performance is observed. In case of several TX beams a RTOA selection/combining strategy is required. This algorithm may need further enhancements.

Proposal 1:	For “low power operation” in combination with interference between positioning-SRS resources, cyclic shift enhancements shall be considered in Rel-17.

Proposal 2: 	Support multi-port transmission in Rel-17.

# References

- RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019.

- R1- 2004650, “Additional Scenarios for performance evaluations”, Ericsson, e-Meeting, May 25th – June 5th, 2020.

# Annex

Table 8.1.1.1.1-1: Rel.16/17 NR positioning - evaluation scenarios and parameters

| Parameter | [Case ID], [Scenario], [Frequency Band], [Technique] Configs 22x | [Case ID], [Scenario], [Frequency Band], [Technique] Configs 32x | [Case ID], [Scenario], [Frequency Band], [Technique] Configs 42x |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF_LOS InF_NLOS_DH |  |  |
| Carrier frequency | 4 GHz |  |  |
| Subcarrier spacing | 30kHz |  |  |
| Reference Signal Transmission Bandwidth | 100MHz |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 1 |  |  |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port |  |  |
| Number of sites | 18, Omni (dual polarized) | 18, 3 panels (1 panel per sector), one dual-polarized antenna element per panel is used | 18, 3 panels, 5 dual polarized beams per panel |
| Number of symbols used per occasion | 1 |  |  |
| number of occasions used per positioning estimate | 1 |  |  |
| Power-boosting level | 6dB (KTC = 4) |  |  |
| Uplink power control (applied/not applied) | Constant TX power setting, TX power = 23dBm, 0dBm, -20dBm |  |  |
| interference modelling (ideal muting, or other) | Interference between 6 UEs sharing same REs |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | - Detection of first arriving path (FAP) by threshold relative to peak. - Rising edge analysis of FAP. |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt |  |  |
| Network synchronization assumptions | ideal |  |  |
| UE/gNB Tx/Rx Calibration Error | Not applicable |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | None | gNB transmits DL-RS signals on all DL beams, UE reports received signal strength on DL-RS, gNB selects 1 beam (with two polarization) for measurements |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | None | None | Virtualization model: 1 TXRU per panel per polarization dimension |
| Additional notes, if any |  |  |  |

| Parameter | [Case ID], [Scenario], [Frequency Band], [Technique] Configs 44x | [Case ID], [Scenario], [Frequency Band], [Technique] Configs 54x | [Case ID], [Scenario], [Frequency Band], [Technique] Configs 62x |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF_LOS InF_NLOS_DH |  |  |
| Carrier frequency | 4 GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz |  |  |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 1 |  |  |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port |  |  |
| Number of sites | 18, 3 panels, 5 dual polarized beams per panel | 18, 3 panels, 7 (4 far, 2 medium coverage, 1 narrow) dual polarized beams per panel |  |
| Number of symbols used per occasion | 4 (sequential transmission) | 4 |  |
| number of occasions used per positioning estimate | 1 | 1 |  |
| Power-boosting level | 6dB (KTC = 4) |  |  |
| Uplink power control (applied/not applied) | Constant TX power setting TX power = 23dBm, 0dBm, -20dBm |  |  |
| interference modelling (ideal muting, or other) | Interference between 6 UEs sharing same REs |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | - Detection of first arriving path (FAP) by threshold relative to peak - Rising edge analysis of FAP |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt |  |  |
| Network synchronization assumptions | ideal |  |  |
| UE/gNB Tx/Rx Calibration Error | Not applicable |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | gNB transmits DL-RS signals on all DL beams, UE reports received signal strength on DL-RS, gNB selects 1 beam (with two polarization) for measurements according reported DL-RS strength |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Virtualization model: 1 TXRU per panel per polarization dimension |  |  |
| Additional notes, if any | Different UE antenna assumptions |  |  |