3GPP TSG RAN WG1 Meeting #103-e	R1-2008776

E-meeting, October 26th – November 13th, 2020

Agenda Item:	7.1

Source:	Huawei, HiSilicon

Title:	Discussion on UL skipping for CG PUSCH

Document for:	Discussion and Decision

Introduction

In RAN1#102-e, it was agreed for dynamic grant PUSCH, when the PUCCH overlaps with a set of PUSCHs, the selected PUSCH from the set cannot be skipped and a MAC PDU for the PUSCH is generated for UCI multiplexing. In addition, how to multiplex UCI on CG PUSCH without UL-SCH is also discussed, three options are listed but no consensus can be reached until the end of the meeting.

Agreement For UL skipping of dynamic UL grant in non-CA and CA case, when there is PUCCH carrying UCI overlapping with a set of PUSCHs, the PUSCH with UCI multiplexing from the set cannot be skipped. MAC generates MAC PDU for the PUSCH and the UCI is multiplexed on the PUSCH. How to handle the issue for CG PUSCH when there is PUCCH carrying UCI overlapping with a set of CG PUSCHs, for non-CA and CA case? - Option 1: (similar solution as DG PUSCH with skipping) - the UE first determines which PUSCH from the set of PUSCHs would carry the UCI, PUSCH_0 - the PUSCH with UCI multiplexing cannot be skipped and MAC generates MAC PDU for the PUSCH_0, and UCI is multiplexed on PUSCH_0 - Option 2: - the UE determines which PUSCH/PUCCH would carry the UCI based on MAC PDU generation: - If MAC generates data for all or a subset of PUSCHs from the set, UCI will be multiplexed on one of the PUSCH(s) with data based on the existing multiplexing rules. - If MAC generates data for none of the PUSCHs, none of the PUSCHs in the set will be transmitted and UCI will be transmitted in the PUCCH. - Option 3: - the UE first determines which PUSCH from the set of PUSCHs would carry the UCI, PUSCH_0 - If MAC generates data for all PUSCHs, all PUSCH will be transmitted and UCI is multiplexed in PUSCH_0 - If MAC generates data only for a subset of the PUSCHs, that subset will be transmitted, and UCI will be multiplexed in PUSCH_0. MAC always generates a PDU for PUSCH_0, whether it is with padding or not. - If MAC generates data for none of the PUSCHs, none of the PUSCHs will be transmitted and UCI will be transmitted in the PUCCH

In this contribution, we continue to investigate the multiplexing rules for configured grant PUSCH and discuss whether the similar solution for dynamic grant can be adopted.

Solutions for UL skipping for the CG PUSCH

Multiple options are proposed in the discussion [1]. Option 1 is similar to the DG PUSCH with skipping, reusing the Rel-15 mechanism to determine a PUSCH from a set for UCI multiplexing and a MAC PDU is still generated although there is no actual data in LCH for transmission. Option 2 is different from option 1. The PUSCH selection does not only depend on the resource mapping, but also relates to the MAC PDU generation. The UCI would only be multiplexed if the PUSCH contains a MAC PDU and if none of PUSCH carries PDU, the UCI fall back on the PUCCH for transmission. Option 3 is like a combination of option 1 and option 2. A PUSCH is determined first to piggyback HARQ-ACK/CSI, if the PUSCH belongs to the set which MAC generates PDU for transmission, then the uplink control information is transmitted on the PUSCH directly. However, if it does not, a pseudo MAC PDU is generated and the PUSCH will not be skipped. On the other hand, if no data is generated for all the PUSCHs, UCI are transmitted on PUCCH.

Unlike dynamic grant PUSCH, the CG PUSCH transmission is configured semi-persistently. So whether there is a PDU pending to be transmitted is likely known by UE in advance and the UE is aware of how to handle the collision between PUCCH and PUSCH once the DL grant is received. For example, assume either CG PUSCH with UL-SCH or PUCCH is able to carry UCI when the overlapping happens. HARQ-ACK would be transmitted on the PUSCH as long as the first timeline condition for overlapping only is satisfied and would be transmitted on the PUCCH instead following the gNB indication if there is no UL-SCH on PUSCH. Therefore, since the status of the LCH is acknowledged ahead and UE can decide whether to transmit a CG PUSCH in an occasion by UE itself, the processing time seems not a quite sever constraint for UCI multiplexing on CG even there is no PDU is delivered from MAC layer.

In addition to the UE processing time, the complexity and performance degradation introduced by blind detection in gNB should be also taken into account when deciding the multiplexing rule. For Option 2, gNB does not get the status of UEs’ logical channels in real time, neither can the gNB figure out which CG PUSCH transmissions would have UL-SCH. So gNB has to blind decode each CG transmission to judge which PUSCH is selected by UE as well as to detect PUCCH to check whether the grant is missed by UE or not. Take the following case as an example in Figure 1, after receiving and decoding CG PUSCH 1 successfully, the gNB is not sure whether the HARQ-ACK is carried on it or not. Only the second PUSCH transmission is definitely received, then gNB knows which CG transmission is used for HARQ-ACK multiplexing. However, if both CG transmissions are absent, the gNB has to go back to check and decode PUCCH which introduce more blind decoding.

Figure 1. HARQ-ACK multiplexed on the CG PUSCH

Referring to Option 3, it is possible to avoid the blind decoding between different PUSCH transmissions, but dependency of other CG transmission will bring another condition on MAC PDU generation. On the other hand, the gNB still needs to blind decode both CG PUSCH and PUCCH to make sense where to achieve UCI exactly. For the option 1, it is similar as the principle for DG PUSCH, and the dedicated CG PUSCH chosen by the common understanding between gNB and UE, blind decoding is not needed. Therefore, to avoid too many blind detections by gNB, option 1 and option 3 can be candidates. However, option 3 will introduce additional judgement on MAC generation and to have a uniform rule with dynamic grant PUSCH, option 1 is supported.

Proposal: For UL skipping of configured grant in non-CA and CA case, when there is PUCCH carrying UCI overlapping with a set of PUSCHs, similar rules of dynamic UL grant is used, i.e. the PUSCH with UCI multiplexing from the set cannot be skipped. MAC generates MAC PDU for the PUSCH and the UCI is multiplexed on the PUSCH.

Conclusions

Based on the discussion above, following proposal is obtained:

Proposal: For UL skipping of configured grant in non-CA and CA case, when there is PUCCH carrying UCI overlapping with a set of PUSCHs, similar rules of dynamic UL grant is used, i.e. the PUSCH with UCI multiplexing from the set cannot be skipped. MAC generates MAC PDU for the PUSCH and the UCI is multiplexed on the PUSCH.

- References

Vivo, “Summary of email discussion [102-e-NR-7.1CRs-11] – phase 2”, R1-2007336, e-Meeting, August 17-28, 2020.