3GPP TSG RAN WG1 Meeting #103-e	R1-2008783

E-meeting, October 26 – November 13, 2020

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	On resource reselection due to dropped retransmissions

Document for:	Discussion and Decision

- Introduction

RAN2 sent an LS to RAN1 [1], where the following question was asked:

RAN2 would like to ask RAN1 whether resource reselection is needed for dropped retransmission caused by prioritization, pre-emption and congestion control.

In this contribution, we discuss the issue, and how to draft the reply LS to RAN2.

- Discussions

There was following agreement made in RAN1#101-e meeting:

- In Step 2, a UE shall select resources so that HARQ retransmission resources can be reserved by a prior SCI, except that

- In case no resource can be found for reservation (e.g., based on the identified candidate set after Step 1) for a retransmission of a TB, the re-transmission can be transmitted on a resource that is not reserved

- After the resource selection is performed, HARQ retransmission on a resource not reserved by a prior SCI is allowed due to transmission dropping caused by prioritization, pre-emption and congestion control

The intention of these agreements is to specify that the UE shall ensure HARQ retransmission resources can be reserved by a prior SCI. However, the condition may not be satisfied under two exceptions. The first one is that a retransmission resource reserved by a prior SCI cannot be found during the resource selection, and the second is when a retransmission in resource reserved by a prior SCI is dropped due to prioritization, pre-emption, or congestion control, in which case retransmission on an unreserved resource is permitted. Thus, the purpose of the sub-bullets of the agreement is that retransmission on resources not reserved by a prior SCI is allowed in these limited cases, whereas the RAN2 question is based on a misinterpretation that the second sub-bullet is a request for additional resource reselection procedures. This means that there is no need for MAC to provide additional procedures of the kind queried by RAN2.

Proposal 1:  Reply to RAN2 as follows:

- The cited RAN1 agreement does not call for any additional resource reselection procedures beyond what are already specified in TS 38.321.

The other point raised in [1] regarding SL-L-CS-RNTI can be handled in editor CRs to TS 38.212 and TS 38.213 without needing any discussion.

- Conclusions

Following proposals were made:

Proposal 1:  Reply to RAN2 as follows:

- The cited RAN1 agreement does not call for any additional resource reselection procedures beyond what are already specified in TS 38.321.

# References

- R2-2008585, “LS on RAN2 agreements and RAN1 related issues”, RAN2 #111-e, August 2020.