3GPP TSG RAN WG1 Meeting #103-e	R1-2008787

E-meeting, October 26th - November 13th, 2020

Agenda Item:	7.1

Source:	Huawei, HiSilicon

Title:	Clarifications on UE features in case of cross-carrier operation

Document for:	Discussion and Decision

Introduction

In RAN1#102-e, the ambiguity issue on the interpretation of UE features in case of cross-carrier operation was discussed. In this contribution, we discuss the remaining Rel-15 UE features which was not agreed in RAN1#102-e.

Discussion

Some UE capabilities are defined as per-band. For a given per-band UE capability, if a UE indicates support of this capability for band A and does not indicates the support for band B, it is not clear whether the UE supports this capability or not in case of cross-carrier operation between band A and band B. There are three possible interpretations as listed in [1]

- Interpretation1: Support of this UE capability is based on the support of this capability for the band of the scheduled/triggered/indicated cell only.

- Interpretation2: Support of this UE capability is based on the support of this capability for the band of the scheduling/triggering/indicating cell only.

- Interpretation3: Support of this UE capability is based on the support of this capability for both the band of the scheduled/triggered/indicated cell and the band of the scheduling/triggering/indicating cell.

After some discussion in RAN1#102-e, the following conclusion was made [2]

Conclusion

- Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on the support of this capability for the band of the scheduled/triggered/indicated cell only.

  - aperiodicTRS

  - beamSwitchTiming

- Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on both the support of this capability for the band of the scheduled/triggered/indicated cell and the support of this capability for the band of the scheduling/triggering/indicating cell.

  - crossCarrierScheduling-SameSCS

However, there are still several UE features: ue-SpecificUL-DL-Assignment, bwp-DiffNumerology and bwp-SameNumerology which were not concluded. It is unclear which interpretation should be followed for these features.

As implied by the UE capability definition listed in the Annex

- ue-SpecificUL-DL-Assignment indicates whether the UE supports dynamic determination of UL and DL link direction and slot format based on Layer 1 scheduling DCI from the scheduling cell. The determination of UL and DL link direction and slot format is based on Layer 1 scheduling DCI for the scheduled cell for a UE and does not have relevance to the scheduling cell carrying the Layer 1 scheduling DCI.

- bwp-DiffNumerology/bwp-SameNumerology indicate whether the UE supports BWP adaptation up to 4 BWPs with the different/same numerologies, via DCI and timer. Similar as above, the DCI and timer based BWP adaptation is for the scheduled cell for a UE which does not have relevance to the scheduling DCI.

In summary, the above UE capabilities relates to operation within the scheduled/triggered/indicated cell which do not require the support of the corresponding capabilities in the scheduling/triggering/indicating cell. Moreover, following this interpretation will provide UE some implementation flexibilities, i.e. a UE does not need to support the given UE capacity on both cells in case of cross-carrier operation.

Therefore, we have the following proposal:

Proposal: In case of cross-carrier operation, the support of the following UE capabilities is based on the support of a capability for the band of the scheduled/triggered/indicated cell only.

- ue-SpecificUL-DL-Assignment

- bwp-DiffNumerology/bwp-SameNumerology

Conclusions

Based on discussion above, we have the following proposal

Proposal: In case of cross-carrier operation, the support of the following UE capabilities is based on the support of a capability for the band of the scheduled/triggered/indicated cell only.

- ue-SpecificUL-DL-Assignment

- bwp-DiffNumerology/bwp-SameNumerology

- References

R1-2006332, “Interpretation of UE Features in Case of Cross-Carrier Operation”, ZTE

RAN1#102-e Chairman’s note

- Annex

| ue-SpecificUL-DL-Assignment Indicates whether the UE supports dynamic determination of UL and DL link direction and slot format based on Layer 1 scheduling DCI and higher layer configured parameter UL-DL-configuration-dedicated as specified in TS 38.213 [11]. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| bwp-DiffNumerology Indicates whether the UE supports BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer. For the UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| bwp-SameNumerology Defines type A/B BWP adaptation (up to 2/4 BWPs) with the same numerology, via DCI and timer. For the UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |