3GPP TSG RAN WG1 Meeting #103-e 	R1-2008795

E-meeting, October 26th – November 13th, 2020

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on the LS on Full slot formats support in TDD UL-DL configuration

Document for:	Discussion and Decision

Introduction

In RAN3 Meeting #109-e, the following LS [1] was sent to RAN1 for response.

1. Overall Description: As part of RIM/CLI work item, RAN3 defined “Intended TDD UL-DL Configuration NR” IE (section 9.2.2.40 of TS 38.423) with slot configuration list, which is exchanged over Xn/F1 interface. RAN3 found that slot formats 46-55 (Table 11.1.1-1 of TS 38.213) cannot be supported except the slot configuration can be used as half-slot configuration. RAN3 would like to confirm with RAN1 whether all the slot formats defined in TS 38.213 shall be supported in the intended TDD UL-DL configuration exchange. XnAP/F1AP specs also quoted tdd-UL-DL-ConfigurationCommon defined in TS 38.331 for the TDD UL-DL configuration of each cell. RAN3 would like to confirm with RAN2 whether all the slot formats defined in TS 38.213 are supported by tdd-UL-DL-ConfigurationCommon. If yes, has maxNrofSlots = 320 already taken half-slot into consideration? 2. Actions: To RAN1: ACTION: RAN3 would like to ask RAN1 whether all the slot formats defined in TS 38.213 shall be supported in the intended TDD UL-DL configuration exchange. To RAN2: ACTION: RAN3 would like to ask RAN2 whether all the slot formats defined in TS 38.213 are supported by tdd-UL-DL-ConfigurationCommon. If yes, has maxNrofSlots = 320 already taken half-slot into consideration?

The following sections will give our understanding regarding to the “Intended TDD UL-DL Configuration NR”.

Discussion

In RAN1#96 meeting, the following agreements were reached regarding to the “Intended TDD UL-DL Configuration NR”

Agreement For “Intended DL/UL configuration”, per-slot based OFDM symbol level DL/UL region indication is supported. - Periodicity and numerology (i.e. subcarrier spacing, CP length) are configured. - A list containing multiple entries of the following: - Information for indicating slot within a periodicity is configured. - DL/UL region indication (i.e. all DL or all UL or number of DL symbols and/or number of UL symbols in a slot) is configured for each slot

In this agreement, the “Intended TDD UL-DL Configuration NR” is conveyed to other gNBs by “DL/UL region indication (i.e. all DL or all UL or number of DL symbols and/or number of UL symbols in a slot) is configured for each slot” whose intention was to reuse the dedicated TDD configuration signaling as follows

TDD-UL-DL-ConfigDedicated ::= SEQUENCE { slotSpecificConfigurationsToAddModList SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig OPTIONAL, -- Need N slotSpecificConfigurationsToReleaseList SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex OPTIONAL, -- Need N ... } TDD-UL-DL-SlotConfig ::= SEQUENCE { slotIndex TDD-UL-DL-SlotIndex, symbols CHOICE { allDownlink NULL, allUplink NULL, explicit SEQUENCE { nrofDownlinkSymbols INTEGER (1..maxNrofSymbols-1) OPTIONAL, -- Need S nrofUplinkSymbols INTEGER (1..maxNrofSymbols-1) OPTIONAL -- Need S } } }

Therefore, the SFI slot format in table 11.1.1-1 of TS 38.213 is in line with the RAN1 agreement and not used for the RAN3 signaling.

Observation: DCI-based SFI is not used to indicate the “Intended TDD UL-DL Configuration NR” gNB-gNB signaling for CLI, but the TDD-UL-DL-ConfigDedicated is reused instead.

Proposal: Send LS reply to RAN3 as the draft in Appendix that it is not required to support all the slot formats in Table 11.1.1-1 of TS 38.213 in the intended TDD UL-DL configuration exchange for CLI.

Conclusion

Based on the discussions, we have the following observation and proposal:

Observation: DCI-based SFI is not used to indicate the “Intended TDD UL-DL Configuration NR” gNB-gNB signaling for CLI, but the TDD-UL-DL-ConfigDedicated is reused instead.

Proposal: Send LS reply to RAN3 as the draft in Appendix that it is not required to support all the slot formats in Table 11.1.1-1 of TS 38.213 in the intended TDD UL-DL configuration exchange for CLI.

- References

- [1]  R1-2007513/R3-205794 Full slot formats support in TDD UL-DL configuration, RAN3#109-e

- Appendix

- Draft LS reply on NR SRS carrier switching

Title: Draft Reply LS on Full slot formats support in TDD UL-DL configuration for CLI Response to: R1-2007513/R3-205794 Release: Rel-16 Work Item: NR_CLI_RIM Source: Huawei, [RAN WG1] To: RAN WG3, RAN WG2 Cc: Contact Person: Name: Frank Long E-mail Address: frank.longyi {@} huawei.com 1. Overall Description: RAN1 would like to thank RAN3 for the LS and provide the following response: Based on the RAN1#96 agreement below, it is not required to support all the slot formats in Table 11.1.1-1 of TS 38.213 in the intended TDD UL-DL configuration exchange for CLI. Agreement For “Intended DL/UL configuration”, per-slot based OFDM symbol level DL/UL region indication is supported. - Periodicity and numerology (i.e. subcarrier spacing, CP length) are configured. - A list containing multiple entries of the following: - Information for indicating slot within a periodicity is configured. - DL/UL region indication (i.e. all DL or all UL or number of DL symbols and/or number of UL symbols in a slot) is configured for each slot. 2. Actions: To RAN3: ACTION: RAN1 respectfully requests RAN3 to take the above response into account. 3. Date of Next TSG-RAN WG1 Meetings: TSG-RAN WG1 Meeting#104-e January 25th – February 5th 2021 E-Meeting TSG-RAN WG1 Meeting#104-bis-e April 12th - 20th 2021 E-Meeting