3GPP TSG RAN WG1 Meeting #103-e	R1-2008797

# E-meeting, October 26 – November 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR |  | rev | - | Current version: | V16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on transmission in preconfigured UL resources |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In legacy NB-IoT, when a UE is configured by higher layers with a NB-IoT carrier for monitoring USS, the UE is not expected to receive NPSS/NSSS/NPBCH on that carrier, no matter that carrier is anchor carrier or non-anchor carrier. PUR works in idle mode, i.e., transmission on preconfigured UL resources and the associated PUR search space monitoring happen in idle mode. UEs in idle mode will use NPSS/NSSS/NPBCH for synchronization as the UE may sleep for a long time. So for a UE configured with PUR, the UE may need to monitor PUR search space as well as receiving NPSS/NSSS/NPBCH during idle mode. If the PUR search space is configured on anchor carrier, according to the current specification, the UE is not expected to receive NPSS/NSSS/NPBCH on that carrier, and the UE may be out of synchronization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Constrain the exclusion of NPSS/NSSS/NPBCH monitoring on a higher-layer configured USS carrier to only C-RNTI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the PUR search space is configured on anchor carrier, the UE may be out of synchronization since it is not expected to receive NPSS/NSSS/NPBCH on that carrier according to the current specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

## 16.6	Narrowband physical downlink control channel related procedures

#### < Unchanged parts are omitted >

For UE-specific search space by C-RNTI, if the UE is configured by high layers with a NB-IoT carrier for monitoring of NPDCCH UE-specific search space,

-	the UE shall monitor the NPDCCH UE-specific search space on the higher layer configured NB-IoT carrier,

-	the UE is not expected to receive NPSS, NSSS, NPBCH on the higher layer configured NB-IoT carrier.

otherwise,

-	the UE shall monitor the NPDCCH UE-specific search space on the same NB-IoT carrier on which NPSS/NSSS/NPBCH are detected.

#### < Unchanged parts are omitted >