3GPP TSG-RAN WG1 Meeting #103-e	R1-2008798

# E-meeting,  October 26 – November 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | <CR#> | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on additional SRS symbols |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_DL_MIMO_EE-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As a uplink signal, the additional SRS symbols are SC-FDMA symbols instead of OFDM symbols. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When referring to additional SRS, the OFDM symbol is replaced by SC-FDMA symbols. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurate technical expressions in normative text, leading to inconsistent or ambiguous multiplexing schemes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.1.2, 5.5.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

##### 5.5.3.1.2	Sequence generation for additional SRS

The sounding reference signal rSRS(p)n is defined by clause 5.5.3.1.1 with the following exceptions

-	nSRScs is given by the higher-layer parameter srs-CyclicShiftAdd

-	Nap is given by the higher-layer parameter srs-AntennaPortAdd

-	KTC is given by the higher-layer parameter srs-TransmissionCombNumAdd

-	the function fgh in clause 5.5.1.3 is given by

fghns,l=0if group hopping is disabledi=072ic8nsNsymbUL+l+i mod 30if group hopping is enabled

where l is the SC-FDMA symbol index within the slot ns and NsymbUL is the number of SC-FDMA symbols per slot

-	the function v in clause 5.5.1.4 is given by

v=cnsNsymbUL+lif group hopping is disabled and sequence hopping is enabled0otherwise

<Unchanged parts are omitted>

##### 5.5.3.2.2	Mapping to physical resources for additional SRS

An additional SRS spans one or more SC-FDMA symbols in the time domain, where

-	the starting SC-FDMA symbol l0 within the subframe is given by the higher-layer parameter srs-StartPosAdd;

-	the duration N in number of SC-FDMA symbols, including potential guard symbols, is given by the higher-layer parameter srs-DurationAdd.

Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions:

-	frequency hopping between SC-FDMA symbols is supported and if a UE is configured by higher layer parameter srs-GuardSymbolFH-Add, a guard symbol is added between every frequency hop;

-	antenna switching within a subframe is supported and if a UE is configured by higher layer parameter srs-GuardSymbolAS-Add, a guard symbol is added between every antenna switching;

-	nSRS=l'R where l' is the additional SRS transmission number not counting guard symbol(s) within the subframe with l'=0 corresponding to the starting SC-FDMA symbol l0, and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter srs-RepNumAdd;

-	BSRS is given by the higher-layer parameter srs-BandwidthAdd;

-	bhop is given by the higher-layer parameter srs-HoppingBandwidthAdd;

-	NFH is the number of frequency hops with the same antenna/antenna pair for additional SRS, derived from N=RNFH+NFH-1GFH if antenna switching is not configured for additional SRS, and from N=RNASNFH+NAS-1GAS+NFH-1NASGFH+NAS-1(1-GAS)GFH if antenna switching is configured for additional SRS, where R is the repetition factor given by the higher-layer parameter srs-RepNumAdd, NAS is the number of antenna switches for additional SRS defined in 8.2 of [4], GAS∈0, 1 is the guard-symbol configuration for antenna switching given by the higher-layer parameter srs-GuardSymbolAS, GFH∈0, 1 is the guard symbol configuration for frequency hopping given by the higher-layer parameter srs-GuardSymbolFH, and N is given by the higher-layer parameter srs-DurationAdd;

-	nRRC is given by the higher-layer parameter srs-FreqDomainPosAdd;

-	Nap is given by the higher-layer parameter srs-AntennaPortAdd;

-	nSRScs is given by the higher-layer parameter srs-CyclicShiftAdd;

-	KTC is given by the higher-layer parameter srs-TransmissionCombNumAdd;

-	kTC is given by the higher-layer parameter srs-TransmissionCombAdd.

<Unchanged parts are omitted>