3GPP TSG RAN WG1 Meeting #103e                                               R1-xxxxxx

E-mail Oct.26.2020-Nov.13.2020

Agenda Item:	8.4.4

Source:	CAICT

Title:	Discussion on the applicability of DFT-S-OFDM for NTN

Document for:	Discussion

Introduction

In R17, OFDM signal waveform has been specified in NTN downlink, but OFDM signal has the characteristics of high PAPR and high EVM. This proposal attempts to discuss the applicability of using DFT-s-OFDM signal waveform in NTN downlink.

Recently, we have tested the DFT-s-OFDM signal both in lab and on orbit.

In lab, we tested multi-carrier performance of DFT-s-OFDM signal using the mature satellite TWTA. By testing the EVM and PAPR of single carrier, two carriers and four carriers, the adaptability of multi-carrier in satellite TWTA is studied.

Meantime, in a real satellite-to-ground environment, the low-orbit satellite and its related gateway and UE were used to verify the DFT-S-OFDM signa. It mainly verifies the transmission rate and transmission quality of the signal in the satellite-to-ground docking scenario.

Lab test

Test configuration

Table.1：Test parameter configuration

| Signal format | Methods of signal generation | modulation mode | Coding rate | Single carrier bandwidth configuration | Multicarrier bandwidth configuration | Demodulat-ion |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Matlab generates baseband signal, imported into the signal source to generate RF signal. | 8PSK | 3/4 | 200MHz 400MHz | 2*400MHz 4*200MHz | Matlab |

Table.2：Test instruments

|  | Type | Firmware version |
| --- | --- | --- |
| Vector signal generator | R&S SMW200A | 04.30.090.19 |
| Vector spectrum analyzer | R&S FSW | 4.50_SP1 |
| Power meter | Keysight N1912A | / |

PAPR, EVM Test

2.2.1 Test scenario：

Scenario 1: Direct connection, test the PAPR and EVM of DFT-s-OFDM signal with typical parameters, such as bandwidth, number of multi-carriers, modulation mode;

Scenario 2: Pass TWTA, test the PAPR and EVM of DFT-s-OFDM signal with different OBO value;

Fig.1  PAPR, EVM Test Scenario

2.2.2 Test Method：

As shown in Figure 2, the test method of DFT-s-OFDM signal is divided into instrument direct link (as shown in A) and RF loop link (as shown in B).

Fig.2  PAPR, EVM Test Connection Diagram

Table.3 Parameter configuration

| Modulation | 8PSK |
| --- | --- |
| Subcarrier | 120kHz |
| Coding method | LDPC |
| Bandwidth | 200MHz、400MHz |
| Multi carrier mode | 2*400MHz、4*200MHz |
| coding rate | 3/4 |

2.2.3 Test Result：

Case 1: Single carrier test (200MHz, 400MHz)，3/4 8PSK

Case 2:Multi carrier test (2*400M, 4*200M), 3/4 8PSK

Case 1:

Fig.3  PAPR test result of 8PSK single carrier

Fig.4  EVM test result of 8PSK single carrier

Test Result of case 1：

a)	In the direct connection test, the PAPR of 200MHz and 400MHz are about 7dB, while the PAPR of 400MHz is slightly higher;

b)	After TWTA, the PAPR of 200MHz and 400MHz are lower than direct connection. With the increase of OBO from 0.5 to 4, the PAPR value increases. Under the same OBO condition, the PAPR of 400MHz bandwidth is slightly higher.

c)	In direct connection, EVM of 200MHz bandwidth and 400MHz bandwidth signal is close, EVM test result is 1.15% at 200MHz bandwidth, EVM test result is 1.37% at 400MHz bandwidth；

d)	After TWTA, the EVM of 200MHz and 400MHz bandwidth signals in the saturated region is close to each other. When the OBO is 0.5, the EVM is about 13%. With the increase of OBO from 0.5 to 4, the EVM of 200MHz and 400MHz bandwidth signals decreased. When OBO is 4, the EVM of 200MHz bandwidth signal is 4.46%, and that of 400MHz bandwidth signal is 5%。

Case 2：

Fig.5  PAPR test result of 8PSK Multi carrier

Fig.6  EVM test result of 8PSK Multi carrier

Test Result of case 2：

a)	In the direct connection test, the PAPR of 2*400MHz signal is lower, which is 8.94dB. While that of 4*200MHz signal is 11.28dB;

b)	After TWTA, the PAPR saturated region is lower than directly connected test. With the increase of OBO, PAPR increases gradually. The change trend of PAPR is consistent under the bandwidth of 2*400MHz and 4*200MHz, and the RAPR of 2*400MHz is slightly lower than 4*200MHz;

c)	In the direct connected test, the EVM of 2*400MHz and 4*200MHz bandwidth are similar, which is about 2%;

d)	After TWTA, the EVM of 2*400MHz and 4*200MHz signals is higher than direct connection test. The EVM decreases with the increase of OBO. For example, when OBO is 3dB, the EVM of two carrier signal is 7.82%, the EVM of four carrier (center carrier) is 11.09%, and the EVM of edge carrier is 12.33%.

Orbit test

Satellite information

This test uses the GalaxySpace’s satellite.

• The satellite height is about 600km
• Service link is on Ka band, and feeder link is on QV band
• Satellite payload channel bandwidth is 600MHz

Fig.7  GalaxySpace’s Gateway

Test Method

The test use instruments combined with GalaxySpace’s Gateway and User’s equipment.

- Signal Generator sends the optimized 5G NR PDSCH (DFT-S-OFDM) channel signal, which is directly connected to the gateway through cable on C-band.

- The gateway station up converts the signal to the QV frequency band and sends it to the satellite via its antenna.

- After the signal is forwarded by the satellite, it is received by the user equipment’s antenna.

- The analyzer receives the signal from the user equipment’s intermediate frequency through the radio frequency cable, and imports the collected data to the key technology verification platform for analysis.

- The terminal receives the signal, down-converted to the C-band and received by the spectrum analyzer. The signal is finally analyzed by the analysis platform.

Fig.8 Test Scenario

Test Configuration

Table.4：Test instruments

| Test instruments | Manufacturer | Type |
| --- | --- | --- |
| Signal Generator | Keysight | M9384B |
| Signal Analyzer | Keysight | M9040B |
| FieldFox Microware Analyzer | Keysight | N9952A |
| Test analysis platform | CAICT/ HwaCreate | / |

Test Process

The test was carried out in different weather conditions and covered a variety of waveform configurations:

- Weather: sunny, cloudy, sandy, raining, etc.

- Carrier bandwidth: 100MHz, 400MHz

- Modulation method: QPSK, 8PSK

- Code rate: 1/2, 3/4, 4/5, 9/10, etc.

Test Result

On the sunny day, using the optimized 5G DFT-S-OFDM signal system, under the 400MHz carrier bandwidth, using 8PSK modulation, 3/4 code rate configuration, removing system overhead, the user rate exceeding 700Mbps.

Conclusion

- With the increase of the number of carriers, the PAPR and EVM of 400MHz, 2*400MHz and 4*200MHz bandwidth signals are both increase;

- For 4 carrier signal, the EVM of center carrier is worse than edge carrier;

- It is necessary to adopt EVM optimization technology to reduce EVM in multi carrier scenario, but it is suggested to consider the system complexity and cost caused by the optimization technology;

- The performance of DFT-S-OFDM signals in the satellite channel scenario meets the design and simulation expectations.

- Later, we will use real stations and terminals to carry out end-to-end verification.

- We expect to discuss the possibility of using DFT-s-OFDM as an optional signal at the next meeting.