3GPP TSG RAN WG1 Meeting #103-e	R1-2008813

E-meeting, October 26 – November 13, 2020

Agenda Item:	8.6.3

Source:	Huawei, HiSilicon

Title:	Functionality for coverage recovery

Document for:	Discussion and Decision

# Introduction

In RAN1#102-e meeting, RAN1 discussed the issue related to coverage recovery due to the device complexity reduction. And the study also includes evaluations of the impact to network capacity and spectral efficiency.

RAN1 made the following agreements related to study of coverage loss/recovery, some agreements related to detailed simulated assumption are provided in Appendix:

Agreements For the channel(s) affected by complexity reduction, the following methodology can be used to determine the target performance for coverage recovery - Step 1: Obtain the link budget performance of the channel based on link budget evaluation - Step 2: Obtain the target performance requirement for RedCap UEs within a deployment scenario - FFS on the target performance requirement - Step 3: Find the coverage recovery value for the channel if the link budget performance is worse than the target performance requirement Agreements: - Link budget evaluation for RedCap should include at least PDCCH/PDSCH and PUCCH/PUSCH Agreements: - For initial access related channels, at least Msg2, Msg3, Msg4 and PDCCH scheduling Msg2/4 are included for link budget evaluation - Other initial access related channels are not precluded Agreements: - The impact of small form factor is considered for all the uplink and downlink channels - A 3dB loss of antenna gain is included in link budget calculation for FR1 - FFS on the application to both FDD and TDD bands or only FDD bands Agreements: Down-selection on the following options for the target performance requirement for RedCap UEs in RAN1#103-e (aim for early in the e-meeting): - Option 1: The target performance requirement for each channel is identified by a target MCL or MIL or MPL within a reasonable deployment - Option 3: The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario - Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL - The details for the target performance requirement are FFS Agreements: For RedCap UE, adopt the following target data rates for link budget evaluation for FR1 Rural. - 1 Mbps on DL and 100kbps in UL Agreements: For RedCap UE, adopt the following target data rates for link budget evaluation for FR1 Urban. - 2 Mbps on DL and 1Mbps in UL Note: The 2Mbps target data rate in downlink is the scaled value of the 10Mbps in the CE SI by a factor of 0.2 Agreements: For RedCap UEs, the target data rates for link budget evaluation for FR2 are as follows: - 25Mbps for BW 50MHz/100MHz on DL and 5Mbps in UL - Optionally, 12.5Mbps for BW 50MHz as the target data rate for DL, assuming the same DL PSD as that of BW 100MHz - Note: in case of 50MHz BW, the maximum supported DL data rate is half that of the 100MHz BW in DL Agreements: - For link budget evaluation, the antenna gain loss due to the small form factor can be applied to all the FR1 bands - For RedCap coverage analysis, the agreements in the Rel-17 CE SI regarding link budget template and antenna array gain are reused. - Continue to discuss and decide the performance metric in RAN1-103 e-meeting

In this contribution, we discuss the coverage reduction and performance impact due to UE complexity reduction and provide potential solutions to support coverage recovery.

# Discussion on coverage recovery

Coverage recovery target and evaluation methodology

Due to the UE complexity reduction, such as UE number of RX antennas and bandwidth reduction, the capability of NR RedCap UEs is obviously lower than that of NR legacy UEs, which will result in coverage loss. Besides UE RX antenna and bandwidth reduction, as discussed in the SID, for FR1, coverage analysis for wearables can consider potential reduced antenna efficiency due to device size limitations as part of the antenna gains. Therefore, some evaluations are necessary.

For the evaluation methodology, the methodology based on link budget has been agreed in last meeting, then coverage deficit for physical channels can be determined if the link budget performance is worse than the target performance requirement. So we think the key point is how to determine the target performance.

In last meeting, it is agreed to down-selection on the following options for the target performance requirement for RedCap UEs:

- Option 1: The target performance requirement for each channel is identified by a target MCL or MIL or MPL within a reasonable deployment

- Option 3: The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario

  - Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL

In our view, typical use cases and scenarios have been defined for NR RedCap UEs in SID. For the sake of business success of REDCAP, we should identify the bottleneck channels taken reasonable deployments into account, rather than just comparing with reference NR UE. So option1 seems more reasonable than option3. And to identify the real deployment for RedCap, some loss should be considered, such as shadow fading, penetration loss, etc. So MPL is more suitable than MIL or MCL for opiton1.

Proposal 1: The target performance requirement for each channel should be identified by a target MPL within a reasonable deployment.

Figure 1. Typical cellular network topology

For the typical cellular network, the well-known hexagonal grid topology can be modeled, as shown in Figure 1. The cell range can be up to  23* ISD from the base station for hexagonal cells. So the target performance can be defined as following:

- Target performance means the MPL at the distance of being 23* ISD from the base station for hexagonal cells.

Proposal 2: Target performance can be defined as the required MPL at the distance of being 23* ISD from the base station for hexagonal cells.

Link level evaluation assumptions

The evaluated scenarios, frequency and channels are summarized in Table 1.

Table 1: The evaluated scenario and frequency

| Parameters | FR1 values |
| --- | --- |
| Scenario and frequency | Urban: 2.6 GHz (TDD) 4 GHz (TDD) 2 GHz (FDD) 700 MHz (FDD) Rural: 700 MHz (FDD) |
| Frame structure for TDD | For 2.6 GHz: DDDDDDDSUU (S: 6D:4G:4U) For 4 GHz: DDDSUDDSUU (S: 10D:2G:2U) |
| Channels | PDCCH, PDSCH, PUCCH, PUSCH, Msg2, Msg3, Msg4 |
| Target data rate | Urban: PDSCH: 2Mbps PUSCH: 1Mbps Rural: PDSCH: 1Mbps PUSCH: 100 kbps |

Some key system assumptions for NR legacy and NR RedCap wearable are provided in Table 2 and Table 3. Other general system assumptions and channel-specific parameters can be found in Appendix 1. As discussed in the SID, coverage analysis for wearables can include consideration of potential reduced antenna efficiency due to device size limitations as part of the antenna gains and the loss of the additional coverage loss is to be limited to 3dB. So the coverage of wearables can be considered to be the worst case of all RedCap use cases. Hence, coverage of wearables is mainly evaluated, while the results of coverage evaluation for the other use cases can be derived by adding 3 dB.

Note that 3dB potential antenna gain loss is considered below for both 1T1R and 1T2R antenna configurations of NR RedCap UEs.

Table 2. Key system assumptions for NR legacy

| Scenario | FR1, Urban | FR1, Urban | FR1, Urban | FR1, Urban | FR1, Rural |
| --- | --- | --- | --- | --- | --- |
| Carrier Frequency | 2.6 GHz (TDD) | 4 GHz (TDD) | 2 GHz (FDD) | 0.7 GHz (FDD) | 0.7 GHz (FDD) |
| Cell BW | 100 MHz (273 PRBs) | 100 MHz (273 PRBs) | 20 MHz (106 PRBs) | 20 MHz (106 PRBs) | 20 MHz (106 PRBs) |
| BWP BW | 100 MHz (273 PRBs) | 100 MHz (273 PRBs) | 20 MHz (106 PRBs) | 20 MHz (106 PRBs) | 20 MHz (106 PRBs) |
| # of UE TX/RX antenna elements | 1 / 4 | 1 / 4 | 1 / 4 | 1 / 4 | 1 / 4 |
| # of UE TX/RX antenna chains | 1 / 4 | 1 / 4 | 1 / 4 | 1 / 4 | 1 / 4 |
| UE antenna gain(dBi) | 0 | 0 | 0 | 0 | 0 |

Table 3. Key system assumptions for NR RedCap wearable

| Scenario | FR1, Urban | FR1, Urban | FR1, Urban | FR1, Urban | FR1, Rural |
| --- | --- | --- | --- | --- | --- |
| Carrier Frequency | 2.6 GHz (TDD) | 4 GHz (TDD) | 2 GHz (FDD) | 0.7 GHz (FDD) | 0.7 GHz (FDD) |
| BWP BW | 20 MHz (51 PRBs) | 20 MHz (51 PRBs) | 20 MHz (106 PRBs) | 20 MHz (106 PRBs) | 20 MHz (106 PRBs) |
| # of UE TX/RX antenna elements | 1 / 1 or 1 / 2 | 1 / 1 or 1 / 2 | 1 / 1 or 1 / 2 | 1 / 1 or 1 / 2 | 1 / 1 or 1 / 2 |
| # of UE TX/RX antenna chains | 1 / 1 or 1 / 2 | 1 / 1 or 1 / 2 | 1 / 1 or 1 / 2 | 1 / 1 or 1 / 2 | 1 / 1 or 1 / 2 |
| UE antenna gain(dBi) | -3 | -3 | -3 | -3 | -3 |

Link level evaluation results

The link budget results for NR legacy UEs and NR RedCap UEs are provided in Table 4 ~ Table 7. The corresponding required SINR are provided in Appendix 2. The highlighted values mean these available MPL are smaller than the target MPL and these channels therefore are coverage limited.

As discussed above, in order to determine the target MPL, we could set a target ISD firstly. 350 meters for Urban and 1732 meters for Rural could be considered as the typical ISD in RedCap SI.

Table 4. The target MPL (dB)

| Scenario | Target MPL |
| --- | --- |
| Urban(O2I) 4GHz (TDD) | 118.2 for ISD 350 m |
| Urban(O2I) 2.6GHz (TDD) | 114.4 for ISD 350 m |
| Rural (O2I) 700MHz (FDD) | 118.9 for ISD 1732 m |

Table 5. The available MPL (dB) for NR legacy UEs

| Scenario | PDCCH USS | PDSCH | PUCCH (2 bits) | PUCCH (22 bits) | PUSCH | PDCCH CSS | Msg2 | Msg4 | Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 2.6GHz (TDD) | 137.18 | 136.51 | 129.77 | 127.47 | 111.22 | 133.18 | 133.37 | 133.16 | 121.88 |
| Urban(O2I) 4GHz (TDD) | 134.20 | 133.50 | 126.71 | 124.94 | 109.30 | 130.20 | 130.39 | 130.15 | 118.93 |
| Rural (O2I) 700MHz (FDD) | 136.29 | 135.29 | 131.85 | 129.60 | 124.15 | 136.29 | 135.29 | 135.97 | 127.67 |

Table 6. The available MPL (dB) for NR RedCap wearable with 1T2R

| Scenario | PDCCH USS | PDSCH | PUCCH (2 bits) | PUCCH (22 bits) | PUSCH | PDCCH CSS | Msg2 | Msg4 | Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 2.6GHz (TDD) | 131.14 | 129.17 | 126.77 | 124.47 | 108.22 | 127.14 | 126.88 | 126.85 | 118.88 |
| Urban(O2I) 4GHz (TDD) | 128.22 | 126.17 | 123.71 | 121.94 | 106.30 | 124.22 | 123.92 | 123.91 | 115.93 |
| Rural (O2I) 700MHz (FDD) | 133.29 | 132.29 | 128.85 | 126.60 | 121.15 | 133.29 | 132.29 | 132.97 | 124.67 |

Table 7. The available MPL (dB) for NR RedCap wearable with 1T1R

| Scenario | PDCCH USS | PDSCH | PUCCH (2 bits) | PUCCH (22 bits) | PUSCH | PDCCH CSS | Msg2 | Msg4 | Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 2.6GHz (TDD) | 128.06 | 125.33 | 126.77 | 124.47 | 108.22 | 124.06 | 122.59 | 122.97 | 118.88 |
| Urban(O2I) 4GHz (TDD) | 124.73 | 122.33 | 123.71 | 121.94 | 106.30 | 120.73 | 119.62 | 120.03 | 115.93 |
| Rural (O2I) 700MHz (FDD) | 129.94 | 128.58 | 128.85 | 126.60 | 121.15 | 129.94 | 127.98 | 128.99 | 124.67 |

The coverage loss due to UE complexity is summarized in Table 8 and Table 9 according to the evaluated results in Table 4, Table 5 and Table 6.

Table 8. Coverage loss (dB) for NR RedCap wearable with 1T2R compared with NR legacy UEs

| Scenario | PDCCH USS | PDSCH | PUCCH (2 bits) | PUCCH (22 bits) | PUSCH | PDCCH CSS | Msg2 | Msg4 | Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 2.6GHz (TDD) | 6.04 | 7.34 | 3.00 | 3.00 | 3.00 | 6.04 | 6.49 | 6.31 | 3.00 |
| Urban(O2I) 4GHz (TDD) | 5.98 | 7.33 | 3.00 | 3.00 | 3.00 | 5.98 | 6.47 | 6.24 | 3.00 |
| Rural (O2I) 700MHz (FDD) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |

Table 9. Coverage loss (dB) for NR RedCap wearable with 1T1R compared with NR legacy UEs

| Scenario | PDCCH USS | PDSCH | PUCCH (2 bits) | PUCCH (22 bits) | PUSCH | PDCCH CSS | Msg2 | Msg4 | Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 2.6GHz (TDD) | 9.12 | 11.18 | 3.00 | 3.00 | 3.00 | 9.12 | 10.78 | 10.19 | 3.00 |
| Urban(O2I) 4GHz (TDD) | 9.47 | 11.17 | 3.00 | 3.00 | 3.00 | 9.47 | 10.77 | 10.12 | 3.00 |
| Rural (O2I) 700MHz (FDD) | 6.35 | 6.71 | 3.00 | 3.00 | 3.00 | 6.35 | 7.31 | 6.98 | 3.00 |

As shown above, For all the uplink channels, the coverage loss is 3 dB for both NR RedCap wearable with 1T2R and 1T1R, which comes from the potential reduced antenna efficiency due to device size limitations.

For NR RedCap UEs, for all the simulated TDD band, only PUSCH and Msg3 is coverage limited where Msg3 is coverage limited for 4GHz only because of 3 dB antenna radiation loss caused by device size limitations. And for all the simulated FDD band, none of the channels is coverage limited.

For msg3, frequency hopping is not enabled in our simulation. However, in current Release, inter-slot frequency hopping has already been supported for msg3. Additional frequency diversity gain can be achieved if frequency hopping is enabled, because frequency bandwidth of msg3 is only 2PRB in current assumption which is apparently lower than the coherent bandwidth assuming 300 ns of delay spread. So frequency hopping of Msg3 for RedCap UEs should also be considered at least within UE bandwidth.

For Msg2, TBS scaling is not enabled in our simulation. However, in current Release, TBS scaling, e.g. 0.5, 0.25, has already been supported for Msg2, which means 2 or 4 times repetition in frequency can be enabled and therefore at least 3~6 dB gain can be achieved additionally.

For all the downlink channels, for NR RedCap UEs with 1T2R, the coverage loss is about 6~8 dB for 4GHz and 2.6 GHz in Urban scenario, which mainly comes from reduced UE RX antenna number, the reduced antenna gain and UE bandwidth reduction. And the coverage loss is 3 dB for 700MHz in rural scenario, which only comes from the reduced antenna gain. For NR RedCap UEs with 1T1R, the coverage loss is about 9~12 dB for 4GHz and 2.6 GHz in Urban scenario and 6~7 dB for 700MHz in rural scenario, which mainly comes from reduced RX antenna number and the reduced antenna gain. The coverage loss by 1T1R compared to 1T2R for NR RedCap UEs is around 3 dB.

In addition, whether the coverage of RedCap UEs is limited or not is determined by target ISD. However, the value of 350 m is already very modest. If a radical target ISD is considered, e.g. 500m, it can be foreseen that the downlink coverage of RedCap UEs with 1T1R will also be limited.

Observation 1: For all the simulated TDD band, PUSCH is coverage limited. Msg3 is coverage limited for 4GHz because of 3 dB antenna radiation loss caused by device size limitations.

Observation 2: For all the simulated FDD band, none of the channels is coverage limited.

Observation 3: For Msg.3, frequency hopping can be considered for NR RedCap UEs.

Observation 4: For Msg.2, TBS scaling has already been supported in Rel-15, which should also be included for RedCap UEs.

Observation 5: The coverage loss by 1T1R compared to 1T2R for NR RedCap UEs is around 3 dB.

Observation 6: If a radical target ISD is considered, e.g. 500m, it can be foreseen that the downlink coverage of RedCap UEs with 1T1R will also be limited.

Half Duplex FDD (HD-FDD) impact on coverage

HD-FDD is one of the potential technique to reduce the UE complexity and cost as discussed in SID. As evaluated in 36.888, type-A HD-FDD has switching interruption only on downlink subframes which results in no change of required SINR of uplink transmission but increased required SINR of downlink transmission for a given TBS at one subframe. This increased required SINR, or equivalent SINR loss, may be compensated by the improved noise figure of a switch-based receiver RF chain. However, because the available time resources within each 10 ms frame are reduced in both downlink and uplink, HD-FDD will apparently result in data rate reduction of PDSCH and PUSCH, thus coverage reduction of PDSCH and PUSCH.

For type B HD-FDD defined in LTE, because both the downlink subframes preceding an uplink subframe and following an uplink subframe from the same UE will not be received, the impact on data rate of PDSCH or PUSCH by type B HD-HDD is even greater than type A HD-FDD.

In summary, if the same reference data rate of downlink or uplink is required, HD-FDD will result in coverage loss of both downlink and uplink compared to full duplex FDD.

Observation 7: HD-FDD will result in loss of downlink and uplink coverage for a given target data rate.

# Discussion on network capacity and spectrum efficiency

In this section, we will discuss network capacity and spectrum efficiency. The observations made in this section hold in general, despite the coverage analysis is done across coverage-limited channels or coverage gap from normal UEs for each channel.

Network capacity evaluation target and methodology

For the purpose of reducing UE complexity and cost, reduction of number of UE RX antennas and downlink modulation order have been proposed at the potential cost of downlink network capacity and spectrum efficiency. So the impact on network capacity by RedCap UE complexity reduction is evaluated.

For evaluation of network capacity, the following agreement has been made in the last meeting.

Agreements:

- For SLS based capacity evaluation, use the assumption in TR 38.802, Table A.2.1-1 as the baseline.

In the offline email discussion, the detail of the evaluation methodology has been discussed. However, in our view, some details of burst buffer traffic evaluation are still needed to be discussed and clarified.

- Traffic model of burst buffer traffic evaluation

We think FTP model 3 should be used for both NR legacy UEs and NR RedCap UEs. IM traffic model should not be used for NR RedCap UEs. As in email discussions, the IM traffic model is suitable for power saving evaluation featured as sparse inter-arrival time and low data rate. However, REDCAP UEs are not featured in texting application but mostly voice and video applications for both wearables and video surveillance use cases, which should be reflected in network capacity and spectral efficiency evaluations for REDCAP UEs by FTP traffic model.

Additionally, maximum 50% UE percentage is limited in current SLS assumptions. Together with the IM traffic model for REDCAP UEs, the maximum ratio of traffic data rate between REDCAP UEs and legacy UEs (FTP model) is just 1:50,  which means that no more than 1/50 traffic in the SLS are contributed from REDCAP UEs. It is obviously far too low for REDCAP evaluations and does not reflect the real situation in practical networks.

Furthermore, the power consumption evaluation is evaluated from UE perspective. Thus network RU is not much important, and UE traffic model with maximum data rate 400kbps like IM model etc. is suitable to reflect the real UE power consumption. However, in evaluations of spectrum efficiency and network RU are from network perspective, IM model with such low data rate is incapable.

Proposal 3: For burst traffic evaluation, FTP model 3 should be used for both NR legacy UEs and NR RedCap UEs, and IM traffic model for RedCap UEs can be optional.

- UE number of burst buffer traffic evaluation

As discussed in the offline email discussion, the number of UEs including both eMBB and RedCap UEs can be based on the following options:

- Option 1: The number of UEs can be different for different RedCap UE ratios in the cell (e.g. using the target RU to determine the number of UEs for each  RedCap UE ratio independently)

- Option 2: With respect to a target RU, the total number of UEs is same for all the RedCap UE ratios in the cell (e.g. firstly determine the number of UEs assuming 0% RedCap UE ratio for a target RU and use the same total number to other RedCap UE ratios)

Comparing both options above, we suggest that Option2 is a more reasonable way. For option1, the total UE numbers becomes the tuning variable to achieve the target RU for different percentages of RedCap UEs and different RedCap UE capabilities, which will much increase the simulation load. More importantly, the evaluation is to evaluate the impact of RedCap UE capability, rather than the impact of the variance of RedCap UE number. So if the number of RedCap UEs varies with different UE capability, it is obvious not reasonable to evaluate the impact on the performance of legacy UEs and RedCap UE self, e.g. UPT. So we have the follow proposal:

Proposal 4: For burst traffic evaluation, the number of UEs should be same for all the RedCap UE ratios in the cell (e.g. firstly determine the number of UEs assuming 0% RedCap UE ratio and use the same number to other RedCap UE ratios).

SLS evaluation assumptions

Some key SLS evaluation assumptions are provided as following, the other assumptions can be find in appendix A.3.

Table 10.  SLS evaluation assumptions

| Parameters | FR1 values |
| --- | --- |
| Scenario and frequency | Dense Urban: 2.6 GHz (TDD) (primary choice) 4 GHz (TDD) (secondary choice) |
| UE antenna configuration | TDD: 1T4R for NR legacy UE 1T2R (2 layer) /1T1R for NR RedCap UE |
| Traffic model | Full buffer: Non-full buffer traffic: Downlink; FTP traffic. Model; FTP model 3. Packet size; 0.125 Mbytes. Mean inter-arrival time; 200 ms. Uplink; FTP traffic. Model; FTP model 3. Packet size; 0.05 Mbytes. Mean inter-arrival time; 200 ms. |
| Traffic load | Full buffer traffic: 10 users per cell including both RedCap and reference NR UEs Non-full buffer traffic: Low (e.g. <30%) and medium (e.g. 30%-50%) loading (resource utilization) |
| Percentage of RedCap UEs among total number of UEs Note: Other UEs are the reference NR UEs | Full buffer traffic: 0, 20%, 50% ,100% Non-full buffer traffic: 0, 25%, 50%, 100% |

SLS evaluation results

Some downlink simulation results are provided in Figure 2~4 and the detailed simulation results are provided in the capacity evaluation template.

Figure 2. Cell downlink spectrum efficiency for full buffer traffic evaluation

Figure 3. Cell downlink spectrum efficiency for burst buffer traffic evaluation

Figure 4. RU (%) for burst buffer traffic evaluation

For full buffer evaluation, for 2.6GHz (TDD) in Urban, if UE RX antennas are reduced from 4RX to 2RX (2Layers) and downlink maximum modulation order is reduced from 256QAM to 64QAM for NR RedCap UEs, when 100% UEs in network are NR RedCap UEs, the loss of downlink cell spectrum efficiency is about 30%. While, if UE RX antennas is further reduced to 1RX, the cell spectrum efficiency loss can be up to 46%. And we can also see that the more the percent of NR RedCap UEs, the larger loss of SE.

For burst buffer evaluation, for 2.6GHz (TDD) in Urban, if UE RX antennas are reduced from 4RX to 2RX (2Layers) and downlink maximum modulation order is reduced from 256QAM to 64QAM for NR RedCap UEs, when 100% UEs in network are NR RedCap UEs, the loss of downlink cell spectrum efficiency is about 54% and meanwhile the RU will be increased by about 104%. While, if UE RX antennas is further reduced to 1RX, the cell spectrum efficiency loss can be up to 70%, meanwhile the RU will be increased by about 166%.

Observation 8: For full buffer traffic evaluation, the loss of cell downlink SE is about 30% for NR RedCap UEs with 1T2R & 64QAM compared with NR legacy UEs with 1T4R & 256QAM, and the loss will be up to nearly 50% if UE RX antennas are further reduced to 1RX, assuming 100% of NR RedCap UEs in network.

Observation 9: For burst buffer traffic evaluation, the loss of cell downlink SE is about 54% and RU will be increased by 104% for NR RedCap UEs with 1T2R & 64QAM compared with NR legacy UEs with 1T4R & 256QAM, and the SE loss will be up to nearly 70% and RU will be increased by 166% if UE RX antennas are further reduced to 1RX, assuming 100% of NR RedCap UEs in network.

- Uplink evaluation

Some uplink simulation results are provided in Figure 5~7 and the detailed simulation results are provided in the capacity evaluation template.

For full buffer evaluation, for 2.6GHz (TDD) in Urban, if uplink maximum modulation order is reduced from 64QAM to 16QAM for RedCap UEs, the loss of uplink cell spectrum efficiency is about 43%.

For burst buffer evaluation, for 2.6GHz (TDD) in Urban, the loss of downlink cell spectrum efficiency is about 50% and meanwhile the RU will be increased by about 57%.

In addition, Table 5.1.3.1-1 MCS index table 1 in TS 38.214 [4] is used in uplink evaluation, so the performance loss for 16QAM compared to 64QAM comes from modulation order reduction and code rate reduction.

Observation 10: For full buffer traffic evaluation, the loss of cell uplink SE is about 43% if uplink maximum modulation order of NR RedCap UEs is reduced from 64QAM to 16QAM, assuming 100% of NR RedCap UEs in network.

Observation 11: For burst buffer traffic evaluation, the loss of cell uplink SE is about 50% and RU will be increased by 57% if uplink maximum modulation order of NR RedCap UEs is reduced from 64QAM to 16QAM, assuming 100% of NR RedCap UEs in network.

Figure 5. Cell uplink spectrum efficiency for full buffer traffic evaluation

Figure 6. Cell uplink spectrum efficiency for burst buffer traffic evaluation

Figure 7. Uplink RU (%) for burst buffer traffic evaluation

# Potential solutions for DL and UL enhancements

In this section, we will discuss some potential enhancement for DL and UL channels.

- UL coverage enhancement

As evaluated in section 2, we can see that the PUSCH is coverage limited gravely for NR RedCap UEs and NR legacy UEs for TDD 4GHz/2.6GHz in urban. So some potential UL coverage enhancements should be considered. As coverage enhancement of PUSCH is also a key target of R17 Coverage enhancement SI and common assumptions in e.g. UE bandwidth can be assumed, general schemes discussed in R17 CE SI could also be considered for NR RedCap, such as repetition, cross-slot channel estimation, etc. Considering the further loss of uplink coverage for NR RedCap UEs due to potential UE antenna gain loss, some additional UL enhancements outside R17 CE SI could also be considered as a general tool for UL enhancements.

SUL

According to the evaluation results in section 2, coverage of PUSCH is worst in all the channels for 2.6GHz and 4GHz, so the coverage of a cell is limited by PUSCH. We also evaluate the uplink coverage performance for SUL band, such as 700MHz and 2GHz. The detail LLS assumptions and results are provided in Appendix 1~2. In contrast, link budget results and coverage gain for SUL compared with TDD band in Urban are provided in Table 11 and Table 12.

Table 11. The available MPL (dB) for NR RedCap wearable with 1T for SUL/FDD band

| Scenario | PUCCH (2 bits) | PUCCH (22 bits) | PUSCH (1Mbps) | Msg3 |
| --- | --- | --- | --- | --- |
| Urban(O2I) 700MHz (FDD) | 125.98 | 124.11 | 110.56 | 121.88 |
| Urban(O2I) 2GHz (FDD) | 125.99 | 124.34 | 110.85 | 121.57 |

Table 12.  Coverage gain analysis of SUL compared with TDD band for RedCap UEs

| Carrier frequency | MPL of PUSCH 1Mbps (dB) | MPL gain(dB) | Path loss gain(dB) | Total gain(dB) |
| --- | --- | --- | --- | --- |
| 2.6GHz (TDD) | 108.22 | 2.3 | 10.3 | 12.6 |
| 700MHz (FDD) | 110.56 |  |  |  |
| 4GHz (TDD) | 106.30 | 4.6 | 6.1 | 10.7 |
| 2GHz (FDD) | 110.85 |  |  |  |

To show the coverage enhancement effect of SUL directly, the maximum coverage range (cell radius) in Urban scenario are provided in Figure 8.

Figure 8. Analysis of Maximum coverage range (cell radius)

for RedCap UEs (PUSCH 1Mbps)

As shown above, SUL can achieve 10 ~ 13 dB coverage gain and maximum cell range can be increased by 80% ~ 120%. So we propose that SUL could be considered for NR RedCap UEs, it is beneficial for UL coverage without increase of UE complexity because it does not require concurrent UL transmission nor concurrent DL reception. Additionally, SUL can also promote UE data rate and improve user experience.

Observation 12: SUL can achieve 10 ~ 13 dB coverage gain and maximum cell range can be increased by 80% ~ 120%.

- DL performance improvement

As evaluated in section 2 and section 3, DL channels are not the coverage bottleneck in case of UE complexity reduction, however, performance degradation of PDSCH/PDCCH and loss of downlink spectrum efficiency are still observed, especially when NR RedCap UEs penetration ratio is very high.

Further, the coverage may need to be compensated especially considering the potential gap with normal UEs due to complexity reduction of RedCap UEs.

DCI size reduction

As evaluated, due to UE RX antennas reduction and potential antenna gain loss, the performance of PDCCH will degrade for not only cell edge UEs but also all the UEs in a cell, which means more CCEs will be consumed for NR RedCap UEs to retain the same performance as NR legacy UEs. When RedCap UEs coexist with normal UEs, the same performance is desirable for network scheduling/configuration. As one example, DCI size reduction could be considered.

- Techniques applicable to both UL and DL

Optimization for stationary UEs and UEs with limited mobility

Additionally since the devices are stationary or limited mobility for industrial wireless sensors and video surveillance, the optimization for stationary UEs and UEs with limited mobility can be studied, such as DMRS overhead reduction.

BWP switching in a larger system bandwidth

In our companion contribution [3], we have evaluated that UE bandwidth reduction will cause loss of frequency selective gain in downlink as shown in Figure 6. Therefore, BWP switching in a larger bandwidth from system perspective could be considered to achieve frequency scheduling gain. Additionally, BWP switching can also achieve fast load balancing between narrow bands in a cell.

Considering there is no change on UL Tx and BS antenna configurations, the gain on UL may be limited; while from specification point of view, there may be no need to restrict this technique to DL only, unless other issues identified.

Figure 6．Performance loss by fixed 20MHz BW location over flexible 20MHz within 100MHz for PDSCH

Proposal 5: Performance enhancements could be considered for NR RedCap UEs, such as UE BWP switching within a larger gNB bandwidth, DMRS overhead reduction for stationary UEs and UEs with limited mobility, DCI size reduction for DL and SUL for UL.

# Conclusions

According to the previous discussion, we have the following observations and proposals.

Observation 1: For all the simulated TDD band, PUSCH is coverage limited. Msg3 is coverage limited for 4GHz because of 3 dB antenna radiation loss caused by device size limitations.

Observation 2: For all the simulated FDD band, none of the channels is coverage limited.

Observation 3: For Msg.3, frequency hopping can be considered for NR RedCap UEs.

Observation 4: For Msg.2, TBS scaling has already been supported in Rel-15, which should also be included for RedCap UEs.

Observation 5: The coverage loss by 1T1R compared to 1T2R for NR RedCap UEs is around 3 dB.

Observation 6: If a radical target ISD is considered, e.g. 500m, it can be foreseen that the downlink coverage of RedCap UEs with 1T1R will also be limited.

Observation 7: HD-FDD will result in loss of downlink and uplink coverage for a given data rate.

Observation 8: For full buffer traffic evaluation, the loss of cell downlink SE is about 30% for NR RedCap UEs with 1T2R & 64QAM compared with NR legacy UEs with 1T4R & 256QAM, and the loss will be up to nearly 50% if UE RX antennas are further reduced to 1RX, assuming 100% of NR RedCap UEs in network.

Observation 9: For burst buffer traffic evaluation, the loss of cell downlink SE is about 54% and RU will be increased by 104% for NR RedCap UEs with 1T2R & 64QAM compared with NR legacy UEs with 1T4R & 256QAM, and the SE loss will be up to nearly 70% and RU will be increased by 166% if UE RX antennas are further reduced to 1RX, assuming 100% of NR RedCap UEs in network.

Observation 10: For full buffer traffic evaluation, the loss of cell uplink SE is about 43% if uplink maximum modulation order of NR RedCap UEs is reduced from 64QAM to 16QAM, assuming 100% of NR RedCap UEs in network.

Observation 11: For burst buffer traffic evaluation, the loss of cell uplink SE is about 50% and RU will be increased by 57% if uplink maximum modulation order of NR RedCap UEs is reduced from 64QAM to 16QAM, assuming 100% of NR RedCap UEs in network.

Observation 12: SUL can achieve 10 ~ 13 dB coverage gain and maximum cell range can be increased by 80% ~ 120%.

Proposal 1: The target performance requirement for each channel should be identified by a target MPL within a reasonable deployment.

Proposal 2: Target performance can be defined as the required MPL at the distance of being 23* ISD from the base station for hexagonal cells.

Proposal 3: For burst traffic evaluation, FTP model 3 should be used for both NR legacy UEs and NR RedCap UEs, and IM traffic model for RedCap UEs can be optional.

Proposal 4: For burst traffic evaluation, the number of UEs should be same for all the RedCap UE ratios in the cell (e.g. firstly determine the number of UEs assuming 0% RedCap UE ratio and use the same number to other RedCap UE ratios).

Proposal 5: Performance enhancements could be considered for NR RedCap UEs, such as BWP switching in a larger bandwidth, DMRS overhead reduction for stationary UEs and UEs with limited mobility, DCI size reduction for DL and SUL for UL.

# References

- RP- 201386, “Revised SID on Study on support of reduced capability NR devices”, Ericsson.

- R1-2005256, “Evaluation on the baseline performance for FR1”, Huawei, HiSilicon.

- R1- 2003303, “Functionality for coverage recovery”, Huawei, HiSilicon.

- 3GPP TS 38.214, NR;Physical layer procedures for data, v16.3.0.

# Appendix

# Link level simulation assumption for coverage evaluation

The general system assumptions and link level simulation for coverage evaluation is provided in Table A1-1~ Table A1-10.

Table A1-1: General system assumptions for NR legacy and NR RedCap

| Scenario | FR1, Urban | FR1, Urban | FR1, Urban | FR1, Urban | FR1, Rural |
| --- | --- | --- | --- | --- | --- |
| Carrier Frequency | 2.6 GHz (TDD) | 4 GHz (TDD) | 2 GHz (FDD) | 0.7 GHz (FDD) | 0.7 GHz (FDD) |
| SCS | 30 kHz | 30 kHz | 15 kHz | 15 kHz | 15 kHz |
| Frame structure for TDD | DDDDDDDSUU (S: 6D:4G:4U) | DDDSUDDSUU (S: 10D:2G:2U) | - | - | - |
| gNB total transmit power(dBm) | 53 | 53 | 46 | 46 | 46 |
| UE power class | Class 3 | Class 3 | Class 3 | Class 3 | Class 3 |
| # of gNB TX/RX antenna elements | 192 | 192 | 48 | 16 | 16 |
| # of gNB TX/RX antenna port | 4 | 4 | 4 | 4 | 4 |
| Channel Model | TDL-C, NLOS | TDL-C, NLOS | TDL-C, NLOS | TDL-C, NLOS | TDL-C, NLOS |
| UE antenna correlation | Low | Low | Low | Low | Low |
| Delay spread | 300 ns | 300 ns | 300 ns | 300 ns | 300 ns |
| UE velocity | 3 km/h | 3 km/h | 3 km/h | 3 km/h | 3 km/h |

Table A1-2. Simulation parameter setting for PUSCH eMBB

| scenario | Channel model | Delay spread | DMRS configuration | Moving speed | Scheduled PRB | MCS | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 4GHz (TDD) | TDL-C | 300ns | 1 FL | 3 km/h | 30 | 3 | 2280 |
| Urban(O2I) 2.6GHz (TDD) | TDL-C | 300ns | 1 FL | 3 km/h | 30 | 3 | 2280 |
| Urban(O2I) 2 GHz (FDD) | TDL-C | 300ns | 1 FL | 3 km/h | 15 | 3 | 1160 |
| Urban(O2I) 700 MHz (FDD) | TDL-C | 300ns | 1 FL | 3 km/h | 15 | 3 | 1160 |
| Rural (O2I) 700MHz (FDD) | TDL-C | 300ns | 1 FL | 3 km/h | 4 | 0 | 128 |
| Other parameters | - Metric: 10% BLER for eMBB - Antenna number: 1T4R - DFT-S-OFDM - No repetition, No retransmission, No frequency hopping - 14OS - TDD frame structure: DDDDSUDDSUU for 4GHz DDDDDDDDSUU for 2.6GHz - Precoding cycling |  |  |  |  |  |  |

Table A1-3. Simulation parameter setting for PUCCH format1

| scenario | Channel model | Delay spread | Scheduled PRB | Performance metric |
| --- | --- | --- | --- | --- |
| Urban(O2I) 4GHz (TDD) | TDL-C | 300ns | 1 | NACK to ACK 0.1% DTX to ACK 1% ACK missed 1% |
| Urban(O2I) 2.6GHz (TDD) | TDL-C | 300ns | 1 |  |
| Urban(O2I) 2 GHz (FDD) | TDL-C | 300ns | 1 |  |
| Urban(O2I) 700 MHz (FDD) | TDL-C | 300ns | 1 |  |
| Rural (O2I) 700MHz (FDD) | TDL-C | 300ns | 1 |  |
| Other parameters | - Payload size: 2 bits - Antenna number: 1T4R - DFT-S-OFDM - No repetitions, No retransmissions - 14 OS scheduled - Frequency hopping: enabled |  |  |  |

Table A1-4. Simulation parameter setting for PUCCH format3

| scenario | Channel model | Delay spread | Scheduled PRB | Performance metric |
| --- | --- | --- | --- | --- |
| Urban(O2I) 4GHz (TDD) | TDL-C | 300ns | 1 | CSI : 10% |
| Urban(O2I) 2.6GHz (TDD) | TDL-C | 300ns | 1 |  |
| Urban(O2I) 2 GHz (FDD) | TDL-C | 300ns | 1 |  |
| Urban(O2I) 700 MHz (FDD) | TDL-C | 300ns | 1 |  |
| Rural (O2I) 700MHz (FDD) | TDL-C | 300ns | 1 |  |
| Other parameters | - Payload size: 22 bits - Antenna number: 1T4R - DFT-S-OFDM - No repetitions, No retransmissions - 14 OS scheduled - Frequency hopping: enabled - DMRS configuration: 2 symbol |  |  |  |

Table A1-5. Simulation parameter setting for Msg3

| scenario | Channel model | Delay spread | Moving speed | DMRS configuration | TBS |
| --- | --- | --- | --- | --- | --- |
| Urban(O2I) 4GHz (TDD) | TDL-C | 300ns | 3 km/h | 1 FL+2 Add | 56bits |
| Urban(O2I) 2.6GHz (TDD) | TDL-C | 300ns | 3 km/h |  |  |
| Urban(O2I) 2 GHz (FDD) | TDL-C | 300ns | 3 km/h |  |  |
| Urban(O2I) 700 MHz (FDD) | TDL-C | 300ns | 3 km/h |  |  |
| Rural (O2I) 700MHz (FDD) | TDL-C | 300ns | 3 km/h |  |  |
| Other parameters | - Metric: 10% BLER - Scheduled PRB: 2 - MCS 0 - Antenna number: 1T4R - DFT-S-OFDM - No repetition, No retransmission, No frequency hopping - 14 OS - Frequency hopping: disabled - Precoding cycling |  |  |  |  |

Table A1-6. Simulation parameter settings for PDSCH of NR legacy UE

| Scenario | Channel model | Delay spread | DMRS configuration | Moving speed | Scheduled PRB | MCS | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 4GHz (TDD) | TDL-C | 300ns | 1 FL | 3 km/h | 271 | 0 | 8456 |
| Urban(O2I) 2.6GHz (TDD) | TDL-C | 300ns | 1 FL | 3 km/h | 241 | 0 | 7296 |
| Rural (O2I) 700MHz (FDD) | TDL-C | 300ns | 1 FL | 3 km/h | 36 | 0 | 1128 |
| Other parameters | - Metric: 10% BLER - Antenna number: 4T4R for TDD, 4T2R for FDD - CP-OFDM - No repetition, No retransmission, No interleaving - 12OS - Precoding cycling |  |  |  |  |  |  |

Table A1-7. Simulation parameter settings for PDSCH of NR RedCap UE

| Scenario | Channel model | Delay spread | DMRS configuration | Moving speed | Scheduled PRB | MCS | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 4GHz (TDD) | TDL-C | 300ns | 1 FL | 3 km/h | 48 | 1 | 2064 |
| Urban(O2I) 2.6GHz (TDD) | TDL-C | 300ns | 1 FL | 3 km/h | 49 | 0 | 2064 |
| Rural (O2I) 700MHz (FDD) | TDL-C | 300ns | 1 FL | 3 km/h | 36 | 0 | 1128 |
| Other parameters | - Metric: 10% BLER - Antenna number: 4T2R/4T1R for TDD, 4T1R for FDD - CP-OFDM - No repetition, No retransmission, No interleaving - 12OS - Precoding cycling |  |  |  |  |  |  |

Table A1-8. Simulation parameter setting for PDCCH

| Scenario | Channel model | Delay spread | Moving speed | DCI payload |
| --- | --- | --- | --- | --- |
| Urban(O2I) 4GHz (TDD) | TDL-C | 300ns | 3 km/h | 64 bits (including 24 bits CRC) |
| Urban(O2I) 2.6GHz (TDD) | TDL-C | 300ns | 3 km/h |  |
| Rural (O2I) 700MHz (FDD) | TDL-C | 300ns | 3 km/h |  |
| Other parameters | - Metric: 1% BLER - AL=16 - CORESET: 2 symbols & 48 PRBs - Antenna number: 4T4R/4T2R/4T1R for TDD 4T2R/4T1R for FDD - REG bundle size: 6 REGs - CCE-to-REG mapping type: interleaved - CORESET interleaver size: 2 REG bundles - Precoding cycling |  |  |  |

Table A1-9. Simulation parameter settings for Msg2 of NR RedCap UE

| Scenario | Channel model | Delay spread | DMRS configuration | Moving speed | Scheduled PRB | MCS | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 4GHz (TDD) | TDL-C | 300ns | 1 FL+2 A | 3 km/h | 18 | 0 | 456 |
| Urban(O2I) 2.6GHz (TDD) | TDL-C | 300ns | 1 FL+2 A | 3 km/h | 18 | 0 | 456 |
| Rural (O2I) 700MHz (FDD) | TDL-C | 300ns | 1 FL+2 A | 3 km/h | 18 | 0 | 456 |
| Other parameters | - Metric: 10% BLER - Antenna number: 4T2R/4T1R for TDD, 4T1R for FDD - CP-OFDM - No repetition, No retransmission, No interleaving - 12OS - Precoding cycling |  |  |  |  |  |  |

Table A1-10. Simulation parameter settings for Msg4 of NR RedCap UE

| Scenario | Channel model | Delay spread | DMRS configuration | Moving speed | Scheduled PRB | MCS | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(O2I) 4GHz (TDD) | TDL-C | 300ns | 1 FL+1 A | 3 km/h | 38 | 0 | 1068 |
| Urban(O2I) 2.6GHz (TDD) | TDL-C | 300ns | 1 FL+1 A | 3 km/h | 38 | 0 | 1068 |
| Rural (O2I) 700MHz (FDD) | TDL-C | 300ns | 1 FL+1 A | 3 km/h | 38 | 0 | 1068 |
| Other parameters | - Metric: 10% BLER - Antenna number: 4T2R/4T1R for TDD, 4T1R for FDD - CP-OFDM - No repetition, No retransmission, No interleaving - 12OS - Precoding cycling |  |  |  |  |  |  |

# On required SINR of different channels for coverage evaluation

The required SINRs for link budget are provided in Table A2-1 ~ Table A2-5.

Table A2-1: Required SINR for UL channels

| Required SNR (dB) | PUSCH |  | PUCCH |  | Msg3 |
| --- | --- | --- | --- | --- | --- |
|  | 128kbps | 1Mbps | format1 | format3 | No retrans |
| Urban(O2I) TDD (7:3) 4GHz | N/A | -4.56 | -10.38 | -7.98 | -6.43 |
| Urban (O2I) TDD(8:2) 2.6GHz | N/A | -3.48 | -10.30 | -8.04 | -6.38 |
| Urban (O2I) FDD 2GHz | N/A | -3.78 | -10.52 | -8.10 | -6.35 |
| Urban (O2I) FDD 700MHz | N/A | -3.84 | -10.30 | -8.05 | -5.81 |
| Rural (O2I) FDD 700MHz | -5.3 | N/A | -10.30 | -8.05 | -5.81 |

Table A2-2: Required SINR for PDSCH

| Required SNR (dB) | 4T4R Note.1 | 4T2R | 4T1R |
| --- | --- | --- | --- |
| Urban(O2I) TDD (7:3) 4GHz | -7.47 | -3.14 | 0.70 |
| Urban (O2I) TDD(8:2) 2.6GHz | -8.35 | -4.15 | -0.54 |
| Rural (O2I) FDD 700MHz | N/A | -3.91 | -0.20 |

Note. 4T is for gNB TX antenna port and 4R is for UE RX antenna port, so do the other cases.

Table A2-3: Required SINR for PDCCH

| Required SNR (dB) | 4T4R | 4T2R | 4T1R |
| --- | --- | --- | --- |
| Urban(O2I) TDD (7:3) 4GHz | -11.25 | -8.27 | -4.78 |
| Urban (O2I) TDD(8:2) 2.6GHz | -11.23 | -8.19 | -5.11 |
| Rural (O2I) FDD 700MHz | N/A | -8.23 | -4.88 |

Table A2-4: Required SINR for Msg2

| Required SNR (dB) | 4T4R | 4T2R | 4T1R |
| --- | --- | --- | --- |
| Urban(O2I) TDD (7:3) 4GHz | -6.82 | -2.64 | 2.46 |
| Urban (O2I) TDD(8:2) 2.6GHz | -8.36 | -4.89 | -0.59 |
| Rural (O2I) FDD 700MHz | N/A | -3.91 | 0.40 |

Table A2-5: Required SINR for Msg4

| Required SNR (dB) | 4T4R | 4T2R | 4T1R |
| --- | --- | --- | --- |
| Urban(O2I) TDD (7:3) 4GHz | -8.12 | -4.88 | -1.0 |
| Urban (O2I) TDD(8:2) 2.6GHz | -8.13 | -4.82 | -0.94 |
| Rural (O2I) FDD 700MHz | N/A | -4.59 | -0.61 |

# System level simulation assumptions for downlink SE evaluation

The simulation assumptions of system level simulation for downlink spectrum efficiency evaluation are provided in Table A3-1.

Table A3-1. The simulation assumption of link level simulation for PDSCH

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500 m(Dense Urban)、1732 m(rural) |
| Scenario and frequency | Dense Uran:2.6 GHz (TDD) |
| system bandwidth | 20 MHz |
| UE bandwidth | 20 MHz for both reference UE and RedCap UE |
| SCS | 30 kHz for TDD 15 kHz for FDD |
| Channel model | 3DUma for DU、3DRma for Rural |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h |
| BS antenna configuration | 192* (M,N,P,Mg,Ng;Mp,Np) = (12,8,2,1,1;1,1) for 2.6GHz TDD |
| BS TRxU configuration | 64 TRxU for 2.6GHz TDD |
| UE antenna configuration | TDD: 1T4R for NR legacy UE 1T2R /1T1R for NR RedCap UE |
| UE antenna gain(dBi) | 0 for NR legacy UEs and NR RedCap UEs |
| CP/DFT-S-OFDM | CP-OFDM |
| SU/MU-MIMO | MU-MIMO |
| Codebook/Non-codebook | Codebook |
| Frame structure for TDD | For 2.6 GHz: DDDDDDDSUU (S: 6D:4G:4U) For 4 GHz: DDDSUDDSUU (S: 10D:2G:2U) DDDSU (S: 10D:2G:2U) |
| DMRS configuration | Type1 with 1front DMRS + 1symbol additional DMRS |

# RAN1 agreements on coverage simulation assumptions

Agreements: - For RedCap coverage evaluation, the Rel-17 CE SI agreements on gNB antenna configuration, # gNB Tx/Rx chains, channel model and delay spread are reused with the following revision and/or addition Parameters; FR1 values; FR2 values. Channel model; TDL-C; TDL-A CDL-A(optional). Delay spread; 300ns; 30ns. UE velocity; 3 km/h; 3 km/h. Antenna correlation; Low; Low. # gNB Tx chains; 2 or 4; 2. # gNB Rx chains; 2 or 4; 2. - For RedCap coverage evaluation, adopt the following table for the reference NR UE. Parameters; FR1 values; FR2 values. # UE Tx chains; 1; 1. # UE Rx chains; Urban: 4 and Rural: 2; 2. UE BW; Urban: 100 MHz (273 PRBs) Rural: 20 MHz (106 PRBs); 100 MHz (66 PRBs). - For RedCap coverage evaluation, adopt the following table for the RedCap UE. - Other UE BWs are not precluded Parameters; FR1 values; FR2 values. # UE Tx chains; 1; 1. # UE Rx chains; 1 or 2; 1 or 2. UE BW; Urban: 20 MHz (51 PRBs) Rural: 20 MHz (106 PRBs); 50 MHz (32 PRBs) or 100 MHz (66 PRBs). Agreements: - For RedCap coverage evaluation, reuse the Rel-17 CE SI agreements on channel specific parameters with the following revision and/or addition - TBS/PRB/MCS of PDSCH (except for Msg2)/PUSCH for the RedCap UE are based on the agreed target data rates or message sizes and reported by companies - Adopt the following table for Msg2 evaluation - Note: the TBS scaling is not precluded in the table entry “PRBs/TBS/MCS” Parameters; Values. PRBs/TBS/MCS; MCS is fixed to zero. Companies to report the used number of PRBs and corresponding TBS value. PDSCH duration; 12 OS. DMRS configuration; Type I, 3 DMRS symbol, no multiplexing with data. Waveform; CP-OFDM. HARQ configuration; No retransmission. Agreements: - For SLS based capacity evaluation, use the assumption in TR 38.802, Table A.2.1-1 as the baseline. - For calibration purposes, the following settings can be used: Parameters; FR1 values; FR2 values. Layout; Single layer Macro layer: Hex. Grid; Single layer Indoor floor: (12BSs per 120m x 50m) Candidate TRP numbers: 3, 6, 12. Inter-BS distance; 500m; 20m. Scenario and frequency; Dense Urban: 2.6 GHz (TDD) (primary choice) 4 GHz (TDD) (secondary choice) Other scenarios (e.g. Rural 700MHz) are not precluded.; Indoor: 28 GHz (TDD). Frame structure for TDD; For 2.6 GHz: DDDDDDDSUU (S: 6D:4G:4U) For 4 GHz: DDDSUDDSUU (S: 10D:2G:2U); DDDSU (S: 10D:2G:2U). Channel model; 3Duma; 5GCM office. UE distribution; 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h; 100% Indoor: 3km/h. Traffic model; Full buffer (Optional) Non-full buffer traffic, e.g. FTP traffic model 3 for the reference NR UEs and the IM traffic model from TR 38.840 for RedCap UEs. Traffic load; Full buffer traffic (Optional): 10 users per cell including both RedCap and reference NR UEs Non-full buffer traffic: Low (e.g. <30%) and medium (e.g. 30%-50%) loading (resource utilization). Percentage of RedCap UEs among total number of UEs Note: Other UEs are the reference NR UEs; Full buffer traffic (Optional): 0, 20%, 50% (i.e. 0, 2 or 5 RedCap UEs per cell), 100% (as applicable) Non-full buffer traffic: 0, 25%, 50%, 100% (optional, as applicable).