3GPP TSG-RAN WG1 Meeting #103-e	R1-2008930

e-Meeting, October 26th – November 13th 2020

Agenda Item:	8.9.1

Source:	Ericsson

Title:	Support of 16-QAM for unicast in UL and DL in NB-IoT

Document for:	Discussion and Decision

# 1	Introduction

In the Work Item (WI) on “Rel-17 enhancements for NB-IoT and LTE-MTC” [1], one of the objectives is to specify the following enhancement for NB-IoT:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

In RAN1 #102e, a set of agreements were made for both UL and DL [2]. In this contribution we go through each of the agreements as to provide our view on the technical aspects that have been left for further study (FFS) in RAN1 #102e [2]. In the sections below we treat UL and DL separately starting with the latter one.

# 2	Support of 16-QAM for unicast in DL

## 2.1	Maximum TBS to support 16-QAM in DL

In RAN1 #102e, three possible options towards down selecting the new maximum TBS for DL were agreed [2]:

Agreement At least for standalone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is select one option from following: - Option 1: 4968 bits with ISF=7 - Option 2: 5072 bits with ISF=7 - Option 3: 5736 bits with ISF=7 - FFS on ISF>7 for this maximum TBS FFS for inband deployments

Today, a Cat-NB2 device can support in DL a TBS up to 2536 bits. According to the WID, in DL there should be an “increase in maximum TBS of e.g. 2x the Rel-16 maximum”. Doubling the max TBS with respect to Rel-16 is reasonable from the perspective that 16-QAM has twice the number of bits per M-ary symbol than QPSK, but not going beyond that (i.e., no Option 3).

Strictly speaking, doubling the maximum TBS of Rel-16 will lead to have a new maximum TBS of 5072 bits, however it is worth noting that such a transport block size has not been previously used in the standard. In LTE, the closest value to twice the max TBS in Rel-16 is a TBS = 4968 bits (see Transport block size table 7.1.7.2.1-1 in [3]), meaning that if LTE were used as design reference the throughput won’t be exactly doubled but almost doubled (i.e., ⁓248 kbps rather than ⁓253.6 kbps).

Thus, from the options to be down selected, “Option 1: 4968 bits with ISF=7” seems to be a better choice as to avoid using a TBS that has not been previously used in the standard, or going beyond twice the max TBS in Rel-16 which won’t be in line with the increase in the number of bits per M-ary symbols introduced by 16-QAM with respect to QPSK.

On the new max TBS to be supported for 16-QAM in DL, “Option 1: 4968 bits with ISF =7” seems to be a better choice as to avoid using a TBS that has not been previously used in the standard, or going beyond twice the max TBS in Rel-16.

The maximum TBS to support 16-QAM for unicast in DL for stand-alone and guard-band deployments is “Option 1: 4968 bits with ISF =7”.

The agreement cited in this section contains one “FFS on ISF>7”, and one “FFS for inband deployments”, which are covered in the next section when the TBS/MCS table design for 16-QAM in DL will be discussed.

## 2.2	TBS/MCS table design, resource assignment and TBS allocation to support 16QAM in DL

In RAN1 #102e, a set of technical considerations towards the TBS/MCS table design, resource assignment and TBS allocation to support 16-QAM in DL were agreed [2]:

Agreement Further study on TBS/MCS table design, resource assignment and TBS allocation to support 16QAM in DL considering at least: - MCS field size - Achievable code rates - Avoidance of link-adaptation issues (i.e., large SINR differences between different entries within one TBS row or between different entries in adjacent TBS rows) - The break point between different modulation schemes - Impacts of deployment modes - Indication of modulation scheme for retransmissions - Applicability of repetitions - UE data rate

Moreover, a set of DL simulation assumptions were agreed to evaluate and verify that the proposed TBS/MCS table designs fulfil the technical considerations above:

Agreement Adopt the following evaluation assumptions for support of 16QAM in DL and UL for NB-IoT Simulation assumptions for DL Parameter; Value/Description. Operation mode for DL; Stand-alone, Guard-band, and In-band with 2 or 4 CRS ports. Number of antennas; 1T or 2T, 1R. Channel model; AWGN. Frequency Resource; 1 PRB. Number of repetitions; Baseline number of repetitions = 1 (Companies can provide results for other repetition). Modulation Order; QPSK, 16-QAM. Noise Estimation; Ideal. Channel Estimation; Realistic. Frequency Offset; 0. Time Offset; 0. .

In the subsection below, we present first a TBS/MCS Table design for stand-alone and guard-band deployments describing one-by-one how the technical considerations agreed in RAN1 #102e are fulfilled, thereafter we present the TBS/MCS Table for the in-band deployment as a subcase of the other deployments performing the same analysis.

### 2.2.1	Stand-alone and Guard-band deployments

In Table 1 we present a TBS/MCS table candidate for stand-alone and guard-band deployments, the table is divided in the following sections:

- Table 1a: Presents the entries of the TBS/MCS table, which design principle consists in re-using TBS entries from the legacy LTE Table 7.1.7.2.1-1 in TS 36.213 with only three changes:

- TBS = 328 bits is replaced by TBS = 296 as to avoid a performance crossing issue with respect to TBS = 336 located in the next adjacent row (i.e., ITBS = 17), see Figure A.1 in Annex A.1.

- TBS = 552 bits is replaced by TBS = 536 as to avoid a performance overlapping issue with respect to TBS = 600 located in the next adjacent row (i.e., ITBS = 15), see Figure A.1 in Annex A.1.

- TBS = 2472 bits is replaced by TBS = 2536 bits as to have the possibility of transmitting the max TBS in Rel-16 with half of the time-domain resources when 16-QAM is used.

- Table 1b: Achievable code rates of the TBS/MCS table for stand-alone and guard-band deployments (assuming two NRS ports).

- Table 1c: Performance of the TBS/MSC table for stand-alone and guard-band deployments at 10% BLER.

- Table 1d: 3GPP Technical Considerations on the TBS/MCS table design, where with respect to the proposed TBS/MSC table we provide one-on-one responses to the technical considerations agreed in RAN1 #102e

| Table 1: TBS/MCS Table for 16-QAM in DL in the case of “stand-alone” and “guard-band” deployments. |  |
| --- | --- |
| Table 1a: All legacy TBS entries for QPSK, 16-QAM TBS entries from legacy LTE Table 7.1.7.2.1-1 in [3] with only three changes to avoid a performance crossing (296) and overlapping issue (536), and to transmit the max Rel-16 TBs with half of the time domain resources (2536). Modulation Scheme; Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. QPSK only; 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 16-QAM only; 14; 256; 536; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 296; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2536; 2984; 4008; 4968. . | Table 1b: Achievable Code Rates of TBS/MCS table for stand-alone and guard-band deployments. Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. 0.13; 0.09; 0.09; 0.09; 0.09; 0.10; 0.10; 0.09. 0.16; 0.13; 0.12; 0.14; 0.13; 0.13; 0.12; 0.12. 0.18; 0.16; 0.18; 0.16; 0.15; 0.15; 0.14; 0.15. 0.21; 0.21; 0.22; 0.19; 0.18; 0.19; 0.19; 0.19. 0.26; 0.24; 0.25; 0.23; 0.23; 0.24; 0.24; 0.23. 0.32; 0.28; 0.27; 0.29; 0.29; 0.29; 0.29; 0.29. 0.37; 0.33; 0.31; 0.34; 0.35; 0.34; 0.34; 0.35. 0.42; 0.41; 0.39; 0.41; 0.40; 0.39; 0.41; 0.41. 0.47; 0.46; 0.46; 0.46; 0.46; 0.46; 0.46; 0.45. 0.53; 0.53; 0.53; 0.53; 0.53; 0.53; 0.53; 0.52. 0.55; 0.58; 0.58; 0.58; 0.59; 0.58; 0.58; 0.58. 0.66; 0.66; 0.67; 0.66; 0.67; 0.67; 0.67; 0.67. 0.76; 0.76; 0.77; 0.76; 0.76; 0.75; 0.75; 0.76. 0.82; 0.84; 0.84; 0.87; 0.84; 0.86; 0.84; 0.84. 0.46; 0.46; 0.47; 0.47; 0.47; 0.48; 0.47; 0.47. 0.5; 0.51; 0.51; 0.51; 0.52; 0.5; 0.51; 0.52. 0.53; 0.54; 0.54; 0.54; 0.54; 0.54; 0.54; 0.54. 0.61; 0.59; 0.6; 0.59; 0.6; 0.6; 0.59; 0.6. 0.66; 0.66; 0.65; 0.64; 0.66; 0.65; 0.64; 0.66. 0.71; 0.71; 0.72; 0.72; 0.72; 0.72; 0.72; 0.71. 0.76; 0.76; 0.77; 0.78; 0.78; 0.77; 0.78; 0.76. 0.84; 0.84; 0.82; 0.83; 0.84; 0.82; 0.83; 0.82. |
| Table 1c: Performance of the TBS/MSC table for stand-alone and guard-band deployments at 10% BLER |  |
| Table 1d: 3GPP Technical Considerations on the TBS/MCS table design for DL stand-alone and guard-band Agreement Further study on TBS/MCS table design, resource assignment and TBS allocation to support 16QAM in DL considering at least: - MCS field size For the proposed TBS/MCS table, 5 bits are required to keep all legacy TBS entries for QPSK and add on top of it entries for 16-QAM. - Achievable code rates The achievable code rates for the TBS entries used for 16-QAM in DL are between 0.46 and 0.84. - Avoidance of link-adaptation issues (i.e., large SINR differences between different entries within one TBS row or between different entries in adjacent TBS rows) - The largest “SINR differences between different entries within one TBS row” are: ITBS14 ⁓ 2 dB, ITBS15 ⁓ 1.69dB, ITBS16 ⁓ 1.23dB, ITBS17 ⁓ 1.45dB, ITBS18 ⁓ 1.22dB, ITBS19 ⁓ 1.34dB, ITBS20 ⁓ 1.3dB, ITBS21 ⁓ 1.09dB. - The largest SINR differences “between different entries in adjacent TBS rows” are: ITBS15-to-ITBS14 ⁓ 0.33 dB, ITBS16-to-ITBS15 ⁓ 0.46 dB, ITBS17-to-ITBS16 ⁓ 0.22 dB, ITBS18-to-ITBS17 ⁓ 0.23 dB, ITBS19-to-ITBS18 ⁓ 0.12 dB, ITBS20-to-ITBS19 ⁓ 0.04 dB, ITBS21-to-ITBS20 ⁓ 0.21 dB. - The break point between different modulation schemes The average SINR difference at the breaking-point is ITBS14_16QAMavg-to-ITBS13_QPSKavg = abs(8.94 dB – 8.28 dB) ⁓ 0.66 dB. - Impacts of deployment modes This TBS/MCS Table applies only for Stand-alone and Guard-band deployments. The TBS/MCS Table for the in-band deployment is a subset of the TBS/MCS table for guard-band and stand-alone deployments (see section 2.2.2). - Indication of modulation scheme for retransmissions In our view it should be up to eNB implementation to select the transmission format, the UE can anyway get the information explicitly as in the case of a new transmission. - Applicability of repetitions In our view a high modulation order as 16-QAM requires good radio conditions, hence we focused on the case where the NPDCCH and NPDSCH repetition level is equal to 1, which assumption will also plays an important role in the design of the channel quality reporting. - UE data rate The proposed TBS/MCS table makes possible to double the throughput with respect to Rel-16 in two different ways: 1) By transmitting the max TBS in Rel-16 (i.e., 2536 bits) using half of the time-domain resources, and 2) By transmitting a new max TBS of 4968 bits using 10 NPDSCH subframes. |  |

For stand-alone and guard-band deployments, the TBS/MCS Table to support 16-QAM in DL consists of:

- All legacy QPSK entries.

- The entries for 16-QAM are from legacy LTE Table 7.1.7.2.1-1 in TS 36.213 ranging from a TBS = 256 bits to TBS = 4968 bits, with only three changes:

  - To avoid a performance crossing issue replacing legacy TBS = 328 bits by TBS = 296 bits

  - To avoid a performance overlapping issue replacing legacy TBS = 552 bits by TBS = 536 bits, and

  - To transmit the max Rel-16 TBS with half of the time domain resources replacing legacy TBS = 2472 bits by TBS = 2536 bits.

### 2.2.2	In-band deployments

For an “in-band” deployment there are less resource elements available for NB-IoT since some of them are reserved for LTE (e.g., PDCCH, CRS), being ITBS = 10 the maximum usable index in the legacy TBS/MCS table. Due that in an “in-band” deployment the resource elements used by LTE increase the code rates, 16-QAM for “in-band” deployments can be introduced as a subcase of the TBS/MCS table used for stand-alone and guard-band deployments by simply using a different range of ITBS indices as shown in Table 2.

| Table 2: TBS/MCS Table for 16-QAM in DL in the case of “In-band” deployments. |  |
| --- | --- |
| Table 2a: All legacy QPSK TBS entries, 16-QAM TBS entries are from legacy LTE Table 7.1.7.2.1-1 in [3]. The TBS/MCS table for in-band below is a subset of the TBS/MCS table for other deployment modes. Modulation Scheme; Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. QPSK only; 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 16-QAM only; 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 536; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 296; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; -; -; -; -; -; -; -; -. 19; -; -; -; -; -; -; -; -. 20; -; -; -; -; -; -; -; -. 21; -; -; -; -; -; -; -; -. . | Table 1b: Achievable Code Rates of TBS/MCS table for In-band deployments. Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. 0.19; 0.13; 0.13; 0.13; 0.14; 0.14; 0.14; 0.13. 0.23; 0.19; 0.18; 0.2; 0.19; 0.19; 0.17; 0.18. 0.27; 0.23; 0.27; 0.24; 0.22; 0.22; 0.21; 0.22. 0.31; 0.31; 0.32; 0.28; 0.27; 0.28; 0.28; 0.28. 0.38; 0.35; 0.37; 0.34; 0.34; 0.35; 0.35; 0.34. 0.46; 0.4; 0.4; 0.42; 0.43; 0.42; 0.42; 0.43. 0.54; 0.48; 0.45; 0.5; 0.51; 0.5; 0.5; 0.51. 0.62; 0.6; 0.56; 0.6; 0.58; 0.56; 0.6; 0.6. 0.69; 0.67; 0.67; 0.67; 0.68; 0.67; 0.67; 0.66. 0.77; 0.77; 0.77; 0.77; 0.77; 0.77; 0.77; 0.75. 0.81; 0.85; 0.85; 0.85; 0.86; 0.85; 0.85; 0.85. 0.48; 0.48; 0.49; 0.48; 0.49; 0.49; 0.49; 0.49. 0.56; 0.56; 0.56; 0.56; 0.55; 0.55; 0.55; 0.55. 0.6; 0.62; 0.62; 0.63; 0.62; 0.63; 0.62; 0.62. 0.67; 0.67; 0.69; 0.69; 0.69; 0.71; 0.69; 0.69. 0.73; 0.75; 0.74; 0.75; 0.75; 0.73; 0.75; 0.75. 0.77; 0.79; 0.79; 0.79; 0.78; 0.78; 0.79; 0.78. 0.87; 0.87; 0.87; 0.87; 0.88; 0.87; 0.87; 0.88. -; -; -; -; -; -; -; -. -; -; -; -; -; -; -; -. -; -; -; -; -; -; -; -. -; -; -; -; -; -; -; -. |
| Table 1c: Performance of the TBS/MSC table for In-band deployments at 10% BLER |  |
| Table 1d: 3GPP Technical Considerations on the TBS/MCS table design for DL in-band Agreement Further study on TBS/MCS table design, resource assignment and TBS allocation to support 16QAM in DL considering at least: - MCS field size For the proposed TBS/MCS table, 5 bits are required to keep all legacy TBS entries for QPSK and add on top of it entries for 16-QAM. - Achievable code rates The achievable code rates for the TBS entries used for 16-QAM in DL are between 0.48 and 0.88. - Avoidance of link-adaptation issues (i.e., large SINR differences between different entries within one TBS row or between different entries in adjacent TBS rows) - The largest “SINR differences between different entries within one TBS row” are: ITBS11 ⁓ 1.98dB, ITBS12 ⁓ 1.48dB, ITBS13 ⁓ 1.77dB, ITBS14 ⁓ 1.61dB, ITBS15 ⁓ 1.71dB, ITBS16 ⁓ 1.48dB, ITBS17 ⁓ 2.27dB. - The largest SINR differences “between different entries in adjacent TBS rows” are: ITBS12-to-ITBS11 ⁓ 0.5 dB, ITBS13-to-ITBS12 ⁓ 0.29 dB, ITBS14-to-ITBS13 ⁓ 0.16 dB, ITBS15-to-ITBS14 ⁓ 0.10 dB, ITBS16-to-ITBS15 ⁓ 0.23 dB, ITBS17-to-ITBS16 ⁓ 0.79 dB. - The break point between different modulation schemes The average SINR difference at the breaking-point is ITBS11_16QAMavg-to-ITBS10_QPSKavg = abs(8.98 dB – 8.08 dB) ⁓ 0.9 dB. - Impacts of deployment modes This TBS/MCS Table applies only for In-band deployments, which is a subset of the TBS/MCS table for stand-alone and guard-band deployments (see section 2.2.1). - Indication of modulation scheme for retransmissions In our view it should be up to eNB implementation to select the transmission format, the UE can anyway get the information explicitly as in the case of a new transmission. - Applicability of repetitions In our view, a high modulation order as 16-QAM requires good radio conditions, hence we focused on the case where the NPDCCH and NPDSCH repetition level is equal to 1, which assumption will also plays an important role in the design of the channel quality reporting. - UE data rate In legacy, for an in-band deployment ITBS = 10 is the maximum available index, for which 1736 bits is the largest selectable TBS entry. According with the proposed TBS/MCS table it will be possible to go beyond doubling the throughput with respect to Rel-16 in two different ways: 1) By transmitting a TBS = 1800 bits using half of the time-domain resources, and 2) By transmitting a TBS of 3624 bits using 10 NPDSCH subframes. |  |

For in-band deployments, the TBS/MCS Table to support 16-QAM is a sub-case of the TBS/MCS Table used for stand-alone and guard-band deployments.

- The entries for 16-QAM ranges from a TBS = 176 bits to TBS = 3624 as to keep below 0.88 the achievable code rates in in-band deployments.

## 2.3	Power control for 16-QAM in DL: Data to Pilot Power Ratios

The WID’s objective includes the “necessary changes to DL power allocation for NPDSCH and DL TBS”. In [4] it was mentioned that “currently a UE may assume the ratio of NPDSCH EPRE to NRS EPRE is 0 dB for an NB-IoT cell with one NRS antenna port and -3 dB for an NB-IoT cell with two NRS antenna ports. For 16-QAM, the power ratio is required in order to determine the appropriate scaling of the LLR values”. Similarly, in [5] it was mentioned that with the introduction of 16-QAM “the UE needs to have a correct assumption on the relative power between pilots and data”.

In RAN1# 102e the following agreement was reached in relation to the DL power allocation for 16-QAM:

Agreement For DL power allocation, support signaling the ratio of NPDSCH EPRE to NRS EPRE. FFS signaling details, including how/whether to signal the ratio for the following cases - NPDSCH in symbols without NRS and CRS - NPDSCH in symbols with CRS (only for “In-band” deployment) - NPDSCH in symbols with NRS

In our view, to define the data-to-power ratios for 16-QAM in DL, the DL power control definitions in LTE (clause 5.2 in 36.213) should be used as baseline. In line with it, the data-to-pilot power ratios for the support of 16-QAM in DL can be described as follows:

First, the NRS EPRE for anchor and non-anchor carriers are assumed to follow the legacy definitions:

- Anchor Carrier:

NRS EPRE = nrs-power (configured in SIB2-NB, cell specific).

- Carrier specific NRS EPRE:

NRS EPRE = nrs-power + nrs-powerOffsetNonAnchor (configured in Msg4 and SIB22-NB, carrier specific).

- Stand-alone and Guard-band deployments:

  - Type A refers to the NPDSCH symbols without NRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB]

  - Type B refers to the NPDSCH symbols with NRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB]

Where:

ρ_a = PA [dB]

PB is the index that refers to the linear ratio between ρ_b and ρ_a (i.e.,10ρ_b1010ρ_a10)

- In-band deployments:

  - Type A refers to the NPDSCH symbols without NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB]

  - Type B refers to the NPDSCH symbols with NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB]

  - Type C refers to the NPDSCH symbols without NRS and with CRS: NPDSCH EPRE = NRS EPRE + ρ_c [dB]

Where:

ρ_a = PA [dB]

PB is the index that refers to the linear ratio between ρ_b and ρ_a (i.e.,10ρ_b1010ρ_a10)

PC is the index that refers to the linear ratio between ρ_c and ρ_a (i.e.,10ρ_c1010ρ_a10)

The data-to-power ratios for 16-QAM in DL, uses as baseline the DL power control definitions in LTE

- Stand-alone and Guard-band deployments:

- Type A refers to the NPDSCH symbols without NRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB]

- Type B refers to the NPDSCH symbols with NRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB]

- In-band deployments:

- Type A refers to the NPDSCH symbols without NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB]

- Type B refers to the NPDSCH symbols with NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB]

- Type C refers to the NPDSCH symbols without NRS and with CRS: NPDSCH EPRE = NRS EPRE + ρ_c [dB]

- Where:

- ρ_a = PA [dB]

- PB is the index that refers to the linear ratio between ρ_b and ρ_a

- PC is the index that refers to the linear ratio between ρ_c and ρ_a

- FFS: signaling details of ρ_a, ρ_b, ρ_c. or PB, PC.

## 2.4	Channel Quality Reporting to support 16QAM in DL

The downlink (DL) channel quality reporting is defined as the NPDCCH repetition level of hypothetical NPDCCH BLER of 1%.

The reported values for NB-IoT are based on an 8-bit mapping table in TS 36.133 clause 9.1.22.15 [6], including the following NPDCCH repetition levels (1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048).

### 2.4.1	CQI reporting definition

Towards the support of the channel quality reporting for 16-QAM in DL, we can possibly re-use the legacy CQI reporting definition for LTE-MTC in clause 7.2.3 [3], which can be adapted for NB-IoT as shown in the Table below.

Table 3. CQI reporting definition

| Legacy CQI reporting definition as per clause 7.2.3 of [3] | Adaptation of the CQI reporting definition to NB-IoT for the support of 16-QAM in DL |
| --- | --- |
| - A single PDSCH transport block with a combination of modulation scheme and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding 0.1 | - A single NPDSCH transport block with a combination of modulation scheme and transport block size corresponding to the Reported value in Table 9.1.22.15-1 of TS36.133, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding 0.1 |

The CQI reporting definition to support 16-QAM in DL is as in clause 7.2.3 of TS 36.213 for LTE-MTC with the corresponding updates to adapt it to NB-IoT.

### 2.4.2	CQI mapping Table

The CQI mapping table in TS 36.133 clause 9.1.22.15 [6] is used as a baseline to introduce the channel quality reporting for 16-QAM in DL.

Due that a high modulation order as 16-QAM requires good radio conditions, we used as a design criterion the case where “NPDCCH repetition level” is equal to 1.

Moreover, the CQI mapping table in TS 36.133 clause 9.1.22.15 currently uses 13 out of 16 entries, being the three unused fields utilized to incorporate the reporting for 16-QAM in DL:

Table 9.1.22.15-1: Downlink channel quality measurement report mapping of CQI-NPDCCH-NB when the DL channel quality reporting is supported

| Reported value | NPDCCH repetition level | NPDSCH TBS: 16-QAM index with transport block error probability not exceeding 0.1 |  |
| --- | --- | --- | --- |
|  |  | Guard-band and Stand-alone deployments | In-band deployments |
| noMeasurement | No measurement reporting | No measurement reporting |  |
| candidateRep-A | 1 | N/A |  |
| candidateRep-B | 2 | N/A |  |
| candidateRep-C | 4 | N/A |  |
| candidateRep-D | 8 | N/A |  |
| candidateRep-E | 16 | N/A |  |
| candidateRep-F | 32 | N/A |  |
| candidateRep-G | 64 | N/A |  |
| candidateRep-H | 128 | N/A |  |
| candidateRep-I | 256 | N/A |  |
| candidateRep-J | 512 | N/A |  |
| candidateRep-K | 1024 | N/A |  |
| candidateRep-L | 2048 | N/A |  |
| candidateRep-M | 1 | 0 | 0 |
| candidateRep-N | 1 | 1 | 1 |
| candidateRep-O | 1 | 2 | 2 |

Table 3a. NB-IoT 16-QAM CQI index

| CQI Index | ITBS |  |
| --- | --- | --- |
|  | Guard-band and Stand-alone deployments | In-band deployments |
| 0 | A | D |
| 1 | B | E |
| 2 | C | F |

The three new reports (i.e., candidateRep-M, candidateRep-N, and candidateRep-O) proposed to be used for 16-QAM, use as metric TBS indices to reflect the channel conditions. The TBS indices (i.e., ITBS =A, ITBS =B, ITBS =C, ITBS =D, ITBS =E, and ITBS =F) associated to the reports M, N, and O respectively, depend on the selected TBS/MCS table, and the TBS indices are expected to be different between the guard-band/stand-alone deployments and the in-band deployment due to the effective coding rates.

Below, we provide an example that simplifies CQI index table, where the ITBS thresholds are based on the TBS/MCS Tables in sections 2.2.1 and 2.2.2 respectively (i.e., where the TBS/MCS for the in-band deployment is a subcase of the stand-alone and guard-band deployments).

| Reported value | NPDCCH repetition level | NPDSCH TBS: 16-QAM index with transport block error probability not exceeding 0.1 |  |
| --- | --- | --- | --- |
|  |  | Guard-band and Stand-alone deployments | In-band deployments |
| candidateRep-M | 1 | 1 | 0 |
| candidateRep-N | 1 | 2 | 1 |
| candidateRep-O | 1 | 3 | 2 |

Table 3a. NB-IoT 16-QAM CQI index

| CQI Index | ITBS |
| --- | --- |
| 0 | 11 |
| 1 | 14 |
| 2 | 17 |
| 3 | 21 |

The usage of the reserved fields in Table 9.1.22.15-1 to introduce the channel quality reporting for the support of 16-QAM in DL can be illustrated through the following example:

- Let’s assume that the channel quality reporting of 16QAM is based on the measurements that the UE has performed, where the radio conditions are found to be good as to correspond to an NPDCCH repetition level equal to 1. At this point based on the updated Table 9.1.22.15-1 there are four possible candidates:

  - candidateRep-A, candidateRep-M, candidateRep-N, or candidateRep-O

- Let’s further assume that the UE has an implementation dependent algorithm that allows it to determine whether any of the 16-QAM candidate reports are suitable to be used or if rather candidateRep-A (i.e., QPSK) should be reported.

- Let’s assume that among the CQI indices (associated to the thresholds ITBS =A/D, ITBS =B/E, and ITBS =C/F) it was found that the radio conditions are optimal as to correspond to CQI index = 0, hence the UE reports to the eNodeB candidateRep-M, which implies two things:

  - That under the current radio conditions 16-QAM could even handle the largest TBS in TBS/MCS table, and

  - That the repetition level for both NPDCCH (explicit in the table) and NPDSCH (implicit under the assumption 16-QAM is used only for 1 repetition) is equal to 1.

- The eNodeB receives the report (i.e., candidateRep-M in this example) as a recommendation reflecting the radio conditions, but the eNodeB has the final say towards scheduling the upcoming DL transmission.

The three unused entries in the legacy CQI mapping Table in clause 9.1.22.15 of TS 36.213 (i.e., Table 9.1.22.15-1) are used for the CQI reporting of 16-QAM in DL.

- The NPDCCH and NPDSCH repetition level is equal to 1.

  - candidateRep-M is reported when the SINR is suitable for 16-QAM with ITBS =A/D.

    - FFS: A for stand-alone and guard-band deployments, and D for in-band deployments.

  - candidateRep-N is reported when the SINR is suitable for 16-QAM with ITBS =B/E.

    - FFS: B for stand-alone and guard-band deployments, and E for in-band deployments.

  - candidateRep-O is reported when the SINR is suitable for 16-QAM with ITBS =C/F.

    - FFS: C for stand-alone and guard-band deployments, and F for in-band deployments.

# 3	Support for 16-QAM for unicast in UL

## 3.1	TBS/MCS table design, resource assignment and TBS allocation to support 16QAM in UL

For the support of 16-QAM in UL, the Work Item Description (WID) is more restrictive than in DL since it states: “For UL, the maximum TBS is not increased”. In relation with the above, a Cat-NB2 device can support in UL a TBS up to 2536 bits.

The NPUSCH Format 1 transmissions using a 15 kHz subcarrier spacing can be scheduled to use a single-tone (ℼ/2-BPSK, ℼ/4-QPSK) or a multi-tone allocation (QPSK) consisting of either 3, 6, or 12 subcarriers. Moreover, in the time domain a given TBS can be mapped over one or more resource units.

The TS 36.213 states that “the UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2-1 if ,  otherwise” [2]. Moreover, the index IRU is expressed in terms of number of Resource units by using Table 16.5.1.1-2 in [2].

One aspect to consider is that the larger the modulation order, the higher the required SNR. On this matter, although resource allocations smaller than 12 subcarriers are mainly targeted towards low SNR regimes (especially single-tone allocations), still there are scenarios (e.g., NPRACH and NPUSCH allocation coexisting within a PRB) where multi-tone allocations could benefit from higher order modulation.

Thus, it is recommended to support 16-QAM in UL for both full-PRB allocations (i.e., 12 allocated subcarriers) and multi-tone allocations consisting of 6 and 3 allocated subcarriers.

Although 16-QAM requires a high SNR and resource allocations < 12 subcarriers are mainly targeted towards low SNR regimes (especially single-tone allocations), still there are scenarios (e.g., NPRACH and NPUSCH coexistence) where multi-tone allocations could benefit from a higher order modulation.

The support of 16-QAM in UL is only for NPUSCH Format 1 using both full-PRB allocations and multi-tone allocations consisting of 6 and 3 allocated subcarriers.

For introducing 16-QAM in UL, we followed the same design principle we used to introduce 16-QAM in DL, which consists in re-using the TBS entries in the legacy Table 7.1.7.2.1-1 in TS 36.213 [3].

| Table 4: TBS/MCS Table for 16-QAM in UL. |  |
| --- | --- |
| Table 4a: All legacy TBS entries for QPSK, plus 16-QAM TBS entries from legacy LTE Table 7.1.7.2.1-1 in [3] with only 1 change to transmit the max Rel-16 TBs with half of the time domain resources (2536). Modulation Scheme; Number of RUS. 1; 2; 3; 4; 5; 6; 8; 10. QPSK only; 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 16-QAM only; 14; 280; 600; 904; 1224; 1544; 1800; 2472; -. 15; 328; 632; 968; 1288; 1608; 1928; -; -. 16; 336; 696; 1064; 1416; 1800; 2152; -; -. 17; 376; 776; 1160; 1544; 1992; 2344; -; -. 18; 408; 840; 1288; 1736; 2152; -; -; -. 19; 440; 904; 1384; 1864; 2344; -; -; -. 20; 488; 1000; 1480; 1992; 2536; -; -; -. . | Table 4b: Achievable Code Rates of TBS/MCS table for all deployment modes. Number of RUs. 1; 2; 3; 4; 5; 6; 8; 10. 0.14; 0.10; 0.090; 0.10; 0.10; 0.10; 0.10; 0.10. 0.17; 0.14; 0.13; 0.15; 0.14; 0.13; 0.12; 0.13. 0.19; 0.17; 0.19; 0.17; 0.16; 0.16; 0.15; 0.16. 0.22; 0.22; 0.23; 0.20; 0.19; 0.20; 0.20; 0.21. 0.28; 0.25; 0.27; 0.24; 0.24; 0.25; 0.25; 0.24. 0.33; 0.29; 0.29; 0.31; 0.31; 0.31; 0.31; 0.31. 0.39; 0.35; 0.32; 0.36; 0.37; 0.36; 0.36; 0.36. 0.44; 0.43; 0.41; 0.43; 0.42; 0.43; 0.44; 0.43. 0.50; 0.49; 0.48; 0.49; 0.49; 0.48; 0.49; 0.49. 0.56; 0.56; 0.56; 0.56; 0.56; 0.56; 0.56; 0.54. 0.58; 0.61; 0.61; 0.61; 0.62; 0.59; 0.61; 0.61. 0.69; 0.69; 0.70; 0.69; 0.71; 0.70; 0.71; 0.71. 0.81; 0.81; 0.81; 0.89; 0.80; 0.80; 0.79; 0.80. 0.86; 0.89; 0.89; 0.92; 0.89; 0.91; 0.89; 0.89. 0.53; 0.54; 0.54; 0.54; 0.54; 0.53; 0.54; -. 0.61; 0.57; 0.57; 0.57; 0.57; 0.56; -; -. 0.63; 0.63; 0.63; 0.63; 0.63; 0.63; -; -. 0.69; 0.69; 0.69; 0.68; 0.70; 0.69; -; -. 0.75; 0.75; 0.76; 0.76; 0.76; -; -; -. 0.81; 0.81; 0.81; 0.82; 0.82; -; -; -. 0.89; 0.89; 0.87; 0.88; 0.89; -; -; -. |
| Table 4c: Performance of the TBS/MSC table for all deployment modes at 10% BLER |  |
| Table 4d: 3GPP Technical Considerations on the TBS/MCS table design for UL Agreement Further study on TBS/MCS table design, resource assignment and TBS allocation to support 16QAM in UL based at least on the following: - MCS field size For the proposed TBS/MCS table, 5 bits are required to keep all legacy TBS entries for QPSK and add on top of it entries for 16-QAM. - Achievable code rates The achievable code rates for the TBS entries used for 16-QAM in DL are between 0.53 and 0.89. - Avoidance of link-adaptation issues (i.e., large SINR differences between different entries within one TBS row or between different entries in adjacent TBS rows) - The largest “SINR differences between different entries within one TBS row” are: ITBS14 ⁓ 0.41dB, ITBS15 ⁓ 1.4dB, ITBS16 ⁓ 0.65dB, ITBS17 ⁓ 0.75dB, ITBS18 ⁓ 0.55dB, ITBS19 ⁓ 0.34dB, ITBS20 ⁓ 0.89dB. - The largest SINR differences “between different entries in adjacent TBS rows” are: ITBS15-to-ITBS14 ⁓ 0.99 dB, ITBS16-to-ITBS15 ⁓ 0.75 dB, ITBS17-to-ITBS16 ⁓ 0.1 dB, ITBS18-to-ITBS17 ⁓ 0.2 dB, ITBS19-to-ITBS18 ⁓ 0.21 dB, ITBS20-to-ITBS19 ⁓ 0.55 dB. - The break point between different modulation schemes The average SINR difference at the breaking-point is ITBS14_16QAMavg-to-ITBS13_QPSKavg = abs(4.23 dB – 3.69 dB) ⁓ 0.54 dB. - Indication of modulation scheme for retransmissions In our view it should be up to eNB implementation to select the transmission format, the UE can anyway get the information explicitly as in the case of a new transmission. - Applicability of repetitions In our view, a high modulation order as 16-QAM requires good radio conditions, for example in DL towards the design of the channel quality reporting it has been assumed that the NPDCCH repetition level is equal to 1, and that 1 repetition would also be used for NPDSCH. For UL, the same design criteria can be followed using 1 repetition for the 16-QAM transmissions using NPUSCH. - Applicability to different number of subcarriers The proposed TBS/MCS Table is intended to support 16-QAM in UL for NPUSCH Format 1 using both full-PRB allocations and multi-tone allocations consisting of 6 and 3 allocated subcarriers. |  |

The TBS/MCS Table to support 16-QAM in UL consists of:

- All legacy QPSK entries.

- The entries for 16-QAM are from legacy LTE Table 7.1.7.2.1-1 in TS 36.213 ranging from a TBS = 280 bits to TBS = 4968, with only two changes:

  - In the above-mentioned TBS range, the TBS > 2536 are excluded as to be compliant with the objective of preserving for UL the max TBS for Rel-16.

  - To transmit the max Rel-16 TBS with half of the time domain resources replacing TBS = 2472 bits by TBS = 2536 bits.

# Conclusion

In the previous sections we made the following observations for the support of 16-QAM for unicast in UL and DL for NB-IoT:

Observation 1	On the new max TBS to be supported for 16-QAM in DL, “Option 1: 4968 bits with ISF =7” seems to be a better choice as to avoid using a TBS that has not been previously used in the standard, or going beyond twice the max TBS in Rel-16.

Observation 2	Although 16-QAM requires a high SNR and resource allocations < 12 subcarriers are mainly targeted towards low SNR regimes (especially single-tone allocations), still there are scenarios (e.g., NPRACH and NPUSCH coexistence) where multi-tone allocations could benefit from a higher order modulation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The maximum TBS to support 16-QAM for unicast in DL for stand-alone and guard-band deployments is “Option 1: 4968 bits with ISF =7”.

Proposal 2	For stand-alone and guard-band deployments, the TBS/MCS Table to support 16-QAM in DL consists of:

	All legacy QPSK entries.

	The entries for 16-QAM are from legacy LTE Table 7.1.7.2.1-1 in TS 36.213 ranging from a TBS = 256 bits to TBS = 4968 bits, with only three changes:

o	To avoid a performance crossing issue replacing legacy TBS = 328 bits by TBS = 296 bits

o	To avoid a performance overlapping issue replacing legacy TBS = 552 bits by TBS = 536 bits, and

o	To transmit the max Rel-16 TBS with half of the time domain resources replacing legacy TBS = 2472 bits by TBS = 2536 bits.

Proposal 3	For in-band deployments, the TBS/MCS Table to support 16-QAM is a sub-case of the TBS/MCS Table used for stand-alone and guard-band deployments.

	The entries for 16-QAM ranges from a TBS = 176 bits to TBS = 3624 as to keep below 0.88 the achievable code rates in in-band deployments.

Proposal 4	The data-to-power ratios for 16-QAM in DL, uses as baseline the DL power control definitions in LTE

	Stand-alone and Guard-band deployments:

-	Type A refers to the NPDSCH symbols without NRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB]

-	Type B refers to the NPDSCH symbols with NRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB]

	In-band deployments:

-	Type A refers to the NPDSCH symbols without NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB]

-	Type B refers to the NPDSCH symbols with NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB]

-	Type C refers to the NPDSCH symbols without NRS and with CRS: NPDSCH EPRE = NRS EPRE + ρ_c [dB]

Where:

ρ_a = PA [dB]

PB is the index that refers to the linear ratio between ρ_b and ρ_a

PC is the index that refers to the linear ratio between ρ_c and ρ_a

FFS: signaling details of ρ_a, ρ_b, ρ_c. or PB, PC.

Proposal 5	The CQI reporting definition to support 16-QAM in DL is as in clause 7.2.3 of TS 36.213 for LTE-MTC with the corresponding updates to adapt it to NB-IoT.

Proposal 6	The three unused entries in the legacy CQI mapping Table in clause 9.1.22.15 of TS 36.213 (i.e., Table 9.1.22.15-1) are used for the CQI reporting of 16-QAM in DL.

-	The NPDCCH and NPDSCH repetition level is equal to 1.

o	candidateRep-M is reported when the SINR is suitable for 16-QAM with ITBS =A/D.

	FFS: A for stand-alone and guard-band deployments, and D for in-band deployments.

o	candidateRep-N is reported when the SINR is suitable for 16-QAM with ITBS =B/E.

	FFS: B for stand-alone and guard-band deployments, and E for in-band deployments.

o	candidateRep-O is reported when the SINR is suitable for 16-QAM with ITBS =C/F.

	FFS: C for stand-alone and guard-band deployments, and F for in-band deployments.

Proposal 7	The support of 16-QAM in UL is only for NPUSCH Format 1 using both full-PRB allocations and multi-tone allocations consisting of 6 and 3 allocated subcarriers.

Proposal 8	The TBS/MCS Table to support 16-QAM in UL consists of:

-	All legacy QPSK entries.

-	The entries for 16-QAM are from legacy LTE Table 7.1.7.2.1-1 in TS 36.213 ranging from a TBS = 280 bits to TBS = 4968, with only two changes:

o	In the above-mentioned TBS range, the TBS > 2536 are excluded as to be compliant with the objective of preserving for UL the max TBS for Rel-16.

o	To transmit the max Rel-16 TBS with half of the time domain resources replacing TBS = 2472 bits by TBS = 2536 bits.

# 5	References

RP-201306, "WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.

“Chairman’s notes of AI 8.9.1 Additional MTC enhancements for LTE,” RAN1 #102e, August 17th – 28th, 2020.

3GPP TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”, v16.0.0.

R1-2005529, “Support of 16-QAM for NB-IoT,” Nokia, Nokia Shanghai Bell, RAN1 #102e, August 17th – 28th, 2020.

R1-2006192, “Support of 16-QAM for NB-IoT,” Qualcomm Incorporated, RAN1 #102e, August 17th – 28th, 2020.

3GPP TS 36.133, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, v16.6.0.

3GPP TS 36.212, “Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding”, v16.0.0.

# 6	Annex A

## Annex A.1

Figure A.1: For stand-alone and guard-band deployments, the legacy TBS = 552 bits and TBS = 328 bits cause a performance overlapping and crossing issue respectively (See dotted circles).