3GPP TSG-RAN WG1 Meeting #103-e	R1-2008932

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.9.3

Source:	Ericsson

Title:	On the support of a maximum DL TBS of 1736 bits in LTE-MTC

Document for:	Discussion

# 1	Introduction

The Work Item (WI) on “Additional enhancements for NB-IoT and LTE-MTC” has recently added the following objective for LTE-MTC [1]:

- Add a Rel-17 optional UE capability to support a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only. [LTE-MTC] [RAN1, RAN2] - Determine soft buffer size [RAN1] - Capability signaling without introducing a new UE category [RAN2] - There shall be no changes to: DCI formats, TBS tables, CQI tables - This objective begins work from RAN#90, i.e. December 2020

In this contribution we provide an insight on the most recently added Rel-17 objective [1], as to encourage interested companies to provide (if possible) their preliminary views.

# 2	Support of a max DL TBS of 1736 bits

The recently added Rel-17 objective on supporting “a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A” touches upon RAN1 and RAN2 according with the following defined scope:

| - Determine soft buffer size [RAN1] - Capability signaling without introducing a new UE category [RAN2] | - There shall be no changes to: DCI formats, TBS tables, CQI tables - This objective begins work from RAN#90, i.e. December 2020 |
| --- | --- |

In the subsection below we provide a preliminary view on the expected data rate according to [2], and as a function of it we provide an estimate of the required soft buffer size.

## 2.1	Targeted Data Rate

In [2], a set of use cases were discussed to justify the support of a larger DL TBS. The discussed use cases included “watches and wearables”, “High data-rate music”, “Small-format video”, “Firmware download”, and “Point-of-sale transactions”. In [2], the following conclusion can be found:

Conclusion 1: The two relevant use cases to consider regarding necessity of a WID revision are watches/wearables, and small-format video, when delivered by a HD-FDD (half-duplex FDD) Cat. M1 UE.

In RAN# 88e a set of use cases were discussed to justify the support of “a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A”, where 1 Mbps was the peak data rate required for the identified use cases.

The relevant use cases in the conclusion above require “1 Mbps peak rate” [2]. Today, for HD-FDD Cat M1 UEs a peak data rate of 588kbps can be achieved in the physical layer through the combined usage of 10 HARQ processes and the HARQ-ACK bundling. Moreover, when the 3GPP Rel-17 will be completed and 14 HARQ processes can be used along with the HARQ-ACK bundling feature the peak data rate will be increased up to 706kbps.

In Rel-17, when a larger DL TBS will become available, the target peak rate of “1 Mbps” won’t be achievable using the Rel-13 TBS table with single-TB scheduling. This because the Rel-13 TBS table requires 6 PRBs to schedule a TBS of 1736 bits (See Table 7.1.7.2.1-1 in [3]), but the 6 PRBs received by a Cat-M1 UE using single-TB scheduling need to be shared between MPDCCH and PDSCH, which means that PDSCH needs to be scheduled with less than 6 PRBs. Thus, for the Rel-13 TBS table, use of multi-TB scheduling will be needed to reach approximately 1 Mbps as depicted in Table 1.

Table 1: Combined usage of multi-TB using 8 HARQ processes along with the HARQ-ACK bundling for a Cat M1 HD-FDD UE.

| Subframe # | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPDCCH | 0-7 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0-7 |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  |  |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  |  |
| PUCCH (ACK) |  |  |  |  |  |  |  |  |  |  |  | 0 (0-3) | 1 (4-7) |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 |

Based on the scheduling diagram in Table 1, the Rel-13 TBS table with 6PRB and multi-TB using 8 HARQ processes combined with HARQ-ACK bundling would result in the following peak data rate:

- Multi-TB scheduling of 8 TBs with HARQ-ACK bundling and 1736 bits: 992 kbps (i.e., ⁓1 Mbps)

The Rel-13 TBS table with 6PRB and Rel-16 multi-TB scheduling with HARQ-ACK bundling can support ~1 Mbps (992 kbps).

The 1736-bit TBS can be used with single-TB scheduling when the UE is configured with 64QAM. According to Table 7.1.7.1-1 and Table 7.1.7.2.1-1, just 3 or 4 PRBs would be required to schedule a TBS of 1736 bits [3]. Table 2 illustrates the combined usage of 14 HARQ processes and HARQ-ACK bundling without the use of multi-TB scheduling.

Table 2: Combined usage of 14 HARQ processes and HARQ-ACK bundling for a Cat M1 HD-FDD UE.

| Subframe # | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPDCCH | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |  |  |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 12 | 13 |
| PDSCH 7 | 12 | 13 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |  |  |  |  | 10 | 11 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| PUCCH (ACK) |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 (12-1) | 1 (2-5) | 2 (6-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Based on the scheduling diagram in Table 1, the Rel-15 (64QAM) TBS table with 3 or 4 PRBs and single-TB scheduling with 10 or 14 HARQ processes combined with HARQ-ACK bundling would result in the following peak data rates:

- 10 HARQ processes combined with HARQ-ACK bundling and 1736 bits: ⁓1.0212 Mbps

- 14 HARQ processes combined with HARQ-ACK bundling and 1736 bits: ⁓1.2254 Mbps

Regarding the use of multi-TB and the Rel-15 TBS table, multi-TB would still be used along with 8 HARQ processes combined with HARQ-ACK bundling since with multi-TB at most 8 transport blocks can be schedule with a single DCI.

The Rel-15 (64QAM) TBS table with 3 or 4 PRBs and single-TB scheduling with Rel-14 HARQ-ACK bundling can support ~1.02 Mbps with 10 HARQ processes and ~1.23 Mbps with 14 HARQ processes.

The new larger DL TBS of 1736 bits should be usable along with the following combinations:

- Rel-13 TBS table with Rel-16 Multi-TB scheduling with Rel-14 HARQ-ACK bundling and 8 HARQ processes.

- Rel-15 (64QAM) TBS table with 1) single-TB scheduling with Rel-14 HARQ-ACK bundling and 10 or 14 HARQ processes, 2) multi-TB scheduling with Rel-14 HARQ-ACK bundling and 8 HARQ processes.

## 2.2	Determination of the Soft Buffer size

The specified “Total number of soft channel bits” is 25344 regardless of whether 8 or 10 HARQ processes are supported [4]. The reason for this is to allow a Cat-M1 UE designed for 8 processes supported in Rel-13 to be upgraded to support the 10 processes introduced in Rel-14 without a hardware change.

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI (Note 1) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| DL Category M1 | 1000 | 1000 | 25344 | 1 |

If the current soft buffer size is simply scaled up according to the larger TBS, we get the following:

- 1736253441000=43998 soft channel bits.

The above represents a soft buffer size suitable for 8 processes.

If the current soft buffer size designed for 8 processes, upgraded in Rel-14 to support the 10 processes were simply scaled up according to the larger TBS, then 43998 soft channel bits are estimated to be required.

In order to take into account the coding rate of the turbo encoder for higher numbers of processes, the following equation can be used:

- Soft Buffer Size = N323X+24+432						(1)

where N is the number of HARQ processes, 32 ensures that the soft buffer size is a multiple of 32 bits, 3 relates to the coding rate of the turbo encoder, 24 relates to the CRC size, and 4 is related to the tail bits of the turbo encoder. Note: X is the TBS X<=6120.

Using equation (1), the soft buffer size is estimated for the new “maximum DL TBS of 1736 bits” in the case of both 10 and 14 HARQ processes.

- 10 HARQ processes:

Soft Buffer Size = 103231736+24+432 = 53760 soft channel bits.

- 14 HARQ processes:

Soft Buffer Size = 143231736+24+432 = 75264 soft channel bits. Although only 12 out of 14 HARQ processes contribute to the increased peak data rate, all the 14 processes may need to be buffered (e.g., to handle an all-NACK case), so the estimate accounts for the worst-case scenario.

Based on the above estimates, with respect to the “Total number of soft channel bits” supported today which are 25344 soft channel bits, in Rel-17 the soft buffer size is at most expected to be increased about 2.12 times and 2.96 times for 10 and 14 HARQ processes respectively. Recall that depending on whether 10 or 14 HARQ processes are used, the peak data rate can be either ~1.02 Mbps or ~1.23 Mbps.

Using an equation that accounts for the number of HARQ processes, the coding rate of the turbo encoder, etc., the soft buffer size was estimated assuming both 10 and 14 HARQ processes:

    - 10 HARQ processes: In principle, 53760 soft channel bits are estimated to be required.

    - 14 HARQ processes: In principle, 75264 soft channel bits are estimated to be required.

The total number of soft channel bits for Rel-13 Cat-M1 is 25344. In Rel-17, the soft buffer size is at most expected to be increased about 2.12 times and 2.96 times to provide peak data rates of ~1.02 Mbps and ~1.23 Mbps for 10 and 14 HARQ processes respectively.

From UE implementation points of view (e.g. bill-of-material cost and design reusability), it may be desired to adopt a smaller soft buffer size than the one derived theoretically above. From a RAN1 perspective it will be beneficial to gather preliminary thoughts on e.g., what are the soft buffer size options that can be considered, and what aspects should be considered towards performing a suitable analysis for this objective.

From a RAN1 perspective it will beneficial to share preliminary thoughts on e.g., what are the soft buffer size options that can be considered, and what aspects should be considered towards performing a suitable analysis for this objective.

# 3	Conclusion

Based on the discussion in the previous section we made the following observations towards the “Support of a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only”:

Observation 1	In RAN# 88e a set of use cases were discussed to justify the support of “a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A”, where 1 Mbps was the peak data rate required for the identified use cases.

Observation 2	The Rel-13 TBS table with 6PRB and Rel-16 multi-TB scheduling with HARQ-ACK bundling can support ~1 Mbps (992 kbps).

Observation 3	The Rel-15 (64QAM) TBS table with 3 or 4 PRBs and single-TB scheduling with Rel-14 HARQ-ACK bundling can support ~1.02 Mbps with 10 HARQ processes and ~1.23 Mbps with 14 HARQ processes.

Observation 4	The new larger DL TBS of 1736 bits should be usable along with the following combinations:

	Rel-13 TBS table with Rel-16 Multi-TB scheduling with Rel-14 HARQ-ACK bundling and 8 HARQ processes.

	Rel-15 (64QAM) TBS table with 1) single-TB scheduling with Rel-14 HARQ-ACK bundling and 10 or 14 HARQ processes, 2) multi-TB scheduling with Rel-14 HARQ-ACK bundling and 8 HARQ processes.

Observation 5	If the current soft buffer size designed for 8 processes, upgraded in Rel-14 to support the 10 processes were simply scaled up according to the larger TBS, then 43998 soft channel bits are estimated to be required.

Observation 6	Using an equation that accounts for the number of HARQ processes, the coding rate of the turbo encoder, etc., the soft buffer size was estimated assuming both 10 and 14 HARQ processes:

	10 HARQ processes: In principle, 53760 soft channel bits are estimated to be required.

	14 HARQ processes: In principle, 75264 soft channel bits are estimated to be required.

Observation 7	The total number of soft channel bits for Rel-13 Cat-M1 is 25344. In Rel-17, the soft buffer size is at most expected to be increased about 2.12 times and 2.96 times to provide peak data rates of ~1.02 Mbps and ~1.23 Mbps for 10 and 14 HARQ processes respectively.

Observation 8	From a RAN1 perspective it will beneficial to share preliminary thoughts on e.g., what are the soft buffer size options that can be considered, and what aspects should be considered towards performing a suitable analysis for this objective.

# References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.

RP-201288, “Intermediate summary of [R17_NBIOT_eMTC] email discussion”, Huawei, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.

TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures,” v16.2.0.

TS 36.306, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities” v16.1.0.