3GPP TSG RAN WG1 #103-e			R1-2008971

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.11.2.1

Title:	Resource allocation for sidelink power saving

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

The revised Rel-17 sidelink enhancements WID [1] has the following objectives on resource allocation enhancements to optmize UE power consumption:

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

In this contribution we talk about NR-SL power saving techniques. We discuss power saving for transmitting-UEs (TX-UE) and receiving-UEs (RX-UE). For TX-UEs, we propose partial sensing methods and random resource selection based on LTE design with some necessary enhancements. For RX-UE power saving, we discuss sidelink-DRX, sidelink full synchronization search procedure, and sidelink wake-up signal.

# Discussion

The baseline power saving solutions from Rel-14 LTE sidelink help minimize the UE’s workload on the sensing operation at the TX-UE as both monitoring SCI for sensing measurements as well as UE’s sensing computations consume power. Rel-14 LTE sidelink focuses on power saving at TX-UE as the transmission of sidelink BSM messages from the pedestrian VRU UEs to vehicles was an important use case. Rel-17 NR sidelink supports more diverse use cases including VRU, public safety, and some commercial cases (e.g., smart home, smart factory, etc.) where power saving enhancements are needed over Rel-16 NR sidelink design. Power saving solutions should be studied separately from TX-UE’s perspective and RX-UE’s perspective.

Observation 1: Power saving solutions should be studied separately from TX-UE’s perspective and RX-UE’s perspective.

Power consumption at TX-UE is dominated by the sensing operation and the actual transmission of the packets. In regards to sensing operation, 1st SCI monitoring, RSRP measurements, and sensing computations are the main factors that increase power consumption. In regards to power consumption due to packet transmission, the number of blind and HARQ re-transmission has an impact on the consumption. Specifically, performing sensing on a restricted set of slots/resources can lower power consumption. However, such partial sensing may also increase packet collision risks. More collisions will either impact reliability, or more re-transmissions will be needed. Sending more re-transmissions will also cause higher power consumption for TX-UE. Therefore, unnecessary re-transmissions should be avoided by improving transmission reliability and by minimizing packet collisions in sidelink. Therefore, Rel-14 LTE based partial sensing and random resource selection methods should be introduced with necessary enhancements to maintain high transmission reliability.

Power consumption at RX-UE is dominated by SCI monitoring and by the decoding of the received packets. Rel-16 NR sidelink design requires UE to monitor every sidelink slot in a resource pool. Introduction of SL-DRX will help RX-UE enter into short or longer sleep states during which no SCI monitoring may be required. Going into long sleep duration helps minimize power consumption due to SCI monitoring at low traffic. However, such long sleep states may also cause the loss of incoming packets. Such packet loss would cause disruptions to both system reliability and higher sidelink congestion as the lost packets may need to be re-transmitted in the system. Hence, further enhancements should be considered for power saving at RX-UE to improve UE power effiency without impacting sidelink reliability/latency KPIs.

# Power saving for TX-UEs

## Dual-Period Partial Sensing

Partial sensing in LTE V2X is based on monitoring a subset of subframes within the full sensing sliding window, wherein the minimum number of monitored subframes is configured by minNumCandidateSF while the selection of the subframes is up to UE implementation. Then, the sequence of bits indicated in the higher layer parameter gapCandidateSensing determines the candidate resources based on the time position of the subframes where sensing is performed.

Figure 1 LTE partial sensing monitors a subset of subframes for sensing

LTE partial sensing approach works well with periodic sidelink traffic. Since NR sidelink supports both periodic and aperiodic traffic, the resource (re)-selection process of a VRU UE with periodic traffic should perform partial sensing in such a way that both aperiodic and periodic resource reservations by other sidelink users can be detected.

A dual-period partial sensing approach can be considered for NR sidelink (see Figure 2). Similar LTE partial sensing design can be followed to detect periodic traffic based on monitoring a subset of slots within the sensing window periodically. As Rel-16 NR sidelink also supports periodic reservations with {1-99ms} periodicities, the periodicity of the monitored slots (P1) should be configurable. In addition, partial sensing should also be performed in the last 32 slots before resource re-(selection) is triggered as Rel-16 SCI can reserve resources up to 32 slots. See Figure 2.

Figure 2 Dual-period partial sensing

Partial sensing approach increases UE’s power efficiency at the expense of higher collision risks. As more frequent packet collisions may require more HARQ (re)-transmissions, such negative impact on power consumption and system performance can be minimized by supporting Rel-16 re-evaluation and pre-emption methods for UEs operating with partial sensing. If necessary, some simplifications can be considered to Rel-16 re-evaluation and pre-emption UE procedures when partial sensing is used.

We propose the following:

Proposal 1: Support dual-period partial sensing to detect periodic and aperiodic traffic from other sidelink users.

Proposal 2: Re-evaluation and preemption should be supported for UEs operating with partial sensing.

## Random Resource Selection

In Rel-14 LTE random selection, UE selects resources in a resource pool without performing sensing at all. Although this approach severely increases the collision risks, it can be useful in some scenarios. For example, a dedicated resource pool can be pre-configured by the network for a specific service type with low-density low-priority service. Pre-emption and packet priorities can be used to reduce collisions. However, re-evalaution should not be required for UEs operating with random selection.

We propose the following:

Proposal 3: Support random resource selection. UEs using random resource selection are not expected to perform re-evaluation.

# Power saving for RX-UEs

One of the objectives of Rel-17 WI on sidelink enhancements is the introduction of SL-DRX. The motivation for SL-DRX is to make UE fall into the deep sleep by switching off RF frontend for maximized power saving when there is no traffic for a while. However, different than Uu with the synced network and/or the dense period for PSS/SSS transmission, SL has to perform full-time SL sync search for SSB transmitted every 160ms according to Rel-16 specifications due to the potential un-synced syncRefs, e.g., self-promoted syncRef UEs are unlikely to be synced with the SyncRef sources especially in a tunnel or underground scenario. Therefore, SL DRX UE is likely to perform the full-time sync search with no chance to switch off RF completely, i.e., no chance to fall into the deep sleep. The full-time SL sync search with no chance of deep sleep will significantly reduce the power saving gain from SL DRX.

Observation 2: The full-time SL sync search with no chance of deep sleep will significantly reduce the power saving gain from SL DRX.

Proposal 4: The full-time SL sync search should be avoided during SL DRX operation for power saving.

To address such issue, we provide two potential solutions for detailed discussion below:

- SL sync search optimization (see section 4.1)

- Wake-up signal for SL DRX (see section 4.2)

## SL sync search optimization

In TS38.331, the full-time SL sync search requirement is defined as follows:

#### 5.8.6.2 Selection and reselection of synchronisation reference The UE shall: 1> if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList … 1> else if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList … 1> else if the frequency used for NR sidelink communication is included in PreconfigurationNR, … 1> else: 2> perform a full search (i.e. covering all subframes and all possible SLSSIDs) to detect candidate SLSS, in accordance with TS 38.133 [14]

However, for the UE with the syncRef from GNSS/BS or P1/P2/P3/P4 groups UEs (i.e., non-P6 group UEs including self-promoted syncRef UE), the UE can just search the additional SL SSBs with +/- 1 symbol of the current SSB timing. So it implies that UE only need to search at most 2 slots every 160ms SSB period (vs. full-time) because these candidate SyncRef UEs (with the higher priority than current SyncRef) are almost synced with the minor timing difference due to timing error (~12Ts) propagation.

For the UE with syncRef from P6 group UEs including self-promoted UE, they have to perform full-time sync search. In this case, wake-up signal based SL synchronization could be a good choice (see section 4.2).

## Wake-up signal for SL DRX

As stated in R17 WI for sidelink enhancements, introduction of new solutions to reduce power consumption is not precluded. The baseline solutions from LTE (i.e., partial sensing and random resource selection) are useful to reduce power consumption at TX UE as only the transmitting-UE is required to perform sensing in R14 LTE and R16 NR. Power consumption reduction methods in R14 LTE were focused on only TX UE as the transmission of BSM messages by pedestrian VRU UEs were prioritized. However, introduction of additional use cases in R17 NR (including VRU, public safety, commercial cases such as smart home, smart factory, etc.) necessitates power consumption reduction for both TX UE and RX UE. Therefore, new power saving solutions should be considered for RX UE in R17 NR sidelink in addition to SL-DRX.

To further reduce power consumption, the concept of wake-up signal in Uu may be considered in SL. For example, if a TX UE has packet to transmit, the TX UE sends a wake-up signal before the start of SL on duration of the RX UE. If the RX UE receives the wake-up signal, RX UE should monitor PSCCH during the SL on duration. Otherwise, RX UE can skip PSCCH monitoring during SL on duration of this SL DRX cycle.

Especially, as discussed above, for the UE with syncRef from P6 group UEs (especially self-promoted SyncRef UEs), the UE has to perform the full time SL search to find the potential high priority syncRef which may not be synced with the current P6 SyncRef UEs. To address such power consumption, WUS originally used for power saving of DRX_On duration can also be applied as the syncRef. In the other words, UE can utilize the periodic WUS (similar to NR-uu WUS in Rel’16) to perform the SL synchronization.

Although wake-up signal requires additional signalling overhead and configuration, the power saving gain of the Rx UE may be significant when packet arrival rate is low or when on duration is relatively long comparing to SL DRX cycle. For example, consider a power-limited UE with high-reliability/low-latency traffic performing aperiodic packet reception. It is not desirable to configure such a UE with a long DRX cycle and short on duration to maintain reliability/latency KPIs. However, setting a long DRX on duration would also mitigate power efficiency. Use of a wake up signal can help reduce power consumption without impacting service reliability and avoiding full-time SL search especially for P6 group UEs. In view of this, we think wake-up signalling can be useful in the following scenarios:

- Example 1: P6 group UEs to avoid full-time SL sync search in addition to power saving for DRX_On.

- Example 2: For power-limited RX UE - such as the wearable devices or sensors.

- Example 3: For a RX UE with sparse but latency-sensitive traffic

- Example 4: For a TX UE with multiple RX UE

  - If a TX UE has multiple RX UE, the SL on duration of the TX UE may be configured long.

  - Example 4-1: the TX UE is the relay UE, and the RX UEs is the remote UEs

  - Example 4-2: the TX UE is the group lead, and the RX UEs is the group members

In the following we study an analysis on potential power saving gains for RX-UE based on different alternative solutions.

## Analysis of the performance for solutions with SL DRX

We provide an analysis for potential power saving gain based on the following alternative solutions:

- Baseline: without any SL-DRX.

- PS1: SL-DRX is applied with DRX_On=20ms @ Periodicity=100ms

- PS2: DRX + WUS. UE only needs to monitor 1 slot for whether to wake on DRX_ON. SL sync is still based on SSB.

- PS3: DRX + SYNC_DRXOn. SL Sync is limited to +/- 1symbol of the current SSB timing for non-P6 UEs whereas UE has to wakeup for PSCCH monitoring during DRX_On.

- PS4: DRX + SYNC_WUS. SL Sync and wakeup during DRX_On are both based on WUS channel.

Figure 3 Significant power saving gains can be achieved by optimizing SL sync process

In the evaluation, the deep sleep is assumed during DRX_OFF even though it is not reasonable. However, it can be observed that the power saving gain is not much compared to PS3/PS4 because UE still has to monitor SSB all the time which contribute a lot for the power consumption.

Even if WUS is used for power saving during DRX_On but not addressing SL sync issue, the power gain is still limited depending on the DRX_On duration compared to PS3/PS4.

For PS3, the power saving by avoiding the full time SL sync is significant, which is feasible for non-P6 UEs.

For PS4, WUS used for both wakeup purpose and SL synchronization can further reduce the power consumption, especially useful for P6 UEs.

Observation 3: Significant power saving gains can be achieved by optimizing SL synchronization for non-P6-group UEs and by introducing wake-up signaling for P6-group UEs.

Proposal 5: Consider supporting wake-up signaling in sidelink to improve power saving for users including those with syncRef from P6 group UEs.

# Conclusions

We have the following observations:

Observation 1: Power saving solutions should be studied separately from TX-UE’s perspective and RX-UE’s perspective.

Observation 2: The full-time SL sync search with no chance of deep sleep will significantly reduce the power saving gain from SL DRX.

Observation 3: Significant power saving gains can be achieved by optimizing SL synchronization for non-P6-group UEs and by introducing wake-up signaling for P6-group UEs.

We have the following proposals:

Proposal 1: Support dual-period partial sensing to detect periodic and aperiodic traffic from other sidelink users.

Proposal 2: Re-evaluation and preemption should be supported for UEs operating with partial sensing.

Proposal 3: Support random resource selection. UEs using random resource selection are not expected to perform re-evaluation.

Proposal 4: The full-time SL sync search should be avoided during SL DRX operation for power saving.

Proposal 5: Consider supporting wake-up signaling in sidelink to improve power saving for users including those with syncRef from P6 group UEs.

# References

- RP-201516, “WID revision: NR sidelink enhancement”, LG Electronics, September 2020.