3GPP TSG RAN WG1 Meeting #103_e                                                                            R1-2009004

e-Meeting, Oct 26th – Nov 13th, 2020

Agenda Item:		8.13.2

Source: 	Intel Corporation

Title:	On 2-cell scheduling via single DCI

Document for: 	Discussion/Decision

# Introduction

At RAN plenary meeting #86, a work item (WI) for the enhancement of dynamic spectrum sharing (DSS) was agreed [1], the following objective was identified.

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

In RAN1#102_e, the following agreements were made,

Agreements:

- For the study on single DCI scheduling PDSCH on two cells

  - Consider the following scenarios as baseline for evaluation

    - UE configured with Inter-band CA with PCell and an SCell

      - PCell for the UE is operated on a DSS carrier (i.e.,  same carrier is also used for serving LTE users)

      - Case 1: Different SCS for PCell and SCell

      - Case 2: Same SCS for PCell and Scell

  - Additional scenarios can also be evaluated, e.g. as below

    - Intra-band CA case with multiple serving cells having same SCS (all cells operated on non DSS carriers)

    - Inter-band CA case with PCell and more than one SCell (at least the SCells are operated on non DSS carriers)

    - Note: other combinations not precluded

- Note: Further details of evaluation framework (including carrier BW, slot format etc.) to be discussed in next stage

According to the WID, the second objective, i.e. 2-cell scheduling by a single DCI needs to be studies first. Therefore, in this contribution, we provide our investigations on 2-cell scheduling by a single DCI.

# Discussions

In a carrier aggregation scenario, it is generally preferred to configure PCell on a cell in low frequency, which is helpful to improve UL transmission coverage/performance. On the other hand, the low frequency is likely being used by an LTE cell too. Therefore, DSS was introduced in Rel-15 and Rel-16 for the better co-existence between LTE and NR on same frequency. In detail, in a shared carrier of LTE and NR, the NR transmission needs to avoid the REs of LTE CRS and LTE PDCCH. Such DSS operation for NR results in capacity limitation on NR PDCCH transmissions especially for the case when number of NR devices on PCell increases.

The first objective [1], i.e. PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell is one way to mitigate the impact on NR PDCCH on PCell. By this way, the PDCCHs scheduling PCell transmission can be offloaded to a scheduling SCell, which completely avoid the dependence between NR PDCCH and LTE transmissions. The SCell is typically a NR-only cell. More T/F resources on the SCell can be allocated to PDCCH transmission. However, there is still a concern that the capacity on PDCCH may become a problem since more PDCCHs cumulate to the SCell. 2-cell scheduling by a single DCI is then proposed to mitigate PDCCH capacity issue on the SCell. The overhead reduction by 2-cell scheduling by a single DCI is a compromise to the scheduling performance for the two cells. Further, it is also argued that, 2-cell scheduling if introduced, may not be limited to the DSS use case.

NR already support multi-TTI scheduling with multiple TBs on uplink. Within one cell, the adjacent slots are normally corelated which is basis for overhead saving. However, such correlation doesn’t exist in 2-cell scheduling, since the channel conditions on the two cells, even for intra-band case, can be quite different. Therefore, the same scheme handling a field in existing multiple-TTI scheduling may not be applicable to multi-cell scheduling. For the full flexibility of gNB scheduling, all fields of a DCI single-cell scheduling need to be duplicated. In this case, the overhead saving is just the bits of one CRC field, i.e. 24 bis, comparing one DCI for 2-cell scheduling and two DCIs of separated single-cell scheduling. It is possible that some fields in the DCI format can apply to both cells without much impact on the scheduling. On the other hand, overhead reduction of other fields must sacrifice the transmission performance.

In the DSS scenario, PCell and the Scell are likely to be on the different frequency band. Specifically, PCell is on low frequency which co-exists with an LTE cell. On the other hand, the SCell is on a high frequency and is NR-only. Due to the large distance on frequency, channel conditions for PCell and the SCell are independent. Consequently, the proper PRB allocation, transmission schemes and link adaptation are all independent on the two cells in a time. That is, FDRA, MCS/NDI/RV and Antenna ports/TCI need to be duplicated for the two cells. Note: the fields of FDRA, MCS/NDI/RV and Antenna ports/TCI contribute a large percentage of a DCI format. Further, TDRA may be normally different on the two cells. The reason is that there is a limitation on TDRA for cell coexisted with LTE, while there is no such limitation for NR-only cell. The PCell and the SCell may use different subcarrier spacing, which may result in different TDRA too.

Observation 1: To support 2-cell scheduling by a single DCI, at least the following bit fields are likely to be duplicated: FDRA, MCS/NDI/RV and Antenna ports/TCI. TDRA field may be duplicated too.

The size of a joint DCI for 2-cell scheduling can be less than the total size of 2 separate DCIs for single-cell scheduling. However, it is not that straightforward a size reduction turns into PDCCH resource saving. On the other hand, ratio of PDCCH blocking is another important factor for the PDCCH design. Therefore, we evaluate the PDCCH resource saving and ratio of blocking by LLS based on geometry curves obtained by SLS. Detailed simulation parameters for the geometry curve and LLS are provided in Annex.

The geometry SINR CDF for UMi is shown below in Figure 1.

Figure 1: SINR CDF for UMi channel model

The required SINR values for the different DCI sizes with different PDCCH aggregation levels are obtained by LLS and summarized in Table 1.

Table 1: Required SINR for 1% BLER

|  | Required SINR (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | AL 16 | AL 8 | AL 4 | AL 2 | AL 1 |
| 60 bits | -6.8 | -4.45 | -1.56 | 2.48 | 10.5 |
| 72 bits | -6.4 | -4 | -1 | 3.56 | 14 |
| 84 bits | -6 | -3.5 | -0.48 | 4.4 | 24 |
| 96 bits | -5.67 | -3.1 | 0 | 5.1 | - |
| 108 bits | -5.3 | -2.7 | 0.55 | 6.39 | - |
| 120 bits | -4.98 | -2.37 | 1 | 7 | - |

Based on the required SINR values in Table 1, the AL distribution of a DCI size can be derived with the geometry curves. Then, the average number of CCEs for the DCI size can be obtained. In Figure 2, we summarize the potential saving of CCEs for each DCI size for 2-cell scheduling comparing with the use of two DCIs for single-cell scheduling. In the analysis, to schedules transmission on two cells, that gNB transmits one DCI for 2-cell scheduling or two DCIs for single-cell scheduling (baseline).

Figure 2: Ratio of CCE saving using DCI for 2-cell scheduling.

To simulate the impacts on PDCCH blocking ratio, we consider two cases too. It is assumed all UEs are scheduled with transmissions on two cells. In the baseline case, two DCI formats for single-cell scheduling are allocated to a scheduled UE. On the other hand, in case of DCI for 2-cell scheduling, a single DCI is allocated to the UE for the two cells. It is assumed the CORESET has 16 CCEs in total. When multiple UEs are scheduled, the AL for the multiple UEs are randomly determined based on the geometry distribution in Figure 1. If the total number of CCEs exceeds 16, the PDCCH of a UE is dropped randomly until the number of occupied CCEs is not beyond 16. Each dropped PDCCH is counted as one blocking. In the baseline case, if both PDCCHs for a UE cannot be transmitted due to limitation of 16 CCEs, it is counted as two blockings. On the other hand, if only one PDCCH can be transmitted until exceeding 16 CCEs, which is counted as one blocking.

Figure 3: Reduction of PDCCH blocking using DCI for 2-cell scheduling

Observation 2: Based on the required SINR values and geometry curves obtained by LLS and SLS

- The ratio of CCE saving is about 20~40%;

- The reduced PDCCH blocking ratio is observed.

The standardization impacts are expected to be large to specify a DCI for 2-cell scheduling. On the other hand, the allocated TU for the WI is quite limited. Therefore, it needs further study whether/how 2-cell scheduling can be specified. At least the following specification impacts of 2-cell scheduling by single DCI are identified.

- The RRC signalling to configure 2-cell scheduling by single DCI

- Duplication or not needs to be checked for each DCI field separately.

- UE complexity is increased to detect both DCI for singe-cell scheduling and DCI for 2-cell scheduling. The maximum number of monitored PDCCH candidates and non-overlapped CCEs for channel estimation need to be considered.

- HARQ-ACK feedback, i.e. Type1, Type2 and Type3 codebook needs to be revised.

Observation 3: Potential specification impacts include but not limited to

- The RRC configuration

- Separate design for each DCI field

- UE complexity on PDCCH detection.

- HARQ-ACK transmission.

# Conclusion

In this contribution, we provide our views on 2-cell scheduling by a single DCI. we make the following observations.

Observation 1: To support 2-cell scheduling by a single DCI, at least the following bit fields are likely to be duplicated: FDRA, MCS/NDI/RV and Antenna ports/TCI. TDRA field may be duplicated too.

Observation 2: Based on the required SINR values and geometry curves obtained by LLS and SLS

- The ratio of CCE saving is about 20~40%;

- The reduced PDCCH blocking ratio is observed.

Observation 3: Potential specification impacts include but not limited to

- The RRC configuration

- Separate design for each DCI field

- UE complexity on PDCCH detection.

- HARQ-ACK transmission.

# References

[1] RP-193260, New WID on NR Dynamic spectrum sharing (DSS), Ericsson

# ANNEX

Table 1: Parameters for SLS geometry analysis

| Scenario | 5G-UMi-Street Canyon |
| --- | --- |
| ISD | 200m |
| CF | 4GHz |
| BW | 20MHz |
| BS Height | 10m |
| BS Tx Power | 41 dBm |
| UE Distribution | 80% Indoor, 20% Outdoor 10 UEs per TRP (210 total for 21 cells) |
| UE Height | 1.5m (Indoor UEs distributed on multiple floors c.f 38.901) |
| BS Antenna (M,N,P,Mg,Ng) | (1,8,2,1,1) 1x8 column sub-array cross-pol, 2 ports |
| UE Antenna (M,N,P,Mg,Ng) | (1,1,2,1,1) single cross-pol, 2 ports |

Table 2 link level evaluation parameters for PDCCH BLER

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6GHz |
| performance target | 1% BLER |
| Antenna config | 4 by 2 |
| SCS | 15 kHz |
| Channel | TDL-C, 300ns, 3km/hr |
| CORESET size | 2 symbols, 48 PRBs |
| Frequency distributed | Yes |
| Payload | 60, 72, 84, 96, 108, 120 bits |
| Rx combining | MRC |
| Diversity scheme | Precoder cycling every 6 REGs |
| Aggregation level | 1, 2, 4, 8, 16 |