# 3GPP TSG RAN WG1 #103-e							R1-2009113

# e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.9.2

Source: 	Qualcomm Incorporated

Title: 	Support of 14 HARQ processes and scheduling delay

Document for:	Discussion and Decision

# Background

A new work item on additional enhancements for NB-IoT and LTE-MTC was approved in [1]. One of the objectives is to increase the eMTC effective throughput by means of the following technique:

- Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL, for HD-FDD Cat M1 UEs. [LTE-MTC] [RAN1]

We would like to point out that this objective was already discussed during Rel-16 TEI [2,3], but was finally postponed to Rel-17. Our input in this contribution is mainly in line with [3].

In RAN1#102-e, the following was agreed:

Agreement

Introduce a new RRC configuration parameter to enable 14 HARQ processes.

Agreement

For a UE configured with 14 HARQ processes, a PDSCH scheduling delay of 2 BL/CE DL subframes and 7 [FFS subframes type(s)] is supported at least in the PUCCH non-repetition case:

- FFS details of signaling.

- FFS other delay values to account for the presence of non-BL/CE subframes in the PUCCH non-repetition case.

- FFS if the 14 HARQ processes feature is supported in PUCCH repetition case.

Working Assumption

- Introduce a new optional UE capability to support 14 HARQ processes.

- Capability support

One of the remaining issues from RAN1#103 is whether 14 HARQ processes should be an optional UE capability. In our view, this is the only acceptable outcome, since making this feature mandatory would prevent UEs from implementing Rel-17 if this feature is not widely deployed. Thus, we make the following proposal:

Proposal 1: Confirm the following working assumption:

- Introduce a new optional UE capability to support 14 HARQ processes.

- Details of delay indication

In the following we present two different approaches to indicate the scheduling delay for PDSCH: the first approach makes use of unused HARQ process IDs to indicate the scheduling delay (with reduced flexibility), while the second one relies on an additional DCI bit to indicate a scheduling delay for PDSCH.

  - Implementation option 1: No additional bits in DCI

To minimize the necessary changes to enable the above features, a first implementation option is as follows:

- For HARQ processes 0-9, the operation is the same as legacy, i.e., when the DCI indicates a HARQ process ID smaller than 10, no change to PDSCH scheduling delay or HARQ-ACK delay is made.

- When the DCI indicates a HARQ process with an ID greater than or equal to 10, the ‘HARQ ID’ field and ‘HARQ-ACK delay’ field indicate the PDSCH scheduling delay. We exploit the fact that the HARQ-ID field in DCI can indicate maximum 16 HARQ IDs but we only need to use 14 of them to achieve peak data rate as per the proposal above.

- HARQ processes 10-13 have reduced flexibility in terms of HARQ-ACK delay (6 possibilities for HARQ-ACK delay instead of 8). For the cases where retransmissions may be needed for the new HARQ ID, we allow a fallback to legacy default scheduling (N+2) and can still use 6 different scheduling delays.

In Table 1 we show a possible implementation of these methods. The UE, in summary, performs the following operations:

If HARQ_ID < 10

Follow legacy procedures to determine HARQ_ID, HARQ-ACK delay. PDSCH delay = 2

Else

Obtain HARQ_ID and PDSCH delay from ‘HARQ-ACK delay’ field in DCI as per Table 1.

Obtain HARQ-ACK delay from ‘HARQ-ID’ field in DCI as per Table 2.

Table 1 Reinterpretation of ‘HARQ-ACK delay’ field in DCI

| 'HARQ-ACK delay' field in DCI | Used HARQ ID | Scheduling delay for PDSCH |
| --- | --- | --- |
| 000 | 10 | 2 |
| 001 | 10 | 7 |
| 010 | 11 | 2 |
| 011 | 11 | 7 |
| 100 | 12 | 2 |
| 101 | 12 | 7 |
| 110 | 13 | 2 |
| 111 | 13 | 7 |

Table 2 Reinterpretation of ‘HARQ-ID’ field in DCI

| 'HARQ-ID’ field in DCI | HARQ-ACK delay |
| --- | --- |
| 10 | 4 |
| 11 | 5 |
| 12 | 7 |
| 13 | 9 |
| 14 | 11 |
| 15 | 13 |

With these values, a timeline like the one in Figure 4 can be constructed, achieving the desired peak data rate. Note that HARQ processes 0-9 cannot be scheduled with a PDSCH scheduling delay, but HARQ processes 12-13 are scheduled with a delay of 7 (e.g. PDSCH in subframe 0 would be scheduled by MPDCCH in subframe -7).

Figure 4 Example of timeline with proposed scheme.

  - Implementation option 2: One additional bit in DCI

With this second option, the changes are simpler with the cost of 1 additional bit in DCI:

- When 14 HARQ processes are configured, there is an additional bit in DCI that indicates a PDSCH scheduling delay of 2 (legacy) or 7 (new).

- The set of HARQ-ACK delays could be modified when 14 HARQ processes are used as added in the last column of Table 3. Note that this change is not strictly necessary, since the eNB can use “range1” with an alternative pattern (as shown in Figure 5) to achieve the same peak throughput. However, introducing a new pattern gives additional flexibility to the eNB, since with bundling operation the eNB is not going to use delay values of 17 (even 15 is useless and could be removed)

Table 3 Different ranges for HARQ-ACK delay

| 'HARQ-ACK delay' field in DCI | HARQ-ACK delay value when 'ce-SchedulingEnhancement' set to 'range1' | HARQ-ACK delay value when 'ce-SchedulingEnhancement' set to 'range2', or 'ce-SchedulingEnhancement' is not configured and ’ce-HARQ-AckBundling’ is set | HARQ-ACK delay value when ce-pdsch-fourteenProcesses’ is set |
| --- | --- | --- | --- |
| 000 | 4 | 4 | 4 |
| 001 | 5 | 5 | 5 |
| 010 | 7 | 6 | 6 |
| 011 | 9 | 7 | 7 |
| 100 | 11 | 8 | 9 |
| 101 | 13 | 9 | 11 |
| 110 | 15 | 10 | 13 |
| 111 | 17 | 11 | 15 |

Figure 5 Example of timeline without changes to HARQ-ACK delay (using ‘range1’ in Table 3)

Proposal 2: For the indication of PDSCH scheduling delay, downselect among the following options:

- Option 1: Do not introduce a new DCI field, the PDSCH scheduling delay is implicitly determined based on the reinterpretation of some existing field(s) (e.g. HARQ process ID, HARQ-ACK delay).

- Option 2: Introduce a new DCI field explicitly indicating the PDSCH scheduling delay

Proposal 3: For UEs configured with 14 HARQ processes, RAN1 to discuss whether to introduce a new set of HARQ-ACK delays, or whether the sets in Table 7.3.1-2 in TS 36.213 are sufficient.

- Support of invalid subframes and PUCCH repetition

One of the open issues in RAN1#102-e was about the support of invalid subframes and PUCCH repetition. In the following, we present our impact analysis of the presence of invalid uplink/downlink subframes in the proposed scheme:

  - Invalid uplink subframes and PUCCH repetition

During the previous meeting, it was argued that, if some uplink subframes are invalid, then the scheduling delay of 7 would not work, since one of the PUCCH would be postponed. However, we would like to note the following Rel-13 agreement:

RAN1#82bis agreement on valid uplink subframes:

- The set of subframes to be used for uplink transmissions can be explicitly and cell-specifically signalled by the eNB by MTC-SIBx

- If the signaling is not provided, a UE can assume all uplink subframes are valid.

- A UE will postpone transmitting PUSCH/PUCCH repetition (when R > 1) to the next available UL subframe in invalid UL subframe indicated by MTC-SIBx.

  - In case of PUSCH/PUCCH with R = 1, a UL grant or PDSCH transmission overrides the MTC-SIBx configuration, thus a UE will transmit PUSCH/PUCCH

- A UE can drop transmitting PUSCH/PUCCH in subframes unavailable due to UE-specific configuration or behavior such as a measurement gap

  - The subframe unavailable due to UE-specific configuration or behavior is counted in the repetition

which is captured in TS 36.213 as follows:

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layer parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) immediately after subframe n-1, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

Thus, assuming that only single PUCCH repetition is supported, the invalid subframes will not create a postponement of PUCCH.

Observation 1: For the case of single PUCCH repetition, non-BL/CE UL subframes do not create postponement.

For the case of PUCCH repetition, the uplink and downlink delays would need to be adjusted to account for the postponement (and e.g. avoid overlap in PUCCH).

  - Invalid downlink subframes

Current specification TS 36.213 is as follows:

-	subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE DL subframe(s) where, x=k0<k1<…,kN-1 and the value of  is determined by the repetition number field in the corresponding DCI, where are given in Table 7.1.11-1, Table 7.1.11-2 and Table 7.1.11-3, respectively and subframe n+x is the second BL/CE DL subframe after subframe n.

Thus, current scheduling delay is “two BL/CE DL subframes after the end of the MPDCCH”. We assume that the In the following, we analyze the impact of an invalid downlink subframe in different locations, taking as baseline the following Figure (all subframes are valid)

Figure 1 Baseline: all subframes are BL/CE DL subframes

If the invalid subframes are in the middle of the burst (e.g. subframes 0-9), everything would work as expected, with just the corresponding throughput loss of 1 subframe. Since the “N+2” is measured in terms of valid subframes, no change is needed

Figure 2: Subframe 4 is invalid. No change is needed for efficient operation.

If the invalid subframe is at the end of the burst (subframes 10 or 11), we would have the issue that the MPDCCH in subframe 9 would schedule a PDSCH in subframe 12. If we want to support this case, we either have to schedule with “N+7 BL/CE DL subframe” (Alt.1) or move everything to the right by 1 absolute subframe (Alt.2)

Figure 3: Alt.1 For solving an invalid subframe in the “tail” of the burst. Subframe 9 schedules subframe 17 (N+7, with 7 BL/CE DL subframes)

Figure 4: Alt.2 for solving an invalid subframe in the “tail” of the burst. The burst is increased by 1 absolute subframe, which requires the HARQ-ACK delay to be modified.

Yet another case is when subframes in which the UE is transmitting PUCCH are non-BL/CE DL subframes. In that case, if the timeline is N+7 “absolute” subframes, no issue would arise. If the timeline is N+7 “BL/CE DL subframes”, there will be an unnecessary delay of 1ms.

In view of the above (and given the time for this objective), it can be seen that supporting PUCCH repetition and invalid UL/DL subframes would be feasible and would essentially require to adjust (maybe in a configurable way) the set of PDSCH and HARQ-ACK delays. Thus, we make the following proposal:

Proposal 4: RAN1 strive to support the following cases:

- PUCCH repetitions.

- Non-BL/CE DL subframes

- Non-BL/CE UL subframes

To support these cases, the following techniques can be applied:

- Configurable set of HARQ-ACK delays.

- Configurable/dynamic set of PDSCH delays.

- Other changes

In this section we describe other second-order changes needed to enable higher throughput with 14 HARQ processes. This section is based on the TPs in [3].

- When 14 HARQ processes are enabled, the “HARQ process number” field in DCI format 6-1A is 4 bits.

- The PDSCH scheduling delay applies only to PDSCH scheduled from USS. If a new bit is introduced in DCI for indicating PDSCH scheduling delay, it is only present in DCI 6-1A mapped to USS.

- The maximum number of received PDSCH receptions pending HARQ-ACK is set to W = 12 in Sect. 7.3.1 of TS 36.213 when the UE is configured with 14 HARQ processes.

- A UE configured with 14 HARQ processes is also configured with HARQ-ACK bundling.

Proposal 5: Introduce the following specification changes for supporting 14 HARQ processes:

- When 14 HARQ processes are enabled, the “HARQ process number” field in DCI format 6-1A is 4 bits.

- The PDSCH scheduling delay applies only to PDSCH scheduled from USS. If a new bit is introduced in DCI for indicating PDSCH scheduling delay, it is only present in DCI 6-1A mapped to USS.

- The maximum number of received PDSCH receptions pending HARQ-ACK is set to W = 12 (in Sect. 7.3.1 of TS 36.213) when the UE is configured with 14 HARQ processes.

- A UE configured with 14 HARQ processes is also configured with HARQ-ACK bundling.

- Conclusions

In this contribution we presented our views on the necessary changes to enable increased throughput with 14 HARQ processes and PDSCH scheduling delay. We made the following proposals:

Proposal 1: Confirm the following working assumption:

- Introduce a new optional UE capability to support 14 HARQ processes.

Proposal 2: For the indication of PDSCH scheduling delay, downselect among the following options:

- Option 1: Do not introduce a new DCI field, the PDSCH scheduling delay is implicitly determined based on the reinterpretation of some existing field(s) (e.g. HARQ process ID, HARQ-ACK delay).

- Option 2: Introduce a new DCI field explicitly indicating the PDSCH scheduling delay

Proposal 3: For UEs configured with 14 HARQ processes, RAN1 to discuss whether to introduce a new set of HARQ-ACK delays, or whether the sets in Table 7.3.1-2 in TS 36.213 are sufficient.

Proposal 4: RAN1 strive to support the following cases:

- PUCCH repetitions.

- Non-BL/CE DL subframes

- Non-BL/CE UL subframes

To support these cases, the following techniques can be applied:

- Configurable set of HARQ-ACK delays.

- Configurable/dynamic set of PDSCH delays.

Proposal 5: Introduce the following specification changes for supporting 14 HARQ processes:

- When 14 HARQ processes are enabled, the “HARQ process number” field in DCI format 6-1A is 4 bits.

- The PDSCH scheduling delay applies only to PDSCH scheduled from USS. If a new bit is introduced in DCI for indicating PDSCH scheduling delay, it is only present in DCI 6-1A mapped to USS.

- The maximum number of received PDSCH receptions pending HARQ-ACK is set to W = 12 (in Sect. 7.3.1 of TS 36.213) when the UE is configured with 14 HARQ processes.

- A UE configured with 14 HARQ processes is also configured with HARQ-ACK bundling.

# References

[1] RP-201306, WID revision: Additional enhancements for NB-IoT and LTE-MTC

[2] RP-192390, LS on support of high peak data rate for Cat-M1 UEs in HD-FDD

[3] R1-1912694, Increased peak data rate for HD-FDD MTC UEs