3GPP TSG RAN WG1#103-e	R1-2009133

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	UE feedback enhancements for HARQ-ACK

Agenda Item:	8.3.1.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide discussions on UE feedback enhancements for HARQ-ACK with respect to the following agreed objective in [1]:

1. Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering  UE feedback enhancements for HARQ-ACK [RAN1]

Additionally, following agreements on HARQ-ACK enhancements were achieved in RAN1 102e meeting [2]:

Agreements: Support Rel-17 enhancements to avoid SPS HARQ-ACK dropping for TDD due to PUCCH collision with at least one DL or flexible symbol. - This topic is to be considered as high priority - FFS detailed solution(s) Agreements: Study further at least the following schemes: - SPS HARQ skipping for ‘skipped’ SPS PDSCH - PUCCH repetition enhancements (at least for HARQ-ACK), e.g., sub-slot based, etc. - Retransmission of cancelled HARQ - SPS HARQ payload size reduction and / or skipping for ‘non-skipped’SPS PDSCH - Type 1 HARQ codebook based on sub-slot PUCCH config - PUCCH carrier switching for HARQ feedback Companies are encouraged to provide detailed analysis and comparison accordingly

# Discussions on SPS HARQ-ACK enhancements

To avoid SPS PDSCH HARQ-ACK dropping for TDD, deferring HARQ-ACK until the first available valid PUCCH resource could be a direct and simple way.

Proposal 1:

- To avoid SPS PDSCH HARQ-ACK dropping for TDD, the SPS HARQ-ACK is allowed to be transmitted in a later PUCCH by deferring HARQ-ACK until the first available valid PUCCH resource.

In current NR, UE should report HARQ-ACK feedback for each configured SPS PDSCH occasion, even though there is no actual SPS PDSCH transmission. However, due to the sporadic nature of URLLC, it could be a waste of feedback resource, especially when the SPS is configured with shorter periodicities to meet the low latency requirement. Thus, HARQ-ACK transmission should be skipped if SPS PDSCH is not transmitted by the gNB.

Proposal 2:

- SPS HARQ-ACK skipping for ‘skipped’ SPS PDSCH is supported in Rel-17.

A ‘skipped SPS PDSCH’ can be identified based on DM-RS presence detection.

# Discussions on HARQ-ACK reliability enhancements

Sub-slot PUCCH repetition is an effective way to enhance the PUCCH reliability and the low latency requirement could be also met. For short PUCCH, repetitions within and across sub-slots can be supported. For sub-lot based long PUCCH, sub-slot to sub-slot is sufficient.

Proposal 3:

- ‘Sub-slot’ type of PUCCH repetition is supported in Rel-17.

  - For short PUCCH, repetitions within and across sub-slots can be supported.

  - For sub-lot based long PUCCH, PUCCH repetition from sub-slot to sub-slot is supported.

# Discussions on retransmission of cancelled HARQ

It has been agreed to further study retransmission of cancelled HARQ in RAN1 #103-e. We consider Type-3 HARQ-ACK codebook as a potential solution. Although Type-3 HARQ-ACK codebook was introduced as a new type of codebook during Rel-16 specification for NR-U, we observed that exploiting Type-3 HARQ-ACK codebook is beneficial to URLLC operation, especially for HARQ retransmission, due to the following advantages:

- Can be dynamically triggered by DCI as required

- One-shot feedback for all configured HARQ processes

- Specification on Type-3 HARQ-ACK codebook generation has been completed in Rel-16 NR-U

To adopt Type-3 HARQ-ACK codebook in URLLC operation, one remaining issue is when Type-3 HARQ-ACK codebook meets prioritization of HARQ-ACK codebook. More specifically, the problem is that it is not specified how to handle Type-3 HARQ-ACK codebook with mixed priorities. To solve this problem, there might be several alternatives:

- Alt. 1: Associate Type-3 HARQ-ACK codebook with one priority index

  - 1a: The priority index is predetermined, e.g., priority index 1

  - 1b: The priority index is RRC configured

  - 1c: The priority index is indicated by the Priority field in the triggering DCI format

- Alt. 2: Generate two Type-3 HARQ-ACK codebooks, each corresponding to a priority index

Among the alternatives, we are preferable to Alt. 1c because 1) the flexibility by dynamically indication, 2) integration of Type-3 codebook triggering and prioritization within one DCI format, 3) relatively small specification impact, e.g., no introduction of RRC parameter or modification of pseudo-code. However, at the beginning phase of Rel-17 specification, we suggest to firstly clarify the necessity of adopting Type-3 HARQ-ACK codebook in URLLC operation.

Proposal 4:

- As a potential solution for retransmission of cancelled HARQ, support Type-3 HARQ-ACK codebook corresponding to mixed priorities.

# Conclusion

In this contribution, we provide the following proposals:

Proposal 1:

- To avoid SPS PDSCH HARQ-ACK dropping for TDD, the SPS HARQ-ACK is allowed to be transmitted in a later PUCCH by deferring HARQ-ACK until the first available valid PUCCH resource.

Proposal 2:

- SPS HARQ-ACK skipping for ‘skipped’ SPS PDSCH is supported in Rel-17.

Proposal 3:

- ‘Sub-slot’ type of PUCCH repetition is supported in Rel-17.

  - For short PUCCH, repetitions within and across sub-slots can be supported.

  - For sub-lot based long PUCCH, PUCCH repetition from sub-slot to sub-slot is supported.

Proposal 4:

- As a potential solution for retransmission of cancelled HARQ, support Type-3 HARQ-ACK codebook corresponding to mixed priorities.

# References

  - 3GPP WID, RP-201310

  - Chairman’s notes of 3GPP TSG RAN WG1 #102-e