3GPP TSG RAN WG1 Meeting #103-e	R	1-2009137

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	Power Control: Subsequent Contribution

Agenda Item:	8.10.2

Document for:	Discussion and Decision

# Introduction and study of previous developments related to power control

As we mentioned in our contribution for RAN1 #102-e, we considered the relevant bits from the new IAB WID [1] read as follows, with emphasis highlighted:

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

And take a look at this:

Agreement

- Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 2 (simultaneous MT-Rx/DU-Rx)

- Case 6 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 1 (simultaneous MT-Tx/DU-Tx)

  - RAN1 should strive to minimize specification impact due to this feature

- FFS: Whether Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 4 (simultaneous MT-Tx/DU-Rx)

Agreement

- Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 2 (simultaneous MT-Rx/DU-Rx)

- Case 6 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 1 (simultaneous MT-Tx/DU-Tx)

  - RAN1 should strive to minimize specification impact due to this feature

- FFS: Whether Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 4 (simultaneous MT-Tx/DU-Rx)

That’s the sum total of agreements reached in RAN1 #102-e regarding other enhancements for simultaneous operation of IAB-node’s child and parent links (including power control.) [2]

However, there was also an e-mail summary and Feature Lead’s analysis, [3].  Take a look at this excerpt from that Tdoc:

FL Conclusion 5.1:

The following table summarizes the applicability of power control to the multiplexing scenarios under consideration:

|  |  | Power control may be helpful for mitigating: |
| --- | --- | --- |
| Multiplexing scenario | Case 1: Simultaneous MT-Tx/DU-Tx | Tx power imbalance |
|  | Case 2: Simultaneous MT-Rx/DU-Rx | Rx power imbalance |
|  | Case 3: Simultaneous MT-Rx/DU-Tx | Self-interference |
|  | Case 4: Simultaneous MT-Tx/DU-Rx | Self-interference |

Although as we mentioned in [4], we still believe that power control might also be useful and relatively easy on specification effort to manage CLI, however, we certainly don’t disagree with the potential usefulness of mitigating the above imbalances and self-interference.

Following this section is a review of our proposals from 102-e, extended with consideration of the aforementioned texts.

# Exposition

## General Principles

First, let us note that that this topic can become quickly complex to the point of unwieldiness if the scope of the enhancements is not limited to that with minimal specification and signaling impact. To elaborate, Figure 1depicts the IAB network as it has appeared in so many places.  Naturally, a network of IAB nodes might, in its simplest form consist of a tree network, although in general a directed acyclic graph may be employed as the IAB topology. However, as we’ll describe, the lily can be gilded quite rapidly:

Some points to consider are:

- In general, because resources are shared between access and backhaul; so especially for FDD deployments power control will be necessary in general to minimize unwanted interference between backhaul and access.

- Because of cross-link-interference, it is clear that not only parent – child links interfering with their associated RANs might be affected but also those of parental siblings of a child link as well as a child’s siblings.  And that’s where it can start to get de trop, as one can extend this ad nauseum. That is to say, as in any work item, it’s important to minimize specification effort, and naturally, complexity.

- Of course, we already have power control developed for the access network, but in general the link conditions of the backhaul link will be different from the access network.

Thus, we have, for points 1 and 2:

Proposal 1:

The scope of the power control enhancements should be limited.  In the WID, the limitation is mentioned between parent-child links, and so the control should take place between these links, but also CLI measurements should be limited to minimize signaling.

As to points 1 and 3, as we all know, there exists uplink power control signaling defined for NR, e.g., as defined in section 7 of [5]; for example, the PUSCH power control update is given as:

where a few paragraphs following is devoted to the definition of the parameters in said equation, as well as said rules for how the parameters are defined, configured, etc.

Similar approaches apply to the other uplink channels as well as rules for prioritization of transmitted power in the event of approaching the maximum transmitted power limits or when the power headroom limit is approached.

Now, clearly this sort of framework is able to be re-used for IAB.  However, as might be expected for other uplink channels, parameter values would vary according to the link needs of IAB.

On the other hand, there ought to be requirements for explicit downlink (DU transmit) power control from parent to child, signaled either arising from measurements taken at the child’s receiver, or, perhaps, under direction of the parent node’s parent and/or the donor DU/CU.

Thus, we have:

Proposal 2:

Specification of power control for IAB nodes should appropriate as much as is feasible from the existing power control framework of NR.

Figure 1 The well known diagram of an IAB network

## Adapting the Methodology of Usage of Time/Frequency Resources to Power

One other point bears being pointed out: How is power to be allocated to a node? In the access network paradigm, of course, power is allocated to a UE as per power control commands sent from a gNB. Now, there’s lots of details involved, but there are scenarios without a half duplex constraint, in which transmission from a DU and transmission from an MT may take place simultaneously (assuming that both, e.g., are FDD.)  Under circumstances dependent on RF parameters of the transmitters, the total power of both the MT and DU need to be controlled.  Cases can be made that they should be controlled individually or as a totality.  In particular, there can exist circumstances in which the MT may allocate some of its transmit power capacity to the DU. Hence we have

Proposal 3:

The scope of the power control enhancements should consider MT/DU power transmit requirements and capabilities dependent on the RF parameters involved, duplex capability, etc.  We do not see this as in conflict with FL proposal 5.1 from RAN1 #102-e.

## FL Proposal 5.1b: Sharp’s view

The Feature Lead’s proposals, not treated in the meeting, considered the scope of power control. Take a look at this:

FL Proposal 5.1b:

The following power control mechanisms for a given IAB-node should be considered:

- DL power control with assistance information from child MT for Rx imbalance mitigation at child node

- DL power control with assistance information from parent DU for Tx imbalance mitigation at IAB-node

- UL power control with assistance information from child MT for Tx imbalance mitigation at child node

- UL power control with assistance information from parent DU for Rx imbalance mitigation at IAB-node

Any DL power control mechanism should take into account:

- existing base station design principles related to transmission power.

- network constraints in regard to transmitted reference signals.

FL Proposal 5.1b:

The following power control mechanisms for a given IAB-node should be considered:

- DL power control with assistance information from child MT for Rx imbalance mitigation at child node

- DL power control with assistance information from parent DU for Tx imbalance mitigation at IAB-node

- UL power control with assistance information from child MT for Tx imbalance mitigation at child node

- UL power control with assistance information from parent DU for Rx imbalance mitigation at IAB-node

Any DL power control mechanism should take into account:

- existing base station design principles related to transmission power.

- network constraints in regard to transmitted reference signals.

Sharp’s view in this area as that the mechanisms for DL assistance from child or parent links can be useful, and that this information can include, but is not be limited to configured total power allocations for IAB-nodes. In addition, said assistance information may re-use mechanisms from LTE (e)ICIC, reference signals, and so forth.

Thus we have:

Proposal 4:

FL Proposal 5.1b can be agreed with the understanding that “assistance information” can include, but is not be limited to configured total power allocations for IAB-nodes. In addition, said assistance information may re-use mechanisms from LTE (e)ICIC, reference signals, and so forth.

# In Conclusion

So the proposals are:

Proposal 1:

The scope of the power control enhancements should be limited.  In the WID, the limitation is mentioned between parent-child links, and so the control should take place between these links, but also CLI measurements should be limited to minimize signaling.

Proposal 2:

Specification of power control for IAB nodes should appropriate as much as is feasible from the existing power control framework of NR.

Proposal 3:

The scope of the power control enhancements should consider MT/DU power transmit requirements and capabilities dependent on the RF parameters involved, duplex capability, etc. We do not see this as in conflict with FL proposal 5.1 from RAN1 #102-e.

Proposal 4:

FL Proposal 5.1b can be agreed with the understanding that “assistance information” can include, but is not be limited to configured total power allocations for IAB-nodes. In addition, said assistance information may re-use mechanisms from LTE (e)ICIC, reference signals, and so forth.

# References

- RP-19325, New WID on Enhancements to Integrated Access and Backhaul, RAN #86

- Draft minutes of RAN1 #102-e.

- R1-2007352	Summary#3 of [102-e-NR-eIAB-02], Qualcomm (moderator)

- R1-2006581, Power Control: It’s an Enhancement, and Here’s Our Views, Sharp

- TS38.213 v15.8.0

At least since Release 15, to our knowledge, there aren’t the equivalents of the OLI and HII that are in LTE, although they could be designed, presumably by cribbing from LTE.  CLI mechanisms might draw on those areas, but they would have to be designed and extended for IAB.