3GPP TSG RAN WG1 #103-e			R1-2009166

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.12.2 Mechanisms to improve reliability for RRC_CONNECTED UEs

Title:	On reliability enhancement for NR multicast and broadcast

Source:	Convida Wireless

Document for:	Discussion

# Introduction

At RAN #86, a work item on supporting of multicast and broadcast services was approved [1] which was further updated at RAN #88-e as [2]. The objectives of the WI include:

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

In the RAN1 #102-e meeting, the following agreements were reached on reliability enhancement [3]:

Agreements:

For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

In this contribution, we provide our views on the potential reliability enhancement to support the NR multicast and broadcast.

# Discussion

In RAN1 #102-e meeting, it has been agreed that HARQ feedback is supported for NR multicast, where the details on the HARQ feedback scheme are FFS. In rel-16 V2X, two HARQ feedback schemes, i.e., ACK-NACK based HARQ feedback and NACK only based HARQ feedback, were agreed for groupcast case. However, both schemes have pros and cons.

For the ACK-NACK based HARQ feedback scheme, each UE will have dedicated resource to send HARQ feedback. So, the gNB can know which UEs send ACK and which UEs send NACK. However, such benefit either comes with the price of low scheduling flexibility, e.g., as what has been agreed for NR V2X, the association between the PSSCH and PSFCH is fixed and the gNB can’t schedule the PSFCH resources for each UE freely; or comes with the price of high signaling overhead, e.g., if the gNB wants to flexibly schedule the resources for each UE to send feedback, it will require very large amount of bits in the DCI. Since each UE will be allocated with dedicated resource, the resources needed to send the feedback is much higher comparing to the NACK only based scheme. Also, the ACK-NACK based scheme doesn’t work well when the gNB doesn’t know how many UEs are within the group.

For the NACK only based HARQ feedback scheme, shared resource is allocated for all the UEs to send NACK. By doing this, the number of required resources is much lower and the signaling overhead is also very lower than ACK-NACK based scheme. However, the gNB can’t know which UEs send the NACK. A more severe problem is if the UE misses the scheduling DCI or SCI, the UE will not send any feedback. From the transmitter side, since it doesn’t receive any feedback, it will assume that the transmission succeeds. As a result, there will be misunderstanding between the transmitter and the receive which will cause problem to the system.

Considering the pros and cons of the above two schemes, other HARQ feedback schemes may be also considered for NR groupcast. For example, one candidate may be shared ACK-NACK based HARQ feedback. In the shared ACK-NACK based scheme, two resources are allocated to the UEs, one resource is shared by all the UEs to send ACK and the other resource is shared by all the UEs to send NACK. Using this scheme, it can solve the misunderstanding issue occurred in NACK only based case while without significantly increasing the required signaling overhead and the resource.

Proposal 1: Different HARQ feedback schemes should be considered for NR multicast for the UEs in RRC_CONNECTED state. Shared ACK-NACK based scheme can be considered as another alternative of the NACK only based scheme.

In some NR broadcast use cases, e.g., public safety, maintaining high reliability is also very important. Supporting HARQ feedback for NR broadcast to improve the reliability may be considered, where such procedure may be based on best effort.

Proposal 2: Supporting HARQ feedback for NR broadcast service to improve the reliability can be considered for the UEs in RRC_CONNECTED state.

In NR multicast and broadcast, different services with different reliability requirements may be provided. When the reliability requirement of a service is low, HARQ feedback may not need to be supported. Therefore, gNB should have the flexibility to enable and disable the HARQ feedback for different services. Therefore, HARQ feedback enabling and disabling should be supported. Both static enabling/disabling and dynamic enabling/disabling should be considered.

Proposal 3: HARQ feedback enabling and disabling should be supported in NR multicast and broadcast.

# Conclusion

In this contribution, we have the following proposals

Proposal 1: Different HARQ feedback schemes should be considered for NR multicast for the UEs in RRC_CONNECTED state. Shared ACK-NACK based scheme can be considered as another alternative of the NACK only based scheme.

Proposal 2: Supporting HARQ feedback for NR broadcast service to improve the reliability can be considered for the UEs in RRC_CONNECTED state.

Proposal 3: HARQ feedback enabling and disabling should be supported in NR multicast and broadcast.

# References

- RP-193248, “New Work Item on NR support of Multicast and Broadcast Services”, Huawei, RAN #86, Spain, December 9th – 12th, 2019

- RP-201038, “WID revision: NR Multicast and Broadcast Services”, Huawei, RAN #88-e, E-meeting, June 29 - July 3, 2020

- Chairman Notes, RAN1 #102-e, August 17th – 28th, 2020