3GPP TSG RAN WG1 #103-e		                                    		   R1-2009170

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.8.1.2

Source:	InterDigital Inc.

Title:	FR2 baseline coverage performance using LLS

Document for:	Discussion and Decision

# Introduction

In RAN1#102e, simulation assumptions for FR2 are agreed [1]. In this contribution, baseline techniques for both downlink and uplink channels are evaluated. The bottleneck channel analysis is conducted using relative MCL and MIL.

# FR2 baseline coverage performance

## Link level simulation results for PUSCH eMBB

In this section, iBLER performance for eMBB is shown to find the optimum number of DMRS symbols in the simulation. Evaluation assumptions are shown in Appendix. Correlation between antennas are varied to study the effect of inter-Rx antenna correlation. In addition, 1 and 2 DMRS antennas are implemented in the simulation. Performance of PUSCH eMBB are investigated in this simulation. BLER and throughput performance for indoor scenario are shown in Figure 1 and Figure 2, respectively. It should be noted the results are also presented in [2]. From the simulation results, it is clear that the best BLER performance, while losing a slight throughput performance, can be obtained using 2 DMRS symbols.

Figure 1 BLER results, Indoor, FR2, TDD

Figure 2 Throughput performance, indoor FR2, TDD

## Relative MCL/MIL/MPL analysis for identification of bottleneck channel for Indoor scenario

In this section, we summarize the MCL, MPL and MIL obtained using the template agreed in the discussion [3]. As shown in the previous subsection, 2 DMRS symbols are inserted in PUSCH. Correlation between RX antennas is assumed to be low. MCL and MIL are obtained using the SNR value, obtained from the simulations, required to achieve the target performance. The simulation parameters used in the evaluation are summarized in Appendix. The link budget parameters [3] are shown in Table 5. For computation of the metrics, antenna gain correction factor ((4b) and (11b) in the template) of 0 dB is assumed. The performance of PDSCH can be improved by using repetition, HARQ, frequency hopping or distributing the transport block over multiple slots to obtain diversity gain. Performances of control channels are superior compared to PUSCH or PDSCH, due to their compact bandwidth. From the relative analysis of MCL and MIL among uplink channels, PUSCH eMBB is identified as the bottleneck channel among uplink channels.

Table 1 MCL/MIL analysis for FR2 TDD 28GHz Indoor, 3km/hr

|  | MCL | MIL |
| --- | --- | --- |
| PUSCH eMBB (5Mbps) | 104.18 | 134.26 |
| PUCCH PF1 | 124.99 | 155.06 |
| PUCCH PF3 | 120.39 | 150.47 |
| PDCCH | 104.88 | 131.94 |
| PDSCH (25 Mbps) | 101.93 | 123.99 |

Observation 1 : From relative MCL and MIL analysis, the bottleneck uplink channel for Indoor scenario is identified as PUSCH eMBB.

Based on the above proposal, the following proposal is made.

Proposal 1: For Release 17 NR coverage enhancement, study enhancement for PUSCH for FR2 for Indoor scenario

# Conclusion.

In this contribution, the following observation and proposal are made.

Observation 1 : From relative MCL and MIL analysis, the bottleneck uplink channel for Indoor scenario is identified as PUSCH eMBB.

Proposal 1: For Release 17 NR coverage enhancement, study enhancement for PUSCH for FR2 for Indoor scenario

Reference

[1] RAN1 Chairman’s notes, RAN1#102e, Aug., 2020.

[2] R1-2006244, “FR2 baseline coverage performance using LLS,” RAN1#102e, InterDigital, Aug. 2020.

[3] 3GPP, “[102-e-Post-NR-CovEnh-02] Email discussion/approval of Phase 1 - link budget template,” Softbank, 3GPP RAN1 email discussion, Aug. 2020.

- Appendix

Table 2 TDD simulation parameters for eMBB PUSCH/PDSCH, FR2

| Simulation Assumptions (to maximize MCL) | eMBB Values |
| --- | --- |
| Duplexing mode | TDD |
| Carrier Frequency (GHz) | 28 |
| Subcarrier Spacing (kHz) | 120 |
| Tx Bandwidth Configuration (MHz) | 100 for UL, 100 for DL |
| Waveform | W CP DFT-s-OFDM, W CP OFDM for DL |
| CP type | Normal |
| PHY channel | PUSCH/PDSCH |
| Allocation (# of PRB) | 30 for UL, 59 for DL |
| Allocation (# of OFDM symbols) | 14 for UL, 12 for DL |
| DM-RS Type | Configuration Type 1 |
| DM-RS (# of OFDM symbols) | Type B for UL, Type A for DL, 2 DM-RS symbols per slot |
| MCS index, table | #5, Table 6.1.4.1-1 for UL, #11, Table 5.1.3.1-3 for DL , TS 38.214 |
| Repetition/HARQ scheme | No repetition, no HARQ |
| Hopping | Disabled |
| Antenna Configuration | UL : 1 (UE) x 2 (gNB), 2 TXRU at gNB, (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) for indoor gNB, DL: 2 x 2 for with precoder cycling |
| PA impairment | ideal |
| Channel model (TDL type, DS) | TDL-A, DS=30ns for indoor |
| Speed | 3 km/hr |
| Receiver: CHEST | Realistic |
| TX pattern per frame | DDDSU |
| Data rate | 5 Mbps for indoor UL, 25 Mbps for indoor DL, iBLER 10% |

Table 3 PDCCH parameters

| - Parameters | - Values |
| --- | --- |
| - Payload | - 40 bits + CRC |
| - Aggregation level | - 16 |
| - CORESET | - 2 symbols x 48 PRBs |
| - Precoder | - Precoder cycling, CCE level REG bundle = 6 |

Table 4 PUCCH parameters

| - Parameters | - Values for PUCCH Format 1 | - Values for PUCCH Format 3 |
| --- | --- | --- |
| - Payload | - 2 bits | - 22 bits |
| - PUCCH symbols | - 14 | - 14 |
| - # of RB | - 1 | - 1 |
| - Frequency hopping | - Disabled |  |
| - Repetition | - Disabled |  |
| - Performance | - ACK missed detection probability : 1%, NACK to ACK probability : 0.1%, DTX to ACK probability : 1% | - BLER 1% |

Table 5 Link budget parameters for MCL/MIL/MPL computation

| - Parameters | - Values (700MHz, FDD) | - Values (4GHz, TDD, rural) |
| --- | --- | --- |
| - Antenna gain correction factor (4b), (11b) | - 0 dB |  |
| - Total transmit power uplink/downlink (3) | - 23 dBm |  |
| - Cable, connector, body loss (8)/(12) | - 1 dB (UE) / 3dB (BS) |  |
| - Gain of antenna elements - (11c) for uplink, (4c) for downlink | - 8 dB |  |
| - Receiver noise figure (13), uplink/downlink | - 7 dB / 10dB |  |
| - Thermal noise density (14) | - -174 dBm/Hz |  |
| - Receiver interference density (15) | - -165.7 dBm/Hz PUSCH, - -161.7dBm/Hz PUCCH, - -169.3 dBm/Hz PDCCH, PDSCH |  |
| - Receiver implementation margin (20) | - 2 dB |  |