3GPP TSG RAN WG1 #103-e	R1-2009180

e-Meeting, Oct. 26th – Nov. 13th, 2020

Source:	NTT DOCOMO, INC

Title:	Discussion on CSI enhancements

Agenda Item:	8.1.4

Document for: 	Discussion and Decision

# Introduction

In RAN1#102-e meeting, following agreements were made regarding CSI enhancements for Type II port selection codebook and multi-TRP operation [1].

Agreement Taking Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) as a starting point, study following aspects, taking into account trade-off among UE complexity, performance and reporting/RS overhead: - Enhancement on codebook structure, e.g., - (Alt 1) Enhancement based on R16 Type II PS CB type structure - Enhancements on W1 quantization, e.g., - With enhanced port selection in W1 - With modified value range of L taking into account beamforming mechanism for CSI-RS; - With layer-specific port selection - Enhancements on Wf quantization, e.g., - With a smaller value of Mv - With a modified value range of R - With multiple values of Mv for different SD basis - With enhanced FD basis selection in Wf - Restrictions/Relaxation, e.g. - for the size of the PMI indicators for SD basis, FD basis and bitmap. - How UE distinguishes SD basis and FD basis or in a pre-defined set - Enhancement on W2 quantization: coefficients for selected ports - (Alt 2) Enhancement based on R15 Type II PS CB type structure - Enhancement on W1 quantization, e.g., enhanced port selection, X out of P SD-FD pairs are selected - XP (if polarization independent) or P/2 (if polarization common) whereas P PCSI-RS only or P can be larger than PCSI-RS - How to map P SD-FD pairs into PCSI-RS CSI-RS ports and inform to UE - Enhancement on W2 quantization: coefficients for the selected X pairs - etc. - Enhancements on indication/reporting mechanism, e.g.: - Separate triggering for reporting of W1 and Wf (for Alt 1) or reporting of W1 and the rest of the PMI components (for Alt 2) - Report only a subset of PMI components - Enhancement on SD/FD basis indication, selection and reporting mechanism - UE reporting to support gNB calibration including UL/DL time difference; - CQI enhancements, e.g., CQI reporting mechanism considering FDD reciprocity - etc. - Enhancements on RS triggering/signaling/transmission mechanism, e.g. for SRS and/or CSI-RS, CSI-RS utilization conveying one or more SD-FD pairs per port, timing restrictions between SRS and CSI-RS transmission, etc - Other enhancement are not excluded Agreement For CSI enhancement for multi-TRP, study following aspects taking into account trade-off among UE complexity, performance and reporting/RS overhead - Category 1 - For a reporting setting CSI-ReportConfig, more than one CSI-RS port groups in a resource or resources or resource sets are associated to different TRPs/TCI states, - the UE will determine CSI reporting quantities based on pre-defined/indicated/configured/UE-selected channel and interference hypotheses across TRPs /TCI states - and then report one or more CSIs within a single CSI report. - Category 2 – Within an implicit/explicit set of reporting settings CSI-ReportConfigs, which are associated to different TRPs/TCI states, - the UE will determine CSI reporting quantities based on pre-defined/indicated/configured/ UE-selected channel and interference hypotheses - and then report multiple CSIs with multiple CSI reports (including one or more CSIs per report or selected CSI with single CSI report) - Other enhancement are not excluded, e.g. CQI enhancements for multi-TRP transmission including CQI format, CQI reporting mechanism Note that companies are encouraged to clarify applicable transmission schemes/scenarios and strive to unify Rel-17 MTRP CSI framework enhancements

In this contribution, we discuss possible CSI enhancements for both DL multi-TRP/panel transmission and Type II port selection codebook focusing on aforementioned agreements from RAN1#102-e meeting.

# CSI enhancement for MTRP

To support joint transmission from multiple TRPs/panels, suitable Tx-Rx beam pair from each TRP/panel should be selected for high frequency. If beam management and CSI measurement is performed for each TRP/panel individually, the selected beams and precoders from each TRP/panel do not consider inter-TRP/panel/beam interference, which may limit the performance of NCJT. In our companion contribution [3], SLS performance of different transmission schemes with different CSI feedback methods were provided, and it can be observed that multiple CSI feedback considering inter-beam/inter-panel/inter-TRP interference measurement can increase both cell average and cell edge performance in multi-panel/TRP scenario. For Rel.17 MTRP CSI framework enhancements, it is important to have a unified framework for both beam measurement/reporting and CSI measurement/reporting, as well as for different transmission schemes. The enhancements on beam measurement/reporting for simultaneous multi-TRP transmission with multi-panel reception are discussed in our companion contribution [4]. In this section, we mainly discuss the enhancement on CSI measurement and reporting in multiple TRPs/panels with consideration of unified signaling framework.

## Discussion on MTRP CSI framework enhancements

In RAN1#102-e meeting, two categories for CSI enhancement were agreed for further study. In Category 1, only one CSI reporting setting CSI-ReportConfig is needed. Separate CMRs for two TRPs can be configured in a CSI-ReportConfig, as shown in Fig. 2-1, which is also similar as CSI framework enhancement for group-based beam reporting in our companion contribution [4]. To support NCJT transmission, multiple CSIs from multiple TRPs should be reported in a single CSI report. For those multiple CSIs in a CSI report, inter-TRP/panel interference should be considered, e.g., by assuming the CMR of one TRP as the NZP-IMR of the other TRP. UE can select two CSIs from two TRPs/panels with good channel quality as well as less inter-TRP/panel interference as a CSI pair for reporting. To make scheduling at gNB more flexible, UE can be configured to report multiple CSI pairs in a CSI report for combinations of different beams from two TRPs/panels for NCJT. The number of CSI pairs in a CSI report can be RRC configured and up to X. In addition, for each CSI pair in a CSI report, the total rank of two CSIs should be limited to not exceed UE capability. In that case, each CSI pair in a CSI report can be used for NCJT scheduling directly at gNB without further update of the CSIs.

Figure 2-1: An example of Category 1

Figure 2-2: An example of Category 2

In Category 2, two CSI reporting settings CSI-ReportConfig are required, as shown in Fig. 2-2. It is similar as CSI enhancement for non-group-based beam reporting in our companion contribution [4]. By explicit/implicit association configuration of a CSI-ReportConfig for one TRP with the other CSI-ReportConfig for the other TRP, UE can also report the CSI for one TRP by considering the CMR of the reported CSI for the other TRP as NZP-IMR and vice versa. Compared to Category 1, Category 2 requires two CSI-ReportConfig with some duplicated configuration signaling. In addition, Category 1 is more aligned with the CSI framework enhancement for group-based beam measurement/reporting. Hence, Category 1 is preferred.

Proposal 1:

- Support Category 1 - For a reporting setting CSI-ReportConfig, more than one CSI-RS port groups in a resource or resources or resource sets are associated to different TRPs/TCI states,

  - UE can report two CSIs as a CSI pair within a single CSI report by default. The number of CSI pairs to be reported within a single CSI report can be RRC configured and up to X.

## Discussion on CSI enhancements for different transmission schemes

In NR Rel-16, multiple MTRP transmission schemes are supported, such as multi-DCI based MTRP transmission, single-DCI based MTRP transmission, SDMed MTRP repetition, etc. In NR Rel-17, some new MTRP transmission scheme may be further introduced, e.g., SFN. CSI enhancements should consider different MTRP transmission schemes.

For different MTRP transmission schemes, two beams for two TRPs are needed. Hence, based on Category 1, the number of CRIs to be reported per CSI pair is two for MTRP case. However, the required number of reported RI/LI/PMI/CQI per CSI pair for different MTRP transmission schemes may be different, which can be determined and configured by gNB. For example, for multi-DCI based MTRP transmission, 2 RIs, 2 LIs, 2 PMIs, and 2 CQIs can be reported per CSI pair. For single-DCI based MTRP, gNB can configure UE to report 2 RIs, 2 PMIs, and 1 or 2 CQIs, where the number of CQI is related to whether a TB or two TBs are to be scheduled for the UE. For SDMed 1a repetition scheme for URLLC, 1 RI, 2 PMIs, and 1 CQI can be reported per pair. For SFN network, 1 RI, 1 PMI, 1 CQI can be reported per CSI pair. In this case, UE should perform measurement on two CRIs from two TRPs simultaneously when deriving 1 RI/PMI/CQI for the CSI pair.

Proposal 2:

- For a CSI-ReportConfig based on Category 1 for MTRP transmission schemes,

  - Support UE to report two CRIs per CSI pair within a single CSI report,

  - Support RRC to configure the number of RI, LI, PMI, and/or CQI to be reported per CSI pair within a single CSI report.

To facilitate dynamic scheduling between single-TRP transmission and multi-TRP transmission, multiple types of CSIs need to be reported, including fallback CSI for single-TRP transmission and CSI pairs for MTRP transmission. Separate CSI-ReportConfig can be configured for different types of CSIs. However, it may require three reporting settings with large configuration overhead. To save the number of reporting setting and configuration overhead, for a CSI-ReportConfig, UE can be configured to report both single-TRP CSI and MTRP CSI in a single CSI report.

Proposal 3:

- For a CSI-ReportConfig based on Category 1 for MTRP transmission schemes,

  - UE can be configured to report both single-TRP CSI and MTRP CSI in a single CSI report.

# Type II port selection codebook enhancement

In WID for MIMO enhancements in NR Rel-17, it was proposed to study enhancements on Type II port selection (PS) codebook by considering information related to angle(s) and delay(s) estimated at the gNB based on sounding reference signal (SRS) transmission [2]. Further, in RAN1#102-e meeting, some study aspects regarding enhancements on Type II PS codebook structure and indication/reporting mechanism were agreed [1]. In this section, we discuss in detail, possible Type II PS codebook enhancements complying with aforementioned agreements.

## Overview of Rel-16 Type II PS codebook

Type II PS codebook does not require UE to derive spatial domain (SD) beams considering 2D-DFT basis as in regular Type II codebook. Instead, gNB transmits K beamformed CSI-RS ports considering set of SD beams. Out of those K beamformed CSI-RS ports, UE has to identify L (≤K) best CSI-RS ports (associated with different beams) and report their indices as part of precoding matrix indicator (PMI). Let us look at sub-band (SB)-wise precoder generation for layer l∈1, 2, 3, 4 considering Rel. 16 Type II PS codebook structure. In particular, this can be given as,

WlNt×N3=QW1WlWf,lH  (1)

where Wl (Nt×N3) captures precoding vectors for N3 SBs in l-th layer. Here Nt represents number of configured ports. Note that, Q Nt×K in (1) captures K SD beams with beamformed CSI-RS. For instance, Q can consist of K 2D-DFT vectors and hence can be given as, Q=[b1 b2 ⋯bK] where bi,i∈1, 2, ⋯K is the i-th 2D-DFT vector. W1 (Nt×2L) in (1) is a block diagonal matrix where each matrix block consisting of L columns from an K×K identity matrix. Further, Wf,l (N3×M) and Wl (2L×M) capture M DFT basis vectors for frequency domain (FD) compression and non-zero linear combination (LC) coefficients, respectively. Note here that, SD beams within matrix Q are selected transparent to the UE. These SD beams can be determined, for example, considering UL sounding reference signals (SRS) transmission. It is worth remarking here that, due to the FDD transmission along with antenna arrays, the UL dominant sub-space is not the same as that of DL. However, port selection in the DL allows UE to approximately select ports covering the DL dominant sub-space (detailed discussion is provided in section 3.3).

Observation 1:

- Due to the FDD transmission using antenna arrays, the UL dominant sub-space is not the same as that of DL. However, port selection in the DL allows UE to approximately select SD beams which can cover the DL dominant sub-space.

Next, let us look at a general codebook structure for Type II PS codebook considering angle(s) and delay(s) reciprocity.

## Type II PS codebook considering beamformed CSI-RS transmission in angle and delay

As defined in MIMO WID [1], further enhancements on Type II PS codebook needs to consider angle and delay reciprocity. Hence, a general structure for Type II PS codebook specifically considering angle(s) and delay(s) reciprocity can be formulated and given as,

WlNt×N3=QW1WlSWf,lH  (2)

where, S N3×K' captures K' FD bases for CSI-RS beamforming. For instance, S can consist of K' DFT vectors and hence can be given as, S=[f1 f2 ⋯fK'] where fj, j∈1, 2, ⋯K' is the j-th DFT vector. Note that, FD bases in S should represent delays associated with the propagation channel. Further, Wf,l K'×M in (2) is now a matrix consisting of columns of an K'×K' identity matrix.

It is worth remarking here that, the gNB now transmits K×K' beamformed CSI-RS ports both in angle and delay domains. However, based on the way UE is configured to select and report PMI associated with SD and FD bases (whether ‘joint’ or ‘separate’ selection of SD and FD bases), the general codebook structure defined in (2) can be flexibly used for DL precoder generation.

Next, we discuss in detail three different cases where UE can be configured to report selected SD and FD bases and how (2) can be flexibly utilized for generating DL precoders for each of those cases.

Case 1: Each SD beam is paired with a single FD basis; K=L, K'=M, K=K'

Figure 3-1: Each SD beam bi, i∈1, 2, ⋯K is paired with a single FD basis fj, j∈1, 2, ⋯K'. Here, K=K'=6

In case 1, each SD beam is paired with a single FD basis. This is succinctly captured in the example provided in Fig. 3-1. In that, K=6 SD beams are paired uniquely with K'=6 FD bases. Note that, since K=L, and K'=M, SD-FD selection is not allowed in this case. As a result, diagonal blocks in W1 in (2) are K×K identity matrices. Wf,l in (2) also becomes a block diagonal matrix with each block consisting of an K×K identity matrices. This can be given as,

W1=IK00IK Wf,l=IK00IK

where, IK is an (K×K) identity matrix.

gNB: Transmits K CSI-RS ports each beamformed with a unique SD-FD pair. An example of SD-FD bases pairing for CSI-RS beamforming is given in Fig. 3-1 and can be given as, b1,f1,b2,f2, ⋯{bK,fK}

UE: Reports 2K LC coefficients corresponding to the diagonal elements of the matrix Wl

Case 2: Each SD beam is paired with a single FD basis; K≥L, K'≥M, K=K' and L=M

Compared to case 1, in case 2, UE can select L SD-FD pairs out of K beamformed CSI-RS ports since K≥L, K'≥M. In particular, this case represents the ‘joint’ selection of SD and FD bases.

Note that, the diagonal blocks of W1 in (2) are K×L matrices with each matrix block consisting of L columns (corresponding to selected CSI-RS ports) from an K×K identity matrix. Further, based on W1, Wf,l in (2) can be determined since each SD beam is paired with a single FD basis. Hence, reporting only W1 or Wf,l is sufficient here. Wf,l is also a block diagonal matrix with each matrix block consisting of L columns. Further, Wl in (2) is a 2L×2L diagonal matrix for case 2.

gNB: Transmits K CSI-RS ports each beamformed with a unique SD-FD pair. An example of SD-FD bases pairing can be given as, b1,f1,b2,f2, ⋯{bK,fK}

UE:

- Identifies, based on some criteria, L ports out of K and reports to gNB as part of PMI. These L ports correspond to L columns of W1. As discussed, Wf,l can be determined directly using reported W1

  - Further, reports, 2L LC coefficients which are the diagonal elements of the matrix Wl

Case 3: Each SD beam is paired with multiple FD bases; K≥L, K'≥M

Figure 3-2: Each SD beam bi, i∈1, 2, ⋯K is paired with all K' FD bases fj, j∈1, 2, ⋯K'. Here, K=6,  K'=5

As captured in Fig. 3-2, in this case, each SD beam is paired with all K' FD bases. Hence, total number of beamformed CSI-RS ports are KK'. Note that, compared to cases 1 and 2, in case 3, UE has the freedom to ‘separately’ select L SD beams and M FD bases, for each SD beam.

Note that, for this case, the diagonal blocks of W1 in (2) are K×L matrices where each matrix block consisting of L columns (corresponding to selected ports) from an K×K identity matrix. There are two approaches to select M FD bases corresponding to each SD beam. They are,

  - Beam common: same set of M FD bases are selected for all SD beams

  - Beam specific: M FD bases are SD beam specific

For beam common FD bases selection, Wf,l in (2) is K'×M matrix where M columns (corresponding to selected FD bases) are from K'×K' identity matrix. Further, Wl in (2) is a 2L×M matrix consisting of 2LM LC coefficients.

For beam specific FD bases selection, Wf,l in (2) is a K'×2LM matrix where 2LM columns (corresponding to selected FD bases) are from K'×K' identity matrix. In particular, Wf,l can be succinctly represented as,

Wf,l=E1⋯E2L (3)

where, EiK'×M,i∈1, 2, ⋯2L is matrix with columns from K'×K' identity matrix and captures selected M FD bases for i-th SD beam. Further, Wl in (2) is a 2L×2LM block-diagonal matrix which can be represented as,

Wl=c1⋯0⋮⋱⋮0⋯c2L

where, cl1×M, l∈1, 2, ⋯2L is a row vector with LC coefficients of l-th SD beam. An example of aforementioned two FD bases selection approaches are captured in Fig. 3-3 pertaining to the example in Fig. 3-2.

Figure 3-3: For each SD beam bi, i∈1, 2, ⋯6, M=2 FD bases out of {f1 f2 ⋯f5} are selected. Here, (a) beam common, (b) beam specific, FD bases selection

gNB: Transmits KK' CSI-RS ports where each SD beam is paired with all K' FD bases for beamformed CSI-RS. An example of SD-FD bases pairing for CSI-RS beamforming can be given as, bi,fj where i∈1, 2, ⋯K and j∈1, 2, ⋯K'.

UE:

  - Identifies, based on some criteria, L ports out of K SD beams and reports to gNB as part of PMI. These L ports correspond to L columns of W1

  - For beam common FD bases selection: identifies, based on some criteria, M ports out of K' FD bases which are common for all SD beams and reports to gNB as part of PMI. These M ports correspond to M columns of Wf,l

  - For beam specific FD bases selection: identifies, based on some criteria, M ports out of K' FD bases for each SD beam separately and reports to gNB as part of PMI. Selected M ports for i-th SD beam corresponds to the M columns in Ei of (3). Within Wf,l all Ei,i∈1, 2, ⋯2L are collected

  - Further, reports 2LM LC coefficients of the matrix Wl

Based on these discussions, we make the following proposal regarding Type II PS codebook structure.

Proposal 4:

- Consider following Type II PS codebook structure for NR Rel-17:

WlNt×N3=QW1WlSWf,lH

where, Q Nt×K and S N3×K' capture K SD beams and K' FD bases, respectively. Using this codebook structure, UE can be configured for either ‘joint’ or ‘separate’ selection of SD and FD bases.

Next, we look at how to select SD and FD bases for CSI-RS beamforming in the DL considering angles and delays estimated in the UL.

## Determining SD and FD bases for CSI-RS beamforming

In the previous section, we discussed a general structure for Type II PS codebook when CSI-RS is beamformed both in angle and delay domains. In this section, we discuss how to determine SD and FD bases for CSI-RS beamforming considering estimated angles and delays in the UL.

3.3.1 Determination of SD bases for CSI-RS beamforming

Let us first consider SD beam determination for CSI-RS beamforming. Note that, since our focus is on FDD systems and the angles are estimated in the UL using antenna arrays, the effective antenna-spacing is different in two bands (due to same physical spacing but different wavelengths). As a result, when angles are sampled through the phased array, set of angles sampled in UL are different than in DL. However, as succinctly discussed in [5], if beamforming is done in the DL considering dominant signal sub-space of the UL, the UL-DL frequency mismatch results in an attenuation factor.

For understanding this further, let us consider an example as shown in Fig. 3-4. In that, black dots represent different multi-path components (MPCs) of the user channel, each associated with a different angle and a delay. Note that, we can safely assume here that the reciprocity of angles and delays associated with each MPC in the propagation channel is independent of the frequency. Now, assuming that the UL spatial covariance, RUL is available, as shown in Fig. 3-4, there are 3 eigenmodes, {u1 u2 u3} within the dominant UL sub-space. Further, as captured, the dominant UL sub-space covers most of the MPCs.

Figure 3-4: Dominant UL sub-space captures most of the energy by covering large number of MPCs. Here, black dots represent MPCs in angle-delay domains

However, as mentioned previously, due to the difference in effective antenna-spacing in two bands (due to the same physical spacing but different wavelengths), it is not proper to consider the dominant UL sub-space for CSI-RS beamforming in the DL [5].

Under this situation, one way to determine SD beams for CSI-RS beamforming is by selecting set of SD beams which can entirely cover at least the UL dominant sub-space. Further, it is important to let UE to select DL SD beams since UE has the idea of DL dominant sub-space. We have provided an example in Fig. 3-5. In that, pertaining to the example discussed in Fig. 3-4, the selected 2D-DFT beams for CSI-RS beamforming cover the estimated UL dominant sub-space entirely. Once the beamformed CSI-RS is transmitted, UE can select appropriate SD beams and report them to the gNB as part of PMI.

Figure 3-5: SD beam determination for CSI-RS beamforming. Here, the selected 2D-DFT beams entirely cover the dominant UL sub-space

Proposal 5:

- Since the UL dominant sub-space is different than that of DL when considering phased arrays (with fixed inter-element spacing) for transmission, allow UE to pick L SD beams out of the K (≥L) beamformed CSI-RS ports.

Next, we discuss a possible approach for determining FD bases for CSI-RS beamforming.

3.3.2 Determination of FD bases for CSI-RS beamforming

In this section we discuss how to identify delays for determining FD bases for CSI-RS beamforming. In particular, the SD and FD beamforming vectors are captured in Q and S matrices, respectively, in the proposed Type II PS codebook structure in (2). For understanding this further, we can consider the example captured in Fig. 3-4 again. In particular, we focus on a selected SD beam bi as captured in Fig. 3-6.

Assuming that the delays of MPCs are available, as can be seen in Fig. 3-6, SD beam bi captures MPCs with delays τ1 and τ2. Under such situation, a delay τi for beam bi can be calculated considering weighted interpolation of τ1 and τ2 as τi=w1τ1+w2τ2. Subsequently, associated FD basis, fi for delay τi can be identified and pair that with SD beam bi for CSI-Rs beamforming.

Figure 3-6: SD beam bi is selected for CSI-RS beamforming. Note here that, SD beam bi includes MPCs with delays τ2, and τ1

Another, possible approach would be to separately pair SD beam bi with FD bases corresponding to delays τ1 and τ2 and let UE to select most appropriate FD basis. In particular, we think it is important to let UE to select FD bases as well since that can provide higher flexibility for the UE to select one or more appropriate FD bases specifically by looking at the selected SD beams as discussed in the section 3.3.1.

Proposal 6:

- Allow UE to select FD bases as well in order to provide higher flexibility and performance. FD bases selection can be either SD beam common or SD beam specific.

It is worth remarking here that, as discussed under section 3.3.1, it is necessary to first make sure selected SD beams (in SD-FD pairs) for CSI-RS beamforming can entirely cover the UL dominant sub-space. Subsequently, FD bases can be determined for those selected SD beams for CSI-RS beamforming.

# Summary

In this contribution, we discussed possible CSI enhancements for DL multi-TRP/panel and Type II PS codebook. Based on the discussion, we have made following observations and proposals.

Proposal 1:

- Support Category 1 - For a reporting setting CSI-ReportConfig, more than one CSI-RS port groups in a resource or resources or resource sets are associated to different TRPs/TCI states,

  - UE can report two CSIs as a CSI pair within a single CSI report by default. The number of CSI pairs to be reported within a single CSI report can be RRC configured and up to X.

Proposal 2:

- For a CSI-ReportConfig based on Category 1 for MTRP transmission schemes,

  - Support UE to report two CRIs per CSI pair within a single CSI report,

Support RRC to configure the number of RI, LI, PMI, and/or CQI to be reported per CSI pair within a single CSI report.

Proposal 3:

- For a CSI-ReportConfig based on Category 1 for MTRP transmission schemes,

  - UE can be configured to report both single-TRP CSI and MTRP CSI in a single CSI report.

Observation 1:

- Due to the FDD transmission using antenna arrays, the UL dominant sub-space is not the same as that of DL. However, port selection in the DL allows UE to approximately select SD beams which can cover the DL dominant sub-space.

Proposal 4:

- Consider following Type II PS codebook structure for NR Rel-17:

WlNt×N3=QW1WlSWf,lH

where, Q Nt×K and S N3×K' capture K SD beams and K' FD bases, respectively. Using this codebook structure, UE can be configured for either ‘joint’ or ‘separate’ selection of SD and FD bases.

.

Proposal 5:

- Since the UL dominant sub-space is different than that of DL when considering phased arrays (with fixed inter-element spacing) for transmission, allow UE to pick L SD beams out of the K (≥L) beamformed CSI-RS ports.

Proposal 6:

- Allow UE to select FD bases as well in order to provide higher flexibility and performance. FD bases selection can be either SD beam common or SD beam specific.

# References

- 3GPP RAN1#102-e, “RAN1 Chairman’s Notes”, Aug. 2020

- 3GPP RP-193133, “New WID: Further enhancements on MIMO for NR”, RAN#86, Dec. 2019

- 3GPP R1-2006724, “Discussion on CSI enhancements”, RAN#102-e, Aug. 2020

- 3GPP R1-2009177, “Discussion on beam management for MTRP”, RAN1#103-e, Oct. 2020

- S. Haghighatshoar, M. B. Khalilsarai and G. Caire, "Multi-Band Covariance Interpolation with Applications in Massive MIMO," 2018 IEEE Int. Symp. on Info. Theory (ISIT), Vail, CO, 2018, pp. 386-390, Aug. 2018