3GPP TSG RAN WG1 #103-e   	 	  	                 R1-2009234

e-Meeting, October 26th – November 13th, 2020

Agenda item: 7.2.6

Source: Moderator (LG Electronics)

Title: Final summary of email thread [103-e-NR-eMIMO-01]

Document for: Discussion and Decision

- Introduction

This contribution summaries issues to be discussed in email thread [103-e-NR-eMIMO-01], which are selected from [1] as listed below.

- MB.1(H): Specifying TCI state codepoint mapping for DCI format 1_2

- MB.3(E): Aligning RRC parameter names with TS38.331

- MB.4(H2): Adopting the agreed TP for TS38.213

Among above issues, E-rated and H2-rated issues can be endorsed without discussion unless pointed out otherwise.

- Discussion on H-rated issues

  - Specifying TCI state codepoint mapping for DCI format 1_2 (MB.1 in [1])

Reason for change:

In the response LS to RAN2 in R1-2007197, RAN1 confirmed that RAN1 will specify the mapping for TCI codepoints of DCI format 1_2 as captured below:

RAN1 would like to provide replies on the following questions from RAN2 in R2-2005811: Question 1: Could RAN1 specify how the TCI state codepoints are mapped when the number of codepoints in the TCI field of DCI format 1_2 is less than the maximum number of codepoints in the TCI field of DCI format 1_1 taking into account that the MAC CE may signal TCI states corresponding to maximum amount of DCI codepoints.? Answer to Q1: RAN1 will specify the mapping for TCI codepoints of DCI format 1_2, of which details are under discussion.

Proposals from tdocs:

For this issue, Ericsson and Samsung provided TPs for specifying codepoint mapping in their contributions, meanwhile Qualcomm proposed introducing a restriction, which is that the number of codepoints that can be indicated by the bits of the TCI field of DCI format 1_2 should be no less than the number of TCI codepoints activated by the MAC-CE.

TP from Ericsson for clause 5.1.5 of TS 38.214 [5]

### 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP or in a set of CCs/DL BWPs, respectively. When a set of TCI state IDs are activated for a set of CCs/DL BWPs, where the applicable list of CCs is determined by indicated CC in the activation command, the same set of TCI state IDs are applied for all DL BWPs in the indicated CCs. When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command. When the DCI field 'Transmission Configuration Indication' is present in both DCI formats 1_1 and 1_2 and when the number of codepoints S in the 'Transmission Configuration Indication' field of DCI format 1_2 is less than the number of codepoints in the 'Transmission Configuration Indication' field of DCI format 1_1, the activated TCI states mapped to the first S 'Transmission Configuration Indication' field codepoints in DCI format 1_1 are mapped to the 'Transmission Configuration Indication' field codepoints in DCI format 1_2. < Unchanged parts are omitted >

TP from Samsung for clause 5.1.5 of TS 38.214 [4]

### 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP or in a set of CCs/DL BWPs, respectively. When a set of TCI state IDs are activated for a set of CCs/DL BWPs, where the applicable list of CCs is determined by indicated CC in the activation command, the same set of TCI state IDs are applied for all DL BWPs in the indicated CCs. When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command. For a UE configured with DCI format 1_2, the codepoints of the DCI field 'Transmission Configuration Indication' in the DCI format are mapped to one or two TCI states by the activation commands, described in clauses 6.1.3.14 and 6.1.3.24 of [10, TS 38.321], according to ordinal position of the codepoints. < Unchanged parts are omitted >

TP from Qualcomm for clause 7.3.1.2.3 of TS 38.212 [6]

##### 7.3.1.2.3 Format 1_2 < Unchanged parts are omitted > - Transmission configuration indication – 0 bit if higher layer parameter tci-PresentForDCI-Format1-2 is not enabled; otherwise 1 or 2 or 3 bits determined by higher layer parameter tci-PresentForDCI-Format1-2 as defined in Clause 5.1.5 of [6, TS38.214]. The number of bits should be no less than the number of TCI codepoints activated by the MAC-CE. < Unchanged parts are omitted >

Discussion:

Question: Which alternative do you prefer? Also, please share your preferred TP, if any.

- Alt1. The number of codepoints S in the TCI field of DCI format 1_2 can be smaller than the number of TCI codepoints N activated by MAC-CE

  - When S<N, the S codepoints are mapped from the lowest activated codepoint

- Alt2. The number of codepoints S in the TCI field of DCI format 1_2 should be no less than the number of TCI codepoints N activated by MAC-CE

Companies’ view

| Company name | View |
| --- | --- |
| Apple | Support Alt2 since the case of S<N could be easily avoided by gNB implementation, but we are also open to Alt1 if that is majority’s view. One quick question, is S=0 included in this scope? If yes, for Alt1, if S=0, is it correct understanding that the first TCI code-point would be used for PDSCH reception? |
| OPPO | Either Alt can work. Alt1 is slightly preferred. To Apple’s question, our understanding is: if tci-PresentForDCI-Format1-2 is not enabled, DCI 1_2 does not have TCI field and the PDSCH follows the TCI state of DCI 1_2. If tci-PresentForDCI-Format1-2 is enabled, the TCI field has 1 or 2 or 3 bits as configured by the higher layer. |
| vivo | Slightly prefer Alt1. The flexibility of different DCI bits for 1-1 and 1-2 could be kept with Alt1. One more question regarding the TP: it seems that the mapping between codepoints in 1-1 and the activated TCI states is also not specified, can we also cover the mapping of 1-1 and the activated TCI states in the same TP? |
| Samsung | Support Alt1. Current specification allows having less # of TCI codepoints for format 1_2 than 1_1. Alt1 is needed to cover mapping for such case. On vivo’s question: mapping between codepoints in 1_1 and activated TCI states is currently specified in RAN2 spec (TS38.321, Section 6.1.3.14) as follows. The Ti field is set to 1 to indicate that the TCI state with TCI-StateId i shall be activated and mapped to the codepoint of the DCI Transmission Configuration Indication field, as specified in TS 38.214 [7]. The Ti field is set to 0 to indicate that the TCI state with TCI-StateId i shall be deactivated and is not mapped to the codepoint of the DCI Transmission Configuration Indication field. The codepoint to which the TCI State is mapped is determined by its ordinal position among all the TCI States with Ti field set to 1, i.e. the first TCI State with Ti field set to 1 shall be mapped to the codepoint value 0, second TCI State with Ti field set to 1 shall be mapped to the codepoint value 1 and so on. The maximum number of activated TCI states is 8; |
| DOCOMO | Support Alt. 1. TCI state field in DCI format 0_1 is always 3-bit, on the other hand, DCI format 0_2 can be less than 3-bit. Alt.1 is more efficient, in case of the different size of TCI state field in DCI format 0_1/0_2. |
| Ericsson | Support Alt 1. We have similar understanding as Samsung and DOCOMO. |
| Intel | Slight preference to Alt 2. Same size of TCI state filed in DCI would make DCI decoding possible in some cases during reconfiguration to the different number of active TCI codepoints. |
| CATT | Support alt. 1 |
| Huawei, HiSilicon | Support Alt1. |
| ZTE | Support Alt 1 |
| MediaTek | Support Alt 1 |
| Qualcomm | Support Alt 2. Understood that DCI 1_1 always has 3 bits, and DCI 1_2 can have 1, 2, 3 bits. The question is why activated codepoint # can be greater than the # indicated by the DCI 1_2? For example, if gNB knows max activated codepoint # is 4, then it can reconfigure 2 bits for DCI 1_2, right? There is no such misalignment to begin with. Cannot see solid use case, since this is fully manageable by gNB. |
| LG | Support Alt 1 |
| Nokia/NSB | Support Alt 1. Alt 2 may bring unnecessary restriction. |

Based on companies’ inputs, Alt1 is supported by a majority number of companies, concerning on Alt2 that it can introduce unnecessary restriction to use different sizes of TCI state fields in DCI format 0_1 and 0_2. Since we need to select one alt between the two, my suggestion is to agree on Alt1 and continue discussion on the exact TP.

Possible Agreement 1

- The number of codepoints S in the TCI field of DCI format 1_2 can be smaller than the number of TCI codepoints N activated by MAC-CE

  - When S<N, the S codepoints are mapped from the lowest activated codepoint

Draft TP for clause 5.1.5 of TS 38.214:

### 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP or in a set of CCs/DL BWPs, respectively. When a set of TCI state IDs are activated for a set of CCs/DL BWPs, where the applicable list of CCs is determined by indicated CC in the activation command, the same set of TCI state IDs are applied for all DL BWPs in the indicated CCs. When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command. When the DCI field 'Transmission Configuration Indication' is present in DCI format 1_2 and when the number of codepoints S in the DCI field 'Transmission Configuration Indication' of DCI format 1_2 is smaller than the number of TCI codepoints that are activated by the activation command, as described in clause 6.1.3.14 and 6.1.3.24 of [10, TS38.321], only the first S activated codepoints are applied for DCI format 1_2. < Unchanged parts are omitted >

Companies’ view on the TP

| Company name | View |
| --- | --- |
| Qualcomm | I think we may have misunderstanding on this “Alt2 that it can introduce unnecessary restriction to use different sizes of TCI state fields in DCI format 0_1 and 0_2”. To my understanding, Alt2 does not have this issue. What it means is that DCI 1_1 always have 3 bits, while DCI 1_2 can have 1, 2, 3 bits, which is configured to be no less than the max activated TCI states. So DCI 1_1 and 1_2 are allowed to have different bit #. For Alt1, my question is that why the # of activated codepoints can exceed the # of codepoints that can be indicated by the bits in DCI 1_2? My understanding is that this misalignment can be avoided by gNB, which knows max # of activated codepoints and hence can configure the right bit # for DCI 1_2. Not clear why the issue can exist at first place. Perhaps someone can explain the use case a bit? |

- Draft TPs for E-rated and H2-rated issues

  - Aligning RRC parameter names with TS38.331 (MB.3 in [1])

Reason for change:

RRC parameter names in TS 38.213 v16.3.0 and TS 38.214 v16.3.0 are not align with TS 38.331 v16.2.0.

Text proposal [7][8]:

Text proposal for TS 38.214 v16.3.0 6.2.1 UE sounding procedure < Unchanged parts are omitted > When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource by a MAC CE for a set of CCs/BWPs, where the applicable list of CCs is indicated by higher layer parameter simultaneousSpatial-UpdateList1-r16 or simultaneousSpatial-UpdateList2-r16, the spatialRelationInfo is applied for the semi-persistent or aperiodic SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs. < Unchanged parts are omitted > Text proposal for TS 38.213 v16.3.0 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > - if the UE is provided by simultaneousTCI-UpdateList1-r16 or simultaneousTCI-UpdateList2-r16 up to two lists of cells for simultaneous TCI state activation, the UE applies the antenna port quasi co-location provided by TCI-States with same activated tci-StateID value to CORESETs with index p in all configured DL BWPs of all configured cells in a list determined from a serving cell index provided by a MAC CE command < Unchanged parts are omitted > ## 4.1 Cell search For operation with shared spectrum channel access, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location ’typeA’, and ’typeD’ properties, when applicable [6, TS 38.214], if a value of NDM-RSPBCHmodNSSBQCL is same among the SS/PBCH blocks. NDM-RSPBCH is an index of a DM-RS sequence transmitted in a PBCH of a corresponding SS/PBCH block, and NSSBQCL is either provided by ssb-PositionQCL-r16 or, if ssb-PositionQCL-r16 is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-1 with kSSB<24 [4, TS 38.211]. subCarrierSpacingCommon indicates SCS of RMSI only for the case of operation without shared spectrum channel access. The UE can determine an SS/PBCH block index according to NDM-RSPBCHmodNSSBQCL, or according to i modNSSBQCL where i is the candidate SS/PBCH block index. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. < Unchanged parts are omitted > # 5 Radio link monitoring The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP, as described in Clause 12, on the primary cell. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in Clause 13, the UE is expected to perform RLM using the associated SS/PBCH block when the associated SS/PBCH block index is provided by RadioLinkMonitoringRS. If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstants is provided by higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG is monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell. A UE can be configured for each DL BWP of a SpCell [11, TS 38.321] with a set of resource indexes, through a corresponding set of RadioLinkMonitoringRS, for radio link monitoring by failureDetectionResources. The UE is provided either a CSI-RS resource configuration index, by csi-RS-Index, or a SS/PBCH block index, by ssb-Index. The UE can be configured with up to RadioLinkMonitoringRS for link recovery procedures, as described in Clause 6, and for radio link monitoring. From the RadioLinkMonitoringRS, up to RadioLinkMonitoringRS can be used for radio link monitoring depending on Lmax as described in Table 5-1, wherein Lmax is as defined in Clause 4.1, and up to two RadioLinkMonitoringRS can be used for link recovery procedures. For operation with shared spectrum channel access, when a UE is provided a SS/PBCH block index by ssb-Index, the UE is expected to perform radio link monitoring using SS/PBCH block(s) in the discovery burst transmission window as described in Clause 4.1, where the SS/PBCH block(s) have candidate SS/PBCH block index(es) corresponding to SS/PBCH block index provided by ssb-Index. If the UE is not provided RadioLinkMonitoringRS and the UE is provided for PDCCH receptions TCI states that include one or more of a CSI-RS - the UE uses for radio link monitoring the RS provided for the active TCI state for PDCCH reception if the active TCI state for PDCCH reception includes only one RS - if the active TCI state for PDCCH reception includes two RS, the UE expects that one RS is configured with qcl-Type set to ’typeD’ [6, TS 38.214] and the UE uses the RS configured with qcl-Type set to ’typeD’ for radio link monitoring; the UE does not expect both RS to be configured with qcl-Type set to ’typeD’. - the UE is not required to use for radio link monitoring an aperiodic or semi-persistent RS - For , the UE selects the RS provided for active TCI states for PDCCH receptions in CORESETs associated with the search space sets in an order from the shortest monitoring periodicity. If more than one CORESETs are associated with search space sets having same monitoring periodicity, the UE determines the order of the CORESET from the highest CORESET index as described in Clause 10.1. < Unchanged parts are omitted > # 6 Link recovery procedures A UE can be provided, for each BWP of a serving cell, a set of periodic CSI-RS resource configuration indexes by failureDetectionResources and a set of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt-r16 or candidateBeamRSSCellList-r16 for radio link quality measurements on the BWP of the serving cell. If the UE is not provided by failureDetectionResources for a BWP of the serving cell, the UE determines the set to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH and, if there are two RS indexes in a TCI state, the set includes RS indexes configured with qcl-Type set to ’typeD’ for the corresponding TCI states. The UE expects the set to include up to two RS indexes. The UE expects single port RS in the set . The UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in the set . The thresholds Qout,LR and Qin,LR correspond to the default value of rlmInSyncOutOfSyncThreshold, as described in [10, TS 38.133] for Qout, and to the value provided by rsrp-ThresholdSSB or rsrp-ThresholdBFR-r16, respectively. < Unchanged parts are omitted > ### 7.1.1 UE behaviour < Unchanged parts are omitted > - If - the PUSCH transmission is scheduled by DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or - the PUSCH transmission is not scheduled by DCI format 0_0 that does not include an SRI field, or - SRI-PUSCH-PowerControl is not provided to the UE, the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking - If - the PUSCH transmission is scheduled by DCI format 0_0 on serving cell c, - the UE is not provided PUCCH resources for the active UL BWP of serving cell c, and - the UE is provided enableDefaultBeamPL-ForPUSCH0-r16 the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to ’typeD’ in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the serving cell c - If - the PUSCH transmission is scheduled by DCI format 0_0 on serving cell c, - the UE is not provided a spatial setting for PUCCH resources on the active UL BWP of the primary cell [11, TS 38.321], and - the UE is provided enableDefaultBeamPL-ForPUSCH0-r16 the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to ’typeD’ in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of serving cell c < Unchanged parts are omitted > ### 7.3.1 UE behaviour < Unchanged parts are omitted > - If the UE - is not provided pathlossReferenceRS or SRS-PathlossReferenceRS, - is not provided spatialRelationInfo, and - is provided enableDefaultBeamPL-ForSRS-r16, and - is not provided CORESETPoolIndex value of 1 for any CORESET, or is provided CORESETPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212] the UE determines a RS resource index providing a periodic RS resource configured with qcl-Type set to ’typeD’ in - the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP, if CORESETs are provided in the active DL BWP of serving cell c - the active PDSCH TCI state with lowest ID [6, TS 38.214] in the active DL BWP, if CORESETs are not provided in the active DL BWP of serving cell c < Unchanged parts are omitted > ## 7.4 Physical random access channel A UE determines a transmission power for a physical random access channel (PRACH), , on active UL BWP of carrier of serving cell based on DL RS for serving cell in transmission occasion as [dBm], where is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3] for carrier of serving cell within transmission occasion , is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers [11, TS 38.321] for the active UL BWP of carrier of serving cell , and is a pathloss for the active UL BWP of carrier based on the DL RS associated with the PRACH transmission on the active DL BWP of serving cell and calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, where RSRP is defined in [7, TS 38.215] and the higher layer filter configuration is defined in [12, TS 38.331]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in Clause 13, the UE determines based on the SS/PBCH block associated with the PRACH transmission. If a PRACH transmission from a UE is not in response to a detection of a PDCCH order by the UE, or is in response to a detection of a PDCCH order by the UE that triggers a contention based random access procedure, or is associated with a link recovery procedure where a corresponding index is associated with a SS/PBCH block, as described in Clause 6, referenceSignalPower is provided by ss-PBCH-BlockPower. If a PRACH transmission from a UE is in response to a detection of a PDCCH order by the UE that triggers a contention-free random access procedure and depending on the DL RS that the DM-RS of the PDCCH order is quasi-collocated with as described in Clause 10.1, referenceSignalPower is provided by ss-PBCH-BlockPower or, if the UE is configured resources for a periodic CSI-RS reception or the PRACH transmission is associated with a link recovery procedure where a corresponding index is associated with a periodic CSI-RS configuration as described in Clause 6, referenceSignalPower is obtained by ss-PBCH-BlockPower and powerControlOffsetSS where powerControlOffsetSS provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB. If the active TCI state for the PDCCH that provides the PDCCH order includes two RS, the UE expects that one RS is configured with qcl-Type set to ’typeD’ and the UE uses the one RS when applying a value provided by powerControlOffsetSS. < Unchanged parts are omitted > ## 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set based on the search space set associated with the value of searchSpaceID. The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB, the DM-RS antenna port associated with corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, quasi co-location ’typeA’, and ’typeD’ properties, when applicable [6, TS 38.214], if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET. The value for the DM-RS scrambling sequence initialization is the cell ID. A SCS is provided by subCarrierSpacingCommon in MIB. For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set is not configured with the same qcl-Type set to ’typeD’ properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH. < Unchanged parts are omitted > For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI state. For a CORESET with index 0, the UE expects that a CSI-RS configured with qcl-Type set to ‘typeD’ in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block - if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH. The active BWP is defined as the active BWP in the slot when the activation command is applied. < Unchanged parts are omitted > If a UE - is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and - monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have been configured with the same or different qcl-Type set to ’typeD’ properties on active DL BWP(s) of one or more cells the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs having being configured with qcl-Type set to same ’typeD’ properties as the CORESET, on the active DL BWP of a cell from the one or more cells - the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index - the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions - for the purpose of determining the CORESET, a SS/PBCH block is considered to have different QCL ’typeD’ properties than a CSI-RS - for the purpose of determining the CORESET, a first CSI-RS associated with a SS/PBCH block in a first cell and a second CSI-RS in a second cell that is also associated with the SS/PBCH block are assumed to have same QCL ’typeD’ properties - the allocation of non-overlapping CCEs and of PDCCH candidates for PDCCH monitoring is according to all search space sets associated with the multiple CORESETs on the active DL BWP(s) of the one or more cells - the number of active TCI states is determined from the multiple CORESETs If a UE - is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and - monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs where none of the CORESETs has TCI-states configured with qcl-Type set to’typeD’, the UE is required to monitor PDCCH candidates in overlapping PDCCH monitoring occasions for search space sets associated with different CORESETs. <omitted text> # 13 UE procedure for monitoring Type0-PDCCH CSS sets < Unchanged parts are omitted > For operation with shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over slots that include Type0-PDCCH monitoring occasions associated with SS/PBCH blocks that are quasi co-located with the SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set with respect to average gain, quasi co-location ’typeA’, and ’typeD’ properties, when applicable [6, TS 38.214]. For a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two consecutive slots starting from slot n0 include the associated Type0-PDCCH monitoring occasions. The UE determines an index of slot n0 as n0=O⋅2μ+i⋅M mod Nslotframe, μ that is in a frame with system frame number (SFN) SFNC satisfying SFNC mod 2=0 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=0, or in a frame with SFN satisfying SFNC mod 2=1 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=1. M and O are provided by Table 13-11, and μ∈{0, 1} based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots n0 and n0+1 is the first symbol index provided by Table 13-11. The UE does not expect to be configured with M=1/2, or with M=2, when NSSBQCL=1. < Unchanged parts are omitted >

No concern has been received on the TPs in section 3.1 except for a minor comment to fix ‘quasi-colocation’ into ‘quasi co-location’. As the TPs in section 3.1 are E-rated, these TPs will be proposed by FL in email threads [103-e-NR-AlignmentCR-38213] and [103-e-NR-AlignmentCR-38214].

  - Adopting the agreed TP for TS38.213 (MB.4 in [1])

Reason for change:

In TS 38.213, the agreed TP in [2] is captured incorrectly.

Text Proposal [3]:

Text proposal for TS 38.213 v16.3.0 7.1.1 UE behavior < Unchanged parts are omitted > - If - the PUSCH transmission is scheduled by DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or - the PUSCH transmission is scheduled by DCI format 0_1 or DCI format 0_2 that does not include an SRI field, or - SRI-PUSCH-PowerControl is not provided to the UE, the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking < Unchanged parts are omitted >

No concern has been received on the TP.

Possible Agreement 2

- Endorse the TP in section 3.2 of R1-2009234.

- Conclusion

Based on discussion in email thread [103-e-NR-eMIMO-01], the TPs for issue 3.1 were requested to editor, and the following agreements were made for issue 2.1 and 3.2.

Agreement

The TP for clause 5.1.5 of TS 38.214 is endorsed in R1-2009457 (TS38.214 Rel-16, CR#0136, Cat. F).

Agreement

The text proposal in section 3.2 of R1-2009233 is endorsed in R1-2009458 (TS38.213 Rel-16, CR#0148, Cat. F).

References

[1] R1-2008140, Summary for Rel.16 NR eMIMO maintenance, Moderator (Samsung)

[2] R1-2007092, Text proposal on default PL RS for Rel.16 NR_eMIMO Multi-Beam, Moderator (LG Electronics)

[3] R1-2007748, Maintenance of multi-beam operation, ZTE

[4] R1-2008139, On maintenance of Rel.16 multi-beam operation, Samsung

[5] R1-2008638, Draft CR on TCI state codepoint mapping for DCI format 1_2, Ericsson

[6] R1-2008611, Remaining issue on multi-beam operation, Qualcomm Incorporated

[7] R1-2008640, Draft CR on QCL terminology alignment, Nokia, Nokia Shanghai Bell

[8] R1-2008514, Remaining issues on multi-beam operation, MediaTek Inc.