3GPP TSG RAN WG1 #103-e			R1-2009249

e-Meeting, October 26th – November 13th, 2020

Agenda Item:    8.4.4

Source: 	CAICT

Title:	Discussion on beam management of NTN

Document for:	Discussion and Decision

Introduction

At RAN#86 meeting, the study item on NR non-terrestrial networks (NTN)was approved [1]. From RAN1’s perspective, the SI includes the following objectives:

The work item aims to specify the enhancements identified for NR NTN (non-terrestrial networks) especially LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles:

- FDD is assumed for core specification work for NR-NTN.

  - NOTE: This does not imply that TDD cannot be used for relevant scenarios e.g. HAPS, ATG

- Earth fixed Tracking area is assumed with Earth fixed and moving cells

- UEs with GNSS capabilities are assumed.

The detailed objectives are to specify enhancing features to Rel-15 & Rel-16’s NR radio interface & NG-RAN as follows:

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified (see TR 38.821):

- Timing relationship enhancements[RAN1,RAN2]

- Enhancements on UL time and frequency synchronization [RAN1,RAN2]

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

In addition, the following topics should be specified if beneficial and needed

- Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2].

- Feeder link switch [RAN2,RAN1]

- Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2]

  - Including signalling of polarization mode

As stated above, the WI should specify beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse.

In last RAN1 meeting, there is an agreement about beam management:One-beam per cell and multiple-beam per cell are supported in existing NR specifications and are baseline for NR NTN.

- FFS: The need for potential enhancement for beam management

- FFS: The need for potential enhancement on association of SSBs, beams and BWPs

This contribution discusses beam management and Bandwidth Parts (BWP) and other issues.

# 2. Beam management

In NTN, beam footprint (or spot beam) created by satellite beam can be handled as either “cell” or “beam” as defined in Rel.15.Satellite beams or satellites are not considered to be visible from UE perspective in NTN.

Both options a) same PCI for several satellite beams and b) one PCI per satellite beam, can be considered in NTN. A satellite beam can consist of one or more SSB beams. One cell (PCI) can have maximum of L SSB beams, where L can be 4, 8 or 64 depending on the band.Similar to TN, one or several SSB index can be used per PCI to separate SSB transmission on different beams.In TN, the mapping of antenna ports or physical beams to SSB index is left for implementation. In NTN, the association between satellite beams and SSBs index is left for implementation (i.e. it will not be specified)[2].

# Figure 1: Options for PCI mapping into satellite beams

For option a, same PCI for several satellite beams, which means the SSBs are beam specific, could avoid the packet interruption and large signalling overhead introduced by cell handover. While for option b, only one beam mapped per PCI, the Rel-15 NR beam management is not suitable to this case, and the cell handover could be unavoidable. So option a is more preferred, because beam specific SSBs can be used for beam management to avoid cell handover.

As option a, the NTN beam can be deployed in the same frequency or different frequency, when the NTN beams are deployed in the same frequency, the allocated bandwith of each beam is very large, but the UEs served by one beam may be co-channel interfered by neighboring beams, but if the NTN beams are deployed in different frequency with frequency reuse factors, although the allocated bandwidth is much smaller for each beam, the co-channel interference is largely reduced.

When the frequency reuse factor is larger than 1, the system bandwidth can be individed into many BWPs with each beam transmitted in one BWP. For example, the SSBs and SIBs of all beams can be transmitted in the initial BWP(BWP0), the initial access UEs firstly detect SSBs and SIBs, and then finish uplink RACH access in BWP0, and when the RRC procedure is connected, the BWP configuration for each beams can be configured to the UEs.

In this case, the beam switch procedure for this BWP specific beam management could be optimized by downlink signalling or optimized by UE procedure of beam switch/beam failure recovery. As in the Rel-15 NR, when stronger beams are detected and reported in UEs, the beam switch can be realized by downlink signalling. But for NR BWP switching, the BWP index is introduced independantly in the DCI scheduling data for downlink and uplink, if this DCI indication is still reused in NTN beam switch, it should be optimized to support both UL and DL switched to the same BWP.

The other possible case of satellite beam switching is the SSBs are transmitted in the initial BWP (BWP0), but the SIBs and even RACH may be transmitted in other BWPs, which means the UEs can be initial accessed in the suitable BWPs and beams. This case needs much more specification.

Another possible case of satellite beam switching is the SSBs are transmitted in different BWPs, different beams are located on different BWPs, so the SSBs associated with the corresponding beams can exist on the BWPs where the beam is located. In this case, the UE searches for the synchronous raster to find the corresponding SSBs, and accordingly connects to the corresponding beam and the BWP where it is located. For mobility issue considerations, the UE also needs to know the direction of other beams and the BWPs where other beams are located.

Proposal 1: Enhance BWP switching used for NTN beam switching to reduce beam switching latency.

# The setting of the beams

Itwasobserved that the impact of earth-fixed beam is expected to be much smaller than earth-moving beams  [3]. Accordingto[4], in order to reduce the standardization work, the table here below identifies the worst case NTN scenarios to be considered for the delay constraint.

Table 1: NTN scenarios versus delay constraints

| NTN scenarios | A | B | C1 | C2 | D1 |  | D2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |  |
| Satellite altitude | 35786 km |  | 600 km |  |  |  |  |
| Relative speed of Satellite with respect to earth | negligible |  | 7.56 km per second |  |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |  |
| Maximum Delay variation as seen by the UE(note 2) | Negligible |  | Up to +/- 40 µs/sec (Worst case) |  |  | Up to +/- 20 µs/sec |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario |  |  |  |  |  |  |  |

- Observation 1:In LEO scenarios, relative speed of satellite with respect to earthis very fast, several kilometers per second, link switching high frequency.

While Satellite altitude at 600km, relative speed of Satellite with respect to earth is 7.56 km per second, and typical minimal beam foot print diameter is 50 km. According to above data, all users under one beam need to switch once every 6.61 seconds.

- Observation 2:Depending on ephemeris information, the switch caused by the satellite's movement can be predicted

Proposal2: The setting of the beam to be regular and uniformly distributed to reduce the complexity of switching.

Figure 2 Satellite beamspatternillustration

- Observation 3: The projection of the satellite beam on the earth's surface is irregular due to the elevation fluctuations of the earth's surface.

As show in Table 1, typical NTN beam foot print diameter, min beam is 50km, max beam is 1000km, max/min ratio is 20.

Proposal 3: Different beam sizes should be considered for timing and other parameters.

- Observation 4: The variation of frequency and timing under different elevation angles.

Proposal 4:Parameter setting should consider the timing change rate at low elevation Angle, so it is very necessary to compensate according to ephemeris and pre-compensation.

Conclusion

In this contribution, we give our analysis and views on theNTN beam managment and other issues, the following proposals are given:

Proposal1 : Enhance BWP switchingused for NTN beam switching to reduce beam switching latency.

Proposal2: The setting of the beam to be regular and uniformly distributed to reduce the complexity of switching.

Proposal 3: Different beam sizes should be considered for timing and other parameters.

Proposal 4: Parameter setting should consider the timing change rate at low elevation Angle, so it is very necessary to compensate according to ephemeris and pre-compensation

References

- RP-193234, “Solutions for NR to support non-terrestrial networks (NTN)”

- TR 38.821 V1.0.0 “3rd Generation Partnership Project;Technical Specification Group Radio Access Network;Solutions for NR to support non-terrestrial networks (NTN)”

- R1-2006678,“Other RAN1 aspects for NR NTN”

- 3GPP 38.821,”Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”