| 3GPP TSG-RAN WG1 Meeting #103-e e-Meeting, October 26th – November 13th, 2020 | R1-2009310 |
| --- | --- |

Agenda item:	8.6.3

Source: 	Qualcomm Incorporated

Title: 	Coverage Recovery for RedCap Devices

Document for:	Discussion/Decision

# Introduction

In RAN#88, the revised SID on support of reduced capability NR devices was approved with following objectives for coverage recovery [1]:

- Study functionality that will enable the performance degradation of such complexity reduction to be mitigated or limited, including [RAN1]:

- Coverage recovery to compensate for potential coverage reduction due to the device complexity reduction.

  - Note: For FR1, coverage analysis for wearables can include consideration of potential reduced antenna efficiency due to device size limitations as part of the antenna gains. The extent of additional recovery of coverage loss due to reduced antenna efficiency is to be limited to 3 dB

- The study includes evaluations of the impact to network capacity and spectral efficiency

In this contribution, we first discuss the remaining aspects of the coverage recovery evaluation assumption. Based on the proposed assumption we present the link budget analysis and discuss the coverage loss for the RedCap devices due to UE complexity reduction. We also provide potential solutions to recover such losses for the identified limiting channels.

# Remaining aspects on coverage recovery targets

In RAN1#102-e, we made the following agreements on the targets for coverage recovery for RedCap devices [2].

Agreements

For the channel(s) affected by complexity reduction, the following methodology can be used to determine the target performance for coverage recovery

- Step 1: Obtain the link budget performance of the channel based on link budget evaluation

- Step 2: Obtain the target performance requirement for RedCap UEs within a deployment scenario

  - FFS on the target performance requirement

- Step 3: Find the coverage recovery value for the channel if the link budget performance is worse than the target performance requirement

Agreements: Down-selection on the following options for the target performance requirement for RedCap UEs in RAN1#103-e (aim for early in the e-meeting):

- Option 1: The target performance requirement for each channel is identified by a target MCL or MIL or MPL within a reasonable deployment

- Option 3: The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference UE within the same deployment scenario

  - Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL

- The details for the target performance requirement are FFS

In our views, option 1 makes more sense since the coverage for the RedCap UE is determined not only by the limiting channel of the reference UE but also the target performance dependent on scenario and service. For option 3, the link budget target is only based on the limiting channel of the reference UE. The recovery for the coverage loss due to complexity reduction is needed only when the link budget for a channel does not exceed that of the limiting channel for the reference UE.

For the RedCap UE, the downlink coverage is expected to be severely reduced for the single Rx antenna with a lower antenna efficiency. Based on the link budget analysis, PUSCH is the bottleneck for both the reference and RedCap UE. However, the recovery of coverage loss for a downlink channel may not be needed even there is a significant coverage loss compared to the reference UE.

Option 3 does not consider coverage enhancement supported by the Rel-17 CE SI. Therefore, it is not forward compatible. It is desirable to have the similar coverage for all the Rel-17 capable UEs, no matter whether they are reduced capability or not.

For option 1, the target performance for a given scenario can be determined based on a typical ISD. Table 1 shows the target ISD and MPL for the concerned three scenarios.

Table 1: Target MPL for coverage recovery

|  | ISD (m) | MPL (dB) |
| --- | --- | --- |
| Rural FDD 700MHz | 1732 | 118.9 |
| Urban TDD 2.6 GHz with PSD of 33 dBm/MHz | 350 | 114.4 |
| Urban TDD 4 GHz with PSD of 33 dBm/MHz | 250 | 112.5 |

Proposal 1: Option 1 with a target MPL dependent on the scenario is used to identify performance bottleneck for the RedCap UE.

# Discussion on coverage recovery for FR1

## Parameters for link budget analysis

In RAN#101 and RAN1#102 e-meeting, we made some agreements on the parameters for link-level simulation. All results presented here are in-line with these agreements. We list some our assumptions for clarify.

Table 2 Common link-level parameters

| Parameter | Rural 700MHz | Urban 2.6GHz | Urban 4GHz |
| --- | --- | --- | --- |
| gNB Tx power | 49 dBm for 20 MHz | 53 dBm for 100 MHz | 44 dBm for 100 MHz |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Numerology | 15 kHz SCS | 30 kHz SCS | 30 kHz SCS |
| Channel | TDL-C 300ns | TDL-C 300ns | TDL-C 300ns |
| UE speed | 3km/h | 3km/h | 3km/h |
| # of gNB TxRUs | 2 | 64 | 64 |
| # of Tx chains at gNB | 2 | 4 | 4 |
| # of Rx chains at gNB | 2 | 4 | 4 |
| TDD pattern | N/A | DDDDD DDSUU | DDDSU DDSUU |

Table 3 PDSCH MCS/RB/TBS assumption for reference UE

| Scenario | MCS index (64QAM table) | # PRBs | TBS | Target data rate | Actual data rate |
| --- | --- | --- | --- | --- | --- |
| Rural 700MHz | 0 | 40 | 1128 | 1 Mbps | 1.0152 Mbps |
| Urban 2.6GHz | 1 | 216 | 7944 | 10 Mbps | 10.009 Mbps |
| Urban 4GHz | 2 | 248 | 11280 | 10 Mbps | 10.152 Mbps |

Table 4 PDSCH MCS/RB/TBS assumption for RedCap UE

| Scenario | MCS index (64QAM table) | # PRBs | TBS | Target data rate | Actual data rate |
| --- | --- | --- | --- | --- | --- |
| Rural 700MHz | 0 | 40 | 1128 | 1 Mbps | 1.0152 Mbps |
| Urban 2.6GHz | 1 | 51 | 1928 | 2 Mbps | 2.4293 Mbps |
| Urban 4GHz | 2 | 51 | 2408 | 2 Mbps | 2.1672 Mbps |

Table 5 PUSCH MCS/RB/TBS assumption for reference NR and RedCap UE

| Scenario | MCS index (64QAM table) | # PRBs | TBS | Target data rate | Actual data rate |
| --- | --- | --- | --- | --- | --- |
| Rural 700MHz | 0 | 4 | 128 | 100 kbps | 1.152 kbps |
| Urban 2.6GHz | 4 | 30 | 2600 | 1 Mbps | 0.936 Mbps |
| Urban 4GHz | 2 | 32 | 1736 | 1 Mbps | 0.937 Mbps |

Table 6 Other PDSCH and PUSCH parameters for link-level simulation

| Parameter | PDSCH | PUSCH |
| --- | --- | --- |
| Waveform | CP-OFDM | DFT-S-OFDM |
| Slot structure | 10 data symbols, 2 DMRS symbols | 12 data symbols, 2 DMRS symbols |
| Beam forming | Precoder cycling | Single Tx |
| PDSCH rank | 1 | 1 |
| HARQ | Disabled | Disabled |
| Performance target | 10% iBLER | 10% iBLER |

Table 7 Parameters related to RACH procedure

| Msg 2 PDSCH |  |
| --- | --- |
| Payload | 9 bytes |
| RB, MCS | MCS 0, no TB scaling, 3 PRBs |
| HARQ | N/A |
| Slot structure | 9 data symbols, 3 DMRS symbols |
| Beam forming | Precoder cycling |
| Performance target | 10% iBLER |
| Msg 3 PUSCH |  |
| Payload | 56 bits |
| RB, MCS | 2 RBs, MCS 0 (64QAM table) |
| Slot structure | 11 data symbols, 3 DMRS symbols |
| HARQ | No HARQ |
| Performance target | 10% iBLER |
| Msg 4 PDSCH |  |
| Payload | 130 bytes |
| Slot structure | 9 data symbols, 3 DMRS symbols |
| RB, MCS | 40 RB, MCS 0 (64QAM table) |
| HARQ | No HARQ |
| Precoding | Precoder cycling |
| Performance target | 10% iBLER |

Table 8 PDCCH and PUCCH parameters for link-level simulation

| Broadcast PDCCH |  |
| --- | --- |
| BW | 48 RBs |
| # of symbols | 2 |
| PDCCH aggregation level | 16 |
| DCI size | 40 (+ 24 bits CRC) |
| REG bundle size | 6 |
| Beam forming | Precoder cycling |
| Performance target | 1% iBLER |
| PUCCH PF3 |  |
| UCI payload (bit) | 22 bits |
| PUCCH duration (symbols) | 14 |
| # RBs for long PUCCH | 1 RB |
| Repetition | Disabled |
| Frequency hopping | Enabled |
| Performance target | 1% false alarm and 1% mis-detection |

## Link budget analysis

In the following, we present out results in the link budget analysis for three FR1 scenarios: Rural FDD 700MHz, Urban TDD 2.6GHz and 4GHz. We use the link budget template agreed over email discussion for our link budget analysis. We present MCL, MIL and MPL results, where the MPL is calculated by using the IMT-2020 parameters   as a starting point. Interference margin, shadow fading and penetration margins are as per IMT-2020 assumptions.

For the urban scenario, we assume a 5 dB beamforming gain difference (i.e. the gain for antenna gain component 2) between broadcast and unicast channels, i.e. 7 and 12dB for broadcast and unicast channels respectively. The assumption of 7 dB for broadcast is from the SLS evaluation with 4 SSB beams. It is noted that the exact number can be UE specific and the number of SSB beams.

The SNR required at the target performance metric for each of the channel is given in Appendix.

Rural FDD 700MHz

The results of the link budget analysis for downlink and uplink in rural scenario are presented in the following.

| Rural (TDL-C 300ns, 700MHz FDD, 3kmph) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL |  |  |  | UL |  |  |
|  | Broadcast PDCCH | Msg2 PDSCH | Msg4 PDSCH | Unicast PDSCH | Msg3 PUSCH | Unicast PUSCH | PUCCH PF3 22 bits |
| Reference UE (2Rx) | MCL: 145.37 MIL: 158.40 MPL: 137.45 | MCL: 139.87 MIL: 152.90 MPL: 135.27 | MCL: 141.87 MIL: 154.90 MPL: 137.27 | MCL: 141.47 MIL: 154.50 MPL: 136.87 | MCL: 130.77 MIL: 143.80 MPL: 126.17 | MCL: 128.26 MIL: 141.29 MPL: 123.66 | MCL: 133.77 MIL: 143.79 MPL: 122.84 |
| RedCap UE (2Rx) | MCL: 145.37 MIL: 155.40 MPL: 134.45 | MCL: 139.87 MIL: 149.90 MPL: 132.27 | MCL: 141.87 MIL: 151.90 MPL: 134.27 | MCL: 141.47 MIL: 151.50 MPL: 133.87 | MCL: 130.77 MIL: 140.80 MPL: 123.17 | MCL: 128.26 MIL: 138.29 MPL: 120.66 | MCL: 133.77 MIL: 140.79 MPL: 119.84 |
| RedCap UE (1Rx) | MCL: 141.37 MIL: 151.60 MPL: 130.65 | MCL: 135.37 MIL: 145.40 MPL: 127.77 | MCL: 138.47 MIL: 148.50 MPL: 130.87 | MCL: 138.07 MIL: 148.10 MPL: 130.47 | MCL: 130.77 MIL: 140.80 MPL: 123.17 | MCL: 128.26 MIL: 138.29 MPL: 120.66 | MCL: 133.77 MIL: 140.79 MPL: 119.84 |

Based on the above results, we can make the following observations.

Observation 1: In rural scenario, PUSCH coverage is the bottleneck for both the reference UE and RedCap UE. The coverage of downlink channels of the RedCap UE can exceed that for PUSCH of the reference UE.

Observation 2: In rural scenario, if the coverage recovery is based on the limiting channel for the reference UE, the coverage loss that should be recovered is ~3dB for Msg3 and PUSCH.

Observation 3: In rural scenario, if the coverage recovery is based on an MPL of 118.9 dB target for ISD 1732m, no coverage recovery is needed for the RedCap UE.

Urban TDD 2.6GHz

The results of the link budget analysis for downlink and uplink in Urban 2.6GHz are presented in the following.

| Urban TDD 2.6GHz, DDDDDDDSUU (TDL-C 300ns, 3kmph, DL PSD 33 dBM/MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL |  |  |  | UL |  |  |
|  | Broadcast PDCCH | Msg2 PDSCH | Msg4 PDSCH | Unicast PDSCH | Msg3 PUSCH | Unicast PUSCH | PUCCH PF3 22 bits |
| Reference UE (4Rx) | MCL: 152.57 MIL: 161.34 MPL: 127.53 | MCL: 149.49 MIL: 158.26 MPL: 127.53 | MCL: 150.99 MIL: 159.76 MPL: 129.03 | MCL: 154.59 MIL: 163.36 MPL: 132.63 | MCL: 139.46 MIL: 148.23 MPL: 117.50 | MCL: 130.60 MIL: 139.37 MPL: 108.64 | MCL: 137.76 MIL: 146.53 MPL: 112.72 |
| RedCap UE (2Rx) | MCL: 150.07 MIL: 155.84 MPL: 122.03 | MCL: 146.19 MIL: 151.96 MPL: 121.23 | MCL: 148.49 MIL: 154.26 MPL: 123.53 | MCL: 151.99 MIL: 157.76 MPL: 127.03 | MCL: 139.46 MIL: 145.23 MPL: 114.50 | MCL: 130.60 MIL: 136.37 MPL: 105.64 | MCL: 137.76 MIL: 143.53 MPL: 109.72 |
| RedCap UE (1Rx) | MCL: 146.77 MIL: 152.54 MPL: 118.73 | MCL: 142.29 MIL: 148.06 MPL: 117.33 | MCL: 145.19 MIL: 150.96 MPL: 120.23 | MCL: 148.89 MIL: 154.66 MPL: 123.93 | MCL: 139.46 MIL: 145.23 MPL: 114.50 | MCL: 130.60 MIL: 136.37 MPL: 105.64 | MCL: 137.76 MIL: 143.53 MPL: 109.72 |

Based on the above results, we can make the following observations.

Observation 3: In Urban scenario 2.6GHz with downlink PSD of 33dBm/MHz, PUSCH coverage is the bottleneck for both the reference UE and RedCap UE. The coverage of all the channels of the RedCap UE except for PUSCH can exceed that for PUSCH of the reference UE.

Observation 4: In Urban scenario 2.6GHz with downlink PSD of 33dBm/MHz, if the coverage recovery is based on the limiting channel for the reference UE, the coverage loss that should be recovered is ~3dB for PUSCH.

Observation 5: In Urban scenario 2.6GHz with downlink PSD of 33dBm/MHz, if the coverage recovery is based on an MPL of 114.4 dB target for ISD 350m, coverage recovery is also needed for PUCCH.

Urban TDD 4GHz

The results of the link budget analysis for downlink and uplink in Urban 4GHz are presented in the following.

| Urban TDD 4GHz, DDDSUDDSUU (TDL-C 300ns, 3kmph, DL PSD 24 dBM/MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL |  |  |  | UL |  |  |
|  | Broadcast PDCCH | Msg2 PDSCH | Msg4 PDSCH | Unicast PDSCH | Msg3 PUSCH | Unicast PUSCH | PUCCH PF3 22 bits |
| Reference UE (4Rx) | MCL: 143.57 MIL: 152.34 MPL: 118.53 | MCL: 138.37 MIL: 147.14 MPL: 116.41 | MCL: 139.87 MIL: 148.64 MPL: 117.91 | MCL: 142.57 MIL: 151.34 MPL: 120.61 | MCL: 138.26 MIL: 147.03 MPL: 116.30 | MCL: 131.21 MIL: 139.99 MPL: 109.26 | MCL: 137.76 MIL: 146.53 MPL: 112.72 |
| RedCap UE (2Rx) | MCL: 141.07 MIL: 146.84 MPL: 113.03 | MCL: 135.07 MIL: 140.84 MPL: 110.11 | MCL: 137.37 MIL: 143.14 MPL: 112.41 | MCL: 139.87 MIL: 145.64 MPL: 114.91 | MCL: 138.26 MIL: 144.03 MPL: 113.30 | MCL: 131.21 MIL: 136.99 MPL: 106.26 | MCL: 137.76 MIL: 143.53 MPL: 109.72 |
| RedCap UE (1Rx) | MCL: 137.77 MIL: 143.54 MPL: 109.73 | MCL: 131.17 MIL: 136.94 MPL: 106.21 | MCL: 134.07 MIL: 139.84 MPL: 109.11 | MCL: 136.67 MIL: 142.44 MPL: 111.71 | MCL: 138.26 MIL: 144.03 MPL: 113.30 | MCL: 131.21 MIL: 136.99 MPL: 106.26 | MCL: 137.76 MIL: 143.53 MPL: 109.72 |

Based on the above results, we can make the following observations.

Observation 6: In Urban scenario 4GHz with downlink PSD of 24dBm/MHz, PUSCH coverage is the bottleneck for both the reference UE and RedCap UE. The coverage of all the channels of the RedCap UE except for PUSCH and Msg2 PDSCH can exceed that for PUSCH of the reference UE.

Observation 7: In Urban scenario 4GHz with downlink PSD of 24dBm/MHz, if the coverage recovery is based on the limiting channel for the reference UE, the coverage loss that should be recovered is ~3dB and 2dB for PUSCH and Msg2 PDSCH, respectively.

Observation 8: In Urban scenario 4GHz with downlink PSD of 24dBm/MHz, if the coverage recovery is based on an MPL of 112.5 dB target for ISD 250m, coverage recovery is also needed for broadcast PDCCH, Msg4 and unicast PDSCH in downlink, and Msg3/PUCCH in uplink.

Based on the above observations, we make the following proposals.

Proposal 2: For FR1, consider enhancement to Msg3 and PUSCH performance to compensate the coverage loss due to UE complexity reduction.

Proposal 3: For Urban scenario with downlink PSD of 24dBm/MHz, also consider enhancement to Msg2 PDSCH, Msg4 PDSCH, broadcast PDCCH and unicast PDSCH performance to compensate the coverage loss due to UE complexity reduction.

# Discussion on coverage recovery for FR2

## Parameters for link budget analysis

The following tables list the link-level simulation parameter assumptions for FR2.

Table 9 Common link-level parameters

| Parameter | FR2 Indoor 28 GHz |
| --- | --- |
| SCS | 120 kHz |
| Channel model | TDL-A |
| CP | Normal |
| Delay spread | 30ns |
| UE velocity | 3 km/h |
| Antenna correlation | Low |
| # gNB Tx chains | 2 |
| # gNB Rx chains | 2 |
| TDD pattern | DDDSU (S: 10D:2G:2U) |
| # UE Tx chains | 1 |
| # UE Rx chains | Ref: 2 RedCap: 1 or 2 |
| UE BW | Ref: 100 MHz RedCap: 50 and 100 MHz |
| HARQ (if applicable) | Disabled |

Table 10 PDSCH link-level parameters

| Parameter | Unicast (Target rate = 25 Mbps) |  |  | Msg2 | Msg4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref UE | RedCap UE |  | All UEs | Ref UE | RedCap UE |  |
| BW (MHz) | 100 | 100 | 50 | 100/50 | 100 | 100 | 50 |
| Number of RBs | 66 | 66 | 32 | 4 | 66 | 66 | 32 |
| MCS/TBS | MCS = 3 | MCS = 3 | MCS = 6 | MCS = 0 (TBS = 96) | TBS = 1040 |  |  |
| Precoder Type | Open loop and closed loop SVD based |  |  | Open loop |  |  |  |
| Waveform | CP-OFDM |  |  |  |  |  |  |
| MCS Table | 1 |  |  |  |  |  |  |
| PDSCH Symbols | 3-14 |  |  |  |  |  |  |
| DMRS Symbol | For msg2: symbols 3, 8, 12. Else symbol 3, front loaded |  |  |  |  |  |  |
| Target BLER | 10% |  |  |  |  |  |  |

Table 11 PDCCH link-level parameters

| Parameter | All UEs |
| --- | --- |
| UE BW | 100 MHz |
| Number of RBs | 48 |
| AL | 16 |
| REG bundle size | 6 |
| CORESET size | 1 symbol |
| DCI Size | 40 |
| Target BLER | 1% |

Table 12 PBCH link-level parameters

| Parameter | All UEs |
| --- | --- |
| UE BW | 50/100 MHz |
| SCS | 120 kHz |
| Periodicity | 20 ms |
| Performance metric | Combination of 4 SSBs in 80ms |
| Target BLER | 10% |

Table 13 PUCCH link-level parameters

| Parameter | All UEs |
| --- | --- |
| Number of RBs | 1 |
| Number of PUCCH symbols | 14 |
| Number of bits | 2, 11, 22 |
| Target BLER | 1% |

Table 14 PUSCH link-level parameters

| Parameter | Unicast (Target Rate = 5 Mbps) |  |  | Msg3 |
| --- | --- | --- | --- | --- |
|  | Ref UE | RedCap UE |  | All UEs |
| BW (MHz) | 100 | 100 | 50 | 100/50 |
| Number of RBs | 66 | 66 | 32 | 4 |
| MCS | 1 | 1 | 4 |  |
| MCS Table | 1 |  |  |  |
| Waveform | DFT-s-OFDM |  |  |  |
| PUSCH Symbols | 1-14 (DMRS front loaded) |  |  |  |
| Target BLER | 10% |  |  |  |

## Link budget analysis

In the following, we present our results in the link budget analysis the FR2 scenario (Indoor TDD 28GHz). We also present the SNR required at the target performance metric.

Table 15 FR2 Downlink Link Budget Results

|  |  | Reference UE 100 MHz / 2Rx | RedCap UE 100 MHz / 1Rx | RedCap UE 50 MHz / 2Rx | RedCap UE 50 MHz / 1Rx |
| --- | --- | --- | --- | --- | --- |
| PBCH (SCS = 120 kHz) | SNR (dB) | -11.9 | -8.6 | -11.8 | -8.8 |
|  | MIL | 153.0 | 149.7 | 152.9 | 149.9 |
|  | MCL | 119.9 | 116.6 | 119.8 | 116.8 |
| PDCCH (CSS) | SNR (dB) | -8.3 | -5.0 |  |  |
|  | MIL | 143.4 | 140.1 |  |  |
|  | MCL | 116.3 | 113.0 |  |  |
| PDCCH (USS) | SNR (dB) | -8.3 | -5.0 |  |  |
|  | MIL | 149.4 | 146.1 |  |  |
|  | MCL | 116.3 | 113.0 |  |  |
| PDSCH (Unicast) Open loop precoder | SNR (dB) | -0.8 | 3.4 | 2.6 | 7.7 |
|  | MIL | 141.9 | 137.7 | 138.4 | 133.4 |
|  | MCL | 108.8 | 104.6 | 105.4 | 100.3 |
| PDSCH (Unicast) Closed loop SVD precoder | SNR (dB) | -4.2 | -1.0 | -1.0 | 2.7 |
|  | MIL | 145.2 | 142.1 | 142.1 | 138.4 |
|  | MCL | 112.2 | 109.0 | 109.0 | 105.3 |
| PDSCH (Msg2) | SNR (dB) | -2.8 | 2.6 |  |  |
|  | MIL | 143.9 | 138.5 |  |  |
|  | MCL | 110.8 | 105.4 |  |  |
| PDSCH (Msg4) | SNR (dB) | -6.2 | -2.7 | -3.1 | 0.9 |
|  | MIL | 147.3 | 143.8 | 144.2 | 140.2 |
|  | MCL | 114.2 | 110.7 | 111.1 | 107.1 |

Table 16 FR2 PUSCH Unicast Link Budget Results

|  |  | All UEs 100 MHz / 1Tx | RedCap UE 50 MHz / 1Tx |
| --- | --- | --- | --- |
| PUSCH (Unicast) | SNR (dB) | 4.5 | 7.6 |
|  | MIL | 138.8 | 138.9 |
|  | MCL | 105.7 | 105.8 |

Table 17 FR2 Other Uplink Link Budget Results

|  |  | All UEs / 1Tx |
| --- | --- | --- |
| PRACH (B4) | SNR (dB) | -12.6 |
|  | MIL | 163.4 |
|  | MCL | 130.4 |
| PUCCH (2bits) | SNR (dB) | -9.3 |
|  | MIL | 170.8 |
|  | MCL | 137.7 |
| PUCCH (11bits) | SNR (dB) | -3.2 |
|  | MIL | 164.7 |
|  | MCL | 131.6 |
| PUCCH (22bits) | SNR (dB) | -0.7 |
|  | MIL | 162.2 |
|  | MCL | 129.1 |
| PUSCH (Msg3) | SNR (dB) | 2.1 |
|  | MIL | 147.4 |
|  | MCL | 120.3 |

Observation 9: For FR2 indoor scenario 28GHz, PUSCH unicast (with target data rate of 5 Mbps) is the bottleneck for the reference UE, and PDSCH unicast (with target data rate of 25 Mbps) is the bottleneck for the RedCap UE.

Observation 10: For FR2 indoor scenario 28GHz, if the coverage recovery is based on the limiting channel for the reference UE, the coverage loss that should be recovered is:

- 1.1 dB for PDSCH unicast for RedCap 100MHz with 1 Rx (if open loop precoder is used)

- 5.4 dB for PDSCH unicast for RedCap 50 MHz with 1Rx (if open loop precoder is used)

- 0.4 dB for PDSCH unicast for RedCap 50 MHz with 1Rx (if closed loop precoder is used)

Based on the above observations, we make the following proposals.

Proposal 4: For FR2, consider enhancements to PDSCH performance to recover the coverage loss due to UE complexity reduction.

# Potential Techniques for FR1 Coverage Recovery

In this section we will discuss the potential coverage recovery techniques especially for FR1. We focus only on the channels for which the coverage recovery is needed.

Msg2 PDSCH

From the LB analysis, Msg2 PDSCH coverage may be under the target performance in some cases. The Rel-15 TBS scaling for achieving lower spectral efficiencies than MCS0 (rate 120/1024, QPSK) can be used to improve coverage. A TBS scaling factor 0.5 or 0.25 can be dynamically indicated in the DCI to achieve an improvement of 3 – 6 dB.

However, due to the bandwidth limitation of the RedCap UE there is a restriction on the Msg2 PDSCH payload size when TBS scaling is applied. In case of a single RAR with 9 bytes, 12 RBs are needed for MCS0 with 0.25 scaling factor, and therefore for RedCap UE with maximum 20MHz BW (i.e. 51 RBs for 30 kHz SCS), a maximum 4 RARs can be accommodated in one Msg2 PDSCH.

In order to efficiently support TBS scaling, enhancements for TDMed RAR transmission on can be considered. For example, when a DCI schedules a Msg2 PDSCH with 8 RARs, the first 4 RARs can be transmitted in the first slot and the other 4 RARs are in the following slot. This can be supported by either using multi-TB scheduling (e.g. the first and second 4 RARs associated with different TBs) or TB processing over multi-slot PDSCH (e.g. the first and second 4 RARs within a same TB). The multi-TB scheduling for Msg2 PDSCH may have a benefit for reducing decoding latency and is thus preferred.

Proposal 5: For Msg2 PDSCH with TBS scaling, multi-TB scheduling can be considered for TDMed RAR transmission over multiple slots.

Msg4 PDSCH

Currently, Msg4 PDSCH is ACK-only channel and based on implementation gNB may retransmit Msg4 without any feedback from UE. But the UE does not know whether the TB is same across different transmission and retransmission and may not apply soft combining for Msg4 PDSCH.

For coverage recovery, the slot-aggregation can be used for Msg4 PDSCH transmission. Alternatively, the use of the low-SE MCS table can also be considered. The performance of the low-SE MCS table is expected to be same as the slot-aggregation. The enabling of Msg4 PDSCH coverage recovery can be indicated in the SIB and can be also based on the UE capability reported in the message 3.

Proposal 6: For FR1, consider the slot-aggregation or the low-SE MCS table for coverage recovery of Msg4 PDSCH.

Unicast PDSCH

Based on the above link budget analysis, it is observed that 1-2 coverage recovery is needed for unicast PDSCH for RedCap UE with 1 Rx. We consider two approaches for PDSCH coverage recovery: DMRS bundling and frequency hopping.

DMRS bundling can be used to improve channel estimation performance and thus BLER performance, especially in coverage limited scenarios. Currently, only frequency domain DMRS bundling with a configurable PRG size is supported. For RedCap UE, DMRS time domain bundling with a joint channel estimation across slots can be considered also to compensate the coverage loss due to reduced number of Rx antennas.

DMRS time domain bundling is straightforward for PDSCH with slot aggregation. DMRS can be coherently transmitted over consecutive slots, and at the receiver, DMRS in different slots can be combined coherently to improve channel estimation performance.

Frequency hopping is now supported only in the uplink. This is because distributed VRB-to-PRB mapping can be configured in downlink for achieving frequency domain diversity gain. For RedCap UE, the bandwidth of BWP will be small due to UE bandwidth reduction, and the diversity gain from distributed PRB mapping is quite limited. Frequency hopping across BWP or a large bandwidth may be considered to achieve higher diversity gain.

In Rel-15, the BWP switch delay is large, e.g. requiring several slots depending on UE capability and SCS. If the same delay value is reused for frequency hopping across BWP, there is a significant reduction on data throughput and performance benefits. If the BWP switch does not require a change of the subcarrier switch, the required time can be reduced. In LTE-MTC, the RF returning time for hopping across different narrowband is only 1 or 2 OFDM symbols. It can be studied whether the same RF returning time can be reused for NR if frequency hopping across BWP is considered for coverage recovery.

Proposal 7: For FR1, consider DMRS bundling and frequency hopping across a larger BW for coverage recovery of unicast PDSCH.

Broadcast PDCCH

From the LLS evaluation, for RedCap UE with a single RX, a recovery of 2-3dB coverage is needed for broadcast PDCCH. For RedCap UE with 2 Rx, the coverage of broadcast PDCCH is not the bottleneck when AL=16 is used. However, there are some cases where AL=16 is not supported. For example, if an NR system has dynamic spectrum sharing with an LTE system, the first two symbols of a slot are reserved for LTE, and for PDCCH monitoring case 1-1, the PDCCH CORESET can only be placed on the third OFDM symbol and there is only 8 CCEs available in the CORESET assuming max 20MHz UE bandwidth.

Therefore, coverage enhancement for broadcast PDCCH is needed when AL=16 is not supported and/or a single Rx case. Time-domain repetition or CORESET bundling can be considered for performance improvement. For coverage recovery, a small number of PDCCH repetitions may be required, e.g. up to 4. Both intra-slot and inter-slot repetition can be considered, where the intra-slot PDCCH repetition is preferred considering PDCCH monitoring complexity and power saving.

Proposal 8: For FR1, consider repetition or CORESET bundling for coverage recovery of broadcast PDCCH.

Figure 1: PDCCH performance with 1, 2 and 4 repetitions

UL coverage recovery

According to the link budget analysis, PUSCH is the bottleneck for both the reference and Redcap UEs. Considering the loss of antenna efficiency for the Redcap UE, a 3dB coverage recovery may be needed for UL channels, e.g. Msg3, PUCCH and unicast PUSCH. Since the UL coverage enhancement has been discussed in the Rel-17 CE SI, it is proposed to reuse the solutions for Redcap UL coverage recovery if needed.

Proposal 9: For FR1, coverage recovery techniques for uplink channels (i.e. Msg3, PUCCH unicast PUSCH) may reuse some of the techniques discussed in the Rel-17 CE SI.

# Potential Techniques for FR2 Coverage Recovery

In this section, several high-level views for FR2 coverage recovery are discussed, mainly related to:

- Frequency hopping

- Time domain repetition

- Beam refinement

- UL L1 measurement report payload reduction

- L1/L2 inter-cell diversity

After initial cell search, to reduce the UE’s BW and thus save power, the UE may switch into a narrow BW active BWP (NBWP). For a narrow band UE supported BW, to achieve frequency diversity gains, frequency domain hopping is one of the methods that can be used. However, in FR2, due to beamforming at both gNB and UE in addition to smaller cells, the delay spread is smaller compared to FR1. This leads to a larger coherence BW and hence less gain using frequency hopping (if the hopping was within a limited frequency range). For FR2, to get the frequency diversity hopping gains, the UE may need to hop across a larger system frequency range (across larger system BW). For example, in case the network supports CA, the UE may hop in frequency over multiple CCs (using 1 CC at a time).

In addition to frequency domain hopping, one of the most widely used ways for coverage recovery/enhancement is repetition in time domain. However, time domain repetition may not be a power or resource efficient way. It also causes lengthy time domain blockages for other users leading to increased latency as well as complicates the scheduler design. To reduce these effects, some other more efficient techniques may be studied. Such techniques include beam refinement or repetition across TRPs.

Inter-cell L1/L2 mobility (diversity) may be a desirable option for coverage recovery, where the UE may be connected at the same time to multiple TRPs causing diversity gains. Hence, it may be desirable to have some enhanced L1/L2 inter-cell mobility techniques that apply to RedCap use cases in order to have some coverage recovery, i.e., for diversity and connection reliability, stationary devices need to have enhanced inter-cell procedures, e.g.:

- L1/L2 mobility

- Message repetition

- BFD/BFR procedures

Proposal 10: For FR2, consider power and resource efficient solutions for coverage recovery in FR2, including:

- Beam refinement

- Hopping across a larger system frequency range (e.g., cross CC)

- Enhanced L1/L2 inter-cell mobility (optimized to RedCap use cases)

Among different use cases, industrial wireless sensors and video surveillance cameras may be among the main use cases for RedCap FR2 devices. As indicated in the SID, the UEs associated with these use cases may be stationary and have UL heavy traffic models. Some of these use cases also have large latency requirements which may be utilized in power saving techniques:

- Industrial Wireless Sensors: < 100 ms, safety related: 5-10 ms

- Video Surveillance: < 500 ms

To enhance message coverage, classical time domain repetition can be utilized. However, as discussed, repetition may cause blockages. Given the large latency requirements of some RedCap use cases, and to reduce blockages and/or resources, study different repetition techniques for message.

Proposal 11: For FR2, consider ways to reduce time blockage and/or resources by taking into consideration the larger latency requirements for some use cases.

Since beam management is very crucial for FR2 operation, the L1 measurement report is an extremely important message. However, a single L1 measurement report may be configured to include multiple CSI report settings where the number is limited by the PUCCH/PUSCH payload size (if payload size is not sufficient, reports with lowest priority are dropped). A single report setting carries SSBRI/CRI and the measurement. Depending on the number of CSI-RS resources configured (up to 128 across all resource sets), the CRI may have a considerable number of bits (7 bits each) with a total of up to 7x4 bits per setting. The measurements per setting (e.g., L1-RSRP) can be 7 + up to 3x4 bits long. Hence, the L1 measurement report may end up having a considerable size.

In some RedCap scenarios, we can take advantage of the stationary conditions of the UEs to have a leaner (reduced payload) L1 measurement report without any considerable loss of BM performance. This may be needed to increase the L1 report reliability (i.e., enhance the report’s coverage). These optimizations may be RedCap specific due to some of the use cases used. For example, we can take advantage of the stationarity of the UE to allow for a reduced payload (e.g., no SSBRI/CRI or no measurement) if the beam pattern is known or a limited set of beams are used.

Proposal 12: For FR2, consider techniques to reduce the payload size for the L1 measurement report.

# System level evaluation for network capacity

To evaluate the impact of UE complexity reduction to the network capacity, we have performed the system level simulation. Urban at 2.6GHz and 4GHz are considered for the evaluation. In addition to the RAN1#102e agreements, the following additional assumptions were made.

- Maximum modulation in DL: 256QAM for Ref UE and 64QAM for RedCap

- # Rx antennas: 4 for Ref UE and 2 and 1 for RedCap

- Rank adaptation (max = 4) for 4Rx, (max = 2) for 2Rx, and (max = 1) for 1Rx

- 21 sectors with 8 UEs per sector for an integer number of UEs for the 25% and 50% ratios

Non-full buffer traffic is evaluated. FTP model 3 is used for the reference eMBB UE and the IM model is assumed for the RedCap UE. The detailed traffic model parameters are shown below.  For FTP model 3, mean inter-arrival time was adjusted across frequencies and RedCap UE ratio to achieve the desired 30% resource utilization.

| Parameter | FTP model 3 | Instant Message model |
| --- | --- | --- |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |
| Mean inter-arrival time | 130 ms @ 2.6 GHz, 50% RedCap 190 ms @ 2.6 GHz, 25% RedCap 190 ms @ 4 GHz, 50% RedCap 260 ms @ 4 GHz, 25 % RedCap | 2 sec |

The SLS evaluation results for different RedCap UE ratios and antenna numbers are illustrated in the figures below with 1 and 2 antenna configurations for identical loading of Ref UE and RedCap. It can be seen the user throughput performance of the reference eMBB UE is affected by UE complexity reduction, however, the 1 Rx antenna does not make an appreciable change compared to the 2 Rx antenna. Also, the degradation of the reference eMBB UE is mainly due to changing the traffic ratio with the assumed traffic model.

Figure 2 Ref UE User Perceived Throughput on 2.6 GHz with 25% RedCap

Figure 3 Ref UE User Perceived Throughput on 2.6 GHz with 50% RedCap

Figure 4 Ref UE User Perceived Throughput on 4GHz with 25% RedCap

Figure 5 Ref UE User Perceived Throughput on 4GHz with 50% RedCap

The evaluation results for different RedCap UE ratios and number of Rx antennas are summarized in Table 18-21.

Table 18 SLS evaluation results for 2Rx RedCap UE @ 2.6 GHz

|  | Low loading |  |  |  |
| --- | --- | --- | --- | --- |
| Metric | 0 | 0.25 | 0.5 | 1 |
| RU | 30.20 | 29.40 | 27.98 | 1.99 |
| 50% UPT (eMBB UEs) | 157.52 | 172.97 | 204.66 | n/a |
| 5% UPT (eMBB UEs) | 53.86 | 64.72 | 87.43 | n/a |
| Cell avg. SE (bps/Hz) (eMBB UEs) | 8.80 | 8.94 | 9.70 | n/a |
| 50% UPT (RedCap UEs) | n/a | 43.25 | 43.41 | 71.02 |
| 5% UPT (RedCap UEs) | n/a | 2.56 | 2.14 | 5.68 |
| Cell avg. SE (bps/Hz) (RedCap UEs) | n/a | 5.55 | 5.19 | 8.47 |
| 50% UPT (All UEs) | 157.52 | 131.30 | 84.85 | 71.02 |
| 5% UPT (All UEs) | 53.86 | 12.84 | 5.31 | 5.68 |
| Cell avg. SE (bps/Hz) (All UEs) | 8.80 | 8.10 | 7.44 | 8.47 |

Table 19 SLS evaluation results for 1Rx RedCap UE @ 2.6 GHz

|  | Low loading |  |  |  |
| --- | --- | --- | --- | --- |
| Metric | 0 | 0.25 | 0.5 | 1 |
| RU | 30.20 | 30.15 | 29.58 | 4.04 |
| 50% UPT (eMBB UEs) | 157.52 | 172.78 | 212.95 | n/a |
| 5% UPT (eMBB UEs) | 53.86 | 69.80 | 98.93 | n/a |
| Cell avg. SE (bps/Hz) (eMBB UEs) | 8.80 | 8.74 | 9.63 | n/a |
| 50% UPT (RedCap UEs) | n/a | 34.08 | 31.15 | 41.79 |
| 5% UPT (RedCap UEs) | n/a | 1.12 | 0.92 | 2.28 |
| Cell avg. SE (bps/Hz) (RedCap UEs) | n/a | 3.49 | 3.13 | 3.98 |
| 50% UPT (All UEs) | 157.52 | 126.20 | 61.29 | 41.79 |
| 5% UPT (All UEs) | 53.86 | 10.13 | 2.48 | 2.28 |
| Cell avg. SE (bps/Hz) (All UEs) | 8.80 | 7.43 | 6.38 | 3.98 |

Table 20 SLS evaluation results for 2Rx RedCap UE @ 4 GHz

|  | Low loading |  |  |  |
| --- | --- | --- | --- | --- |
| Metric | 0 | 0.25 | 0.5 | 1 |
| RU | 34.07 | 29.22 | 27.13 | 5.74 |
| 50% UPT (eMBB UEs) | 118.95 | 155.56 | 189.03 | n/a |
| 5% UPT (eMBB UEs) | 44.27 | 52.85 | 77.25 | n/a |
| Cell avg. SE (bps/Hz) (eMBB UEs) | 7.62 | 8.54 | 9.30 | n/a |
| 50% UPT (RedCap UEs) | n/a | 20.64 | 28.90 | 34.61 |
| 5% UPT (RedCap UEs) | n/a | 1.63 | 1.51 | 1.81 |
| Cell avg. SE (bps/Hz) (RedCap UEs) | n/a | 5.55 | 5.19 | 8.47 |
| 50% UPT (All UEs) | 118.95 | 118.55 | 82.69 | 34.61 |
| 5% UPT (All UEs) | 44.27 | 5.85 | 2.29 | 1.81 |
| Cell avg. SE (bps/Hz) (All UEs) | 7.62 | 7.46 | 7.02 | 5.63 |

Table 21 SLS evaluation results for 1Rx RedCap UE @ 4 GHz

|  | Low loading |  |  |  |
| --- | --- | --- | --- | --- |
| Metric | 0 | 0.25 | 0.5 | 1 |
| RU | 34.07 | 29.99 | 29.92 | 10.58 |
| 50% UPT (eMBB UEs) | 118.95 | 167.35 | 197.97 | n/a |
| 5% UPT (eMBB UEs) | 44.27 | 60.54 | 80.16 | n/a |
| Cell avg. SE (bps/Hz) (eMBB UEs) | 7.62 | 8.69 | 9.53 | n/a |
| 50% UPT (RedCap UEs) | n/a | 15.22 | 15.84 | 19.22 |
| 5% UPT (RedCap UEs) | n/a | 0.62 | 0.66 | 0.76 |
| Cell avg. SE (bps/Hz) (RedCap UEs) | n/a | 2.59 | 2.74 | 3.07 |
| 50% UPT (All UEs) | 118.95 | 120.11 | 58.11 | 19.22 |
| 5% UPT (All UEs) | 44.27 | 2.45 | 1.05 | 0.76 |
| Cell avg. SE (bps/Hz) (All UEs) | 7.62 | 7.16 | 6.14 | 3.07 |

Observation 11: For the impact of UE complexity reduction, the 1 Rx antenna does not make an appreciable change on the user throughput performance of the reference UE compared to the 2 Rx antenna.

# Conclusions

In this contribution, we discuss the functionality of coverage recovery for reduced capability NR devices. We have the following observations:

Observation 1: In rural scenario, PUSCH coverage is the bottleneck for both the reference UE and RedCap UE. The coverage of downlink channels for RedCap UE can exceed the coverage of  PUSCH for reference UE.

Observation 2: In rural scenario, if the coverage recovery is based on the limiting channel for the reference UE, the coverage loss that should be recovered is ~3dB for Msg3 and PUSCH.

Observation 3: In rural scenario, if the coverage recovery is based on an MPL of 118.9 dB target for ISD 1732m, no coverage recovery is needed for the RedCap UE.

Observation 3: In Urban scenario 2.6GHz with downlink PSD of 33dBm/MHz, PUSCH coverage is the bottleneck for both the reference UE and RedCap UE. The coverage of all the channels of the RedCap UE except for PUSCH can exceed that for PUSCH of the reference UE.

Observation 4: In Urban scenario 2.6GHz with downlink PSD of 33dBm/MHz, if the coverage recovery is based on the limiting channel for the reference UE, the coverage loss that should be recovered is 3dB for PUSCH.

Observation 5: In Urban scenario 2.6GHz with downlink PSD of 33dBm/MHz, if the coverage recovery is based on an MPL of 114.4 dB target for ISD 350m, coverage recovery is also needed for PUCCH.

Observation 6: In Urban scenario 4GHz with downlink PSD of 24dBm/MHz, PUSCH coverage is the bottleneck for both the reference UE and RedCap UE. The coverage of all the channels of the RedCap UE except for PUSCH and Msg2 PDSCH can exceed that for PUSCH of the reference UE.

Observation 7: In Urban scenario 4GHz with downlink PSD of 24dBm/MHz, if the coverage recovery is based on the limiting channel for the reference UE, the coverage loss that should be recovered is ~3dB and 2dB for PUSCH and Msg2 PDSCH, respectively.

Observation 8: In Urban scenario 4GHz with downlink PSD of 24dBm/MHz, if the coverage recovery is based on an MPL of 112.5 dB target for ISD 250m, coverage recovery is also needed for broadcast PDCCH, Msg4 and unicast PDSCH in downlink, and Msg3/PUCCH in uplink.

Observation 9: For FR2 indoor scenario 28GHz, PUSCH unicast (with target data rate of 5 Mbps) is the bottleneck for the reference UE, and PDSCH unicast (with target data rate of 25 Mbps) is the bottleneck for the RedCap UE.

Observation 10: For FR2 indoor scenario 28GHz, if the coverage recovery is based on the limiting channel for the reference UE, the coverage loss that should be recovered is:

- 1.1 dB for PDSCH unicast for RedCap 100MHz with 1 Rx (if open loop precoder is used)

- 5.4 dB for PDSCH unicast for RedCap 50 MHz with 1Rx (if open loop precoder is used)

- 0.4 dB for PDSCH unicast for RedCap 50 MHz with 1Rx (if closed loop precoder is used)

Observation 11: For the impact of UE complexity reduction, the 1 Rx antenna does not make an appreciable change on the user throughput performance of the reference UE compared to the 2 Rx antenna.

Based on these observations, we made the following proposals:

Proposal 1: Option 1 with a target MPL dependent on the scenario is used to identify coverage bottleneck for the RedCap UE.

Proposal 2: For FR1, consider enhancement to Msg3 and PUSCH performance to compensate for the coverage loss due to UE complexity reduction.

Proposal 3: For Urban scenario with downlink PSD of 24dBm/MHz, also consider enhancement to Msg2 PDSCH, Msg4 PDSCH, broadcast PDCCH and unicast PDSCH performance to compensate for the coverage loss due to UE complexity reduction.

Proposal 4: For FR2, consider enhancements to PDSCH performance to recover the coverage loss due to UE complexity reduction.

Proposal 5: For Msg2 PDSCH with TBS scaling, multi-TB scheduling can be considered for TDMed RAR transmission over multiple slots.

Proposal 6: For FR1, consider the slot-aggregation or the low-SE MCS table for coverage recovery of Msg4 PDSCH.

Proposal 7: For FR1, consider DMRS bundling and frequency hopping across a larger BW for coverage recovery of unicast PDSCH.

Proposal 8: For FR1, consider repetition or CORESET bundling for coverage recovery of broadcast PDCCH.

Proposal 9: For FR1, coverage recovery techniques for uplink channels (i.e. Msg3, PUCCH unicast PUSCH) may reuse some of the techniques discussed in the Rel-17 CE SI.

Proposal 10: For FR2, consider power and resource efficient solutions for coverage recovery in FR2, including:

- Beam refinement

- Hopping across a larger system frequency range (e.g., cross CC)

- Enhanced L1/L2 inter-cell mobility (optimized to RedCap use cases)

Proposal 11: For FR2, consider ways to reduce time blockage and/or resources by taking into consideration the larger latency requirements for some use cases.

Proposal 12: For FR2, consider techniques to reduce the payload size for the L1 measurement report.

# References

- RP-201386, Revised SID on study on support of reduced capability NR devices, Ericsson, 3GPP TSG RAN Meeting #88e, June 29th – July 3rd , 2020

- 3GPP RAN1#102-e, Chairman’s notes, May 2020

# Appendix

Required SNR at the target performance for reference and RedCap UE for FR1.

| Parameter | Rural 700MHz | Urban 2.6GHz | Urban 4GHz |
| --- | --- | --- | --- |
| PDCCH CSS | 2Rx: -8.5 dB 1Rx: -4.7 dB | 4Rx: -11.7 dB 2Rx: -9.2 dB 1Rx: -5.9 dB | 4Rx: -11.7 dB 2Rx: -9.2 dB 1Rx: -5.9 dB |
| Msg2 PDSCH | 2Rx: -3 dB 1Rx: 1.5 dB | 4Rx: -6.5 dB 2Rx: -3.2 dB 1Rx: 0.7 dB | 4Rx: -6.5 dB 2Rx: -3.2 dB 1Rx: 0.7 dB |
| Msg4 PDSCH | 2Rx: -5 dB 1Rx: -1.6 dB | 4Rx: -8 dB 2Rx: -5.5 dB 1Rx: -2.2 dB | 4Rx: -8 dB 2Rx: -5.5 dB 1Rx: -2.2 dB |
| Unicast PDSCH | 2Rx: -4.6 dB 1Rx: -1.2 dB | 4Rx: -6.6 dB 2Rx: -4 dB 1Rx: -0.9 dB | 4Rx: -5.7 dB 2Rx: -3 dB 1Rx: 0.2 dB |
| Msg3 | -2.5 dB | -6 dB | -6 dB |
| Unicast PUSCH | -3 dB | -3.9 dB | -6 dB |
| PUCCH PF3 22 bits | -2 dB | -5 dB | -5 dB |