3GPP TSG- RAN WG1 Meeting #103-e                                                   R1- 2009333

# e-Meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.6

Source:	Apple Inc.

Title:	Outcome on 103-e-NR-eMIMO-02

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide a summary on email discussion 103-e-NR-eMIMO-02.

During the discussion, the following agreements have been achieved.

Agreement There is no consensus to confirm the working assumption to support L1-SINR measured based on both ZP and NZP IMR - The TP in R1-2008571 is endorsed in R1-2009506 (TS38.214 Rel-16, CR#0139, Cat. F). Agreement - For CBRA based PCell/PSCell BFR, if Msg3 or MsgA of CBRA-BFR contains BFR MAC CE, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of CBRA BFR as described in Clause 5.1.5 of 38.321, UE transmits PUCCH in the Pcell/PSCell using the same spatial domain filter as that used for the latest PRACH transmission and with power control parameters q_u = 0, q_d = q_new and l = 0, if the PUCCH resource is provided with PUCCH-SpatialRelationInfo - q_new is the SSB selected for the PRACH transmission - Send an LS to RAN2 to tell RAN2 that RAN1 has made the agreement above and whether corresponding RRC parameter and UE capability needs to be introduced to enable BFR MAC CE for CBRA based PCell/PSCell BFR is a RAN2 issue. - LS is endorsed in R1-2009519 Agreement When one or two resource settings are configured for L1-SINR measurement - If a UE is not configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the SSB or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. - If a UE is configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SSB or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. - If a UE is not configured with higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig, the UE shall derive the interference measurements for computing L1-SINR reported in uplink slot n based on only the CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement no later than the CSI reference resource associated with the CSI resource setting. - If a UE is configured with higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig, the UE shall derive the interference measurements for computing the L1-SINR reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement associated with the CSI resource setting. Agreement The TP in section 5.1 of R1-2009332 is endorsed for the 38.214 alignment CR Agreement The TP in section 5.2 of R1-2009332 is endorsed for the 38.213 alignment CR Agreement The text proposal on CBRA based BFR for TS38.213 is endorsed in R1-2009685(TS38.213 Rel-16, CR#0173, Cat. F) The text proposal on Measurement Restriction for L1-SINR for TS38.214 is endorsed in R1-2009686 (TS38.214 Rel-16, CR#0146, Cat. F)