3GPP TSG RAN WG1 Meeting #103-e										R1-2009428

e-Meeting, October 26th – November 13th, 2020	(revision of R1-2008709)

Agenda Item:	8.5.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Evaluation of positioning enhancements

Document for:	Discussion & Decision

# Introduction

In this contribution, we are evaluating the positioning performance for UL-TDOA in InF scenarios. The evaluations are based on the agreements in RAN1#101-e and RAN1#102-e for the baseline and optional complementary parameters. We provide evaluation results on the achievable positioning accuracy and focus on the impact of interference, antenna configuration and UL transmit power.

We focused on two scenarios:

- UMi:

  - ISD = 200m

  - LOS/NLOS probability according to TR38.901

  - ATOA-Model

    - Baseline (According to UMi TR38.901)

    - Apply ATOA model for NLOS UMi links.

- InF-DH (with r=0.4, hc =2m dClutter=2m)

  - Main focus is on the increase of network and power efficiency

    - The network efficiency can be increased if several UEs share the same resource elements

    - We studied the impact of the transmit power. Lowering the transmit power reduces the interference to other links.

# Rel-16 Analysis

All simulations use “1 occasion” one SRS transmission per UE drop. If the UE uses several TX beams a sequential transmission is assumed.

Table: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | Case 1- config 445 (InF-DH, FR1,UL-TDOA) | Case 2- config1011 (UMi, FR1,UL-TDOA) | Case 3- config config1112 (UMi, FR1,UL-TDOA) |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (r =0.4,hc=2m, dClutter=2m) | UMi (ATOA modelling with mu=-8) | UMi (No ATOA modelling) |
| Carrier frequency | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 4 | KTC = 8 nbSymb = 8 | KTC = 8 nbSymb = 8 (o/w KTC = 4 nbSymb = 4; or KTC = 2 nbSymb = 1) |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port |
| Number of sites | 18 | 19 | 19 |
| Number of symbols used per occasion | 4 | 8 | 8/4/1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | Power boosting by comb factor | Power boosting by comb factor | Power boosting by comb factor |
| Uplink power control (applied/not applied) | +23dBm | +23dBm | +23dBm |
| interference modelling (ideal muting, or other) | No Interference (no resource sharing) | No Interference (no resource sharing) | No Interference (no resource sharing) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt |
| Network synchronization assumptions | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Not applied | Not applied | Not applied |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | - Per site (Rx): 3 sectors (3 panels, 5 dual polarized beams - 1 TXRU per panel per polarization dimension - UE (Tx): Omni pattern per element, 2 dual polarized antennas (1,2,2), random orientation of UE - 4 sequential SRS transmissions | - Per site (Rx): 3 sectors (= 3 panels, 5 dual polarized beams per panel) - 1 TXRU per panel per polarization dimension - UE (Tx): Omni antenna - One polarization used for UE-TX - Random UE orientation (random polarization orientation | - Per site (Rx): Omni antenna - 1 TXRU per panel per polarization dimension - UE (Tx): Omni antenna - One polarization used for UE-TX - Random UE orientation (random polarization orientation |
| Additional notes, if any | No LOS detection | LOS detection not applicable (number of LOS links not sufficient, all Links are used) | LOS detection not applicable (number of LOS links not sufficient, all Links are used) |

Observation 1:	 Following performance was observed based on Rel-16 evaluations:

        - For InF-DH in FR1 a target performance ≤ 0.5m @ 90% is not achieved.

        - For UMi in FR1, due to the low probability of LOS links the target performance ≤ 1m @ 80% is not achieved.

    - With ATOA modelling the error will further increase.

The observation is made based on the following evaluations:

## InF-DH (no LOS detection FR1)

Reliable LOS detection is a key issue for high accuracy positioning. LOS identification enhancements may be achieved with implementation based approaches. In this evaluation only a quality value per link is forwarded to the positioning engine.

- Config 445: No LOS detection (all RTOA values are forwarded to the LMF with signal quality information like RSRP only).

- Config 442 (for comparison): Ideal LOS detection

Figure 1- CDF 2D (horizontal) error for UTDOA in InF for Rel-16 evaluation.

## UMi (no LOS detection FR1)

ATOA modelling:

  - Config 1012: according to TR38.901.

  - Config 1011: Complementary ATOA model.

Figure 2-CDF 2D (horizontal) error for UTDOA in UMi for Rel-16 evaluation

# Rel-17 Positioning evaluation

We compared different configurations to identify relevant enhancements in Rel-17. The details on each configuration used are provided in the table at the end of this section. The methods, signaling and procedures, if needed, are subject to the discussion in agenda item. 8.5.3.

## Impact of gNB antenna configuration

In the SLS-2D- and 3D-results (Figure 3 and Figure 4) we characterize the performance w.r.t. to the gNB antenna configuration. Three different gNB antenna configurations are considered:

- Config 220: Omni antenna.

- Config 320: 3 sectors (3 panels, one element per panel is used).

- Config 420: 3 panels, 5 beams per panel.

The UEs assumed for these results are characterized by an omni antenna and a high transmit power (23dBm). An omni-antenna may receive more multipath components, hence Config 420 outperforms the other configurations. The evaluations in the following sections use configuration gNB antenna configuration Config-4xx for the further detailed analysis on the uplink transmit power and the UE antenna assumptions.

Observation 2:	 Following performance was observed based on Rel-17 evaluations:

        - For InF-DH in FR1 a performance ≤ 0.5m @ 90% is achieved.

        - For InF-DH in FR2 a performance ≤ 0.2m @ 90% is achieved.

Observation 3: 	Using antennas with higher directivity increase the performance.

Figure 4-CDF 2D (horizontal) error for UTDOA, comparison of different gNB antenna configurations

Figure 5 - CDF 3D error for UTDOA, comparison of different gNB antenna configurations

## Impact UE Tx Power in the presence of interference, SRS with one symbol

Due to the low ISD and the high BS antenna gain of the config-4xx configuration a low UE TX-power may be sufficient. We compared for the UE power analysis the following configurations:

- Config 420: +23dBm (applied configuration in Figure 3and Figure 4).

- Config 422:  -10dBm.

- Config 423:  -20dBm.

It can be observed from Figure 5 and Figure 6 that a low UE-TX power is sufficient in the InF scenario. In the example we see that for 0dBm shows almost no performance degradation compared to +23dBm.

Observation 4: 	A low UE-TX power is sufficient for gNBs with beam forming in InF scenarios.

Figure 6 - CDF horizontal error for UTDOA, comparison of different UE TX Power levels

Figure 7 - CDF 3D error for UTDOA, comparison of different UE Tx Power levels.

For this analysis, we assumed interference between UEs sharing the same RE (“cyclic shift multiplex”). The reported high accuracy performances (with low transmission power) in the presence of interference by non-orthogonal signals (e.g. SRS with different root sequence, data or another RS) is not feasible. The low TX-power is only feasible if all other UEs sharing the same resources use orthogonal sequences (either different COMB offset and/or different cyclic shifts and the cyclic shift step size is sufficient).

Observation 5:	When considering interference, the performance with low TX power is only feasible if UEs sharing the same REs use different cyclic shifts for the same SRS root sequence (“cyclic shift multiplex”).

Proposal 1:	For “low power operation” in combination with interference between positioning-SRS resources, cyclic shift enhancements shall be considered in Rel-17.

Using several symbols and staggering with phase correction it is even feasible to further reduce the TX power. Without phase correction in the transmitter (UE) a coherent destaggering in the receiver (gNB) is only feasible, if no other UEs share the same REs.

## Supporting “TX antenna diversity”

The baseline simulation in FR1 assumes an omni-directional antenna. If a low TX power is used the gain variation of a non-ideal UE antenna becomes critical. To study this effect we compare

- Config 423: 	Low power (-20dBm) operation (as above).

- Config 422: 	Medium power (0dBm) operation (as above).

- Config 444:	Low power operation, 4 identical UE antennas, the 4 antennas are at different position at the UE, sequential transmission of the 4 SRS resources.

- Config 443: 	Low power operation, 4 non-ideal UE antennas (app. 5dB gain variation), the 4 antennas are at different position at the UE, sequential transmission of the 4 SRS resources.

Figure 8 - CDF horizontal error (with LOS detection),

Figure 9 - CDF 3D error for UTDOA (with LOS detection),

Observation 6: 	Using TX-diversity (4 transmissions at -20dBm from different antennas) shows a similar performance as single port transmission with 0dBm.

Multi-port SRS transmission reduces the number of OFDM symbols needed compared to sequential transmission.

Proposal 2: 	Support multi-port transmission in Rel-17.

## Uplink beam management evaluation

### Tx-Rx Beam pair selection criteria

One important criteria for uplink based methods (UL-TDOA, multi-RTT) using beam based operation is the choice of the Tx-Rx beam pair for timing. We compared:

- Config 420: CIR analysis is used for the beam selection/spatial relationship establishment.

- Config 421: only power (LOS power and multipath power) is used for selection of the spatial relationship (a beam with low gain for the LOS path was selected).

Figure 11 and Figure 12 show that an efficient beams management needs more than the analysis of total power as a base for selection. An approach for selecting the links based on RSRP reports is not reliable for positioning (Config 421). For better establishing the spatial relationship several methods can be considered. In Config 420 we implemented an enhanced method (not supported in Rel-16) evaluating the CIR and using the power of the first arriving taps only.

Figure 10 - CDF horizontal error UTDOA, Beam pair selection

Figure 11 - CDF 3D error for UTDOA, Beam pair selection

Observation 7: 	The beam selection and spatial relationship establishment criteria have major impact on the performance.

### “Narrow” TX beams

For FR2, the UE may use antenna panels with narrow TX beams [2]. For positioning, each transmit direction may be required. This may demand many SRS resources and advanced ToA analysis at the TRP. To study the performance of a UE with narrow beams compared to a UE with “omni like pattern” we implemented FR2 like antenna panels also at FR1 to evaluate the beam management and spatial relation configuration.

We compared:

- Config 447: 4 antenna panels (similar to FR2) at FR1, one element per panel is used (i.e.. the element has a beam width of app. 120deg).

- Config 552: Same config at FR2; for FR2 a higher TX power is assumed to compensate the higher pathloss at FR2.

- Config 554: 400MHz BW, same antenna config as config 552 and 0dBm Tx power.

- Config 423: For reference, a FR1 configuration with low TX power.

With an ideal beam management and ToA analysis a configuration with (many) narrow beams should perform equal or better than a configuration with wide beams (and low TX power).

Figure 12-CDF horizontal error UTDOA

Figure 13 - CDF 3D error for UTDOA, FR2 Tx Beams

Observation 8: 	When applying narrow beam antenna configuration in FR2 over multiple SRS resources, and in the presence of interference, a degradation in the positioning performance is observed. In case of several TX beams a RTOA selection/combining strategy is required. This algorithm may need further enhancements.

Figure 14 - CDF 2D error for UTDOA, FR2 (400MHz BW)

Table 8.2.1.8.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | Case 4- config 220 (InF-DH, FR1,UL-TDOA) | Case 5- config 320 (InF-DH, FR1,UL-TDOA) | Case 6- config 420 (InF-DH, FR1,UL-TDOA) | Case 7- config 421 (InF-DH, FR1,UL-TDOA) |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) |
| Carrier frequency | 4GHz | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port |
| Number of sites | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 1 | 1 | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | +23dBm | +23dBm | +23dBm | +23dBm |
| interference modelling (ideal muting, or other) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Not applied | Not applied | Not applied | Not applied |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps | RSRP used for Rx beam selection |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | - Per site (Rx): Omni antenna - 1 TXRU per panel per polarization dimension - UE (Tx): Omni antenna | - Per site (Rx): 3 sectors (3 panels, one element per panel) - 1 TXRU per panel per polarization dimension - UE (Tx): Omni antenna | - Per site (Rx): 3 panels, 5 dual polarized beams per panel - 1 TXRU per panel per polarization dimension - UE (Tx): Omni antenna | - Per site (Rx): 3 panels, 5 dual polarized beams per panel - 1 TXRU per panel per polarization dimension - UE (Tx): Omni antenna |
| Evaluated Enhancement for Rel.17 | - Ideal LOS detection | - Ideal LOS detection - CIR analysis is used for the beam selection/spatial relationship establishment | - Ideal LOS detection - CIR analysis is used for the beam selection/spatial relationship establishment | - Ideal LOS detection |
| Additional notes, if any | If the number of LOS links is sufficient, only LOS links are used for the position calculation (ideal LOS detection). If the number of LOS links is not sufficient all ToA-Values together with a signal power based signal quality are used by the positioning algorithm | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) |

| Parameter | Case 8- config 422 (InF-DH, FR1,UL-TDOA) | Case 9- config 423 (InF-DH, FR1,UL-TDOA) | Case 10- config 443 (InF-DH, FR1,UL-TDOA) | Case 11- config 444 (InF-DH, FR1,UL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) |
| Carrier frequency | 4GHz | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port (4 resources) | SRS, 1 Port (4 resources) |
| Number of sites | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 1 | 1 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | 0dBm | -20dBm | -20dBm | -20dBm |
| interference modelling (ideal muting, or other) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Not applied | Not applied | Not applied | Not applied |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | - Per site (Rx): 3 panels, 5 dual polarized beams per panel - 1 TXRU per panel per polarization dimension - UE (Tx): Omni antenna | - Per site (Rx): 3 panels, 5 dual polarized beams per panel - 1 TXRU per panel per polarization dimension - UE (Tx): Omni antenna | - Per site (Rx): 3 panels, 5 dual polarized beams per panel - 1 TXRU per panel per polarization dimension - UE (Tx): 4 non-ideal antennas at different positions | - Per site (Rx): 3 panels, 5 dual polarized beams per panel - 1 TXRU per panel per polarization dimension - UE (Tx): 4 ideal antennas at different positions |
| Evaluated Enhancement for Rel.17 | - Ideal LOS detection - CIR analysis is used for the beam selection/spatial relationship establishment | - Ideal LOS detection - CIR analysis is used for the beam selection/spatial relationship establishment | - Ideal LOS detection - CIR analysis is used for the beam selection/spatial relationship establishment - Gain of TX diversity with non-ideal UE antenna | - Ideal LOS detection - CIR analysis is used for the beam selection/spatial relationship establishment - Gain of TX diversity using 4 ideal UE antennas |
| Additional notes, if any | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) |

| Parameter | Case 12- config 447 (InF-DH, FR1,UL-TDOA) | Case 13- config 552 (InF-DH, FR2,UL-TDOA) | Case 14- config 554 (InF-DH, FR2,UL-TDOA |  |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) |  |
| Carrier frequency | 4GHz | 28GHz | 28GHz |  |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz |  |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400Mhz |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 4 |  |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port (sequential transmissions) |  |
| Number of sites | 18 | 18 | 18 |  |
| Number of symbols used per occasion | 4 | 4 | 12*4 (12 different UE TX beams, 4 symbols per SRS resource) |  |
| number of occasions used per positioning estimate | 1 | 1 | 1 |  |
| Power-boosting level | 6dB | 6dB | 6dB |  |
| Uplink power control (applied/not applied) | 23dBm | 23dBm | 0dBm |  |
| interference modelling (ideal muting, or other) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt |  |
| Network synchronization assumptions | Ideal | Ideal | Ideal |  |
| UE/gNB Tx/Rx Calibration Error | Not applied | Not applied | Not applied |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | - Per site (Rx): 3 panels, 5 dual polarized beams per panel - Virtualization model: 1 TXRU per panel per polarization dimension - UE (Tx): 4 antenna panels (same characteristic as FR2 antenna), one element per panel | - Per site (Rx): 3 panels, 5 dual polarized beams per panel - Virtualization model: 1 TXRU per panel per polarization dimension - UE (Tx): 4 antenna panels | - Per site (Rx): 3 panels, 5 dual polarized beams per panel - Virtualization model: 1 TXRU per panel per polarization dimension - UE (Tx): 4 antenna panels |  |
| Evaluated Enhancement for Rel.17 | - Ideal LOS detection - CIR analysis is used for the beam selection/spatial relationship establishment | - Ideal LOS detection - CIR analysis is used for the beam selection/spatial relationship establishment | - Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment |  |
| Additional notes, if any | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) |  |

# Network efficiency

Network efficiency is evaluated for UL SRS resource utilization for the following cases:

- UL SRS resource utilization w.r.t to the cyclic shift enhancements

- UL SRS resource utilization w.r.t to the number of OFDM symbols

- UL SRS resource utilization w.r.t to the number of antenna ports

UL SRS resource utilization w.r.t to the cyclic shift enhancements:

The number of simultaneous UEs of CS separated is chosen based on the evaluation so that the configured with different cyclic shifts on the same REs are detected without ambiguity.

| Scenario | Rel-16/ Rel-17 enhancements | COMB | nbSym | Cyclic shift range | max number of CSs | Number of simultaneous UEs of CS separated | Improvement factor vs Rel16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Umi | CS enhancement | 4 | 4 | fftlength | 48 | 8 | 4 |
|  | Rel-16 | 4 | 4 | fftlength/KTC | 12 | 2 |  |
| Umi | CS enhancement | 8 | 8 | fftlength | 48 | 8 | 8 |
|  | Rel-16 | 8 | 8 | fftlength/KTC | 6 | 1 |  |
| InF | CS enhancement | 2 | 1 | fftlength/KTC | 8 | 8 | - |
|  | Rel-16 | 2 | 1 | fftlength/KTC | 8 | 8 |  |

Observation 9: 	Cyclic shift enhancements SRS improves the network efficiency for UMi scenarios by a factor of 4-8 .

The following analysis shows that UMi performance with resource sharing doesn’t degrade compared with the case when no resource sharing is configured:

- Config 1011.2 (blue): resource sharing : Comb 4, 4 symbols, CS enhancement, phase correction in transmitter (Rel-17), 6 UEs use the same REs and are separated by different cyclic shift values

- Config 1011.2 (red) , no resource sharing : Comb 4, 4 symbols, no phase correction, Coherent Destaggering with phase correction in receiver (Rel-16)

Figure 15 2D-Error in UMi scenario SRS (Comb 4, 4 symbols) applying ATOA model. Blue: CS enhancement (Rel-17), 6 UEs use the same REs and are separated by different cyclic shift values. Red: no resource sharing.

UL SRS resource utilization w.r.t to the number of OFDM symbols:

For scenarios with low delay spread and low propagation delay difference (e.g. indoor with low ISD) COMB=4 may be sufficient. Rel-16 does not allow SRS for positioning with COMB=4, 1 Symbol. Alternatively a Rel-15 SRS may be supported for positioning purpose.

| Scenario | Rel-16/ Rel-17 enhancements | COMB | nbSym | max number of CSs | Number of simultaneous UEs of CS separated | REs per UE and per RB | Improvement factor vs Rel16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| InF | SRS with one symbol for COMB 4 | 4 | 1 | 12 | 6 | 3 | 2 |
|  | Rel-16 | 4 | 2 | 12 | 6 | 6 |  |

UL SRS resource utilization w.r.t to the number of antenna ports:

For UL SRS resource utilization we compare the improvement factor in terms of the Capacity per UE for Rel-16 and Rel-17 enhancements. We derive the improvement factor as Capacity per UERel-16 Capacity per UERel-17  ,  here  the Capacity per UE=OFDMsymbolsUE CombsizeN.

| Scenario | Rel-17 enhancements | # of Tx Beams | COMB | nbSym | OFDM symbols per UE | Capacity per UE | Improvement factor vs Rel16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1-Umi | 2-ports | 2 | 4 | 4 | 4 | 1 | 2 |
|  | Rel-16 | 2 | 4 | 4 | 8 | 2 |  |
| FR1-InF | 2-ports | 2 | 2 | 1 | 1 | 0,5 | 2 |
|  | Rel-16 | 2 | 2 | 1 | 2 | 1 |  |
| FR2-Umi | 4-ports | 4 | 8 | 8 | 8 | 2 | 4 |
|  | Rel-16 | 4 | 8 | 8 | 32 | 8 |  |
| FR2-InF (SCS=120kHz) | 4-ports | 4 | 4 | 4 | 4 | 1 | 4 |
|  | Rel-16 | 4 | 4 | 16 | 16 | 4 |  |

Observation 10: 	Multi-port SRS transmission improves the network efficiency in terms of SRS resource compared to Rel-16 utilization by a factor:

# 4 in terms of SRS resource utilization for 4 ports SRS.

- 2 in terms of SRS resource utilization for 2 ports SRS.

# Conclusions

We conclude with the following observations and proposals:

Observation 1:	 Following performance was observed based on Rel-16 evaluations:

        - For InF-DH in FR1 a target performance ≤ 0.5m @ 90% is not achieved.

        - For UMi in FR1, due to the low probability of LOS links the target performance ≤ 1m @ 80% is not achieved.

    - With ATOA modelling the error will further increase

Observation 2:	 Following performance was observed based on Rel-17 evaluations:

        - For InF-DH in FR1 a performance ≤ 0.5m @ 90% is achieved.

        - For InF-DH in FR2 a performance ≤ 0.2m @ 90% is achieved.

Observation 3: 	Using antennas with higher directivity increase the performance.

Observation 4: 	A low UE-TX power is sufficient for gNBs with beam forming in InF scenarios.

Observation 5:	When considering interference, the performance with low TX power is only feasible if UEs sharing the same REs use different cyclic shifts for the same SRS root sequence (“cyclic shift multiplex”).

Observation 6: 	Using TX-diversity (4 transmissions at -20dBm from different antennas) shows a similar performance as single port transmission with 0dBm.

Observation 7: 	The beam selection and spatial relationship establishment criteria have major impact on the performance.

Observation 8: 	When applying narrow beam antenna configuration in FR2 over multiple SRS resources, and in the presence of interference, a degradation in the positioning performance is observed. In case of several TX beams a RTOA selection/combining strategy is required. This algorithm may need further enhancements.

Observation 9: 	Cyclic shift enhancements SRS improves the network efficiency for UMi scenarios by a factor of 4-8

Observation 10: 	Multi-port SRS transmission improves the network efficiency in terms of SRS resource compared to Rel-16 utilization by a factor:

- 4 in terms of SRS resource utilization for 4 ports SRS.

- 2 in terms of SRS resource utilization for 2 ports SRS.

Proposal 1:	For “low power operation” in combination with interference between positioning-SRS resources, cyclic shift enhancements shall be considered in Rel-17.

Proposal 2: 	Support multi-port transmission in Rel-17.

# References

- RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019.

- R1- 2004650, “Additional Scenarios for performance evaluations”, Ericsson, e-Meeting, May 25th – June 5th, 2020.

# Annex

# Simulation setup overview

The simulation setup features are summarized in the following table:

| Parameter | Description | Used for R1-2009428 |
| --- | --- | --- |
| Network | The setup supports different deployment scenarios (ISD, number of BS, hexagon or other grid, ….) | InF-grid, 20m, 18BS UMi, ISD=200m, 19BS |
| Channel | All channels as defined by TR38.901 | InF_LOS InF_NLOS_DH UMi (with random LOS/NLOS accorind TR38.901 |
| Noise | Noise is inserted after RX-antenna beamforming (assuming the beam forming losses are already included in the antenna virtualization) |  |
| LOS/NLOS model | Random selection of LOS/NLOS receive scenario. The model as defined by 3GPP supports distance dependent LOS probability. | Inline with the agreements in RAN1#101-e : r = 0.4 hc = 2m dClutter = 2m For UMi: According TR38.901 |
| gNB antenna | Different antennas are supported - Omni (for comparison) - 3 panels per BS, 1 element per panel is used, antenna pattern per element (inline with the agreements) - 3 panels per BS, several beams per panel, beams are generated assuming using the “antenna virtualization” method. | The used antenna config is defined by the config number Xxx; where 2xx omni antennas for BS 3xx Three sectors, 1 element per sector 4xx Three sectors, FR1 antenna panel config, several beams 5xx Three sectors, FR2 antenna panel config, several beams |
| UE antenna | FR1: baseline one Omni Optional: several antenna elements at different positions of the UE, “ideal antenna element”, “non-ideal antenna element with 5dB gain variation”, 3GPP antenna element FR2: 4 panels with 4 elements each - One element/one polarization per panel is used - Several elements (and beam forming) per panel | For FR1: - one Omni, one polarisation - Up to 4 elements (2 dual polarized elements) For FR2: - 4 panels (4 elements each), one element is used - 4 panels, 3 beams per panels (12 beams total) |
| UE dropping | Different dropping modes including - “random dropping” - Random dropping with short tracks to emulate the movement of the UE in case of sequential measurements. - Long tracks (emulating the move movement of the UE and changes of the receive scenario along the track. This mode is especially foreseen for enhanced positioning algorithms using Kalman-Filter - “group dropping” (UEs sharing the same resources are in the same area | Random dropping |
| Frequency range | carrier frequency | FR1 (4GHz) and FR2 (28GHz) |
| Signal bandwidth and subcarrier spacing | According to Rel-16 | FR1: 100MHz, SCS=30kHz |
| SRS pattern | The preferred configuration depends on the network (ISD), the UE capabilities (available TX-power) and BS antenna gain. The COMB factor, number of symbols, staggering on/off, etc. depends on the link budget, the expected delay spread and the level difference of the signals transmitted from different UEs. We assume “cyclic shift multiplex” (several UEs share the same REs and are separated by different cyclic shifts only). KTC= 4 supports 12 cyclic shift steps. This allows that up to 12 UEs can share the same REs. KTC= 8 supports 6 cyclic shift steps | 5 configuration: KTC = 4, nbSym = 1 (not allowed in Rel 16) KTC = 4, nbSym = 4 KTC = 2, nbSym = 1 KTC = 2, nbSym = 2 KTC = 8, nbSym = 8 For InF scenarios there are no/minor link budget issues. Hence 1 OFDM symbol per SRS is sufficient. The delay spread is low. Therefore, high COMB factors can be used. Default configuration is KTC = 4, 6 UEs share the same REs (Interference included in all results) |
| UE antenna ports | Inline, with Rel-16 we assume for SRS for positioning only one TX-port can be configured. The setup supports also UEs with several antennas. Two modes are supported - Several SRS resources are configured targeting different UE spatial TX filter. If only one TX-port is allowed, the SRS are transmitted sequential (no interference between SRS of the same UE) - The UE supports also parallel transmission over several antenna ports (multi-Port SRS transmissions) | One TXRU is active per OFDM symbol. If different SRS resources are configured as sequential transmission is assumed. |
| Beam management | The simulation setup supports beam management. For positioning purpose we used: - The gNB transmits a DL-RS (SSB or DL-PRS) - The UE reports the signal strength for each received RS - The spatial relationship for the SRS transmissions is configured accordingly - The gNB receives the SRS of this beam only. | DL-RS signal power based beam selection. |
| Timing advance | Ideal timing advance setting according to the serving gNB (s-gNB). The nearest gNB is assumed as s-gNB. Optional: non-ideal timing advance | Ideal timing advance |
| TX power control | In case of omni-directional a simplified power control can be used. The configured TX power is a trade-off between the - Generated interference - Probability that a faraway gNB can detect the signal To study this tradeoff we use a fixed setting of the TX power per UE and assume 3 power level - +23dBm - 0dBm - -20dBm Other TX power control methods (especially methods for UEs using non-omni-antennas) are FFS. | Constant TX power setting |
| ToA- Measurements | Cyclic correlation, selection of the time window according the configured cyclic shift, evaluation of the correlation function including threshold based “first arrival path detection” (FAP) and evaluation of the first rising edge of the correlation function (“IFP”). Other ToA-Estimators: MUSIC, Machine Learning | FAP+IFP |
| RTOA data analysis | Especially in case of UE-TX antennas with high directivity the signal quality of the different SRS transmissions may be significantly different. It may happen that the FAP can’t be detected (depending on noise and interference). If several SRS are transmitted per UE the gNB used a signal power based selection of the ToA measurement further used by the positioning algorithm. | Quality selection: Signal power |
| LOS-Detection | Taking into account the high ToA-error introduced by the ATOA model of TR38.901 it is essential to detect the LOS links. Different concept are currently FFS | The LOS detection algorithm is under development and not yet used for the simulation. No or Ideal LOS detection is assumed |
| Positioning algorithm | We distinguish “single shot positioning” (= one SRS resource set is used per positioning event) and “positioning with tracking” (e.g. Kalman filter based positioning algorithms. As single shot positioning algorithm we use - Bancroft - Levenberg-Marquardt | Levenberg-Marquardt |