3GPP TSG RAN WG1 Meeting #103-e 	R1-2009433

E-meeting, October 26th – November 11th, 2020

Agenda Item:	8.5.2

Source:	Huawei, HiSilicon

Title:	Evaluation results for Rel-16 positioning and Rel-17 enhancement

Document for:	Discussion and decision

Introduction

Revised from R1-2007576.

In this contribution, we will provide the evaluation results for Rel-16 positioning method and Rel-17 potential enhancement on the following aspects.

- Rel-16 positioning methods

  - Accuracy

  - Latency

- Rel-17 potential enhancements

  - Accuracy

  - Latency

  - Network efficiency

  - UE efficiency

Evaluations for Rel-16 positioning methods

Accuracy

General description

The scenarios to be presented in this contribution include

- Baseline scenarios with InF-SH/InF-DH with fixed UE/gNB height and without UE/gNB calibration error

- InF-DH with variable UE/gNB height

- InF-SH/In-DH with UE/gNB calibration error

Baseline scenario

Evaluated cases

The following cases in Table 1 are evaluated for FR1 and FR2. We will evaluate DL positioning, UL positioning, and DL+UL positioning solutions in InF-SH and InF-DH. In the following, the results are provided for DL-TDOA, UL-TDOA, UL+TDOA+UL-AOA and Multi-RTT in FR1, DL-TDOA, DL-TDOA+DL-AOD, UL+TDOA+UL-AOA and Multi-RTT in FR2.

Table 1 Evaluated parameter combinations

| Parameter | Case 1 (InF-SH, FR1) | Case 2 (InF-SH, FR1) | Case 3 (InF-SH, FR1) | Case 4 (InF-SH, FR1) | Case 5 (InF-DH, FR1) | Case 6 (InF-DH, FR1) | Case 7 (InF-DH, FR1) | Case 8 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |  |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal |  |  |  |  |  |  |  |
| UE/gNB Tx/Rx Calibration Error | Ideal |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |  |  |  |  |
| Additional notes, if any | Fixed UE/gNB height. |  |  |  |  |  |  |  |
| Parameter | Case 9 (InF-SH, FR2) | Case 10 (InF-SH, FR2) | Case 11 (InF-SH, FR2) | Case 12 (InF-SH, FR2) | Case 13 (InF-DH, FR2) | Case 14 (InF-DH, FR2) | Case 15 (InF-DH, FR2) | Case 16 (InF-DH, FR2) |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |  |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal |  |  |  |  |  |  |  |
| UE/gNB Tx/Rx Calibration Error | Ideal |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping Rx beam sweeping |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based Rx codebook based |  |  |  |  |  |  |  |
| Additional notes, if any | Fixed UE/gNB height. |  |  |  |  |  |  |  |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 2.

Table 2  Positioning accuracy (in unit of meter)

|  | Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 1, InF-SH, FR1, DL-TDOA | All UE | 0.077 | 0.153 | 0.419 | 2.789 | 10.288 |
|  |  | Central UE | 0.053 | 0.090 | 0.224 | 1.964 | 8.759 |
|  | 2, InF-SH, FR1, UL-TDOA | All UE | 0.0871 | 0.1708 | 0.4013 | 1.8678 | 5.8576 |
|  |  | Central UE | 0.0621 | 0.1096 | 0.2410 | 1.0277 | 3.9456 |
|  | 3, InF-SH, FR1, UL-TDOA/AoA | All UE | 0.0465 | 0.0731 | 0.1271 | 0.299 | 0.7889 |
|  |  | Central UE | 0.0434 | 0.0648 | 0.1116 | 0.2682 | 0.5521 |
|  | 4, InF-SH, FR1, Multi-RTT | All UE | 0.0721 | 0.2035 | 0.6628 | 1.9481 | 4.4855 |
|  |  | Central UE | 0.0665 | 0.1798 | 0.5628 | 1.6992 | 4.3519 |
|  | 5, InF-DH422, FR1, DL-TDOA | All UE | 0.861 | 3.448 | 8.103 | 15.212 | 23.030 |
|  |  | Central UE | 0.703 | 2.904 | 8.399 | 15.635 | 23.030 |
|  | 6, InF- DH422, FR1, UL-TDOA | All UE | 0.4309 | 1.6317 | 4.9693 | 9.9453 | 16.8111 |
|  |  | Central UE | 0.3354 | 1.2393 | 4.6237 | 9.6631 | 16.6620 |
|  | 7, InF-DH422, FR1, UL-TDOA/AoA | All UE | 0.0702 | 0.1673 | 0.3825 | 1.0453 | 2.8012 |
|  |  | Central UE | 0.0643 | 0.1529 | 0.3206 | 0.8016 | 2.1747 |
|  | 8, InF- DH422, FR1, Multi-RTT | All UE | 0.0699 | 0.1260 | 0.3916 | 7.8992 | 15.2156 |
|  |  | Central UE | 0.0667 | 0.1164 | 0.3338 | 7.3110 | 14.4738 |
| FR2 | 9, InF-SH, FR2, DL-TDOA | All UE | 0.0089 | 0.0329 | 0.1935 | 1.1281 | 4.4335 |
|  |  | Central UE | 0.0077 | 0.0265 | 0.1586 | 0.9633 | 4.1343 |
|  | 10, InF-SH, FR2, DL-TDOA/AoD | All UE | 0.0118 | 0.0203 | 0.0345 | 0.0918 | 0.2518 |
|  |  | Central UE | 0.0143 | 0.0231 | 0.0387 | 0.0654 | 0.1293 |
|  | 11, InF-SH, FR2, UL-TDOA/AoA | All UE | 0.0102 | 0.0191 | 0.0365 | 0.0991 | 0.3826 |
|  |  | Central UE | 0.0084 | 0.0156 | 0.0296 | 0.0694 | 0.2516 |
|  | 12, InF-SH, FR2, Multi-RTT | All UE | 0.0049 | 0.0143 | 0.0881 | 0.4697 | 1.4975 |
|  |  | Central UE | 0.0048 | 0.0137 | 0.0841 | 0.4496 | 1.3076 |
|  | 13, InF-DH422, FR2, DL-TDOA | All UE | 0.0466 | 0.5795 | 3.6866 | 9.2401 | 16.2349 |
|  |  | Central UE | 0.0321 | 0.4003 | 2.7974 | 9.6798 | 17.9321 |
|  | 14, InF- DH422, FR2, DL-TDOA/AoD | All UE | 0.0755 | 0.1861 | 0.4228 | 0.9619 | 2.1831 |
|  |  | Central UE | 0.0587 | 0.1452 | 0.3221 | 0.7197 | 1.7271 |
|  | 15, InF-DH422, FR2, UL-TDOA/AoA | All UE | 0.0732 | 0.1728 | 0.3750 | 0.9277 | 2.1001 |
|  |  | Central UE | 0.0603 | 0.1309 | 0.2717 | 0.7086 | 1.4854 |
|  | 16, InF- DH422, FR2, Multi-RTT | All UE | 0.0317 | 0.1905 | 1.0572 | 4.1555 | 10.5087 |
|  |  | Central UE | 0.0259 | 0.1425 | 0.8398 | 4.2895 | 11.5554 |

Observation 1:  The hybrid positioning can help to improve the positioning accuracy, e.g. UL-TDOA+UL-AOA, DL-TDOA+DL-AOD.

Observation 2:  For InF-SH, the accuracy of less than 0.2m@90% can be achieved with DL-TDOA+DL-AOD and UL-TDOA+UL-AOA in FR2, and the accuracy of less than 0.5m@90% can be achieved with UL-TDOA+UL-AOA in FR1 and Multi-RTT in FR2.

Observation 3:  For InF-DH, the accuracy target of less than 0.5m@90% cannot be achieved in either FR1 or FR2.

Variable height with modified DH

Evaluated cases

The following cases in Table 3 are evaluated for modified InF-DH with clutter parameters {40%, 3m, 5m}. We will evaluate the 2D localization with fixed UE/gNB height and 3D localization with variable UE/gNB height. In the following, the results are provided for UL-TDOA+UL-AOA and Multi-RTT in FR1 and FR2, respectively.

Table 3  Evaluated parameter combinations

| Parameter | Case 101 (InF-DH, FR1) | Case 102 (InF-DH, FR1) | Case 103 (InF-DH, FR1) | Case 104 (InF-DH, FR1) | Case 105 (InF-DH, FR2) | Case 106 (InF-DH, FR2) | Case 107 (InF-DH, FR2) | Case 108 (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |  |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal |  |  |  |  |  |  |  |
| UE/gNB Tx/Rx Calibration Error | Ideal |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping |  |  |  | Tx beam sweeping Rx beam sweeping |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  | Tx codebook-based Rx codebook-based |  |  |  |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 4.

Table 4 Positioning accuracy (in unit of meter)

|  | Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 101, InF-DH435, FR1, UL-TDOA/AoA | All UE | 0.0742 | 0.1792 | 0.4392 | 1.3838 | 3.3913 |
|  |  | Central UE | 0.0628 | 0.1571 | 0.3844 | 1.3012 | 2.8376 |
|  | 102, InF-DH435, FR1, Multi-RTT | All UE | 0.0713 | 0.1249 | 0.4734 | 8.6557 | 9.0699 |
|  |  | Central UE | 0.0663 | 0.1082 | 0.4952 | 9.8411 | 9.0856 |
|  | 103, InF-DH435-3D, FR1, UL-TDOA/AoA | All UE | 0.3405(H) | 1.0103(H) | 2.3157(H) | 5.1203(H) | 7.9726(H) |
|  |  |  | 0.0905(V) | 0.2923(V) | 0.7287(V) | 1.3239(V) | 1.8648 (V) |
|  |  | Central UE | 0.2923(H) | 0.8122(H) | 2.0088(H) | 4.3405(H) | 6.6352(H) |
|  |  |  | 00682(V) | 0.2182(V) | 0.5301(V) | 1.1585(V) | 1.7265(V) |
|  | 104, InF-DH435-3D, FR1, Multi-RTT | All UE | 0.1721(H) | 1.3411(H) | 7.4143(H) | 15.4966(H) | 23.8238(H) |
|  |  |  | 0.2176(V) | 0.6510(V) | 1.1742(V) | 1.7622(V) | 2.1000(V) |
|  |  | Central UE | 0.1258(H) | 1.4345(H) | 8.9295(H) | 16.0515(H) | 22.7440(H) |
|  |  |  | 0.1666(V) | 0.5005(V) | 1.0346(V) | 1.6675(V) | 2.0241(V) |
| FR2 | 105, InF-DH435, FR2, UL-TDOA/AoA | All UE | 0.0943 | 0.2314 | 0.4973 | 1.3007 | 2.6211 |
|  |  | Central UE | 0.0865 | 0.2101 | 0.4707 | 1.1486 | 2.1881 |
|  | 106, InF-DH435, FR2, Multi-RTT | All UE | 0.3364 | 0.9128 | 2.4777 | 5.7209 | 10.7382 |
|  |  | Central UE | 0.3079 | 0.8594 | 2.2392 | 5.4600 | 10.481 |
|  | 107, InF-DH435-3D, FR2, UL-TDOA/AoA | All UE | 0.1297(H) | 0.4173(H) | 1.1042(H) | 2.6878(H) | 5.1519(H) |
|  |  |  | 0.0361(V) | 0.1075(V) | 0.2471(V) | 0.5573(V) | 0.9884(V) |
|  |  | Central UE | 0.1109(H) | 0.3273(H) | 0.8667(H) | 2.4365(H) | 4.4973(H) |
|  |  |  | 0.0288(V) | 0.0796(V) | 0.1879(V) | 0.4593(V) | 0.7747(V) |
|  | 108, InF-DH435-3D, FR2, Multi-RTT | All UE | 0.5012(H) | 2.2343(H) | 6.6096(H) | 15.6304(H) | 24.2192(H) |
|  |  |  | 0.3436(V) | 0.9022(V) | 1.5000(V) | 1.9555(V) | 2.1766(V) |
|  |  | Central UE | 0.4304(H) | 2.4466(H) | 7.9926(H) | 15.5828(H) | 21.7159(H) |
|  |  |  | 0.2871(V) | 0.7927(V) | 1.3316(V) | 1.8800(V) | 2.1606(V) |

Observation 4: For modified InF-DH, the accuracy of less than 0.5m@90% cannot be achieved without NLOS/LOS detection.

UE/gNB calibration error

Evaluated cases

The following cases in Table 5 are evaluated for InF-SH/In-DH with UE/gNB calibration error. In the following, the results are provided for DL-TDOA, UL-TDOA, UL-TDOA+UL-AOA and Multi-RTT in FR1.

The gNB angle error is modeled as a truncated Gaussian distribution with zero mean and standard deviation of θ degree, with truncation of the distribution to the [-2θ, 2θ]. The following cases show the results with θ=1°, 2°, and 5°.

Table 5  Evaluated parameter combinations

| Parameter | Case 201 (InF-SH, FR1) | Case 202 (InF-SH, FR1) | Case 203 (InF-SH, FR1) | Case 204 (InF-SH, FR1) | Case 205 (InF-DH, FR1) | Case 206 (InF-DH, FR1) | Case 207 (InF-DH, FR1) | Case 208 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | In-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |  |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal |  |  |  |  |  |  |  |
| UE/gNB Tx/Rx Calibration Error | gNB Rx/Tx Time error T1=1.4ns UE Rx/Tx time error T1=5.6ns |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |  |  |  |  |
| Additional notes, if any | Fixed UE/gNB height. |  |  |  |  |  |  |  |
| Parameter | Case 209 (InF-SH, FR1) | Case 210 (InF-SH, FR1) | Case 211 (InF-SH, FR1) | Case 212 (InF-SH, FR1) |  |  |  |  |
| Channel model (baseline, otherwise state any modifications) | InF-SH | In-SH | InF-SH | InF-SH |  |  |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | ZC, single port |  |  |  |  |
| Number of sites | 7 | 7 | 7 | 7 |  |  |  |  |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |  |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UL-AOA PSO |  |  |  |  |  |  |  |
| Network synchronization assumptions | Ideal |  |  |  |  |  |  |  |
| UE/gNB Tx/Rx Calibration Error | Ideal | gNB Rx Angle error θ=1° | gNB Rx Angle error θ=2° | gNB Rx Angle error θ=5° |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |  |  |  |  |
| Additional notes, if any | Fixed UE/gNB height. |  |  |  |  |  |  |  |

Evaluation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized inTable 6.

Table 6 Positioning accuracy (in unit of meter)

|  | Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 201, InF-SH, FR1, DL-TDOA, Group Delay Error | All UE | 0.587 | 0.874 | 1.462 | 3.540 | 8.841 |
|  |  | Central UE | 0.462 | 0.614 | 0.829 | 1.458 | 4.584 |
|  | 202, InF-SH, FR1, UL-TDOA, Group delay error | All UE | 0.5937 | 0.8672 | 1.3135 | 2.6395 | 6.4564 |
|  |  | Central UE | 0.4653 | 0.6145 | 0.8120 | 1.2343 | 3.2444 |
|  | 203, InF-SH, FR1, UL-TDOA/AoA, Group delay error | All UE | 0.0699 | 0.1180 | 0.2034 | 0.3878 | 0.7570 |
|  |  | Central UE | 0.0661 | 0.1094 | 0.1750 | 0.3251 | 0.6576 |
|  | 204, InF-SH, FR1, Multi-RTT, Group delay error | All UE | 1.7167 | 2.3902 | 3.3592 | 4.4076 | 5.7510 |
|  |  | Central UE | 1.7105 | 2.3963 | 3.3574 | 4.2662 | 5.6313 |
|  | 205, InF-DH422, FR1, DL-TDOA, Group delay error | All UE | 1.719 | 4.034 | 8.588 | 15.330 | 24.388 |
|  |  | Central UE | 1.391 | 3.407 | 8.570 | 15.039 | 24.121 |
|  | 206, InF-DH422, FR1, UL-TDOA, Group delay error | All UE | 1.0065 | 2.0631 | 4.7037 | 10.459 | 15.899 |
|  |  | Central UE | 0.8741 | 1.5739 | 4.2751 | 9.4102 | 16.813 |
|  | 207, InF-DH422, FR1, UL-TDOA/AoA, Group delay error | All UE | 0.0810 | 0.184 | 0.4251 | 1.1961 | 3.0589 |
|  |  | Central UE | 0.0717 | 0.1577 | 0.3371 | 0.8662 | 2.1206 |
|  | 208, InF-DH422, FR1, Multi-RTT, Group delay error | All UE | 2.2640 | 3.2695 | 4.7279 | 8.9874 | 14.3457 |
|  |  | Central UE | 2.2784 | 3.3381 | 5.0728 | 9.5701 | 13.7623 |
|  | 209, InF-SH, FR1, UL-AoA | All UE | 0.0446 | 0.0615 | 0.0879 | 0.1383 | 0.2314 |
|  |  | Central UE | 0.0410 | 0.0552 | 0.0758 | 0.1119 | 0.1933 |
|  | 210, InF-SH, FR1, UL-AoA, Angle error 1 degree | All UE | 0.4960 | 0.6675 | 0.8724 | 1.3003 | 2.1484 |
|  |  | Central UE | 0.4629 | 0.6280 | 0.8016 | 1.1676 | 1.8429: |
|  | 211, InF-SH, FR1, UL-AoA, Angle error 2 degrees | All UE | 0.9357 | 1.2729 | 1.6915 | 2.6204 | 5.1113 |
|  |  | Central UE | 0.8848 | 1.1961 | 1.5487 | 2.1732 | 3.8340 |
|  | 212, InF-SH, FR1, UL-AoA, Angle error 5 degrees | All UE | 2.3274 | 3.1571 | 4.2839 | 5.9433 | 9.2281 |
|  |  | Central UE | 2.2187 | 2.9534 | 4.0582 | 5.3982 | 7.5917 |

Observation 5: With the presence of UE/gNB time calibration error

- The positioning accuracy of Rel-16 Multi-RTT is deteriorated greatly than other positioning methods.

- The positioning accuracy of less than 0.5m@90% can be achieved with UL-TDOA+UL-AOA.

Observation 6: The positioning accuracy of Rel-16 UL-AOA is deteriorated greatly with gNB Rx angle error.

Physical layer latency

General description

The physical layer latency for the following positioning methods are provided

- UE-A DL-only positioning

- UL-only positioning

- Multi-RTT

- DL E-CID

- UL E-CID

- UE-B DL-only positioning

UE-assisted DL-only positioning (DL-TDOA and/or DL-AoD)

| Source NW/Destination NW Positioning technique DL-TDOA and/or DL-AoD, type DL, mode UE-A Initial and Final RRC States CONNECTED Gap is requested, and PRS periodicity is 20ms |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| UE Tx higher layer processing | 3ms | UL RRC message preparation for gap request |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| gNB Rx higher layer processing | 3ms | Decoding measurement gap request |
| gNB Tx higher layer processing | 3ms | Reconfiguration with a measurement gap of MGRP = 20ms, MGL = 3ms (MG pattern 10) |
| PDSCH scheduling | 0.5-1ms | Considering TDD configuration DDDSU, using D or S to transmit PDSCH. |
| UE Rx higher layer processing | 10ms | RRC reconfiguration delay in TS 38.331 |
| PRS measurement delay | 23 (1 samp.) 83ms (4 samp. CSSF = 1) 143 (4 samp. CSSF = 2) | According to RAN4, assuming PRS periodicity being 20ms and single frequency layer processing TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Assuming - NRxBeam, i=1, i.e. the number of Rx beams is 1 at UE - NPRS,islotN'LPRS,iN=1, i.e. UE can process all PRS in a slot and UE can buffer all PRS within a P-ms window - Nsample = 1 or 4, i.e., the measurement average time is 1 or 4 - Ti = 20ms as the UE capability T - Tavailable_PRS, i=20ms, i.e., the LCM of PRS periodicity and MGRP is 20ms |
| UE Tx higher layer processing | 3ms | UL LPP message preparation |
| PUSCH scheduling | 0.5ms-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 51.5-66ms (1 samp.) 111.5-126.5ms (4 samp. CSSF = 1) 171.5-186ms (4 samp. CSSF = 2) |  |

| Source NW/Destination NW Positioning technique DL-TDOA and/or DL-AoD, type DL, mode UE-A Initial and Final RRC States CONNECTED Gap is not requested, and PRS periodicity is 160ms |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| PRS measurement delay | 163ms (1 samp.) 643ms (4 samp. CSSF = 1) | According to RAN4, assuming PRS periodicity being 160ms and single frequency layer processing TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Assuming - NRxBeam, i=1, i.e. the number of Rx beams is 1 at UE - NPRS,islotN'LPRS,iN=1, i.e. UE can process all PRS in a slot and UE can buffer all PRS within a P-ms window - Nsample = 1 or 4, i.e., the measurement average time is 1 or 4 - Ti = 20ms as the UE capability T - Tavailable_PRS, i=20ms, i.e., the LCM of PRS periodicity and MGRP is 20ms |
| UE Tx higher layer processing | 3ms | UL LPP message preparation |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 171.5-178.5ms (1 samp.) 651.5-658.5ms (4 samp. CSSF = 1) |  |

UL-only positioning (UL-TDOA and/or UL-AoA)

| Source NW/Destination NW Positioning technique UL-TDOA and/or UL-AoA, type UL Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| SRS measurement delay | 0.5-20ms (1 samp.) 60.5-80ms (4 samp.) | Assuming SRS periodicity is 20ms. Using 1 or 4 samples. |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 6.5-26ms (1 samp.) 66.5-86.5ms (4 samp) |  |

Multi-RTT positioning

The latency for multi-RTT positioning should be the maximum between DL-TDOA/DL-AoD and UL-TDOA/UL-AoA, which is the same as DL-TDOA/DL-AoD considering UL delay is much smaller than DL.

DL E-CID positioning

| Source NW/Destination NW Positioning technique DL E-CID, type DL, mode UE-A Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| DL E-CID measurement | 0ms | Using only existing RRM measurement |
| UE Tx higher layer processing | 3ms | UL LPP message preparation |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 8.5-15ms |  |

UL E-CID positioning

| Source NW/Destination NW Positioning technique UL-E-CID, type DL+UL, mode UE-A Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| AoA measurement delay | 0-20ms | Assuming SRS periodicity is 20ms. Either AoA is not measured or AoA is measured by 1 sample. |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 6-26ms |  |

UE-based DL-only positioning (DL-TDOA and/or DL-AoD)

| Source UE/Destination UE Positioning technique DL-TDOA and/or DL-AoD, type DL, mode UE-B Initial and Final RRC States CONNECTED MO-LR, i.e. UE as the location consumer |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| UE Tx higher layer processing | 3ms | UL RRC message preparation for gap request |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| gNB Rx higher layer processing | 3ms | Decoding measurement gap request |
| gNB Tx higher layer processing | 3ms | Reconfiguration with a measurement gap of MGRP = 20ms, MGL = 3ms (MG pattern 10) |
| PDSCH scheduling | 0.5-1ms | Considering TDD configuration DDDSU, using D or S to transmit PDSCH. |
| UE Rx higher layer processing | 10ms | RRC reconfiguration delay in TS 38.331 |
| PRS measurement delay | 23 (1 samp.) | According to RAN4, assuming PRS periodicity being 20ms and single frequency layer processing TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Assuming - NRxBeam, i=1, i.e. the number of Rx beams is 1 at UE - NPRS,islotN'LPRS,iN=1, i.e. UE can process all PRS in a slot and UE can buffer all PRS within a P-ms window - Nsample = 1 or 4, i.e., the measurement average time is 1 or 4 - Ti = 20ms as the UE capability T - Tavailable_PRS, i=20ms, i.e., the LCM of PRS periodicity and MGRP is 20ms |
| Location calculation | 3ms | UE |
| End trigger |  | Calculation of Location Estimate at the UE |
| Total values | 51-58.5ms (1 samp.) |  |

Observation 7: Physical layer latency is summarized below, with the latency value less than 10ms only achieved by UL E-CID (6ms), UL-only (6.5ms), and DL E-CID (8.5ms).

| Positioning methods | Subcase |  | Physical layer latency |
| --- | --- | --- | --- |
| UE-assisted DL-only positioning (DL-TDOA and/or DL-AoD) and Multi-RTT positioning | Gap request (20ms PRS) | 1 sample | 51.5-66ms |
|  |  | 4 samples/CSSF = 1 | 111.5-126.5ms |
|  |  | 4 samples/CSSF = 2 | 171.5-186ms |
|  | No gap Request (160ms PRS) | 1 sample | 171.5-178.5ms |
|  |  | 4 samples/CSSF = 1 | 651.5-658.5ms |
| UL-only positioning (UL-TDOA and/or UL-AoA) | 1 sample |  | 6.5-26ms |
|  | 4 samples |  | 66.5-86.5ms |
| DL E-CID positioning |  |  | 8.5-15ms |
| UL E-CID positioning |  |  | 6-26ms |
| UE-based DL-only positioning | Gap request and 1 sample |  | 51-58.5ms (1 samp.) |

Evaluations for potential enhancements

LOS/NLOS identification

Accuracy

Evaluated cases

| Parameter | Case 311 (InF-DH, FR1) | Case 312 (InF-DH, FR1) | Case 313 (InF-DH, FR1) | Case 314 (InF-DH, FR1) |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution RAIM [2] | Super resolution LOS/NLOS detection [2] | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO |
| Additional notes, if any |  |  |  |  |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 7.

Table 7 Positioning accuracy (in unit of meter)

| Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| 311, InF-DH422, FR1, UL-TDOA | All UE | 0.4309 | 1.6317 | 4.9693 | 9.9453 | 16.8110 |
|  | Central UE | 0.3354 | 1.2393 | 4.6237 | 9.6631 | 16.6620 |
| 312, InF-DH422, FR1, UL-TDOA w/ RAIM | All UE | 0.0371 | 0.0748 | 0.1912 | 1.4517 | 8.6696 |
|  | Central UE | 0.0301 | 0.0554 | 0.1217 | 0.6569 | 8.6696 |
| 313, InF-DH422, FR1, UL-TDOA w/ LOS/NLOS identification | All UE | 0.0397 | 0.0701 | 0.1354 | 0.4836 | 4.9663 |
|  | Central UE | 0.0311 | 0.0505 | 0.0862 | 0.2111 | 2.9827 |
| 314, InF-DH422, FR1, UL-TDOA w/ ideal LOS selection | All UE | 0.0394 | 0.0608 | 0.1399 | 0.4452 | 4.8362 |
|  | Central UE | 0.0310 | 0.0506 | 0.0855 | 0.2022 | 1.4922 |

Observation 8:  For modified InF-DH

- The LOS/NLOS detection achieves close performance to the Ideal LOS/NLOS detection.

- Both the LOS/NLOS detection and the Ideal LOS/NLOS detection achieve the accuracy of less than 0.5m@90%.

- The LOS/NLOS detection method achieves higher accuracy than the traditional RAIM method by tagging the channel as LOS or NLOS.

PRS/SRS frequency aggregation

Accuracy

Evaluated cases

| Parameter | Case 321 (InF-DH, FR1) | Case 322 (InF-DH, FR1) | Case 323 (InF-DH, FR1) |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO |
| Additional notes, if any | 100MHz contiguous | 200MHz contiguous | 50MHz+100MHz (Gap)+50MHz |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 8.

Table 8 Positioning accuracy (in unit of meter)

| Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| 321, InF-DH422, FR1, UL-TDOA, 100M contiguous | All UE | 0.0394 | 0.0608 | 0.1399 | 0.4452 | 4.8362 |
|  | Central UE | 0.0310 | 0.0506 | 0.0855 | 0.2022 | 1.4922 |
| 322, InF-DH422, FR1, UL-TDOA, 200M contiguous | All UE | 0.0106 | 0.0179 | 0.0332 | 0.0827 | 1.6747 |
|  | Central UE | 0.0082 | 0.0122 | 0.0197 | 0.0384 | 0.1081 |
| 323, InF-DH422, FR1, UL-TDOA, 50MHz+100MHz(Gap)+50MHz | All UE | 0.0148 | 0.0273 | 0.0570 | 0.2244 | 2.5975 |
|  | Central UE | 0.0109 | 0.0189 | 0.0337 | 0.0912 | 1.2261 |

Observation 9: PRS/SRS frequency aggregation with phase continuity can help to improve the positioning accuracy, and discontiguous aggregation can approach the performance of contiguous aggregation with the same frequency span.

1-symbol PRS

Accuracy

Evaluated Cases

| Parameter | Case 331 (InF-SH, FR1) | Case 332 (InF-SH, FR1) | Case 333 (InF-SH, FR1) | Case 334 (InF-SH, FR1) |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 1 symbol) | DL-PRS (Comb-12, 12 symbol) | DL-PRS (Comb-12, 1 symbol) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO |
| Additional notes, if any |  |  |  |  |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 9.

Table 9 Positioning accuracy (in unit of meter)

| Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| 331, InF-SH, FR1, DL-TDOA, Comb-4 and 4-symbol | All UE | 0.0252 | 0.0425 | 0.0718 | 0.1531 | 0.3977 |
|  | Central UE | 0.0196 | 0.0314 | 0.0504 | 0.0939 | 0.1853 |
| 332, InF-SH, FR1, DL-TDOA, Comb-4 and 1-symbol | All UE | 0.0268 | 0.0473 | 0.0836 | 0.1693 | 0.4643 |
|  | Central UE | 0.0210 | 0.0347 | 0.0567 | 0.1123 | 0.2158 |
| 333, InF-SH, FR1, DL-TDOA, Comb-12 and 12-symbol | All UE | 0.0279 | 0.0492 | 0.0835 | 0.1647 | 0.4184 |
|  | Central UE | 0.0219 | 0.0347 | 0.0573 | 0.1091 | 0.2172 |
| 334, InF-SH, FR1, DL-TDOA, Comb-12 and 1-symbol | All UE | 0.0284 | 0.0484 | 0.0866 | 0.1784 | 0.5340 |
|  | Central UE | 0.0209 | 0.0359 | 0.0595 | 0.1199 | 0.3277 |

Observation 10: The single PRS symbol transmission achieves almost the same performance with that of multiple PRS symbols for InF-SH scenarios.

Network efficiency

The network efficiency is evaluated assuming

- PRS transmission periodicity is 160ms

- PRS numerology is 30kHz

- 8 PRS resources with 12 symbols or 4 symbols per PRS resource

- The PRS bandwidth is 100MHz

The total resource utilization dedicated for positioning is summarized in Table 10.

Table 10 PRS resource utilization evaluation results

| Case | Positioning resource utilization |
| --- | --- |
| Case 1: PRS with 12 symbols | 2.14% |
| Case 2: PRS with 4 symbols | 0.714% |
| Case 3: PRS with 1 symbols | 0.179% |

By reducing the PRS symbols from 12 and 4 to 1 for comb-12 and comb-4, respectively, the overhead of PRS transmission is reduced by 11/12 and 3/4, respectively.

Moreover, by allowing 1-symbol PRS transmission, network may reuse CSI-RS for mobility for positioning, further reducing the PRS overhead.

PRS punctured by SSB

Accuracy

Evaluated cases

| Parameter | Case 341 (InF-SH, FR1) | Case 342 (InF-SH, FR1) |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | DL-TDOA PSO | DL-TDOA PSO |
| Additional notes, if any |  | PRS center 20RBs are punctured by SSB |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 11.

Table 11 Positioning accuracy (in unit of meter)

| Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| 341, InF-SH, FR1, DL-TDOA | All UE | 0.0252 | 0.0425 | 0.0718 | 0.1531 | 0.3977 |
|  | Central UE | 0.0196 | 0.0314 | 0.0504 | 0.0939 | 0.1853 |
| 342, InF-SH, FR1, DL-TDOA, 20-RB of PRS punctured by SSB | All UE | 0.0293 | 0.0475 | 0.0846 | 0.1699 | 0.4414 |
|  | Central UE | 0.0213 | 0.0346 | 0.0539 | 0.1090 | 0.2195 |

Observation 11: When the PRS center 20RBs are punctured by SSB, the positioning accuracy is almost not affected.

IDLE/INACITVE state positioning

UE efficiency

UE Power consumption for DL-only positioning

The evaluation methodology is as presented in [1] with PRS periodicity being 160ms, and

- C-DRX cycle 160msec

  - 8 msec on-duration timer

  - 100 msec inactivity timer

- I-DRX cycle 1.28msec

For a UE with very limited 5GC traffic that needs to be in a positioning session, the state transit for IDLE/INACTIVE state and CONNECTED state is given in Figure 1 and Figure 2, respectively.

Figure 1 UE power state transit for IDLE/INACTIVE state

Figure 2 UE power state transit for CONNECTED state (in on-duration and outside on-duration)

The related power consumption is shown below.

| IDLE/INACTIVE state in every 1.28s |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 50 | 2 | 100 | 0.0781% | 0.3789% |
| SSB RRM | 80 | 4 | 320 | 0.1563% | 1.2125% |
| Micro-sleep | 45 | 6 | 270 | 0.2344% | 1.0230% |
| PRS RRM | 200 | 96 | 19200 | 3.7500% | 72.7493% |
| Deep sleep | 1 | 2452 | 2452 | 95.7813% | 24.6363% |
| -- State transit | 450 | 9 | 4050 |  |  |
| Total |  | 2560 | 26392 | 100.0000% | 100.0000% |
| CONNECTED state inside on-duration in every 1.28s |  |  |  |  |  |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 100 | 32 | 3200 | 1.2500% | 11.2549% |
| PRS RRM | 200 | 96 | 19200 | 3.7500% | 67.5295% |
| Deep sleep | 1 | 2432 | 2432 | 95.0000% | 21.2155% |
| -- State transit | 450 | 8 | 3600 |  |  |
| Total |  | 2560 | 28432 | 100.0000% | 100.0000% |
| CONNECTED state outside on-duration in every 1.28s |  |  |  |  |  |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 100 | 128 | 12800 | 5.0000% | 33.7410% |
| PRS RRM | 200 | 96 | 19200 | 3.7500% | 50.6116% |
| Deep sleep | 1 | 2336 | 2336 | 91.2500% | 15.6474% |
| -- State transit | 450 | 8 | 3600 |  |  |
| Total |  | 2560 | 37936 | 100.0000% | 100.0000% |

UE power consumption for UL-only positioning

The evaluation methodology is as presented in [1] with SRS periodicity being 160ms, and

- C-DRX cycle 160msec

  - 8 msec on-duration timer

  - 100 msec inactivity timer

- I-DRX cycle 1.28msec

The slot format is DDDSU where the SRS is only transmitted on S slots. In particular for IDLE/INACTIVE state, the following slot assignment is assumed.

| D | D | D | S | U | D | D | D | S | U | D | D | D | S | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Sync (2ms) |  |  |  | Micro sleep |  |  |  | SRS | MS | Paging |  | Deep sleep |  |  |
| D | D | D | S | U | D | D | D | S | U | D | D | D | S | U |
| SSB Sync (2ms) |  |  |  | Micro sleep |  |  |  | SRS | Deep sleep |  |  |  |  |  |

For a UE with very limited 5GC traffic that needs to be in a positioning session, the state transit for IDLE/INACTIVE state and CONNECTED state is given in Figure 3 and Figure 4, respectively.

Figure 3 UE power state transit for IDLE/INACTIVE state

Figure 4 UE power state transit for CONNECTED state

The related power consumption is shown below.

| IDLE/INACTIVE state in every 1.28s |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 50 | 2 | 100 | 0.0781% | 0.8396% |
| SSB RRM | 80 | 32 | 2560 | 1.2500% | 21.4945% |
| Micro-sleep | 45 | 33 | 1485 | 1.2891% | 12.4685% |
| SRS Tx | 210 | 8 | 1680 | 0.3125% | 14.1058% |
| Deep sleep | 1 | 2485 | 2485 | 97.0703% | 51.0915% |
| -- State transit | 450 | 8 | 3600 |  |  |
| Total |  | 2560 | 11910 | 100.0000% | 100.0000% |
| CONNECTED state outside on-duration in every 1.28s |  |  |  |  |  |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 100 | 120 | 12000 | 4.6875% | 60.8766% |
| SRS Tx | 210 | 8 | 1680 | 0.3125% | 8.5227% |
| Deep sleep | 1 | 2432 | 2432 | 95.0000% | 30.6006% |
| -- State transit | 450 | 8 | 3600 |  |  |
| Total |  | 2560 | 19712 | 100.0000% | 100.0000% |

Summary

In this section, we evaluated the power consumption at the UE if UE is only participating in positioning related procedures. The power consumption is summarized as below.

Table 12 Power consumption evaluation for IDLE/INACTIVE state

| Cases |  | Power consumption | Power saved |
| --- | --- | --- | --- |
| DL-only | IDLE/INACTIVE state | 26392 | 7.2% to inside on-duration 30.4% to outside on-duration |
|  | CONNECTED state with PRS inside on-duration | 28432 | - |
|  | CONNECTED state with PRS outside on-duration | 37936 | - |
| UL-only | IDLE/INACTIVE state | 11910 | 39.6% |
|  | CONNECTED state | 19712 | - |

Based on the evaluation, we have the following observation.

Observation 12: IDLE/INACTIVE state positioning can save about 7%-40% power consumption compared to C-DRX configuration.

AoA with ULA

Accuracy

Evaluated cases

| Parameter | Case 361 (InF-DH, FR1) | Case 362 (InF-DH, FR1) | Case 363 (InF-DH, FR1) |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UL-TDOA+UL-AoA PSO | UL-TDOA+UL-AoA PSO | UL-TDOA+UL-AoA PSO |
| Additional notes, if any | UPA 4x4x2 | ULA 1x4x1 assuming legacy AoA | ULA 1x4x1 with modified AoA |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 13.

Table 13 Positioning accuracy (in unit of meter)

| Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| 361, InF-DH422, FR1, UL-TDOA/AoA | All UE | 0.0216 | 0.0318 | 0.0460 | 0.0735 | 0.1199 |
|  | Central UE | 0.0203 | 0.0276 | 0.0396 | 0.0665 | 0.0973 |
| 362, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ legacy AoA | All UE | 2.4977 | 3.3503 | 4.1807 | 4.9986 | 5.6053 |
|  | Central UE | 2.2164 | 3.1038 | 3.9067 | 4.8161 | 5.3759 |
| 363, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ modified AoA | All UE | 0.0406 | 0.0626 | 0.1027 | 0.2058 | 0.5794 |
|  | Central UE | 0.0348 | 0.0525 | 0.0821 | 0.1694 | 0.4306 |

Observation 13: For ULA,

- The positioning accuracy is greatly reduced using legacy AOA reporting.

- The enhanced AoA reporting can approach the UPA positioning accuracy.

E-CID enhancement

Accuracy

Evaluated cases

| Parameter | Case 371 (InF-DH, FR1) | Case 372 (InF-DH, FR1) |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Multi-RTT PSO | E-CID (RTT+AoA only from the serving cell) |
| Additional notes, if any |  |  |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 14.

Table 14 Positioning accuracy (in unit of meter)

| Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| 371, InF-DH422, FR1, Multi-RTT | All UE | 0.0423 | 0.0591 | 0.0901 | 0.1756 | 0.7138 |
|  | Central UE | 0.0388 | 0.0555 | 0.0871 | 0.1694 | 0.5306 |
| 372, InF-DH422, FR1, E-CID w/ single cell RTT/AoA | All UE | 0.0471 | 0.0651 | 0.1068 | 0.2497 | 1.1151 |
|  | Central UE | 0.0473 | 0.0647 | 0.1040 | 0.2395 | 0.9800 |

Observation 14: The positioning accuracy of E-CID (RTT+AOA) approaches the Multi-
RTT with Ideal LOS/NLOS detection.

Latency

| Source NW/Destination NW Positioning technique UL-E-CID, type DL+UL, mode UE-A, Initial and Final RRC States CONNECTED Assuming UE Rx – Tx time difference is already available |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| gNB Rx – Tx time difference measurement and AoA measurements | 0-20ms | Assuming SRS periodicity is 20ms |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 6-26ms |  |

| Source NW/Destination NW Positioning technique UL-E-CID, type DL+UL, mode UE-A, Initial and Final RRC States CONNECTED Assuming UE Rx – Tx time difference is not available |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| gNB Tx higher layer processing | 3ms | Configuration of UE Rx – Tx time difference based on SRS and TRS/CSI-RS |
| PDSCH scheduling | 0.5-1ms | Considering TDD configuration DDDSU, using D or S to transmit PDSCH. |
| UE Rx higher layer processing | 10ms | RRC reconfiguration delay in TS 38.331 |
| UE Rx – Tx time difference measurement/gNB Rx – Tx time difference measurement and AoA measurements | 20ms | Assuming TRS/CSI-RS periodicity is 20ms Assuming SRS periodicity is 20ms |
| UE Tx higher layer processing | 3ms | UL RRC message preparation for RRM measurement reporting, including UE Rx – Tx time difference measurement |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| gNB Rx higher layer processing | 3ms | Processing UL-SCH containing the measurement and including gNB measurements |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 46-53.5ms |  |

Network efficiency

Since enhanced E-CID may only use existing communication signals, it has not extra positioning overhead.

Synchronization error compensation

Accuracy

Evaluated cases

| Parameter | Case 381 (InF-SH, FR1) | Case 382 (InF-SH, FR1) | Case 383 (InF-SH, FR1) | Case 384 (InF-SH, FR1) | Case 385 (InF-SH, FR1) | Case 386 (InF-SH, FR1) | Case 387 (InF-SH, FR1) | Case 388 (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO |
| Network synchronization assumptions | 0ns | 0.2ns | 0.5ns | 1ns | 0ns | 0.2ns | 0.5ns | 1ns |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Additional notes, if any | 100MHz contiguous | 100MHz contiguous | 100MHz contiguous | 100MHz contiguous | 200MHz contiguous | 200MHz contiguous | 200MHz contiguous | 200MHz contiguous |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 15.

Table 15 Positioning accuracy (in unit of meter)

| Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| 381, InF-SH, FR1, UL-TDOA, 100M，0ns gNB Sync error | All UE | 0.0369 | 0.0622 | 0.1026 | 0.1867 | 0.3242 |
|  | Central UE | 0.0263 | 0.0433 | 0.0654 | 0.1136 | 0.2113 |
| 382, InF-SH, FR1, UL-TDOA, 100M，0.2ns gNB Sync error | All UE | 0.0828 | 0.1153 | 0.1643 | 0.2672 | 0.5100 |
|  | Central UE | 0.0662 | 0.0855 | 0.1096 | 0.1452 | 0.1847 |
| 383, InF-SH, FR1, UL-TDOA, 100M，0.5ns gNB Sync error | All UE | 0.1811 | 0.2482 | 0.3330 | 0.5173 | 0.8804 |
|  | Central UE | 0.1454 | 0.1877 | 0.2306 | 0.2828 | 0.3486 |
| 384, InF-SH, FR1, UL-TDOA, 100M，1ns gNB Sync error | All UE | 0.3500 | 0.4709 | 0.6348 | 1.0315 | 1.6249 |
|  | Central UE | 0.2731 | 0.3530 | 0.4353 | 0.5372 | 0.6274 |
| 385, InF-SH, FR1, UL-TDOA, 200M, 0ns gNB Sync error | All UE | 0.0086 | 0.0144 | 0.0240 | 0.0456 | 0.0770 |
|  | Central UE | 0.0065 | 0.0101 | 0.0149 | 0.0250 | 0.0395 |
| 386, InF-SH, FR1, UL-TDOA, 200M, 0.2ns gNB Sync error | All UE | 0.0710 | 0.0971 | 0.1330 | 0.2158 | 0.3453 |
|  | Central UE | 0.0558 | 0.0726 | 0.0901 | 0.1102 | 0.1297 |
| 387 InF-SH, FR1, UL-TDOA, 200M, 0.5ns gNB Sync error | All UE | 0.1725 | 0.2328 | 0.3055 | 0.4895 | 0.7868 |
|  | Central UE | 0.1406 | 0.1790 | 0.2178 | 0.2659 | 0.2962 |
| 388 InF-SH, FR1, UL-TDOA, 200M, 1ns gNB Sync error | All UE | 0.3490 | 0.4658 | 0.6330 | 0.9466 | 1.4384 |
|  | Central UE | 0.2735 | 0.3582 | 0.4407 | 0.5340 | 0.6381 |

Observation 15: According to the simulation results, the residue synchronization error should at least meet following requirement to reach the corresponding accuracy target.

| SRS bandwidth | Synchronization error threshold to meet the requirement |  |
| --- | --- | --- |
|  | 0.2m@90% | 0.5m@90% |
| 100M | 0.5ns | 1ns |
| 200M | 0.5ns | 1ns |

TP to TR

The TP to the TR can be found in the attached file.

Conclusion

Observation 1:  The hybrid positioning can help to improve the positioning accuracy, e.g. UL-TDOA+UL-AOA, DL-TDOA+DL-AOD.

Observation 2:  For InF-SH, the accuracy of less than 0.2m@90% can be achieved with DL-TDOA+DL-AOD and UL-TDOA+UL-AOA in FR2, and the accuracy of less than 0.5m@90% can be achieved with UL-TDOA+UL-AOA in FR1 and Multi-RTT in FR2.

Observation 3:  For InF-DH, the accuracy target of less than 0.5m@90% cannot be achieved in either FR1 or FR2.

Observation 4: For modified InF-DH, the accuracy of less than 0.5m@90% cannot be achieved without NLOS/LOS detection.

Observation 5: With the presence of UE/gNB time calibration error

- The positioning accuracy of Rel-16 Multi-RTT is deteriorated greatly than other positioning methods.

- The positioning accuracy of less than 0.5m@90% can be achieved with UL-TDOA+UL-AOA.

Observation 6: The positioning accuracy of Rel-16 UL-AOA is deteriorated greatly with gNB Rx angle error.

Observation 7: Physical layer latency is summarized below, with the latency value less than 10ms only achieved by UL E-CID (6ms), UL-only (6.5ms), and DL E-CID (8.5ms).

| Positioning methods | Subcase |  | Physical layer latency |
| --- | --- | --- | --- |
| UE-assisted DL-only positioning (DL-TDOA and/or DL-AoD) and Multi-RTT positioning | Gap request (20ms PRS) | 1 sample | 51.5-66ms |
|  |  | 4 samples/CSSF = 1 | 111.5-126.5ms |
|  |  | 4 samples/CSSF = 2 | 171.5-186ms |
|  | No gap Request (160ms PRS) | 1 sample | 171.5-178.5ms |
|  |  | 4 samples/CSSF = 1 | 651.5-658.5ms |
| UL-only positioning (UL-TDOA and/or UL-AoA) | 1 sample |  | 6.5-26ms |
|  | 4 samples |  | 66.5-86.5ms |
| DL E-CID positioning |  |  | 8.5-15ms |
| UL E-CID positioning |  |  | 6-26ms |
| UE-based DL-only positioning | Gap request and 1 sample |  | 51-58.5ms (1 samp.) |

Observation 8:  For modified InF-DH

- The LOS/NLOS detection achieves close performance to the Ideal LOS/NLOS detection.

- Both the LOS/NLOS detection and the Ideal LOS/NLOS detection achieve the accuracy of less than 0.5m@90%.

- The LOS/NLOS detection method achieves higher accuracy than the traditional RAIM method by tagging the channel as LOS or NLOS.

Observation 9: PRS/SRS frequency aggregation with phase continuity can help to improve the positioning accuracy, and discontiguous aggregation can approach the performance of contiguous aggregation with the same frequency span.

Observation 10: The single PRS symbol transmission achieves almost the same performance with that of multiple PRS symbols for InF-SH scenarios.

Observation 11: When the PRS center 20RBs are punctured by SSB, the positioning accuracy is almost not affected.

Observation 12: IDLE/INACTIVE state positioning can save about 7%-40% power consumption compared to C-DRX configuration.

Observation 13: For ULA,

- The positioning accuracy is greatly reduced using legacy AOA reporting.

- The enhanced AoA reporting can approach the UPA positioning accuracy.

Observation 14: The positioning accuracy of E-CID (RTT+AOA) approaches the Multi-
RTT with Ideal LOS/NLOS detection.

Observation 15: According to the simulation results, the residue synchronization error should meet following requirement to meet the corresponding accuracy target.

| SRS bandwidth | Synchronization error threshold to meet the requirement |  |
| --- | --- | --- |
|  | 0.2m@90% | 0.5m@90% |
| 100M | 0.5ns | 1ns |
| 200M | 0.5ns | 1ns |

- Reference

3GPP R1-2007575, Scenarios and evaluation assumption for IIoT positioning, Huawei, HiSilicon, RAN1#103-e, e-Meeting, 26th October – 11th November, 2020.

3GPP R1-2007577, Positioning enhancement in Rel-17, Huawei, HiSilicon, RAN1#103-e, e-Meeting, 26th October – 11th November, 2020.