3GPP TSG-RAN WG1 #103-e		R1-2009486

eMeeting, Oct 26th – Nov 13th, 2020

Source: 	Moderator (Ericsson)

Title:      	Summary of Email discussion [103-e-NR-DSS-01]

Agenda item:	8.13.1

Document for:	Discussion and Decision

# 1 Introduction

This document summarizes the discussions for email thread [103-e-NR-DSS-01] under agenda item 8.13.1 on Cross-carrier scheduling (from Scell to Pcell) for the Rel17 WI on NR Dynamic spectrum sharing (DSS).

# 2. Discussion

## 2.1	Moderator Summary

Below is a short moderator summary based on tdocs [1-19] submitted for RAN1#103-e

- Search Space and DCI format handling when CCS from an SCell (sSCell) to PCell/PSCell is configured

  - Type 0/0A/1/2 CSS sets for any associated DCI formats

    - monitored only on PCell/PSCell – [1],[2],[4],[6],[7],[10],[11],[13],[14]

  - DCI formats 0_0 and 1_0 for scheduling PDSCH/PUSCH on PCell/PSCell in any associated SS set

    - monitored only on PCell/PSCell – [2],[3],[7],[10],[14],[16]

  - DCI formats 0_1,1_1,0_2,1_2 for scheduling PDSCH/PUSCH on PCell/PSCell in USS set(s)

    - Not monitored on PCell/PSCell and can be monitored only on sScell – [2],[3],[6?],[8],[11?],[19]

    - Can be monitored on both PCell/PSCell and the sSCell – [1],[7],[13],[14],[15],[16],[17?],[18]

    - Study further – [4],[5],[10]

  - DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set and DCI format 2_5

    - monitored on PCell/PSCell/sSell according to existing specifications – [1?],[2],[4],[6],[10],[18]

    - applicable to PCell/PSCell even when monitored on PCell/PSCell sSell in some cases – [14]

  - DCI format 2_6 in type 3 CSS set

    - Monitored only on PCell/PSCell – [10]

    - Can be monitored on sSCell – [1],[6],

  - DCI formats 0_1,1_1 with SCell dormancy indication

    - Monitored only on PCell/PSCell –

    - Can be monitored on the sSCell – [1]

  - SS set overbooking and associated UE procedures

    - Applied only for PCell – [1],[6],[19]

    - Applied considering both PCell and sSCell – [14?]

- Scheduling framework when CCS from an SCell (sSCell) to PCell/PSCell is configured

  - Activation/deactivation of sSCell

    - Discussed in [2],[4],[6],[7],[9],[18]

  - Dormancy/non-dormancy for sSCell

    - Discussed in [2],[4],[18]

  - No out of order scheduling between a) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on sSCell

    - Discussed in [1],[18],[19]

  - No simultaneous rx/tx of unicast PDSCH/PUSCH on PCell/PSCell scheduled from PCell/PSCell and unicast PDSCH/PUSCH on PCell/PSCell scheduled from sSCell

    - Discussed in [18],[19]

  - Simultaneous rx of PDCCH on sSCell used for scheduling PCell/PCell and PDCCH for self-scheduling on PCell/PSCell

    - Discussed in [5]

  - PDCCH for initial transmission and retransmission can be on different cells for same TB

    - Discussed in [1],[5]

- BD/CCE limit handling when CCS from an SCell (sSCell) to PCell/PSCell is configured

  - Use multi-TRP for BD/CCE handling as starting point – [1]

  - PCell is counted as a scheduled cell for both the PCell and sSCell – [6]

  - BD/CCE limits are counted over both scheduling cells (PCell and SCell) and determined for scheduled PCell, overbooking is allowed [14]

  - Both PCell self-scheduling and SCell to PCell are considered – [18]

  - Per-scheduling-CC limit is determined as (per-CC BD or CCE limit) – (max of the number of BDs or CCEs at a slot across all the slots on the PCell/PSCell) – [19]

  - Study further – [2],[10],[11],[13],

- Other aspects

  - Further details on CCS configuration

    - CrossCarrierSchedulingConfig details – [2],[3],[10].[11]

  - Dynamic switching of PDCCH candidates/SS sets/SS groups between PCell/PSCell and sSCell – [13],[14],[15],[17]

  - Further details on search space handling

    - UE is not configured with complete USS in P(S)cell for self-scheduling/only parts of USS are moved – [2],[10]

    - Light SS linking – [2]

    - SS set linking between PCell/PSCell and sSCell by using same index – [4],[10]

    - Use n_CI for SS hashing – [19]

    - Independent config. of parameters related to PDCCH monitoring occasions on sSCell for scheduling PCell/PSCell and for scheduling sSCell – [19]

    - Configuration of individual DCI format(s) (e.g. only DCI format 0-1, only DCI format 1-1) – [18]

    - CIF = 0 used for sSCell or PCell? – [11],[19]

  - sSCell can schedule dynamic and CG PDSCH/PUSCH – [2]

  - Multi/single-TRP operation should remain independent on the PCell and the SCell – [6]

  - Is SCell scheduling PCell configured per UE, per BWP or per SS? – [11]

  - Reuse Rel16 preparation time between sSCell PDCCH and PCell/PSCell PDSCH/PUSCH – [19]

  - Clarify whether PCell and/or sSCell can be unlicensed cell(s) – [16]

  - Clarify allowed #UL DCI and #DL DCI combination – [16]

  - DCI size alignment clarifications – [12]

  - Impact of BWP change on sScell on Scell to Pcell scheduling – [6]

  - SCell to PCell/PSCell scheduling has no impact on PUCCH – [6],[14]

  - Clarify that one sSCell per CG can be configured (in case wording in previous agreement is not clear) – [9]

  - BFR operations/mechanism for the sSCell should be studied – [9]

## 2.1	Proposals for discussion

Below are some proposals for discussion

### Proposal 1

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

Companies are requested to indicate their view about the above proposal in the Table below

| Company Name | support/not support | Comments (Proposal 1) |
| --- | --- | --- |
| Intel | support | Type0/0A/1/2 CSS sets are necessary for UE with single cell operation on ‘PCell’. Specifically, they are used before a CA UE enters CA mode. It is possible ‘SCell’ has its own Type0/0A/1/2 CSS sets, since ‘SCell’ can be PCell for some other UEs. However, this is not related to ‘SCell scheduling PCell’ under current scope of WI. |
| Qualcomm | Support |  |
| CATT | Support |  |
| vivo | support | Type0/0A/1/2 CSS sets remains on PCell/PSCell as NR Rel-15/16 |
| Nokia | Support | This approach is aligned to the current specifications and limit the specification change needs. |
| InterDigital | Support |  |
| Samsung | Support | This is the main usage of scheduling combination (a) from RAN1#102e (“self-scheduling on PCell/PSCell is allowed”) |
| ZTE | Support | We are generally fine with the proposal. Just to make it clear, we propose to clarify that Type 0/0A/2 CSS is only allowed in PCell while Type 1 CSS is allowed in both PCell and PSCell. Thus, we propose the following. ### Proposal 1 - When CCS from an SCell (sSCell) to PCell/PSCell is configured, - UE monitors Type 0/0A/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell and not on the sSCell - UE monitors Type 1 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell |
| LG | Support | Agree with ZTE’s proposal for more clarity. |
| DOCOMO | Support |  |
| Apple | Support |  |
| ETRI | Support |  |
| Xiaomi | Support |  |
| OPPO | Support | Type0/0A/1/2 CSS sets keep on PCell/PSCell as NR Rel-15/16 |
| Huawei, HiSilicon | support | It is not reasonable to use SCell PDCCH to schedule the PCell/PSCell cell-specific information transmission, since the PCell/PSCell PDCCHs for scheduling cell-specific information transmission should still be transmitted for the legacy UEs so that the PCell/PSCell PDCCH in these CSS sets can’t be offloaded. |
| Ericsson | support | The impact on PCell operation should be reduced as much as possible when SCell to PCell scheduling is configured. |
| Lenovo, Motorola Mobility | Support |  |

Revised proposal below, based on above discussion

### Proposal 1v2

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

  - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell and PSCell

Companies are requested to indicate their view about the above proposal in the Table below

| Company Name | support/not support | Comments (Proposal 1v2) |
| --- | --- | --- |

### Proposal 2

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors DCI formats 0_0 and 1_0 for scheduling PDSCH/PUSCH on PCell/PSCell (in any associated SS set(s)) only on the PCell/PSCell and not on the sSCell

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 2) |
| --- | --- | --- |
| Intel | Support | We are considering two sources of DCI 0_0/1_0 on PCell/PSCell: - a DCI format in CSS, which has same DCI size as DCI 0_0/1_0 can be used for unicast scheduling - a USS containing only DCI 0_0/1_0 may be configured on PCell |
| Qualcomm | Not support | We do not see the need of supporting a USS set on the PCell/PSCell when the UE is configured to monitor USS set(s) on the SCell for cross-carrier scheduling from the SCell to the PCell/PSCell. Therefore, we propose the following. - When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors DCI formats 0_0 and 1_0 for scheduling PDSCH/PUSCH on PCell/PSCell (in any associated CSS set(s)) only on the PCell/PSCell and not on the sSCell |
| CATT | Not support | We support Qualcomm’s update. The motivation of supporting a USS on the scheduled P(s)Cell is far from being justified. The intention of supporting CCS from Scell to PCell is to offload the PDCCH overhead on the scheduled PCell. If a USS can be configured on the PCell for self-scheduling, there is no reason to further configure CCS from SCell to PCell. |
| vivo |  | We understand the key point of this proposal is not to allow DCI 0_0 and 1_0 from sScell to PCell/PSCell scheduling. If this is the correct understanding, we support it; otherwise we agree with QC’s update. |
| Nokia | Support | DCI formats 0_0/1_0 do not carry the CIF field, there is no need to monitor them in the scheduling SCell for cross-carrier scheduling. In other words, DCI formats 0_0/1_0 are monitored for self-scheduling in PCell in CSS and USS, and for self-scheduling in the SCell in the USS. |
| InterDigital | Support | We think that USS on PCell is needed when the SCell quality drops. |
| Samsung |  | For CSS, support (UE anyway monitors non-Type3 CSS on PCell). For USS, consider together with P4. Suggest to resolve P4 first. |
| ZTE | Support | Our understanding is that the legacy UE behaviors of DCI format 0-0 and DCI format 1-0 are the same as before. Thus, DCI format 0-0 and DCI format 1-0 in both CSS and USS should still be kept in PCell. Regarding Qualcomm’s comment above, fallback DCI on PCell/PSCell should be used to schedule PDSCH/PUSCH in case fallback scheduling is needed. Because there are many restrictions only using fallback DCI with C-RNTI in CSS (For example, only aggregation level 4/8/16 can be used for Type 0/0A/2 CSS; Type 1 CSS can be only used if the UE has not been provided a Type 3 CSS or a USS and the UE has received a C-RNTI; Type 3 CSS can be used if this optional search space is configured and only configured on PCell.) From network vendor’s perspective, we see the need to kept fallback DCIs in USS in PCell/PSCell. |
| LG | Need discussion | We think that USS associated with fallback DCI may need to be monitored on at least one of P(S)Cell and SCell, with consideration of potential PDCCH blocking/congestion in UE-common CSS resources in case of scheduling fallback DCI. |
| DOCOMO |  | We have similar view as Samsung. |
| Apple | In principle support | We think the fall back DCI is only monitored in SpCell |
| ETRI | Support |  |
| Xiaomi | Support |  |
| OPPO | Support | We share view with Nokia. |
| Huawei, HiSilicon | support | Since there is no carrier indicator field in DCI format 0_0/1_0, when CCS from an SCell to PCell/PSCell and self-scheduling on the SCell are configured, a UE specific DCI format 0_0/1_0 on the SCell cannot indicate the scheduled carrier index. |
| Ericsson | support | The impact on PCell operation should be reduced as much as possible when SCell to PCell scheduling is configured. Regarding QC suggestion “…UE monitors DCI formats 0_0 and 1_0 for scheduling PDSCH/PUSCH on PCell/PSCell in CSS set(s)) only on the PCell/PSCell and not on the sSCell”, we would like to better understand how DCI 0_0,1_0 for scheduling PDSCH/PUSCH on PCell/PSCell using USS should then be handled. i.e., a) DCI 0_0,1_0 for scheduling PDSCH/PUSCH on PCell/PSCell using USS is not supported? or b) DCI 0_0,1_0 for scheduling PDSCH/PUSCH on PCell/PSCell is monitored on sSCell USS by adding CIF for these DCI formats? or c) some other option? |
| Lenovo, Motorola Mobility | Support | To avoid additional specification impact, we prefer to keep the legacy setting to not allow cross-carrier scheduling with DCI format 0_0/1_0. |

### Proposal 3

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set configured on PCell/PSCell/sSell, and applicability of the information in the DCI formats is as per Rel15/Rel6 framework

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 3) |
| --- | --- | --- |
| Intel | Support | Our understanding is the support of DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 on a SCell is already supported in Rel-15. Therefore, it can be used on sSCell |
| Qualcomm | Need a clarification | It is better to clarify that a UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set configured on either of PCell/PSCell or sSell. |
| CATT | Support |  |
| vivo | Need updates | It seems 2_5 introduced in NR Rel-16 is missed in the proposal. Besides, 2_6 is already supported in PCell/PSCell and whether to extend to sScell can be FFS. Combined with QC’s update, we suggest the proposal updated below: - When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4, 2_5 in Type 3 CSS set configured on either of PCell/PSCell or sSell and 2_6 in PCell/PSCell, and applicability of the information in the DCI formats is as per Rel15/Rel6 framework - FFS: whether/how to support DCI 2_6 in sScell |
| Nokia | Support | It is desirable to have at least the CSS type 3 for power control of the PCell configurable in the SCell (DCI format 2_2/2_3) so that all the normal operations of an ongoing connection can be handled via the SCell. We see no obvious reasons why any of the DCI format, search space or PCell related limitations would need to be revisited. |
| InterDigital | Support |  |
| Samsung | Need clarification | What additional to Rel-16 is to be agreed? Is this for DCI format 2_6? |
| ZTE | Support | Two comments, 1. It seems DCI format 2_5 can also be included in the above proposal. 2. Similar comment as Qualcomm. We prefer not to use “either …. or” here, as it seems that these DCI formats are not precluded to be configured on both PCell/PSCell and sSCell. ### Proposal 3 When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 or 2_5 in Type 3 CSS set configured on PCell/PSCell or/sSell, and applicability of the information in the DCI formats is as per Rel15/Rel6 framework |
| LG | Support | Also fine with vivo’s update. |
| DOCOMO | Support |  |
| Apple | Need discussion | We also have 2_5 and 2_6. Also similar concern as Qualcomm |
| ETRI | Support |  |
| Xiaomi | Support |  |
| OPPO | Support | Fine with vivo’s update |
| Huawei, HiSilicon | support | In current specification, Type3-PDCCH CSS set can be configured in both in a PCell/PSCell and an SCell. According to the PDCCH loads of a PCell/PSCell and an SCell, gNB can select the preferred cell to configure Type3-PDCCH CSS set and the RNTIs needs to be monitored. Thus, there is no need to change this. |
| Ericsson | support | We do not see the need to change anything from Rel15/Rel16 framework for these DCI formats when sSCell to PCell/PSCell scheduling is configured. |
| Lenovo, Motorola Mobility | Support |  |

Revised proposal below, based on above discussion

### Proposal 3v2

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set configured on PCell/PSCell/sSell [PCell/PSCell or sSell], and applicability of the information in the DCI formats is as per Rel15/Rel6 framework

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

Companies are requested to indicate their view about the above proposal in the Table below

| Company Name | support/not support | Comments (Proposal 3v2) |
| --- | --- | --- |

### Discussion point 4

- For USS handling, below options should be further discussed

  - Option 1

    - When CCS from an SCell (sSCell) to PCell/PSCell is configured, regarding DCI formats 0_1,1_1,0_2,1_2 that schedule PDSCH/PUSCH on PCell/PSCell, UE does not monitor them on PCell/PSCell USS set(s), and monitors them only on the sSCell USS set(s)

  - Option 2

    - When CCS from an SCell (sSCell) to PCell/PSCell is configured, regarding DCI formats 0_1,1_1,0_2,1_2 that schedule PDSCH/PUSCH on PCell/PSCell, UE can monitor them on both PCell/PSCell USS set(s) and sSCell USS set(s)

Companies are requested to indicate their view about the above options in the Table below. If companies prefer to study further, please indicate FFS and the aspects to be considered.

| Company Name | Preferred option | Comments (Discussion point 4) |
| --- | --- | --- |
| Intel | Option 2 | We prefer to allow the flexibility to configure a USS on PCell/PSCell or sSCell. By this way, operator can select a most proper configuration based for a deployment. Configuration of a USS on PCell/PSCell or sSCell could be per BWP. In fact, it is possible that the NR BW and LTE BW are only partially overlapped. In this case, it is possible that, a first BWP is overlapped with LTE hence coordination is needed, while a second BWP is not overlapped with LTE. For the latter case, it is preferable to allow USS configured on PCell. Even for the overlap case, it doesn’t mean that PCell cannot transmit PDCCH. In fact, UE anyway needs to monitor DCI 0_0/1_0 and DCI 2_x on PCell. Therefore, it doesn’t introduce new behavior to monitor a USS on PCell. The choice of configuration of a USS or not on PCell can be left to implementation. |
| Qualcomm | Option 1 | So far, for a given scheduled cell, there was only one scheduling cell that is configured by higher-layer signalling semi-statically. We do not think we need to change this principle. The reason why a UE configured with cross-carrier scheduling from an SCell to the PCell/PSCell has to monitor PDCCH on the PCell/PSCell is because of UE-common functions and fallback operation using CSS set(s). This does not mean we should enable USS on the PCell/PSCell. Allowing USS on the PCell/PSCell will cause further trouble if the subcarrier-spacings are different between PDCCH on the PCell/PSCell and PDCCH on the scheduling SCell. |
| CATT | Option 1 | We don’t see the necessity of supporting USS configuration on the scheduled P(S)Cell. The case here is similar to the current cross carrier scheduling scheme in some regard, wherein UE only monitoring USS on the scheduling cell. gNB has the freedom to configure the combination of scheduling cell and scheduled cell, and any proper search space on the scheduling cell. The flexibility is already fully respected from the current mechanism. As we commented on proposal 2, it doesn’t make sense to configure USS on the scheduled cell considering it can already been scheduled by a USS on the scheduling cell. It will cause more efforts of monitoring PDCCH on different cells in vain. |
| Vivo | Option 1 | In our view, there is no use case to monitor USS simultaneously from Pcell/PScell and sScell. It makes the handling of DCI size budget, BD/CCE budget more complicated. |
| Nokia | Option 2 | When cross carrier scheduling from SCell to PCell, (for formats 0_1,1_1,0_2,1_2) UE monitors USS on scheduling SCell. UE can be configured to monitor USS on PCell as well. We do not see a need to have any restriction on the PCell USS. It can be left to the implementation to configure USS on SCell and/or PCell. This also serves as a fall-back for the case when UE runs out of coverage on SCell. In situations when scheduling cell fall-back happens, gNB may switch SearchSpace group and within 2ms move the majority of USS PDCCH candidates from Scell to Pcell. R16 SS group switching feature may be used for this purpose. |
| InterDigital | Option 2 | Having USS only on SCell can create reliability issue for downlink control channels that schedule PCell. For example, the radio link quality of SCell can drop while the PCell radio link quality is still good. In that case, the UE cannot be scheduled on PCell using USS and RRC reconfiguration is needed to configure USS on PCell. |
| Samsung | Discuss further | We understand the concerns for complexity of option 2. However, if the PCell is viewed as another scheduled cell for the scheduling sSCell, Rel-16 can be directly re-used. Although the per-cell limits on BDs/CCEs may not apply (resulting total of BDs/CCEs can be larger), we don’t think that is a problem because it does not affect UE complexity. Otherwise, if the per cell limits on BDs/CCEs need to be kept, option 1 is of course simpler. But then there is no ‘D’ in DSS. At least, if option 1 is selected, self/cross-carrier scheduling should be BWP-specific, not cell-specific. |
| ZTE | Option2 | Option 1 may be simpler while Option 2 can offer higher flexibility for network implementation. Both alternatives are feasible, more studies are needed for these two alternatives. Since USS with corresponding DCI formats are configured by RRC, Option 2 supports dynamic switching of the PDCCH for PCell scheduling. When the UE is close to gNB, the scheduling PDCCH is transmitted on sSCell. When the UE moves to cell edge area, the PDCCH carrying non-fallback DCI can reside in PCell (usually low frequency and better for coverage) to guarantee the performance. While if Option 1 is supported, this switching will rely on RRC reconfiguration which is not flexible. |
| LG | Option 2 |  |
| DOCOMO | Discuss further | In the current specifications, scheduling cell for a serving cell is semi-statistically configured by schedulingCellInfo. Therefore, when PDCCH capacity of P(S)Cell becomes sufficient and NW would like to change from cross-carrier scheduling to self-carrier scheduling for the P(S)Cell, RRC reconfiguration is necessary. Option2 doesn’t have the concern. On the other hand, if option1 is selected, we may need to consider enhancement to address the above point (e.g. dynamic switching of scheduling cell for P(S)Cell). |
| Apple | Option 1 | When SCell is configured to schedule PCell, the main purpose is to schedule non-fallback DCI. Then we should maintain the same Rel-15 principle as much as possible, i.e. only one scheduling cell can schedule a scheduled cell. The reason we agree simultaneous PCell self-scheduling is due to the fall back and support of initial acquisition etc. |
| ETRI | Option 2 | Option 2 may prevent potential throughput loss when PDCCH reception from sSCell is suddenly disabled (due to e.g., link failure, dynamic activation/deactivation). |
| Xiaomi | Option 1 | In our view, there is no need to monitor USS simultaneously from Pcell/PScell and sScell. It makes the handling of DCI size budget, BD/CCE budget more complicated. |
| OPPO | Option 2 | It is not necessary to restrict USS configuration in sSCell. Moreover, there is reliability and coverage issue for sSCell. However, to avoid exceeding BD/CCE budget, USS configuration can be configured in either PCell/PSCell or sSCell. It is left to implementation. To cover flexible configuration in implementation, we update option 2 - Option 2 - When CCS from an SCell (sSCell) to PCell/PSCell is configured, regarding DCI formats 0_1,1_1,0_2,1_2 that schedule PDSCH/PUSCH on PCell/PSCell, UE can monitor them on both PCell/PSCell USS set(s) and/or sSCell USS set(s) |
| Huawei, HiSilicon | Option 2 | It is preferred to monitor USS set(s) on the SCell and CSS set(s) on the PCell/PSCell, which means there will be anyway two carriers used for scheduling the PCell/PSCell. If the USS can be only configured in SCell in this case, there may be benefits for UE power saving; if the USS can be configured on both PCell/PSCell and SCell, more scheduling opportunity can be available at the cost of UE power saving. From network flexibility point of view, it may be beneficial to allow the latter as one configuration option, which can be turned ON/OFF based on e.g. UE capability and network configuration. |
| Ericsson | Option 2 | The impact on PCell operation should be reduced as much as possible when SCell to PCell scheduling is configured. Non-fallback DCI formats such as 0_1,1_1 are the normal mechanism of scheduling data on a cell. From NW perspective, Option 2 allows broad configuration of sSCell to PCell scheduling and the sSCell PDCCH resources can be opportunistically used to reduce NR PDCCH load on DSS carrier. With Option 1, this is not possible and NW has to upfront make a decision to limit or not the normal data scheduling for PCell/PSCell (i.e., during configuration of sSCell to PCell scheduling), and this imposes a trade-off for configuring sSCell to PCell scheduling which we do not prefer. From UE procedures perspective, since CCS with different SCS is supported in Rel16, our expectation is the Rel16 framework can be mostly reused. Also, if the total PDCCH blind decoding budget is not changed, there should not be much UE complexity impact with this. |
| Lenovo, Motorola Mobility | Option 2 (fine also with FFS) | In our view, there could be advantageous of allowing such monitoring: for instance, - DCI format x_2 for PCell scheduling can be configured to be monitored with 0 bits for carrier indicator (saving URLLC DCI bits). - in case of PCell and sSCell having different SCS, the resulting UE PDSCH reception preparation time may not be desirable for URLLC operation (e.g., DCI format(s) x_2) |

### Proposal 5

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, CA activation/deactivation of the sSCell is supported

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 5) |
| --- | --- | --- |
| Intel |  | We are fine with the proposal. However, it seems the proposal is not as basic as the other 4 proposals. It is fine to leave this issue for further study. |
| Qualcomm |  | Agree with Intel. |
| CATT |  | Share similar views with Intel. |
| vivo |  | Agree with Intel. In addition, dormancy behavior of sSCell should also be discussed for future study. |
| Nokia | Support | We agree with Vivo. |
| InterDigital |  | We agree with Intel |
| Samsung |  | Support in principle. This also relates to P4 and having self/cross-carrier scheduling be BWP specific (e.g. when sSCell moves to dormant, PCell becomes self-carrier scheduled) – can discuss together with P4. In addition, above proposals 1 ~ 4 require corresponding updates with this proposal 5, e.g.: Proposals 1 ~ 4: “When CCS from an SCell (sSCell) to PCell/PSCell is configured activated, ….” |
| ZTE | Support | We are ok with the proposal and we are also fine to either clarify it in this meeting or in the future meeting. |
| LG |  | Agree with vivo. |
| DOCOMO |  | We agree with Intel. |
| Apple | Need discussion | First of all, it is related to whether non-fallback DCI is only monitored on sSCell which is our preferred solution. In that case, if NW uses MAC-CE to deactivate the sSCell, UE cannot receive non-fallback DCI scheduled data on PCell, i.e., UE can only be scheduled by fallback DCI. This is partly like to deactivate the PCell. |
| ETRI | Support | We support the proposal, and also agree with Intel. |
| Xiaomi |  | We agree with Intel. |
| OPPO | Support | We agree with Intel and vivo |
| Huawei, HiSilicon | support | When USS set(s) are configured in PCell/PSCell and SCell, CA activation/deactivation of the SCell can be supported obviously. When USS is configured only on the SCell and not on the PCell/PSCell, fallback mechanism can be considered. |
| Ericsson | support | CA activation/deactivation should be applicable for sSCell as well. |
| Lenovo, Motorola Mobility | FFS | We prefer to leave this issue FFS for now. |

# 3 Conclusions

TBD

# 4 References

- R1-2007579	Discussion on SCell PDCCH scheduling P(S)Cell PDSCH or PUSCH	Huawei, HiSilicon

- R1-2007695	Discussion on Scell scheduling Pcell	vivo

- R1-2007839	Disucssion on cross-carrier scheduling from Scell to Pcell	CATT

- R1-2008062	Discussion on cross-carrier scheduling from SCell to Pcell	LG Electronics

- R1-2008110	Discussion on cross-carrier scheduling from SCell to Pcell	Spreadtrum Communications

- R1-2008195	Cross carrier scheduling from SCell to Pcell	Samsung

- R1-2008284	Discussion on cross-carrier scheduling from Scell to Pcell	OPPO

- R1-2008451	Views on Rel-17 DSS SCell scheduling PCell	Apple

- R1-2008695	Discussion on cross-carrier scheduling from SCell to PCell	ASUSTeK

- R1-2008830	Discussion on Cross-Carrier Scheduling from SCell to PCell	ZTE

- R1-2009003	On SCell scheduling PCell transmissions	Intel Corporation

- R1-2009023	Cross-carrier scheduling from SCell to Pcell	ETRI

- R1-2009040	Discussion on Cross-carrier scheduling from SCell to Pcell	Xiaomi

- R1-2009046	Cross-carrier scheduling from SCell to Pcell	Nokia, Nokia Shanghai Bell

- R1-2009085	Search space monitoring to support SCell scheduling PCell 	InterDigital, Inc.

- R1-2009110	Cross-carrier scheduling (from Scell to Pcell)	Lenovo, Motorola Mobility

- R1-2009195	Discussion on cross-carrier scheduling enhancements for NR DSS	NTT DOCOMO, INC.

- R1-2009206	Enhanced cross-carrier scheduling for DSS	Ericsson

- R1-2009277	Views on cross-carrier scheduling from an SCell to the PCell/PSCell	Qualcomm Incorporated

# 5 Annex A – Agreements from previous meetings

## Agreements from RAN1#102-e

Agreements:

- Following scheduling combinations are allowed/not allowed when cross-carrier scheduling from an SCell to PCell/PSCell is configured

  - self-scheduling on PCell/PSCell is allowed

  - cross-carrier scheduling from PCell/PSCell to another SCell is not allowed

  - self-scheduling on the ‘SCell used for scheduling PCell/PSCell’ is allowed

  - cross-carrier scheduling from the ‘SCell used for scheduling PCell/PSCell’ to another serving cell is allowed

  - cross-carrier scheduling from another serving cell to the ‘SCell used for scheduling PCell/PSCell’ is not allowed

- FFS: Search space and DCI format handling for the allowed cases above

Agreements:

- Configuring 2 or more Scells to schedule the PCell/PSCell is not allowed