3GPP TSG-RAN WG1 #103-e	R1-2009537

# e-Meeting, October 26th – November 13th, 2020‎

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0158 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on HARQ-ACK generation for DL transmission with single TB when multi-TB is configured |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-11-5 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In current specification for Type-1 HARQ-ACK codebook, for a serving cell configured with reception of maximum two transport blocks, the HARQ-ACK generation for a PDSCH occasion in response to a PDCCH with DCI format 1_0 is not defined, which may lead to misunderstanding that only one HARQ-ACK bit is generated for the PDSCH occasion when HARQ-ACK spatial bundling is not configured. Note that since there is description in the pseudo-code that “if the UE receives one transport block, the UE assumes ACK for the second transport block”, there should be no confusion when HARQ-ACK spatial bundling is configured. In addition, HARQ-ACK generation for a SPS PDSCH release for a serving cell configured with reception of maximum two transport blocks is missing. - In current specification for Type-2 HARQ-ACK codebook, in case UE is configured with reception of maximum two transport blocks, the HARQ-ACK feedback generation for a SPS PDSCH release is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete the condition “an occasion for a candidate PDSCH reception can be in response to a PDCCH with DCI format 1_1 and if” in clause 9.1.2.1 to include PDSCH occasions in response to a PDCCH with DCI format 1_0. - Add “or a SPS PDSCH release” in clauses 9.1.2.1 and 9.1.3.1 to clarify the HARQ-ACK generation in response to a SPS PDSCH release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Undefined UE behaviour of HARQ-ACK generation for a PDSCH occasion in response to a PDCCH with DCI format 1_0 for a serving cell configured with reception of maximum two transport blocks without HARQ-ACK spatial bundling for Type-1 HARQ-ACK codebook - Undefined UE behaviour of HARQ-ACK generation for SPS PDSCH release for both Type-1 and Type-2 HARQ-ACK codebooks in case UE is configured with reception of maximum two transport blocks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1, 9.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact on HARQ-ACK feedback bit generation for a DL transmission with a single TB and for SPS PDSCH release when multi-TB is configured for a serving cell. The corrections in this CR are common understandings which may have no impact on both UE and gNB implementations. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<unchanged part is omitted>

If maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block or a SPS PDSCH release, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

<unchanged part is omitted>

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

<unchanged part is omitted>

For a PDCCH monitoring occasion with DCI format scheduling PDSCH reception or SPS PDSCH release in the active DL BWP of a serving cell, when a UE receives a PDSCH with one transport block or a SPS PDSCH release and the value of maxNrofCodeWordsScheduledByDCI is 2, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

<unchanged part is omitted>