3GPP TSG RAN WG1 Meeting #103-e	                         R1-2009559

e-Meeting, October 26th – November 13th, 2020

Source:                Moderator (Lenovo)

Title:                     Feature lead summary#1 on multi-cell scheduling via a single DCI

Agenda item:       8.13.2

Document for:     Discussion and Decision

# Introduction

This document summarizes the contributions submitted under the “Multi-cell PDSCH scheduling via a single DCI” agenda item of the Rel-17 work item on “Dynamic spectrum sharing (DSS)”.

The revised DSS WID [1] contains the following objective related to this agenda item:

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS): - PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2] - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - The number of cells can be scheduled at once is limited to 2 - The increase in DCI size should be minimized - Note: The total PDCCH blind decoding budget should not be changed as a result of this work - Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

In RAN1 #102-e meeting, the following agreements under the “Multi-cell PDSCH scheduling via a single DCI” agenda item of was reached:

Agreements: - For the study on single DCI scheduling PDSCH on two cells - Consider the following scenarios as baseline for evaluation - UE configured with Inter-band CA with PCell and an SCell - PCell for the UE is operated on a DSS carrier (i.e., same carrier is also used for serving LTE users) - Case 1: Different SCS for PCell and SCell - Case 2: Same SCS for PCell and SCell - Additional scenarios can also be evaluated, e.g. as below - Intra-band CA case with multiple serving cells having same SCS (all cells operated on non DSS carriers) - Inter-band CA case with PCell and more than one SCell (at least the SCells are operated on non DSS carriers) - Note: other combinations not precluded - Note: Further details of evaluation framework (including carrier BW, slot format etc.) to be discussed in next stage

In Section 2, candidate schemes for multi-cell PDSCH scheduling, evaluation assumptions as well as evaluation results are summarized. Companies’ views on whether to support this feature are also summarized at the end of Section 2. Based on majority companies’ views, some proposals are listed for discussion purpose.

In Section 3, the standard impacts on DCI format design and HARQ-ACK codebook determination are summarized. Several open questions are listed in this section and companies are encouraged to provide their inputs for each open question.

In Section 4, miscellaneous issues are listed which can be treated in low priority.

# Summary of contributions

The section summarises key proposals and observations from submitted contributions.  A few proposals and questions to resolve based on the general leaning of the companies are captured below.

## Candidate schemes for multi-cell PDSCH scheduling

Regarding number of TBs carried by the multiple PDSCHs scheduled by a single DCI, many companies show their views in below contributions. Based on the summary, most companies think the scheduled multiple PDSCHs carrying a single TB will lead to significant standard impact on CA framework, HARQ and protocol layer design and propose the scheduled multiple PDSCHs carry different TBs.

For two PDSCHs on two carriers scheduled by a single DCI, companies’ views on same or different TBs are summarized below:

| Company | Key Proposals/Observations |
| --- | --- |
| Huawei, HiSilicon | Scheme 1: one DCI schedules two PDSCHs over two cells Scheme 2: one DCI schedules one PDSCH over two cells |
| vivo | Proposal 2. The study focuses on the case that the two PDSCH jointly scheduled by a single DCI correspond to different TBs. |
| CATT | Proposal 2: Two TBs should be scheduled separately on different serving cells for multi-cell PDSCH scheduling via a single DCI. |
| Spreadtrum | Clarify which the scheme is in the scope. |
| ETRI | Observation 4: For multi-cell joint scheduling, the principle that one PDSCH does not span multiple cells can be kept to minimize the workload. |
| Lenovo, Motorola Mobility | Observation 5: Scheduling a single TB on two carriers is not in the scope of Rel-17 DSS. |
| NTT DOCOMO | Observation 11: - Whether the same TB and/or different TBs is/are scheduled on multiple cells can be considered. |

Based on the majority companies’ views, below proposal can be discussed:

FL proposal#1:

For Rel-17 DSS, when multiple PDSCHs are scheduled by a single DCI, the multiple PDSCHs carry different TBs.

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | Support |
| Apple | Support, even though we do not see the need to even allow single DCI to schedule two PDSCH in different CCs In the current CA, we do not have HARQ process sharing between two CCs, so different TB is a nature baseline. |
| InterDigital | Support |
| Charter Communications | We think both a single TB and two TBs can be scheduled. |
| Huawei, HiSi | Open to either with benefits. |
| Spreadtrum | Support |
| vivo | Agree. Support of the scheduling of a same TB on two cells will involve a lot of RAN2 spec change which is undesirable. |
| Qualcomm | Agree. It would be fair to say “when and if multiple PDSCHs are scheduled by a single DCI, …” |
| Lenovo, Motorola Mobility | Support |
| Moderator | @Qualcomm: updated proposal 1 as suggested. |
| CATT | Support |
| OPPO | Open to either. |

### Updated proposals

FL proposal#1:

For Rel-17 DSS, when and if multiple PDSCHs are scheduled by a single DCI, the multiple PDSCHs carry different TBs.

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| ZTE | It seems that we may need to first determine whether to support one PDCCH schedules two PDCHs on two carriers or not. If supported, then we are fine with the above proposal. |
| NTT DOCOMO | OK to support. |
| CATT | Support |
| Intel | Support, maybe better to also emphasis that a TB carried by a PDSCH on a carrier is associated with a HARQ process of the carrier |
| vivo | Support |
| OPPO | Open to either. |
| Ericsson | We prefer below formulation given this is under study “For the study on using single DCI for scheduling multiple PDSCHs, it is assumed for the evaluations that the multiple PDSCHs carry different TBs.” |

## Evaluation

### Evaluation assumptions:

| Parameters | Values |
| --- | --- |
| Scenario | Inter-band CA Intra-band CA |
| Bandwidth | PCell: 10/20MHz SCell: 10/20/100MHz |
| Simulation cases | Case 1: two DCIs with baseline payload size schedule two PDSCHs on two carriers, respectively Case 2: a single DCI with larger payload size schedules two PDSCHs on two carriers |
| DCI payload size (excluding CRC) | Single PDSCH scheduling: 60 bits Multi-cell PDSCH scheduling: 72/84/96/108/120 bits |
| Evaluation metrics | PDCCH blocking probability, CCE saving ratio, PDSCH throughput improvement, UE power saving |

### PDCCH blocking probability

Since NR transmission can’t use REs occupied by LTE CRS and LTE PDCCH region on a carrier shared with LTE, NR PDCCH capacity on this shared carrier is limited especially when this shared carrier is configured as PCell for NR. The insufficient NR PDCCH capacity on the NR PCell will lead to system performance degradation especially when more NR devices are camped on the NR PCell.

Supporting cross-carrier scheduling from NR SCell to NR PCell results in requiring additional PDCCH capacity of the scheduling SCell due to the need for self-scheduling on the SCell as well cross-carrier scheduling on the (shared carrier) PCell. Thus, the PDCCH capacity on the SCell may be a potential issue when a large number of UEs are configured on the SCell or the SCell is not configured with a large enough bandwidth. This issue can be addressed by allowing a single DCI on one carrier to schedule PDSCHs on two carriers. In detail, two PDSCHs on two carriers are scheduled by a single DCI format, which saves PDCCH scheduling overhead compared to scheduling two PDSCHs on two carriers by two DCI formats. Since the number of required PDCCHs is reduced, many companies think the PDCCH blocking probability can be alleviated.

On the other hand, in inter-band CA, the payload size of the single DCI increases significantly when it schedules two PDSCHs on two carriers due to different channel conditions. Some companies think it may increase PDCCH blocking rate since a high AL is needed for this DCI. For intra-band CA, some companies think the payload size of the single DCI does not increase significantly by sharing many fields of the DCI.

Regarding PDCCH blocking probability, companies’ views are summarized as below:

| Company | Key Proposals/Observations |
| --- | --- |
| Huawei, HiSilicon | Observation 1: For 700MHz + 800MHz scenario, one PDCCH scheduling PDSCH(s) on two cells can achieve up to 23%~50% average CCE saving ratio. Observation 2: For 800MHz + 1800MHz scenario, one PDCCH scheduling PDSCH(s) on two cells can achieve up to 18.2%~50.4% average CCE saving ratio. Observation 3: For 1800MHz + 3500MHz scenario, one PDCCH scheduling PDSCH(s) on two cells can achieve up to 12.8%~43% average CCE saving ratio. Observation 4: Single DCI scheduling PDSCH(s) on two cells can reduce the PDCCH blocking probability obviously |
| Vivo | Observation 1. Compared with using single DCI, a joint DCI scheduling two PDSCHs on two cells brings around 44.87% and 44.51% CCE saving gain if the joint DCI size (excluding CRC) is less than 60% of the total size of two single DCI at 2.6GHz and 4GHz respectively, and the gain becomes less significant if the compression rate of joint DCI size increases. Observation 3. Compared with using single DCI, a joint DCI scheduling two PDSCHs on two cells brings around 13.95% or 13.57% reduction in PDCCH blocking rate if the joint DCI size (excluding CRC) is less than 60% of the total size of two single DCI at 2.6GHz and 4GHz respectively, and the reduction rate becomes less significant if the compression rate of joint DCI size increases. Observation 4. PDCCH blocking rate reduction by joint DCI scheduling can only bring a marginal <=3.31% throughput gain in practical scenarios at 4GHz. |
| CATT | Multi-cell scheduling via a single DCI can significantly reduce the possibility of PDCCH blocking. Consequently, the PDCCH capacity increase remarkably. The benefits harvested from DSS-DCI in terms of PDCCH capacity is impacted on the DCI design, i.e. the smaller size the DSS-DCI has, the more significant benefit can be got. |
| OPPO | Observation 2: The smaller DCI size increases, the more UEs achieve gain from one-to-two scheduling. |
| MediaTek | Observation 2: Around 30% DCI overhead reduction is observed for Rel-17 DCI aggregation for x-carrier scheduling, assuming that the aggregated DCI size is scaled with the number of scheduled cells. |
| Lenovo, Motorola Mobility | Observation 1: Compared to a single DCI scheduling a single PDSCH, the payload size of a single DCI scheduling two PDSCHs on two carriers needs to be increased about 21~54%. Observation 2: Compared to two DCIs scheduling two PDSCHs, the payload size of a single DCI scheduling two PDSCHs on two carriers can save 23% ~ 39% overhead. |
| Intel | Observation 2: Based on the required SINR values and geometry curves obtained by LLS and SLS - The ratio of CCE saving is about 20~40%; - The reduced PDCCH blocking ratio is observed. |
| Samsung | Observation 4: Coverage and relative BLER comparisons for DCI format C2 further worsen for operation under less favorable conditions such as with some correlation or blockage of UE receiver antennas or for 2 UE receiver antennas. Observation 5: For DSS, a maximum gain in resources per slot from scheduling PDSCH on 2 cells using a single DCI format is ~0.35% for a BWP of 20 MHz and ~0.07% for a BWP of 100 MHz on the scheduling cell. |
| Nokia, NSB | Observation 1: Two-cell DCI format reduces overhead by at least 24 CRC bits, and if single DCI field applies to both cells (at least for HARQ-related parameters), further significant PDCCH overhead reduction is expected. Observation 6: - A 120 bits (>106bits) PDCCH payload does not fit anymore to 1CCE, this means that for 50-70% users, aggregation level must be doubled. For the AL>2, the 120bit PDCCH payload probability is not anymore significantly increased compared to 80 and 100bit, and could be also compensated by increasing PDCCH TX power. - When comparing 80bit and 100bits PDCCH payload, the AL probability is similar |
| InterDigital | Observation 2: PDCCH blocking probability and CCE utilization can be reduced by using a single DCI scheduling PDSCH on two cells. |
| DOCOMO | Observation 1: - PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI can improve PDCCH resource efficiency. |
| Ericsson | Initial evaluations indicate that DCI scheduling PDSCH on two cells (mc-DCI) provides no/marginal performance gains. |
| Qualcomm | Observation 1: The gain from multi-cell PDSCH scheduling compared to single-cell PDSCH scheduling in inter-band CA for DSS scenario is mainly comes from the omission of 24-bit CRC. In intra-band CA scenario, higher gain would be achievable by compressing some DCI fields. |

### PDSCH throughput improvement

Based on Rel-15 NR CORESET design principle, the unused control resource REs can be scheduled for PDSCH transmission, thus some companies observed PDSCH throughput improvement due to the reduction of PDCCH overhead.

However, some companies observed PDSCH throughput loss due to the inappropriate scheduling parameters that have to be shared between the two carriers even though PDCCH blocking probability is decreased.

Regarding PDSCH throughput improvement, companies’ views are summarized as below:

| Company | Key Proposals/Observations |
| --- | --- |
| Huawei, HiSilicon | Observation 5: For two carriers of 10MHz @ 700MHz&800MHz frequency with or without DSS, when the traffic load is high, one PDCCH scheduling PDSCH(s) on two carriers can achieve 7.2% to 15% throughput gain. |
| Vivo | Observation 2. CCE saving by joint DCI scheduling can bring up to around 6.41% throughput gain if the PDSCH can rate match around PDCCH. |
| OPPO | Observation 1: One-to-two scheduling has 13.5% throughput gain due to PDCCH overhead reduction. |
| ZTE | Observation 2: For Case B (most of the DCI fields are separate indicated for two carriers), the cell throughput performance is almost the same as the baseline and the gain of PDCCH blocking rate is marginal. For Case C (most of the DCI fields are shared between two carriers), the cell throughput performance is decreased by 13~16% compared with the baseline though the PDCCH blocking rate is decreased. |
| Samsung | Observation 4: Coverage and relative BLER comparisons for DCI format C2 further worsen for operation under less favorable conditions such as with some correlation or blockage of UE receiver antennas or for 2 UE receiver antennas. Observation 5: For DSS, a maximum gain in resources per slot from scheduling PDSCH on 2 cells using a single DCI format is ~0.35% for a BWP of 20 MHz and ~0.07% for a BWP of 100 MHz on the scheduling cell. |

### UE blind detection reduction and power saving

Using a single DCI format scheduling two PDSCHs on two carriers can save UE’s power consumption since UE needs to monitor the DCI in the search space of only one carrier where the DCI format is transmitted. This is especially true when the scheduling cell is configured with small bandwidth and the scheduled cell has ultra-wide carrier.

Regarding UE power saving, companies’ views are summarized as below:

| Company | Key Proposals/Observations |
| --- | --- |
| Huawei, HiSilicon | Observation 10: A single PDCCH scheduling PDSCH over two cells can save up to 6.67%~15% power consumption comparing with two separate PDCCHs for scheduling. |
| MediaTek | Observation 3: There is 50% and 15% reduction are observed for UE blind decoding complexity and power consumption, respectively. |
| Charter Communications | Observation 1: The ability to schedule PDSCH on multiple cells using a single DCI will help reduce UE power consumption when operating with ultra-wide carriers, for e.g., NR beyond 52.6 GHz. |
| Lenovo, Motorola Mobility | Observation 3: Using single DCI scheduling two PDSCHs on two carriers can save UE’s power consumption. |
| Nokia, NSB | Observation 2: Two-cell DCI format may reduce UEs monitoring burden as UE needs to monitor search-space set(s) of only single scheduling cell compared to R16, given that design is based on DCI format 1_1. |

### Preliminary observations

Preliminary observations on PDCCH blocking probability using a single DCI to schedule multiple PDSCHs on multiple carriers are summarized below:

- 14 companies [Huawei, HiSilicon, vivo, CATT, OPPO, MediaTek, Lenovo, Motorola Mobility, Intel, Nokia, NSB, InterDigital, DOCOMO, Qualcomm] observed decreased PDCCH blocking probability or reduced CCE consumptions.

- 3 companies [ZTE, Samsung, Ericsson] observed marginal performance gain in PDCCH blocking.

Preliminary observations on PDSCH throughput improvement by scheduling unused control resource REs for PDSCH transmission are summarized below:

- 4 companies [Huawei, HiSilicon, vivo, OPPO] observed about 6~15% performance gain.

- 1 company [ZTE] observed 13~16% performance loss due to shared scheduling information for the two carriers.

Preliminary observations on UE blind decoding reduction and power saving are summarized below:

- 8 companies [Huawei, HiSilicon, MediaTek, Charter Communications, Lenovo, Motorola Mobility, Nokia, NSB] think multi-cell scheduling by a single DCI can save UE power.

### Whether to support multi-cell PDSCH scheduling by single DCI?

Regarding whether to support multi-cell PDSCH scheduling by a single DCI, below companies show clear views which are summarized in below table.

Company views:

| Company | Key Proposals/Observations |
| --- | --- |
| CATT | Proposal 1: Multi-cell PDSCH scheduling via a single DCI should be further studied considering it can bring significant benefits in terms of PDCCH capacity, PDSCH throughput and network flexibility. |
| Vivo | Observation 1. Compared with using single DCI, a joint DCI scheduling two PDSCHs on two cells brings around 44.87% and 44.51% CCE saving gain if the joint DCI size (excluding CRC) is less than 60% of the total size of two single DCI at 2.6GHz and 4GHz respectively, and the gain becomes less significant if the compression rate of joint DCI size increases. |
| Spreadtrum | Support multi-cell PDSCH scheduling via a single DCI. |
| ASUSTeK | Proposal 1: NR DSS supports PDCCH scheduling PDSCHs on two cells using a single DCI. |
| Samsung | Proposal: A DCI format that schedules PDSCH receptions on two cells is not introduced. |
| Apple | Proposal 1: We do not observe enough justification and motivation to allow single DCI to schedule PDSCH on multiple cells. |
| ZTE | Proposal 1: RAN1 further discusses the necessity, gain, open issues and possibility of timely completion of single DCI scheduling two PDSCHs on two carriers. |
| Charter Communications | Proposal 1: Consider enhanced multi-carrier operation where a single DCI can schedule PDSCH on two non-DSS cells with the same SCS, including SCells with a dormant BWP, for energy-efficient and low-latency NR performance. |
| Lenovo, Motorola Mobility | Proposal 1: Support using a single DCI to schedule two PDSCHs on two carriers. |
| MediaTek | Proposal 4: Support DCI aggregation for cross-carrier scheduling in Rel-17 DSS. - FFS whether DCI aggregation for cross-carrier scheduling is 1-stage or 2-stage |
| Nokia, NSB | Proposal 1: Support multi-cell DCI in R17, focus on multiple SCell (2 or more) with the same/similar carrier size and SCS first. Strive to keep DCI format 1_1 payload <106bits (including CRC). |
| InterDigital | Proposal 1: Support a single DCI to schedule two PDSCH in different cells. |

So far, 10 companies [CATT, Spreadtrum, ASUSTeK, Charter Communications, MediaTek, Lenovo, Motorola Mobility, Nokia, NSB, InterDigital] clearly propose to support such multi-carrier scheduling. 2 companies [Samsung, Apple] suggests not to introduce this feature in Rel-17 DSS.

Based on majority companies’ views, below proposal is made:

FL Proposal#2:

- Support using a single DCI to schedule two PDSCHs on two carriers.

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | Support |
| Apple | We do not see the need. |
| Samsung | Do not agree with the proposal. The WID mentions “Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI”. A statement such as “X companies support, Y companies do not support, and X > Y” is not any sort of study or an analysis for benefits/drawbacks/trade-offs of the proposal. |
| InterDigital | Support |
| Charter Communications | Support |
| Huawei, HiSi | Support the proposal and Ok with continuing some discussion per SS comment. We think the FL proposal is naturally to collect companies view based on the presented study of contributions. |
| Spreadtrum | Support |
| vivo | Since this is a study objective, we think we should start with some discussions about the expected/achievable gain by this feature from different aspects, hopefully some observations can be drawn from RAN1 perspective. |
| Qualcomm | OK to support, if we make sure that the design is forward looking for intra-band CA and for more than 2 carriers. |
| Lenovo, Motorola Mobility | Support |
| ZTE | Discuss and summarize the potential gain first. Based on companies’ input, simulations under different assumptions have been performed and different observations have been made by companies. From our perspective, we believe it may be better to first have some discussion on the simulation results and try to make some observations. If the observation is obvious, then we can discuss whether to support this enhanced scheduling or not based on the observations. If the observation is diverging, then we may need to further align simulation assumptions. |
| CATT | Support. As summarized by FL, the benefits coming with multiple PDSCHs via single DCI have been studied from several aspects. We think it is fair enough to at least say the feature is studied. |
| LG | Similar view with Samsung. Negative to the proposal. Basically, we think this multi-CC scheduling is quite different from the multi-TTI scheduling (already introduced in Rel-16 NR-U for PUSCH and currently being discussed in Rel-17 SI for PDSCH), at least in terms of the benefit to save DCI overhead. The benefit expected to be obtained by the multi-CC scheduling is very limited compared to the multi-TTI scheduling due to basically uncorrelated MCS/RA and other HARQ parameters across multiple CCs. In addition, from the SI procedure perspective, we agree with ZTE that the evaluation results and relevant observations are to be summarized/discussed first before deciding whether to support this feature according to clear statements in the SID. |
| OPPO | Support |
| Ericsson | In our view, focus at this stage should be on evaluations and obtaining conclusions from the evaluations. Our initial evaluations indicate that DCI scheduling PDSCH on two cells (mc-DCI) provides no/marginal performance gains. |

### Updated proposals

Discussion points on simulation results:

Preliminary observations on CCE overhead saving by using a single DCI scheduling two carriers compared to using two legacy DCIs scheduling two carriers are summarized below:

- 2 companies [Huawei, HiSilicon] observed 12.8%~50.4% CCE saving ratio achieved.

- 1 company [vivo] observed 44.87% and 44.51% CCE saving gain if the joint DCI size (excluding CRC) is less than 60% of the total size of two single DCI at 2.6GHz and 4GHz respectively, and the gain becomes less significant if the compression rate of joint DCI size increases.

- 1 company [Intel] observed about 20~40% CCE saving.

Preliminary observations on PDCCH blocking probability by using a single DCI scheduling two carriers compared to using two legacy DCIs scheduling two carriers are summarized below:

- 13 companies [Huawei, HiSilicon, CATT, OPPO, MediaTek, Lenovo, Motorola Mobility, Intel, Nokia, NSB, InterDigital, DOCOMO, Qualcomm] observed decreased PDCCH blocking probability.

- 1 company [vivo] observed 13.95% or 13.57% reduction in PDCCH blocking rate if the joint DCI size (excluding CRC) is less than 60% of the total size of two single DCI at 2.6GHz and 4GHz respectively, and the reduction rate becomes less significant if the compression rate of joint DCI size increases.

- 3 companies [ZTE, Samsung, Ericsson] observed marginal performance gain in PDCCH blocking.

Preliminary observations on PDSCH throughput by scheduling unused control resource REs for PDSCH transmission are summarized below:

- 2 companies [Huawei, HiSilicon] observed 7.2% to 15% throughput gain.

- 1 company [OPPO] observed 13.5% throughput gain.

- 1 company [vivo] observed up to 3.31% throughput gain.

- 1 company [ZTE] observed 13~16% performance loss due to shared scheduling information for the two carriers.

Preliminary observations on UE blind decoding reduction and power saving are summarized below:

- 2 companies [Huawei, HiSilicon] observed 6.67%~15% UE power saving by using single DCI scheduling two carriers compared with two legacy DCIs.

- 6 companies [MediaTek, Charter Communications, Lenovo, Motorola Mobility, Nokia, NSB] think multi-cell scheduling by a single DCI can save UE power.

Companies’ views on whether to support multi-cell scheduling via a single DCI are summarized below:

- Support (14): Nokia, NSB, InterDigital, Charter Communications, Huawei, HiSilicon, Spreadtrum, Qualcomm, CATT, Lenovo, Motorola Mobility, OPPO, ASUSTeK, MediaTek

- Not support (3): Samsung, LG, Apple

Proposals:

Further study the multi-cell PDSCH scheduling via a single DCI with following evaluation assumptions and scenarios:

| Parameters | Values |
| --- | --- |
| Scenario | Inter-band CA (700MHz + 3.5GHz) Intra-band CA |
| SCS | 15 kHz for 700MHz 30 kHz for 3.5GHz |
| Bandwidth | PCell: 10/20MHz SCell: 10/20/100MHz |
| Number of symbols for CORESET | 2 |
| CORESET BW (contiguous PRB allocation) | 24/48/96 RBs depending on SCell bandwidth and SCS |
| CCE-to-REG mapping | interleaved |
| REG bundle size | 6 |
| Interleaver size | 2 |
| DCI payload size (excluding CRC) | Single PDSCH scheduling: 60 bits as baseline payload size Multi-cell PDSCH scheduling: 72/84/96/108/120 bits |
| BLER target for multi-cell scheduling DCI | Option 1: 1% Option 2: 0.5% |
| Evaluation metrics | CCE saving ratio PDCCH blocking probability PDSCH throughput UE power consumption |

Note: PDCCH is transmitted on SCell for cross-carrier scheduling PCell.

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Intel | Some clarifications. Is PCell on the lower carrier frequency? Will PDCCH be transmitted on PCell or SCell? Since we are targeting DSS, it is better to have an assumption on number of CRS ports for LTE carrier. The number of PRBs in CORESET is impacted by the BW of a carrier. E.g. It only has about 50 PRB for 10MHz BW with SCS 15KHz. |
| ZTE | First of all, regarding the evaluation metrics, as the key issue for DSS WI is to increase the PDCCH capability, it seems that “UE power saving” may not be a key metric here. Secondly, LLS and geometry are needed for “CCE saving ratio” and “PDCCH blocking probability” simulation. SLS is needed for “PDSCH throughput” simulation. Since companies’ views are diverging on whether to support one-to-two scheduling, it would be good for companies to align the simulation assumptions and compare simulation results to see whether there is any gain. We should at least try to align the simulation assumption for LLS and geometry. If possible, we would also like to align the SLS simulation assumption. Usually, 2GHz and 4GHz are typical carrier frequency for LLS and SLS, e.g, like what we did in Coverage WI. Thus, we prefer to add 2GHz and change 3.5GHz to 4GHz for inter-band CA. For intra-band CA, we would prefer to use 2GHz. Currently, the CORESET BW is 96 RBs. To support 96 RBs CORESET, we may need at least 20MHz bandwidth assuming 15KHz. Thus, the 10MHz can be deleted from our perspective. For DCI size, we prefer to align with what we discussed in last meeting, i.e., 72, 84, 85, 96 and 110 bits. Regarding the detailed SLS assumption, we have provided our assumptions in our tdoc x8831. |
| vivo | Share the same view with intel, whether the PDCCH is on Pcell and Scell should be clarified first. Given that the key issue is PDCCH capacity on DSS carrier and multi-cell scheduling is expected to reduce the PDCCH load of DSS carrier, it is typical to assume that the PDCCH is on the Pcell on the lower frequency and the Scell should be scheduled by PDCCH on Pcell. If CORESET BW=96 RB is assumed, it seems 10MHz for 15kHz case and 10/20MHz for 30kHz case can be removed from the table. Or, we need to provide more options for CORESET bandwidth, e.g., 24/48 RB. Moreover, we share the same view as ZTE that 2.6GHz and 4GHz are typical and should also be considered. So we would like to update the assumptions by following: Scenario; Inter-band CA (700MHz or 2.6GHz for Pcell + 3.5GHz or 4GHz for Scell) Intra-band CA. SCS; 15 kHz for 700MHz or 2.6GHz 30 kHz for 3.5GHz or 4GHz. Note: PDCCH is located on Pcell and the Scell is scheduled by Pcell |
| OPPO | Support |
| Huawei, HiSi | We think the discussion should be: - Draw initial observations/conclusions based on the common part of presented simulations - Further discuss additional simulations that are needed – this should have a clear necessity on top of the above presented results For LLS results, we are not comparing different schemes to down-select one, rather, (almost) all link-level results have justified the benefits of using single PDCCH scheduling multiple carriers in same/different scenarios. Therefore, we should be able to draw some conclusions already, based on the presented simulation results where as summarized below there are large proportion of commonality in the link-level simulation assumptions. For the parameters having different values, it could be understood that there will not be obvious impact on the possible conclusion/observation, e.g. frequency carrier, frequency distributed or not, Rx combining. |
| Moderator | @Intel: yes, PCell is transmitted on low frequency. PDCCH is transmitted on SCell for cross-carrier scheduling PCell. Regarding number of RBs for CORESET, 48 RBs are added. @ZTE: (1) UE power saving can be an optional metric. I see some companies have such results. (2) I agree with you. That is also my intention. (3) I think 2.6GHz or 4GHz will not impact on simulation results compared with 700MHz or 3.5GHz. (4) 48 RBs are added. (5) 72/84/96/108/120 bits are evenly increased which seem more reasonable. @vivo: For DSS, I think the typical case is PDCCH is transmitted on SCell and cross-carrier schedules PCell. I added one note below the table. Meanwhile, I added 24/48 RBs for CORESET bandwidth. Regarding the carrier frequency, I think 2.6GHz or 4GHz will not impact on simulation results compared with 700MHz or 3.5GHz. |
| Ericsson | Suggest following updates for the evaluation assumptions to be consistent across different BWs chosen Bandwidth – 40MHz should be included at least for SCell (10/20/40/100MHz) CORESET Size – 273RBs should be included as 100MHz is considered (24/48/96/273 RBs) Number of symbols for CORESET – 1 or 2 On evaluation metrics – suggest replacing “CCE saving ratio” with “Control channel overhead” as what we need to measure is to what extent the control channel overhead can be reduced. |

|  | MTK (R1-2008963) | CATT(R1-2007840) | Ericsson(R1-2009207) | Huawei(R1-2007580) | Intel(R1-2009004) | Interdigital(R1-2009086) | Nokia(R1-2009047) | OPPO(R1-2008285) | Samsung(R1-2008196) | vivo(R1-2007696) | ZTE(R1-2008831) | Observations | Huawei comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Freq. Carrier | 2.1GHz/4.4GHz | 700/800MHz | 4GHz | 700/800/1800/3500 MHz | 2.6GHz | 2 GHz |  | 4 GHz | 2.6 GHz | 2.6GHz 4GHz | 700MHz/2 GHz | 700MHz: 3 companies 800MHz: 2 companies 1.8GHz: 1 company 2/2.1GHz: 3 companies 2.6GHz: 3 companies 3.5GHz: 1 company 4/4.4 GHz: 4 companies | The parameter difference will have no obvious impact on the results for F+F and F+T respectively. Almost aligned. |
| System bandwidth | 20MHz for 2.1GHz 100MHz for 4.4GHz |  | 100MHz |  |  | 10 MHz |  |  | 20 MHz | 2.6GHz:20MHz/106 RB 4GHz:100MHz/273RB | 20 MHz | 10/20 MHz for FDD+FDD 20/100MHz for FDD+TDD | The parameter difference will have no obvious impact on the results for F+F and F+T respectively. Almost aligned. |
| SCS | 15 kHz for 2.1GHz 30 kHz for 4.4GHz | 15kHz | 30kHz | 15 kHz for 700/800/1800 MHz 30 kHz for 3500 MHz | 15kHz | 15kHz |  | 15kHz |  | 2.6GHz:15kHz 4GHz:30kHz | 15 KHz | 15 kHz for FDD, 30 kHz for TDD | Common for FDD+FDD and FDD+TDD. Aligned. |
| CORESET RBs |  | 96 |  | 96 | 48 |  | 48 |  | 20 MHz | 96 | 48 | 48/96 | For certain CORESET no obvious difference. Aligned. |
| CORESET symbol | 3 | 3 |  | 3 | 2 | 3 | 2 | 3 | 2 | 2 | 2 | 3: 5 companies 2: 5 companies | No obvious impact on LLS results. 2 and/or 3 CORESET symbol can be based for conclusion. |
| Channel model | TDL_C DS=100,300ns | TDL_C DS=300ns | TDL-A, medium correlation DS=300ns | TDL_A DS=30ns | TDL_C DS=300ns | TDL-B (delay spread: 100ns) | AWGN | TDL-B DS=100ns | TDL_A DS=30ns | TDL_C, Low correlation,DS=300ns | TDL_C DS=300ns | TDL_C: 5 companies TDL_A: 3 companies TDL_B: 2 companies AWGN: 1 company | TDL-A DS=30ns and TDL-C DS=300ns are used in most cases. Results based on TDL-B also demonstrates performance gain. |
| Tx number | 4 | 4 |  | 4 | 4 |  |  | 2 | 2 | 2 |  | 4: 4 companies 2: 2 companies | The difference on Tx will have no obvious impact on LLS results |
| Rx number | 4 | 2 |  | 2Rx for 700/800/1800 MHz 4Rx for 3500 MHz | 2 | 2 |  | 2 | 4 | 2 | 2 | 2 for FDD, 4 for TDD (vivo and OPPO use 2 for 2.6/4GHz) | Mostly aligned with 2 for FDD, 4 for TDD |
| Target BLER | 0.01 |  |  | 0.01 | 0.01 | 1% for 2 DCI scheme 0.5% for single DCI schemes |  |  | 1% for 2 DCI scheme 0.5% for single DCI schemes | 0.01 |  | 1% in most sourcing companies; two sourcing companies uses 0.5% for single DCI | Discussion point |
| Tx type | Interleaved | Interleaved |  | Interleaved |  | Interleaved | Non-interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Almost the same | No impact on the possible conclusion |
| Interleaver size |  | 2 |  | 2 |  | R=3 for 3OS |  |  | 2 | 2 | R=3 for 3OS,others,R=2 | 2: 5 companies 3 for 3OS: 2 companies | No impact on the possible conclusion |
| Bundle size | 6 | 6 |  | 6 |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 | Aligned |
| Modulation | QPSK |  |  | QPSK |  | QPSK |  | QPSK | QPSK |  | QPSK | QPSK | Aligned |
| UE Speed | 3km/h | 3km/h | 3km/h | 3km/h | 3km/h | 3km/h | 3km/h | 3km/h | 3km/h | 3km/h | 3km/h | 3km/h | Aligned |
| Tx Diversity |  | one port precoder cycling |  | One port precoder cycling | Precoder cycling every 6 REGs | One port precoder cycling |  |  | Precoder cycling | One port precoder cycling | 1-port precoder cycling | 1-port precoder cycling | Aligned |
| DCI size (excluding CRC) bits | 60/84 | 60/72/82/85/96/110/120 |  | 60/72/82/85/96/110/120 | 60/72/84/96/108/120 | 60 bits for 2 DCI scheme 76/86/96/106 bits for single DCI scheme | 36/56/76/96 | 58/68/82/92/106bits | 60 bits for legacy DCI 104 bits for Single DCI | Legacy DCI: 60 bits, One-to-two scheduling DCI: 72/ 84/96/110 | Legacy DCI: 60 bits, One-to-two scheduling DCI: 72/84/ 85/96/110 | 60/72/84/96/110/120 | A large proportion of common DCI sizes are assumed: 60bits for baseline 72/84/96/110 for single DCI. |
| Receiver type |  |  |  |  |  | MMSE |  |  |  |  | MMSE | MMSE | Aligned |
| Channel estimation |  |  |  |  |  | Realistic |  |  |  |  | Practical | Practical | Aligned |

# Standard impact

## DCI format design

If scheduling multiple PDSCHs on multiple carriers via a single DCI is supported, one important thing is to design the DCI format. Based on the simulation results, for reducing PDCCH blocking probability, the DCI payload should be further compressed. So many fields in the DCI need to be shared for the PDSCHs scheduled on two carriers. However, this scheduling inflexibility may lead to throughput loss for inter-band CA case. Due to the large frequency separation between the scheduled carriers in inter-band CA, the channel conditions are less correlated. It is difficult to assume same link adaptation property on the scheduled carriers and use single fields for indicating same MCS, frequency domain resource allocation as well as time domain resource allocation. For full flexibility scheduling two PDSCHs on two carriers by a single DCI, almost all the related fields in the scheduling DCI need to be doubled except DAI, HARQ timing, PRI, TPC and 24-bit CRC. However, the larger the DCI payload size, the lower the transmission reliability and less coverage. As a result, further overhead reduction is required for the two-carrier scheduling DCI at the cost of potential reduction in scheduling flexibility.

In addition, in order not to increase UE’s PDCCH blind decoding budget as one target of Rel-17 DSS, another open issues is whether the multi-carrier scheduling DCI needs to schedule not only a single PDSCH but also two PDSCHs on two carriers when the UE is configured with such feature.

Regarding DCI format design, companies’ views are summarized as below:

| Company | Key Proposals/Observations |
| --- | --- |
| Huawei, HiSilicon | Observation 7: For the DCI scheduling multi-carrier scheduling, some DCI fields can be predefined to be independent for separate PDSCHs, some fields can be predefined to be common for 2 PDSCHs, and the other fields can be configurable to be independent or common based on network decisions. Observation 8: It is possible to reduce the DCI payload around 18% when using single DCI joint scheduling two carriers instead of two separate DCIs, still with significant performance improvement without losing network scheduling flexibility. |
| vivo | Proposal 4. For joint DCI with DCI payload compression, study the trade-off between system capacity improvement due to CCE saving/PDCCH blocking rate reduction and the spectrum efficiency loss due to degraded scheduling flexibility. Proposal 5. The fields type (shared or cell-specific) in joint DCI needs to be investigated. |
| CATT | Proposal 3: The DCI content for multi-cell PDSCH scheduling and HARQ feedback procedure need to be further studied. |
| LG | Proposal #2: At least following issues would need to be addressed, and relevant specification impacts (and standardization workload for them) are expected, if the single DCI based multi-cell PDSCH scheduling is introduced. - How to indicate the multiple cells with PDSCH transmission by single scheduling DCI - How to compose (and signal) the DCI fields in the multi-cell PDSCH scheduling DCI - How to construct PDCCH search space for the multi-cell scheduling DCI transmission - How to allocate (and handle) PDCCH BD candidates for the multi-cell scheduling DCI |
| Spreadtrum: | - The DCI fields should be discussed and study whether or not the scheduling information should be same or different for the multiple PDSCHs. |
| Samsung | Observation 1: Consideration of a DCI format scheduling PDSCH receptions on two cells should avoid any throughput degradation over using two DCI formats and any new design requirements on the gNB scheduler. Observation 2: For TDD operation and no scheduling restrictions, a DCI format scheduling 2 cells can avoid duplication for only the CRC bits. Observation 3: DCI format size 1_1 is preferable to DCI format C2 across the geometry CDF, particularly for UEs above the 30% point of the geometry CDF which are more likely to operate with CA. Observation 4: Coverage and relative BLER comparisons for DCI format C2 further worsen for operation under less favorable conditions such as with some correlation or blockage of UE receiver antennas or for 2 UE receiver antennas. Observation 5: For DSS, a maximum gain in resources per slot from scheduling PDSCH on 2 cells using a single DCI format is ~0.35% for a BWP of 20 MHz and ~0.07% for a BWP of 100 MHz on the scheduling cell. Observation 6: The size matching of DCI format 0_1 with DCI format 1_1 that is required when using DCI format C2 reduces overhead savings from using DCI format C2. Observation 7: Introduction of a DCI format scheduling PDSCH on two cells by parallelizing use of field for scheduling on a single cell does not provide any material benefit over a DCI format scheduling PDSCH on a single cell. |
| OPPO | Observation 2: The smaller DCI size increases, the more UEs achieve gain from one-to-two scheduling. |
| APPLE | Observation 2: Rel-16 Single-DCI Multi-TRP solutions significantly restrict PDSCH scheduling flexibility, and, is only designed for PDSCH repetition, i.e. single TB case |
| ASUSTeK | Proposal 2-1: DCI fields about reporting HARQ-ACK information are shared between the multiple PDSCHs scheduled by a single DCI. Proposal 2-2: DCI fields about resource assignment and transmission parameters are separate between the multiple PDSCHs scheduled by a single DCI. Proposal 3: Constrain one of the two cells scheduled by a single DCI to be the scheduling cell. |
| ZTE | Observation 3: If single DCI scheduling two PDSCHs on two carriers is supported, to guarantee at least the moderate flexibility, the minimum size of this enhanced DCI for one-to-two scheduling is 93bits compared with 60 bits for legacy scheduling. Observation 4: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to handle the Rel-16 newly introduced DCI fields in DCI format 1_1. Observation 5: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study whether to apply DCI format 1_1/1_2 or introduce a new DCI format for one-to-two scheduling. Observation 6: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to indicate the two scheduled carriers. Observation 7: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to guarantee the current BD/CCE budget. Observation 8: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to perform the corresponding HARQ-ACK feedback. |
| Charter Communications | Proposal 1: Consider enhanced multi-carrier operation where a single DCI can schedule PDSCH on two non-DSS cells with the same SCS, including SCells with a dormant BWP, for energy-efficient and low-latency NR performance. |
| Lenovo, Motorola Mobility | Observation 4: Only non-fallback DCI can be configured to schedule one PDSCH on one carrier or two PDSCHs on two carriers. |
| MediaTek | Proposal 3: Both 1-stage DCI aggregation and 2-stage DCI aggregation for cross-carrier scheduling should be included for the evaluation. |
| Intel | Observation 1: To support 2-cell scheduling by a single DCI, at least the following bit fields are likely to be duplicated: FDRA, MCS/NDI/RV and Antenna ports/TCI. TDRA field may be duplicated too. |
| ETRI | Observation 1: Multi-cell scheduling via a single DCI should be generally applicable for both the DSS scenario and the non-DSS scenario. Observation 2: Decision on support of the joint multi-cell scheduling should be made by considering both the DSS scenarios and general CA scenarios. If agreed to specify, the design should also target both the scenarios. Observation 3: The multi-cell joint scheduling should allow a sufficiently wide range of scheduling flexibility to support different scenarios. Observation 4: For multi-cell joint scheduling, the principle that one PDSCH does not span multiple cells can be kept to minimize the workload. Observation 5: For multi-cell joint scheduling, scheduling more than two cells using a joint DCI can be considered. |
| Nokia, NSB | Observation 3: The baseline design would be to determine DCI format fields based on primary of the two-cells and interpret the fields for the secondary of the two-cells as in case of BWP switching R15. Some fields could be further optimized or doubled in the DCI format which is FFS. Observation 4: In DSS a typical DCI size with CRC would be 76 bits for single cell DCI, while for double cell DCI could be around 110bits when allowing for fully flexible allocation of TDRA, FDRA, MCS, HARQ ID, RV, NDI and Antenna ports. Observation 5: In FR2 CA a typical DCI format 1_1 size with CRC would 80bits, while four-cell DCI format could be around 88bits, when allowing for NDI field and 1-bit scheduling indication per cell. |
| InterDigital | Support a single DCI to schedule two PDSCH in different cells. |
| DOCOMO | Observation 2: - The required DCI size for the agreed scenarios should be discussed prior to performance gain discussion. Observation 3: - In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), CRC field attached to DCI (i.e. 24-bit) can be shared between the scheduled multiple cells. Observation 6: - In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), it may be better to separate Time domain resource assignment field for each scheduled cell. Observation 7: - In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), it may be better to separate Frequency domain resource assignment field for each scheduled cell. Observation 8: - Whether/how to support some indications in DCI for multiple scheduled cells can be considered. - e.g. rate matching indicator, BWP indicator, CSI request and SRS request Observation 9: - How to determine the size of DCI scheduling PDSCH on multiple cells can be considered. |
| Ericsson | Following design aspects for single DCI scheduling PDSCH on two cells should be considered in the study - When single DCI is used to schedule PDSCH on two cells, whether the two scheduled cells are allowed to have different configuration for at least the following attributes: - Numerology used on each scheduled cell - Channel BW (and BWP BW) of each scheduled cell - MIMO configuration of each scheduled cell - HARQ processes/TBs/MCSs of each scheduled cell - FDRA/TDRA (including type) used for each scheduled cell - When single DCI is used to schedule PDSCH on two cells, whether the corresponding DCI format always schedules PDSCH on both cells or whether it is also used to schedule single cell PDSCH. - When UE monitors the DCI format for single DCI scheduling PDSCH on two cells, whether the UE can be configured to also monitor existing DCI format(s) scheduling PDSCH on single cell (i.e. 1-0/1-1/1-2). - Handling DCI size budget and DCI size-matching when UE is configured to monitor the DCI format for single DCI scheduling PDSCH on two cells. - Whether the DCI format supports the functionality of all the DCI fields specified for existing DCI formats or whether it supports only a limited subset of DCI fields. - For each DCI field of the DCI format, whether the DCI field jointly indicates the functionality for both cells or whether separate DCI fields for each cell are used to indicate the respective functionality. |
| Qualcomm | Proposal 1: Consider further DCI compression schemes that are effective for the cases where the scheduled cells are highly correlated (e.g., intra-band CA). Proposal 2: Following should be realized: - The UE is able to know a set of PDCCH candidates for the DCI format scheduling PDSCHs for a given set of scheduled cells, based on the higher-layer configurations, before blind decodes - NW can schedule one cell or multiple cells by the DCI format for multi-cell PDSCH scheduling. Proposal 3: The UE monitors a set of PDCCH candidates for a DCI format that can schedule PDSCH(s) on a set of cell(s). A field in the DCI format tells the UE on which cell(s) data is actually scheduled amongst the set of cells. |

Discussion points:

### Q1: For two PDSCHs on two carriers scheduled by a single DCI, there may be two options:

- Option 1: One PDSCH is cross-carrier scheduled and another PDSCH is self-scheduled.

- Option 2: Both PDSCHs are cross-carrier scheduled;

E.g., in Option 1, the DCI transmitted on cell 1 schedules one PDSCH on cell 1 via self-scheduling and another PDSCH on cell 2 via cross-carrier scheduling; in Option 2, the DCI transmitted on cell 1 schedules one PDSCH on cell 2 via cross-carrier scheduling and another PDSCH on cell 3 via cross-carrier scheduling.

Shall we support at least Option 1 or both options?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | Should not preclude the possibility that both carriers are cross-carrier scheduled. |
| Apple | If this feature is supported, both options are okay |
| InterDigital | We are fine with both options |
| Charter Communications | Fine with both options |
| Huawei, HiSi | Ok with both options but consider Option 1 should be at least included, e.g. for the scenario of DSS+one NR carrier. |
| Spreadtrum | Fine with both options |
| vivo | Fine with both options |
| Qualcomm | Fine with both options |
| Lenovo, Motorola Mobility | We are fine with both options |
| ZTE | If one-to-two scheduling is introduced, we are fine with both options. |
| NTT DOCOMO | We are fine with both options. |
| CATT | Both should be supported. |
| Intel | If the feature is introduced, both can be supported |
| LG | If this feature is introduced, Option 1 may need to be prioritized by considering main DSS scenario. |
| ETRI | Fine to support both options. |
| OPPO | Fine to support both options. |
| Ericsson | Performance and complexity impact of both options can be considered in the study |

### Q2: Will a new DCI format be introduced, or an existing non-fallback DCI be reused to support multi-cell PDSCH scheduling by a single DCI?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | This is perhaps a specification technicality than something that impacts design and in that sense not a critical item to address at this stage. That said, it might be beneficial to adopt this as a subcase for format 1_1 rather than a new format in order to avoid having to update all the instances of the spec where the format type is mentioned. |
| Apple | The more important question is how to treat different DCI field. It is not good to restrict the DCI format at this stage without detailed discussion on the DCI field. |
| InterDigital | New DCI format can be introduced |
| Huawei, HiSi | Open for discussion. Share the feeling that this can be addressed in WI phase. |
| Spreadtrum | We are open for further discussion. At this stage, we prefer non-fallback DCI can be reused. |
| vivo | Same view as apple. It may be too early to conclude whether a new DCI format is needed at this stage. We prefer to discuss the DCI field type first, e.g., which information fields should be shared or CC-specific. |
| Qualcomm | Should be discussed once detailed mechanisms are further clarified. |
| Lenovo, Motorola Mobility | We are open for further discussion although we prefer reusing existing non-fallback DCI. |
| ZTE | We also think it may be too premature to discuss this kind of detailed design at this stage. We may need to come back to this issue in the future meeting. |
| CATT | Further discussion is needed. The fundamental issue on whether reusing existing non-fallback DCI or introducing a new DCI format is whether we need to define some new bit fields compared to the current DCI format, i.e. the bit field is exactly same as the current DCI format or new bit fields will be introduced (including totally new bit fields and the current bit fields but doubled in order to support two PDSCHs). |
| Intel | As commented by other companies, it is too early to discuss this proposal. Above all, we prefer the keep existing budget on the number of DCI sizes |
| LG | Same view with other companies. It seems premature to discuss/decide this kind of details. |
| ETRI | We do not have a strong view for now. Can be discussed further. |
| OPPO | Share view with other companies. It is too early to discuss details. |
| Ericsson | Performance and complexity impact of new DCI format vs. reusing existing DCI formats can be considered in the study |

### Q3: Whether the multi-cell scheduling DCI can be also used to schedule a single PDSCH on one carrier?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | Yes. The DCI format should be able to schedule either one of the two PDSCH carriers without scheduling the other carrier. |
| Apple | This is related to Q4. When one DCI can schedule two PDSCH in different CCs, we also need to support basic operation, i.e. one DCI schedules only on PDSCH. But Q3 and Q4 can achieve the same thing |
| InterDigital | Yes. |
| Charter Communications | Yes |
| Huawei, HiSi | Yes |
| Spreadtrum | Yes |
| vivo | Yes. It is beneficial to support dynamic switching between the single-cell scheduling and multi-cell scheduling considering the traffic status, TDD configurations etc. |
| Qualcomm | Yes |
| Lenovo, Motorola Mobility | Yes |
| ZTE | Although our answer is yes, we still believe it is too premature to discuss the detailed DCI design at this stage. |
| NTT DOCOMO | Yes |
| CATT | Yes |
| Intel | Though it is not efficient to schedule one PDSCH with the DCI for 2-cell scheduling, however, it is better to allow it. This is also the design principle of multi-TTI PUSCH scheduling in NR-U. |
| LG | If this feature is introduced, yes. |
| ETRI | Yes |
| OPPO | Yes |
| Ericsson | Performance and complexity impact of allowing/not allowing this can be considered in the study. |

### Q4: When UE monitors the DCI format which can schedule two PDSCHs on two cells, can the UE be configured to also monitor existing DCI (i.e., 1-0/1-1/1-2) scheduling a single PDSCH on a single cell?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | Yes, no need to change the DCI monitoring rules. |
| Apple | This is related to Q3 |
| InterDigital | Depends on the outcome of Q3. |
| Huawei, HiSi | Yes |
| Spreadtrum | Yes for 1-0. We can further check 1-2 if 1-1 is used for multi-cell PDSCH scheduling. |
| vivo | Yes if the BD budget can be kept. |
| Qualcomm | “Maybe no”. This question relates to the discussions on BD budget and DCI size budget. If the question is asking whether a UE shall monitor all the DCI formats 1_0, 1_1, 1_2, and multi-cell scheduling DCI format on the same cell, then without introducing a new DCI size alignment, the DCI size budget will exceed 3. |
| Lenovo, Motorola Mobility | Yes |
| ZTE | This issue is highly depending on the outcome of Q3. We may need to first discuss Q3. |
| CATT | Yes. The potential issue on DCI size budget raised by Qualcomm is good point. We are open to discuss it. |
| Intel | DCI 1_0 is anyway needed which is for fallback and robust operation. Regarding DCI 1_1/1_2, better to conclude on Q3 first. |
| LG | This issue would have dependency with the above Q2 in terms of which DCI format is used for multi-CC scheduling. |
| ETRI | Agree with Intel and LG. |
| OPPO | Yes. The issue on DCI size budget raised by QC needs to be considered. |
| Ericsson | Monitoring of existing DCI formats should be assumed as the baseline for the study and performance/complexity from other options should be compared wrt. baseline. |

### Q5: When UE monitors the DCI format which can schedule two PDSCHs on two cells, whether the BD/CCE budget defined in Rel-15 is kept?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | Yes |
| Apple | We first need to discuss how BD/CCE is computed since now we break the one scheduling to one scheduled cell mapping rule. |
| InterDigital | Yes |
| Huawei, HiSi | In general yes as required by WID. But Ok with Apple suggestion. |
| Spreadtrum | Yes. Also support Apple’s comments. |
| vivo | We should keep in mind there is a note in the WID, which gave clear guidance to keep the BD budget unchanged. Note: The total PDCCH blind decoding budget should not be changed as a result of this work |
| Qualcomm | Yes. |
| Lenovo, Motorola Mobility | Yes |
| ZTE | Yes, The WID explicitly mentions that PDCCH blind decoding budget should not be changed. |
| CATT | Yes |
| Intel | Agree with Apple to check how to define BD/CCE. |
| LG | If this feature is introduced, yes. |
| ETRI | Yes |
| OPPO | Share view with Apple. |
| Ericsson | Existing BD/CCE limits should be assumed as the baseline for the study and performance/complexity from other options should be compared wrt. baseline. |

### Q6: Whether the fields in the single DCI which schedules the two PDSCHs on two carriers are shared or separate for each PDSCH?

Companies are encouraged to provide names for each field in the corresponding column of the below table.

Table 1: Fields in the DCI which can schedule two PDSCHs on two carriers

| - DCI fields | - Shared indication for the two scheduled PDSCHs | - Separate indication for each scheduled PDSCH |
| --- | --- | --- |
| - Identifier for DCI formats | - Nokia, HW |  |
| - Carrier indicator | - Nokia, HW |  |
| - Bandwidth part indicator |  |  |
| - Frequency domain resource assignment | - Nokia (at least for FR2 intra-band CA), HW (Common PRB numbering can be assumed), | - Nokia (FR1 DSS) |
| - Time domain resource assignment | - Nokia (at least for FR2 intra-band CA) | - Nokia (FR1 DSS) |
| - VRB-to-PRB mapping | - Nokia |  |
| - PRB bundling size indicator | - Nokia |  |
| - Rate matching indicator |  | - Nokia |
| - ZP CSI-RS trigger |  | - Nokia |
| - Modulation and coding scheme | - Nokia (at least for FR2 intra-band CA) | - Nokia (FR1 DSS), HW |
| - New data indicator |  | - HW |
| - Redundancy version |  | - HW |
| - HARQ process number |  | - HW |
| - Downlink assignment index | - Nokia, HW |  |
| - TPC command for scheduled PUCCH | - Nokia, HW |  |
| - PUCCH resource indicator | - Nokia, HW |  |
| - HARQ timing indicator | - Nokia, HW |  |
| - Antenna port(s) | - Nokia (at least for FR2 intra-band CA) | - Nokia (FR1 DSS) |
| - Transmission configuration indication | - Nokia (at least for FR2 intra-band CA) | - Nokia (FR1 DSS) |
| - SRS request |  | - Nokia |
| - CBGTI | - N/A |  |
| - CBGFI | - N/A |  |
| - DMRS sequence initialization | - Nokia, HW |  |
| - 24-bit CRC | - Nokia, HW |  |

| Company | View |
| --- | --- |
| Huawei, HiSi | We provided our view above as current preference. However we also want to express that some of the fields may be predefined or RRC configurable for single DCI usage. We are open to it. |

## HARQ-ACK codebook design

Regarding HARQ-ACK codebook design, there is no issue for Type 1 HARQ-ACK codebook due to the semi-static codebook size. However, for Type 2 HARQ-ACK codebook, since each non-fallback DCI can schedule one or two PDSCHs, when the DCI is missed by UE, there may be misunderstanding between gNB and UE on the number of scheduled PDSCHs. In that sense, HARQ-ACK codebook ambiguity may happen. As a result, how to construct the Type 2 HARQ-ACK codebook needs to be considered in order to synchronize the same understanding between gNB and UE.

Company views:

| Company | Key Proposals/Observations |
| --- | --- |
| Vivo | Observation 6. To support multi-cell scheduling, the following issues need to be resolved - DCI field design - Any restrictions on the scheduled cells to be paired for multi-cell scheduling - Framework of multi-cell scheduling - Whether to introduce a new DCI format - PDCCH BD budget maintenance if multi-cell scheduling is enabled - HARQ-ACK codebook determination if multi-cell scheduling is enabled |
| CATT | The HARQ-ACK feedback procedure may also need to be further studied accordingly, e.g. the SCS and scheduling/feedback timing may be different for the different scheduled cells. We also provide some tentative insights below from our side: - For type1 HARQ-ACK codebook, current mechanism can be directly reused if two separate PDSCHs are scheduled on different cells respectively. - Design of C-DAI and T-DAI in one DCI for counting multiple PDSCHs scheduled by one DCI should be considered. - HARQ-ACK timing needs to be further considered as the scheduling timing and feedback timing may be both different on the two scheduled cells. |
| Intel | Observation 3: Potential specification impacts include but not limited to - The RRC configuration - Separate design for each DCI field - UE complexity on PDCCH detection. - HARQ-ACK transmission. |
| ZTE | Observation 8: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to perform the corresponding HARQ-ACK feedback. |
| Lenovo, Motorola Mobility | Observation 6: HARQ-ACK feedback for the two PDSCHs scheduled by a single DCI is included in same HARQ-ACK codebook. |

### Q7: Whether HARQ-ACK codebook determination needs to be enhanced when multiple PDSCHs are scheduled by a single DCI?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | No |
| Apple | No |
| InterDigital | No |
| Huawei, HiSi | Given companies contributions we think it is not necessary to make decision for this question for now. Can be left to WI. |
| Spreadtrum | We are open to further discussion. And agree with CATT’s proposal in principle. |
| Vivo | TBD. Prefer to address the previous questions first. |
| Qualcomm | TBD. Prefer not to close the door for any changes. |
| Lenovo, Motorola Mobility | We are open for further discussion. |
| ZTE | We believe it is too premature to discuss the detailed design at this stage. |
| CATT | TBD. For type1-codebook, we intend to say no as the HARQ-ACK codebook is generally determined by the semi-static configurations for PDSCH, HARQ-ACK timing, etc. For type2-codebook, it is determined by a dynamic behaviour, i.e. the received DCI. However, the key difference is that two PDSCHs are scheduled by a single DCI. The current mechanism may need to be modified. |
| Intel | Better to wait for progress on design details of multi-cell scheduling. |
| LG | Similar view with other companies. It seems premature to discuss/decide this kind of details. |
| OPPO | TBD. |
| Ericsson | Impact on HARQ-ACK codebook (if any) can be considered in the study |

## Other issues

Regarding other issues not mentioned above, companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |

# Miscellaneous (Low priority)

Regarding some low priority issues, companies’ views are summarized as below:

| Company | Key Proposals/Observations |
| --- | --- |
| Huawei, HiSilicon | Observation 11: Using single DCI scheduling multi-carriers has large potential to be deployed with other NR features, deployment scenarios and services. |
| vivo | Proposal 3. Clarify whether PUSCH multi-cell scheduling should be studied. |
| ZTE | Proposal 2: If TU permits, RAN1 considers one DCI scheduling two PDSCHs on the same carrier instead of one DCI scheduling two PDSCHs on two carriers. |
| MediaTek | Proposal 3: Both 1-stage DCI aggregation and 2-stage DCI aggregation for cross-carrier scheduling should be included for the evaluation. |
| ETRI | Observation 5: For multi-cell joint scheduling, scheduling more than two cells using a joint DCI can be considered. |
| Nokia, Nokia Shanghai Bell | Proposal 1: Support multi-cell DCI in R17, focus on multiple SCell (2 or more) with the same/similar carrier size and SCS first. Strive to keep DCI format 1_1 payload <106bits (including CRC). |

Discussion points:

One company [13] propose both 1-stage DCI aggregation and 2-stage DCI aggregation for cross-carrier scheduling. For 1-stage DCI, DCIs for the scheduled cells are aggregated into a 1-stage DCI in a PDCCH. For 2-stage DCI, DCIs for the scheduled cells are aggregated into a 2-stage DCI, where 1st stage DCI is carried in a PDCCH and 2nd stage DCI is carried in a set of REs indicated in the 1st stage DCI.

### Q8: Whether to support two-stage DCI for scheduling multiple carrier PDSCHs?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | No |
| Apple | Do not prefer two-stage DCI |
| InterDigital | No |
| Charter Communications | No |
| Huawei, HiSi | Open to discuss. |
| vivo | No |
| Qualcomm | No |
| Lenovo, Motorola Mobility | No. Since we have quite limited TU for this topic, it is not possible to fully discuss the two-stage DCI design. |
| Intel | No |
| LG | No |
| ETRI | No |
| OPPO | No |

Discussion point:

### Q9: Whether to support more than 2 carriers scheduled by a single DCI?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | Yes |
| Apple | No |
| InterDigital | No |
| Charter Communications | Concerns over DCI payload size |
| Huawei, HiSi | Yes. We see a larger potential in such case, and this is also some operator’s interest. |
| vivo | This is not allowed by the WID scope - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - The number of cells can be scheduled at once is limited to 2 - The increase in DCI size should be minimized |
| Qualcomm | Yes |
| Lenovo, Motorola Mobility | It is better to only support up to 2 carriers in Rel-17 and can be extended to more than 2 carriers in next phase. |
| ZTE | No, unless the WID is updated. |
| Intel | No |
| LG | No |
| ETRI | Yes |
| OPPO | No, out of scope. |

### Q10: Whether to support multiple PDSCHs on same carrier scheduled by a single DCI?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | Not a priority item, can be discussed in a later stage. Should not include multi-TRP operation. |
| Apple | We do not need to discuss this at this stage. This can be discussed if we agree to support two PDSCHs in different CCs, then we can discuss whether two CCs can be the same |
| InterDigital | We should focus on the different carrier scenario |
| Charter Communications | Focus only on multi-cell scenario |
| Huawei, HiSi | Focus on the scope and the above can be discussed in a later stage. |
| vivo | No, this aspect is out of the scope. |
| Qualcomm | For this, we need to discuss how to support multi-cell PDSCH scheduling on the serving cells having different SCSs. |
| Lenovo, Motorola Mobility | It is better to only focus on current scope in Rel-17 and can be extended in next phase. |
| ZTE | The question may be misleading. Our original proposal is trying say that RAN1 doesn’t support one PDCCH schedules two PDSCHs on two carriers, and instead RAN1 supports one PDCCH schedules two PDSCHs on the same carrier. For one PDCCH schedules two PDSCHs on the same carrier, almost all the DCI fields can be shared between the two PDSCHs. According to the Rel-16 PUSCH design, maybe only 1-3 additional bits should be sufficient. Besides, the standard effort would be minor since we already define multi-PUSCH scheduling in NRU. We can try to reuse the same design there. @Nokia, this is not related to M-TRP design. M-TRP is trying to schedule one TB from two TRPs via one PDCCH. However, the proposal here is trying to schedule two TBs. @Apple, the proposal here is trying to replace one PDCCH schedules two PDSCHs on two carriers. Thus, we didn’t see the need to bundle this two together. |
| NTT DOCOMO | We think it’s better to focus on multiple PDSCHs on different carrier since a main target of the WID is to improve PDCCH capacity in DSS and CA case. |
| Intel | We prefer to limit the discussion to two PDSCHs on two carriers |
| LG | No, it seems out of WI scope. |
| ETRI | No |
| OPPO | No |

### Q11: Whether to support multiple PUSCHs on multiple carriers scheduled by a single DCI?

Companies are encouraged to provide comments in the table below.

| Company | View |
| --- | --- |
| Nokia | No. Could be discussed in Rel-18. |
| Apple | It is out of the WID scope |
| InterDigital | We don’t think we will have time to discuss and support it. |
| Charter Communications | Agree with Apple |
| Huawei, HiSi | Yes. We see a larger potential in such case, but perhaps this can also be discussed later. |
| vivo | We prefer to focus on the two PDSCH scheduling which is clearly included in WID, whether to extend the solution to PUSCH scheduling should be addressed later at RAN plenary. |
| Qualcomm | Open for this. |
| Lenovo, Motorola Mobility | It is better to only focus on current scope in Rel-17 and can be extended in next phase. |
| ZTE | No, unless the WID is updated. |
| Intel | No, out of scope. |
| LG | No, it seems out of WI scope. |
| ETRI | UL scheduling can be considered in a next release considering the limited timeline of this WI. |
| OPPO | Open for this. |

# References

- R1-2007580	Discussion on multi-carrier scheduling using single PDCCH	Huawei, HiSilicon

- R1-2007696	Discussion on joint scheduling	vivo

- R1-2007840	Discussion on multi-cell PDSCH scheduling via a single DCI	CATT

- R1-2008063	Discussion on multi-cell PDSCH scheduling via a single DCI	LG Electronics

- R1-2008111	Discussion on multi-cell PDSCH scheduling via a single DCI	Spreadtrum Communications

- R1-2008196	On the use of one DCI format for scheduling on two cells	Samsung

- R1-2008285	Discussion on multi-cell PDSCH scheduling via a single DCI	OPPO

- R1-2008452	Views on Rel-17 DSS Multi-cell PDSCH scheduling via a single DCI	Apple

- R1-2008696	Discussion on multi-cell PDSCH scheduling via a single DCI	ASUSTeK

- R1-2008831	Discussion on Multi-cell PDSCH Scheduling via a Single DCI	ZTE

- R1-2008835	Multi-cell scheduling and dormancy	Charter Communications

- R1-2008929	Discussion on multi-cell PDSCH scheduling via a single DCI	Lenovo, Motorola Mobility

- R1-2008963	Evaluation on On Multi-cell PDSCH Scheduling via Single DCI	MediaTek Inc.

- R1-2009004	On 2-cell scheduling via single DCI	Intel Corporation

- R1-2009024	Discussion on multi-cell PDSCH scheduling via a single DCI	ETRI

- R1-2009047	On support of Single DCI scheduling two cells	Nokia, Nokia Shanghai Bell

- R1-2009086	Discussion on the support of single DCI scheduling multi-cell 	InterDigital, Inc.

- R1-2009196	Discussion on multi-cell PDSCH scheduling via a single DCI for NR DSS	NTT DOCOMO, INC.

- R1-2009207	Study on single DCI scheduling PDSCH on multiple cells	Ericsson

- R1-2009278	Views on multi-cell PDSCH scheduling via a single DCI	Qualcomm Incorporated