3GPP TSG RAN WG1 #103-e		R1-2009582

e-Meeting, Oct. 26th – Nov. 13th, 2020

Source:	Panasonic

Title: 	Baseline coverage performance analysis in FR1

Agenda Item:		8.8.1.1

Document for:	Discussion

Introduction

A study item on NR coverage enhancement was approved [1]. One of objectives of this study item is to identify baseline coverage performance for both DL and UL for the following scenarios and services based on link-level simulation.

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2

  - TDD and FDD for FR1

  - VoIP and eMBB services for FR1

  - eMBB service as first priority and VoIP as second priority for FR2

  - LPWA services and scenarios are not included.

In this document, we provide baseline coverage performance analysis in FR1.

This is the update version of R1-2008377. Figure 1 is updated. Based on the updated results, Observation 1, 3 and Proposal 2 are also updated.

Baseline coverage performance

Simulation assumptions

In our link-level simulation, PUSCH, PUCCH, PDSCH, PDCCH and Msg.3 are evaluated. General assumptions used in our evaluation are summarized in Table 1.

Table 1: General simulation parameters

| Scenario | Urban | Rural |
| --- | --- | --- |
| eMBB Target data rate | DL: 10Mbps UL: 1Mbps | DL: 1Mbps UL: 100kbps |
| VoIP | A packet size of 320 bits with 20 ms data arriving interval is adopted. |  |
| Latency requirements for voice | 50 ms |  |
| Frequency | 4 GHz (TDD) | 4 GHz (TDD), 2 GHz (FDD), 700 MHz (FDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) DDDSUDDSUU (S: 10D:2G:2U) |  |
| Pathloss model | NLOS |  |
| BWP | 100 MHz for 4 GHz 20 MHz for 2 GHz and 700 MHz |  |
| SCS | 30 kHz for TDD, 15 kHz for FDD |  |
| Number of antenna elements for BS | 192 for 4 GHz | 64 for 4 GHz, 32 for 2 GHz, 16 for 700 MHz |
| Number of TxRUs for BS | 2 for 2 GHz and 700 MHz 64 TxRUs for 4 GHz |  |
| Number of transmit/receive chains modelled in LLS | 4 for 4 GHz 2 for 2 GHz and 700 MHz |  |
| Channel model | TDL-C |  |
| Delay spread | 300 ns |  |
| UE velocity | 3 km/h | 3 km/h for indoor, 120 km/h for outdoor |

Channel-specific parameters for link-level evaluation are summarized in Table 2.

Table 2: Channel-specific parameters for link-level evaluation

| PUSCH |  |
| --- | --- |
| Target BLER | 10% iBLER for eMBB 2% rBLER for VoIP |
| Waveform | DFT-s-OFDM |
| Number of UE transmit chains | 1 |
| PUSCH duration | 14 OFDM symbols |
| Frequency hopping | w/ or w/o frequency hopping |
| DMRS configuration | PUSCH mapping Type A 3 km/h: Type 1, 2 DMRS symbol, no multiplexing with data For frequency hopping: Type 1, 2 DMRS symbols for each hop, no multiplexing data 120 km/h: Type 1, 3 DMRS symbol, no multiplexing with data For frequency hopping: Type 1, 2 DMRS symbol for each hop, no multiplexing data |
| PRBs / TBS / MCS for eMBB | 30 PRBs for 1Mbps, 4 PRBs for 100kbps TBS and MCS is derived based on e.g., the number of PRBs, target data rate, frame structure and overhead. TDD: MCS4 for ‘DDDSU’ MCS2 for ‘DDDSUDDSUU’ FDD: MCS0 |
| PRBs / MCS for VoIP | 4 PRBs / MCS4 |
| Repetition | eMBB w/o repetition VoIP w/o repetition or w/ repetition (2 repetitions) |
| HARQ configuration | eMBB w/o HARQ VoIP w/ HARQ (The maximum number of HARQ transmission is 8.) |
| PUCCH |  |
| PUCCH format | Format 1: 2 bits UCI Format 3: 11 or 22 bits UCI |
| Target performance | PUCCH format 1: DTX to ACK probability: 1% NACK to ACK probability: 0.1% ACK missed detection probability: 1% PUCCH format 3: BLER for ACK/NACK, SR: 1% |
| Duration | 14 OFDM symbols |
| Number of UE transmit chains | 1 |
| Number of PRBs | 1 PRB |
| Frequency hopping | Enabled |
| DMRS configuration | 2 DMRS symbols per hop for PUCCH format 3 |
| Number of repetitions | w/o repetition |
| PDSCH |  |
| Target BLER | 10% iBLER for eMBB |
| Waveform | CP-OFDM |
| Number of UE receive chains | 4 for 4 GHz 2 for 2 GHz and 700 MHz |
| PRBs / MCS / TBS | 155 PRBs for 10 Mbps, 16 PRBs for 1 Mbps TBS and MCS is derived based on e.g., the number of PRBs, target data rate, frame structure and overhead. TDD: MCS3 for ‘DDDSU’ MCS4 for ‘DDDSUDDSUU’ FDD: MCS4 |
| PDSCH duration | 12 OFDM symbols |
| DMRS configuration | PDSCH mapping Type A 3 km/h: Type 1, 2 DMRS symbol, no multiplexing with data 120 km/h: Type 1, 3 DMRS symbol, no multiplexing with data |
| Repetition | w/o repetition |
| HARQ configuration | w/o HARQ |
| PDCCH |  |
| Target performance | 1% BLER |
| Number of UE receive chains | 4 for 4 GHz 2 for 2 GHz and 700 MHz |
| CORESET size | 2 symbols, 48 PRBs |
| Aggregation level | 16 |
| Payload | 40 bits |
| Tx diversity | REG-bundle with cycle |
| Msg.3 |  |
| Target performance | 10% iBLER |
| Waveform | DFT-s-OFDM |
| Number of UE transmit chains | 1 |
| Number of PRBs | 2 |
| Frequency hopping | Enabled |
| PUSCH duration | 14 OFDM symbols |
| Number of DMRS symbols | 2 DMRS symbols per hop |
| TBS | 56 bits |
| HARQ configuration | w/o HARQ |

Baseline coverage performance

In baseline coverage performance evaluation, the required SINR is obtained for the physical channels under target scenarios and service/reliability requirements by link-level simulation. In the next step, baseline performance is obtained based on the required SINR and link budget template [2]. The detailed values for link budget template are summarized in the attached excel spreadsheet in this tdoc. Figure 1 shows the MIL calculation results for FR1 for Urban and Rural scenarios.

As shown in Fig.1, PUSCH is bottleneck channel in all scenarios, where its MIL is at most 18 dB lower than the best channel. Therefore, the coverage improvement of PUSCH should be the most essential part of the study.

For PUCCH, PUCCH format 1 is the 2nd best channel in urban and rural 4 GHz scenarios and only the PUCCH format 3 with larger payload size could be bottleneck of PUCCH coverage. On the other hand, in this evaluation, PUCCH repetition is not evaluated. It is expected that PUCCH repetition can provide a gain of several dB for the baseline coverage performance. Therefore, at least the enhancement of increasing the maximum number of PUCCH repetition would not be necessary.

For PDSCH, there are at most 8 dB MIL lower than the best channel in urban and rural 4 GHz, while PDSCH is the 2nd best channel in rural 2 GHz and 700 MHz. If the target for a channel is formulated as the amount of difference of MILs between the channel and the best channel among all UL and DL channels, PDSCH should be enhanced for TDD scenarios. On the other hand, it is necessary to consider several aspects, such as the relative amount of MIL difference and satisfactions of other target metrics (e.g., ISD, MPL, etc), to decide whether to enhance a channel or not. Even if there is relative amount of MIL difference between the channels, but the channels can achieve target MPL, so that the improvement may not necessary.

The MIL difference between Msg.3 and the best channel is at most 15 dB. Therefore, the coverage improvement of Msg.3 could be potential candidate of enhancement.

Observation 1: PUSCH is bottleneck channel in urban and rural scenarios.

Observation 2: PUCCH format 3 with larger payload size could be bottleneck of PUCCH coverage.

Observation 3: PDSCH needs to be enhanced if the performance target is formulated as relative amount MIL difference compared with the best channel.

Based on the above observations, we propose following.

Proposal 1: The coverage improvement of PUSCH should be the most essential part of the study.

Proposal 2: The enhancement of PUCCH format 3 and Msg.3 could be considered as the second priority for FR1 coverage enhancement.

(a) Urban, 4GHz, TDD

(b) Rural (Indoor), 4 GHz, TDD

(c) Rural (Outdoor), 4 GHz, TDD

(d) Rural (Indoor), 2 GHz, FDD

(e) Rural (Outdoor), 2 GHz, FDD

(f) Rural (Indoor), 700 MHz, FDD

(g) Rural (Outdoor), 700 MHz, FDD

Fig.1 MIL calculation results for FR1

Conclusion

In this contribution, we provide baseline coverage performance analysis in FR1. We made following observations and proposals.

Observation 1: PUSCH is bottleneck channel in urban and rural scenarios.

Observation 2: PUCCH format 3 with larger payload size could be bottleneck of PUCCH coverage.

Observation 3: PDSCH needs to be enhanced if the performance target is formulated as relative amount MIL difference compared with the best channel.

Proposal 1: The coverage improvement of PUSCH should be the most essential part of the study.

Proposal 2: The enhancement of PUCCH format 3 and Msg.3 could be considered as the second priority for FR1 coverage enhancement.

Reference

[1]		RP-193240, “New SID on NR coverage enhancement,” China Telecom, RAN#86, December 2019.

[2]	R1-2007480, “[102-e-Post-NR-CovEnh-02] Summary on email discussion/approval of Phase 1 - link budget template,” Moderator (Softbank), RAN1#102e.