3GPP TSG RAN WG1 Meeting #103-e	R1-2009610

E-meeting, October 26 – November 13, 2020

Agenda Item:	8.2.3

Source:	Huawei, HiSilicon

Title:	Link level and System level evaluation for NR system operating in 52.6GHz to 71GHz

Document for:	Discussion and Decision

Introduction

In RAN1#102-e, the link level and system level evaluation assumptions were continued to be discussed and new agreements were made [1]. According to the agreed assumptions, we provide new results to study the phase noise impairment impact in various numerologies for PDSCH/PUSCH/PRACH/SSB in link level evaluation and channel access mechanism under indoor and outdoor scenario in system level evaluation.

Assumptions and results for Link level evaluation

Simulation assumptions

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 60 GHz |
| Duplexing | TDD |
| Waveform | CP-OFDM for DL/UL, DFT-s-OFDM for UL |
| Numerology | [120 kHz 240 kHz 480 kHz 960 kHz] |
| Antenna Configuration (Mg,Ng,M,N,P) | - (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) |
| PN model | Example 2 phase noise model scaling to 60 GHz in 38.803 |
| Channel coding | LDPC code |
| frame structure | 2(PDCCH)/1(DMRS)/11(data+PTRS) |
| Transmission BW | 400 MHz：256/128/64/32 PRB for 120/240/480/960 kHz |
| TTI | 1 slot |
| DMRS | 1/2 in frequency in each port |
| PTRS | For CP-OFDM: K=2 in frequency domain, L=1 in time domain For DFT-s-OFDM: 8*4 for 120 kHz and 240 kHz, 4*4 for 480 kHz, 4*2 for 960 kHz |
| Receiver | CPE, ICI compensation |
| Channel estimation | Realistic |

Simulation results

PDSCH

Normal CP length is used for evaluation of different SCSs. The results illustrated from Figure 01 to Figure 09 show that the performance gap among different SCSs is ignorable for QPSK and 16QAM in scenarios with short delay spread. Further, when ICI compensation based on block-based PTRS is used, the phase noise impact reduces significantly for 64QAM when small a SCS is used; resulting in a similar BLER performance to a large SCS. And by taking place of NCP with ECP for large delay spread, the BLER performance of 960 kHz is improved, but the SE performance is deteriorated due to the high overhead of ECP. From the aspect of peak data rate, the simulation results have shown that, the small SCSs can work at MCS28 if a high order of ICI estimation and compensation can be used.

Figure 01. BLER performance with different numerologies for MCS7/16/22 for CP-OFDM

(CDL-B DS=20ns, CPE compensation)

Figure 02. BLER performance with different numerologies for MCS7/16/22 for CP-OFDM

(CDL-B DS=50ns, CPE and ICI compensation)

Figure 03. BLER performance with different numerologies for MCS7/16/22 for CP-OFDM

(CDL-D, DS=20ns, left: MCS7/16, right: MCS22)

Figure 04. BLER performance with different numerologies for MCS7/16/22 for CP-OFDM

(CDL-D DS=30ns, CPE compensation)

Figure 05. BLER performance with different numerologies for 22 for CP-OFDM with ICI compensation

(left: CDL-B, DS=20ns; right: CDL-D, DS=30ns)

Figure 06. BLER performance of different SCSs for MCS28

(left: CDL-B, 20ns; right: CDL-B, 50ns)

Figure 07. BLER performance of different SCSs for MCS28

(left: CDL-D, 20ns; right: CDL-D, 30ns)

Figure 08. BLER/SE performance comparison between NCP and ECP for 960 kHz

(CDL-B, 50ns, MCS22, MCS24, and MCS26)

In addition, a new phase noise model proposed in [4] is used for performance evaluation of different SCSs with only CPE compensation, and the error floor of 120 kHz is lower than 10-2, which can be improved further if ICI compensation is applied.

Figure 09. BLER performance of different SCSs with new phase noise model for MCS22

BLER performance and SE performance between block PTRS and Rel-15 PTRS are compared in Figure 10 and Figure 11 respectively, where K means the PTRS frequency density is every K-th RB for Rel-15 PTRS, and CN means chunk number, CS means chunk size for block PTRS. By putting Rel-15 PTRS in adjacent subcarriers as a block, the BLER performance is lightly worse than that of distributed PTRS, about 0.1-dB loss at BLER of 10%. But when a sequence which has constant module in both time domain and frequency domain is used with block PTRS, the performance of block PTRS is significant improved, which is slightly better than that of Rel-15 PTRS, about 0.5dB. And the similar phenomenon can be observed for SE performance. Moreover, the matrix inversion operation in the ICI compensation algorithm can be simplified with block PTRS.

Figure 10. BLER performance comparison between block PTRS and distributed PTRS

Figure 11. SE performance comparison between block PTRS and distributed PTRS

Observation 1: For QPSK and 16QAM, SCSs larger than 120 kHz do not achieve a significantly better BLER. For 64QAM, a larger SCS performs better than a smaller SCS without ICI compensation. Block-based PTRS enables ICI compensation for smaller SCSs and helps a smaller SCS to perform even better than a larger SCS.

Observation 2: If CDL-B DS=50ns channel model is considered, the BLER performance of a subcarrier spacing larger than 480 kHz decreases a lot due to the short CP length.

Observation 3: When both the impact of phase noise and CP length on BLER performance are considered, simulation results show that a smaller SCS (120 kHz or 240 kHz) with NCP is the best solution if block-based PTRS for ICI compensation is introduced.

Observation 4: Using ECP improves the BLER performance of 960 kHz in scenarios with large delay spread, but makes no contribution to SE or throughput due to the high overhead.

Observation 5: Phase noise model is an important factor of feasibility assessment for different SCSs.

PUSCH

Evaluation results for PUSCH of CP-OFDM are the same as PDSCH, and only the results for PUSCH of DFT-s-OFDM is illustrated with details in this section.

In the evaluations, the same PTRS overhead of about 2% is applied to 240 kHz, 480 kHz and 960 kHz, which is similar to the PTRS overhead for CP-OFDM. In turn, the overhead for 120 kHz is only about 1%, which is the highest overhead that can be configured for 120 kHz as the maximum number of PTRS patterns defined in Rel-15 is only 32. With parameters of PTRS pattern being set to NgroupPT-RS*Nsamplegroup=8*4, the BLER performance of 120 kHz for 64QAM shows a performance floor above 10-2 due to the longest interpolation range between two PTRS groups. In addition, BLER performance of 960 kHz for 16QAM is the worst, as the noise reduction performance per PTRS group is the worst in a low SNR region. And BLER performances of different SCSs are almost the same for QPSK without phase noise compensation, as the phase noise for QPSK is not the prime impact.

Figure 10. BLER performance with different numerology for DFT-s-OFDM MCS7(QPSK), MCS16(16QAM) and MCS22(64QAM) (CDL-B DS=20ns)

Figure 11. BLER performance with different numerology for DFT-s-OFDM MCS7(QPSK), MCS16(16QAM) and MCS22(64QAM) (CDL-B DS=50ns)

Figure 12. BLER performance with different numerology for DFT-s-OFDM MCS7(QPSK), MCS16(16QAM) and MCS22(64QAM) (CDL-D DS=20ns)

Figure 13. BLER performance with different numerology for DFT-s-OFDM MCS7(QPSK), MCS16(16QAM) and MCS22(64QAM) (CDL-D DS=30ns)

Observation 6: Simulation results for DFT-s-OFDM show that, SCSs larger than 240 kHz do not achieve a better BLER performance than 120 and 240 kHz SCSs for QPSK and 16QAM. BLER performance of 120 kHz for 64QAM reaches a floor above 10-2 due to a lower overhead of PTRS and the longest interpolation range.

PRACH

Preamble sequence length is set to be 139 in the PRACH evaluations. As shown in Figure 14 and Figure 15, the detection SINR for 960 kHz is about 1.3dB lower than that of 120 kHz under CDL-B with different delay spread since 960 kHz SCS enjoys a more frequency division gain due to its larger bandwidth. To further verify the influence of the frequency selectivity, detection performances are compared under CDL-D with different delay spread which show that the detection performance for different SCSs are almost the same since the frequency selectivity of CDL-D is similar to that of the AWGN channel.

In addition, when taking a fixed total transmission power into consideration, which is a typical practice in uplink transmission, the SNR gap of 99% detection probability between 120 kHz SCS and 960 kHz SCS can be compensated by the 9 dB energy gap of the received signal between 120 kHz SCS and 960 kHz SCS.

Figure 14. Detection performance of PRACH with different numerology

(left: CDL-B, DS=20ns; right: CDL-B, DS=50ns)

Figure 15. Detection performance of PRACH with different numerology

(left: CDL-D, DS=20ns; right: CDL-D, DS=30ns)

Observation 7: The detection performance gap between 960 kHz and 120 kHz is about 1.3dB under CDL-B, and the detection performances for different SCSs are almost the same under CDL-D.

Observation 8: By accounting for both the impacts of the received signal energy gap and the detection performance gap for different SCSs, a lower SCS has a wider PRACH coverage.

Assumptions and results for system level evaluation

Simulation assumptions for indoor scenario

| Parameters | Assumptions |
| --- | --- |
| Layout | Indoor scenario-A: Office box 120m x 50m，12 BS per operator, 2 operator.BS randomly deployed within 10m x 10m virtual box. BS mounting on the ceiling. Indoor scenario-C: Office box 120m x 50 m, 12 BS per operator, 1 operator |  |
| Inter-BS distance | Random with minimum distance of 2m for scenario-A |  |
| UE distribution | 100% Indoor, randomly distributed within the office box, 5 users per BS |  |
| Carrier bandwidth | 400MHz/2000MHz |  |
| Numerology | 120kHz for 400MHz/960kHz for 2000MHz |  |
| Carrier frequency | 60GHz |  |
| Channel model | InH mixed/open channel model in 38.901 |  |
| Max. allowed BS Tx power(EIRP) | 40dBm |  |
| Max. allowed UE Tx Power(EIRP) | 25dBm |  |
| BS antenna configurations | See 38.802 Table A.2.1-7 for ceiling mount(antenna gain = 5dB) |  |
| UE antenna configuration | See 38.802 Table A.2.1-8, boresight parallel to ground(antenna gain = 5dB) |  |
| BS antenna height | 3m |  |  |
| UE antenna height | 1.5m |  |  |
| BS receiver noise figure | 7dB |  |  |
| UE receiver noise figure | 10dB |  |
| BS antenna array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), Dh = Dv = 0.5 λ |  |
| UE antenna array configuration | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ |  |
| MCOT | 5ms |  |
| Traffic model | FTP3 with packet size of 8/27 Mbytes. |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| Feedback assumption | Realistic |  |
| Channel estimation | Realistic/ideal |  |
| Traffic type | 50% DL, 50% UL |  |

Simulation assumptions for outdoor scenario

| Parameters | Assumptions |
| --- | --- |
| Layout | Scenario Outdoor-B: Dense Urban with 2 layers Macro layer (sub 7GHz-not simulated): Hexagonal grid, single layer, 3 sectors per site, 1 sites locations BS height 25m, UE height 1.5m, ISD = 100m, fixed BS position Micro layer (above 52.6 GHz): BS height 10m, UE height 1.5m, 2 operator, 2 BS per hexgrid per operator, random position within macro hexagonal grid per operator, minimum distance between TRP(gNB or UE) and TRP(gNB or UE): 10m |  |
| Inter-BS distance | Random with minimum distance of 10m |  |
| UE distribution | 100% Outdoor, randomly distributed within macro hexagonal grid, 5 users per BS |  |
| Carrier bandwidth | 2000MHz |  |
| Numerology | -960kHz |  |
| Carrier frequency | 60GHz |  |
| Channel model | gNB-to-gNB and gNB-to-UE links: UMI street canyon channel & PL models from tr38.901 UE-to-UE links: D2D channel& PL models from TR36.843 Section A.2.1.2 |  |
| Max. allowed BS Tx power(EIRP) | 40dBm |  |
| Max. allowed UE Tx Power(EIRP) | 25dBm |  |
| BS antenna configurations | See 38.901 Table 7.3-1, 3 Panel single sector gNB with {0,+120,-120} degree boresight orientations(antenna gain = 5dB). |  |
| UE antenna configuration | See 38.802 Table A.2.1-8, boresight parallel to ground(antenna gain = 5dB) |  |
| BS antenna height | 10m |  |
| UE antenna height | 1.5m |  |
| BS receiver noise figure | 7dB |  |
| UE receiver noise figure | 10dB |  |
| BS antenna array configuration | (M, N, P, Mg, Ng) = (8, 16, 2, 3, 1), Dh = Dv = 0.5 λ |  |
| UE antenna array configuration | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ |  |
| MCOT | 5ms |  |
| Traffic model | FTP3 with packet size of 27 Mbytes. |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| Feedback assumption | Realistic |  |
| Channel estimation | Realistic/ideal |  |
| Traffic type | 50% DL, 50% UL |  |

Simulation results

In RAN1 #102e, simulation scenarios and assumptions were continued to be discussed. The deployment of indoor scenario-A and scenario-C was agreed to maintain the room size of 120m*50m with the cell selection threshold of -71 dBm + 10 log10 (BW/2GHz). Figures 16 to Figure 23 are the RSRP distribution for serving BS to UE links, BS-to-UE interference links, BS-to-BS interference links, and UE-to-UE interference links for indoor and outdoor scenario. In BS-to-UE interference link, the statistics are calculated assuming gNB is sending signals to its served UEs and the received power at other UEs are calculated with Omni-dir antenna pattern. Same approach is used for other types of interference links.

Figure 16. Serving link RSRP of Indoor scenario-A/C

Figure 17. BS-to-UE interference link RSRP of Indoor scenario-A/C

Figure 18. BS-to-BS interference link RSRP of Indoor scenario-A/C

Figure 19. UE-to-UE interference link RSRP of Indoor scenario-A/C

Figure 20. Serving link RSRP of outdoor scenario-B

Figure 21. BS-to-UE interference link RSRP of Indoor scenario-B

Figure 22. BS-to-BS interference link RSRP of Indoor scenario-B

Figure 23. UE-to-UE interference link RSRP of Indoor scenario-B

We also provide the SLS results with various channel access mechanisms based on the indoor and outdoor scenarios. The results presented here are according to the SLS evaluation methodology agreed in [1] and the adaptively rule specified in the latest draft of EN 302 567 to verify the impact of LBT on the performance of various antenna array configurations in 60GHz band. In EN 302 567, LBT is mandatory to facilitate spectrum sharing. Before a transmission or a burst of transmissions on an Operating Channel, the equipment shall perform a Clear Channel Assessment (CCA) check using "energy detect". The energy detection threshold for the CCA shall be -47 dBm + (40 dBm - Pout (dBm)) irrespective to the channel bandwidth, where Pout is the RF output power (EIRP).

Figure 24. Simulation results with various channel access mechanism for 60GHz band in indoor scenario-A

Figure 25. Simulation results with various channel access mechanism for 60GHz band in indoor scenario-C

Figure 26. Simulation results with various channel access mechanism for 60GHz band in outdoor scenario-B (1 site)

Figure 27. Simulation results with various channel access mechanism for 60GHz band in outdoor scenario-B (7 sites)

Simulation results in Figures 24, 25, 26 and 27 show mean value of throughput and the 5th percentile user perceived throughput (UPT) for LBT schemes with both DL and UL FTP3 traffic loads with the file size of 27 Mbytes for all the related scenarios above. More details results corresponding to Figures 24, 25 and 26 could be found in Appendix B in Tables 01-02, Tables 03-04, Table 05-06 and Tables 07-08, respectively.

Figure 28. Simulation results with various channel access mechanism for 60GHz band in indoor scenario-A with InH mixed channel model for BS to UE links

Simulation results in Figure 28, show mean value of throughput and the 5th percentile user perceived throughput (UPT) for LBT schemes with both DL and UL FTP3 traffic loads with the file size of 8Mbytes in indoor scenario-A with InH mixed channel models for BS to UE links. More detailed results could be found in the appendix B Tables 09-10.

Figure 29. Simulation results of No LBT/Receiver-assisted LBT/Receiver-only directional LBT in indoor scenario with InH open channel model for BS to UE links

Figure 30. Simulation results of No LBT/Receiver-assisted LBT/Receiver-only directional LBT in indoor scenario with InH mixed channel model for BS to UE links

Simulation results in Figure 29 and 30 show mean value of throughput and the 5th percentile user perceived throughput (UPT) for LBT schemes with No LBT/received-assisted LBT/receiver-only directional LBT in indoor scenario A with InH open/mixed channel models for BS to UE links respectively. More details results could be found in the appendix B table 11 and 12 respectively.

A fixed contention window size of 127 contention slots with energy detection threshold of  -47 dBm for gNB and -32dBm for UE is used in LBT procedure based on EN 302 567. The maximum channel occupancy times (COT) is 5ms. The No-LBT scheme uses unconstrained NR-like TDD operation for both operators with no medium access protocol or LBT mechanism. LBT with energy detection considering no array gain is called Omni-dir-LBT, and LBT with energy detection via narrow beam is called Direct-LBT. LBT with energy detection via narrow beam at the receiver side is called as receiver-assisted LBT, which is proposed in our companion contribution [3].

The observations from the simulation results both indoor and outdoor scenarios indicate the following: (1) The LBT mechanism may result in contention overhead and consequent loss in performance in a low traffic load scenario. (2) Receiver-assisted LBT is beneficial for the cell-edge users especially in medium and high traffic load.

Observation 9: LBT mechanism at transmitter may result in contention overhead and loss in performance for both indoor and outdoor scenario.

Observation 10: Receiver-assisted/Receiver-only directional LBT is beneficial for cell-edge users in indoor scenario especially in medium and high traffic load.

Conclusions

In this contribution, we have provided link level and system level simulation results for both licensed and unlicensed bands. Based on the discussion, we have the following proposals and observations:

Observation 1: For QPSK and 16QAM, SCSs larger than 120 kHz do not achieve a significantly better BLER. For 64QAM, a larger SCS performs better than a smaller SCS without ICI compensation. Block-based PTRS enables ICI compensation for smaller SCSs and helps a smaller SCS to perform even better than a larger SCS.

Observation 2: If CDL-B DS=50ns channel model is considered, the BLER performance of a subcarrier spacing larger than 480 kHz decreases a lot due to the short CP length.

Observation 3: When both the impact of phase noise and CP length on BLER performance are considered, simulation results show that a smaller SCS (120 kHz or 240 kHz) with NCP is the best solution if block-based PTRS for ICI compensation is introduced.

Observation 4: Using ECP improves the BLER performance of 960 kHz in scenarios with large delay spread, but makes no contribution to SE or throughput due to the high overhead.

Observation 5: Phase noise model is an important factor of feasibility assessment for different SCSs.

Observation 6: Simulation results for DFT-s-OFDM show that, SCSs larger than 240 kHz do not achieve a better BLER performance than 120 and 240 kHz SCSs for QPSK and 16QAM. BLER performance of 120 kHz for 64QAM reaches a floor above 10-2 due to a lower overhead of PTRS and the longest interpolation range.

Observation 7: The detection performance gap between 960 kHz and 120 kHz is about 1.3dB under CDL-B, and the detection performances for different SCSs are almost the same under CDL-D.

Observation 8: By accounting for both the impacts of the received signal energy gap and the detection performance gap for different SCSs, a lower SCS has a wider PRACH coverage.

Observation 9: LBT mechanism at transmitter may result in contention overhead and loss in performance for both indoor and outdoor scenario.

Observation 10: Receiver-assisted/ Receiver-only directional LBT is beneficial for cell-edge users in indoor scenario especially in medium and high traffic load.

- References

[1] Chairman’s Notes RAN1#102-e

[2] ETSI EN 302 567 Multiple-Gigabit/s radio equipment operating in the 60 GHz band; V2.1.20

[3] R1-2007605 Channel access mechanism for 60 GHz unlicensed operation, Huawei, HiSilicon.

[4] R4-2011494, “On PN model for 52.6~71GHz”, Huawei.

- Appendix A.

Table 01: SINR in dB achieving PDSCH/PUSCH BLER of 10% /1% for CP-OFDM

| MCS | Channel | 120 kHz @400 MHz |  | 240 kHz @400 MHz |  | 480 kHz @400 MHz |  | 960 kHz @400 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PHN | CPE | ICI | CPE | ICI | CPE | ICI | CPE |
| 7 | CDL-B, 20ns | 2/4.8 | -- | 2/4.8 | -- | 2/4.8 | -- | 2/4.8 |
|  | CDL-B, 50ns | 2.9/4.2 | -- | 2.6/4.2 | -- | 2.7/4.2 | -- | 2.6/3.9 |
|  | CDL-D, 20ns | -0.7/0 | -- | -0.4/0.3 | -- | -0.1/0.6 | -- | -0.3/0.3 |
|  | CDL-D, 30ns | -1.4/-1 | -- | -1.2/-1 | -- | -1.6/-1.4 | -- | -1.7/-1.7 |
| 16 | CDL-B, 20ns | 10.5/12.5 | -- | 10/12.3 | -- | 11.8/12 | -- | 10.7/11.9 |
|  | CDL-B, 50ns | 11.2/12.6 | -- | 11.2/13.4 | -- | 11.6/14.0 | -- | 11.9/13.3 |
|  | CDL-D, 20ns | 6.8/7.4 | -- | 7.2/7.9 | -- | 7.3/7.9 | -- | 7.1/7.6 |
|  | CDL-D, 30ns | 7/7.2 | -- | 6.8/7.1 | -- | 6.7/6.8 | -- | 6.6/6.8 |
| 22 | CDL-B, 20ns | NAN | 16.8/20 | NAN | 15.3/17.2 | 17/-- | 17/19 | 15.7/18 |
|  | CDL-B, 50ns | NAN | 16.8/18.1 | NAN | 15.8/18.2 | 18/26 | 16.9/19.1 | 18.4/20.8 |
|  | CDL-D, 20ns | NAN | 11.9/12.7 | NAN | 12.3/13.0 | 13.7/-- | 12.6/13.3 | 12.5/13.1 |
|  | CDL-D, 30ns | NAN | 12.5/12.9 | NAN | 11.8/12.4 | 13.2/14.8 | -- | 12.1/12.5 |

Table 02: SINR in dB achieving PDSCH/PUSCH BLER of 10% /1% for CP-OFDM and MCS28

| Channel | 120 kHz @400 MHz | 240 kHz @400 MHz | 480 kHz @400 MHz |  | 960 kHz @400 MHz |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ICI-11 | ICI-9 | ICI-5 | ICI-7 | CPE | ICI-3 |
| CDL-B, 20ns | 26.2/30 | 24.3/28.8 | 25.5/33.7 | 25.1/28.8 | NAN/NAN | 25.3/30.5 |
| CDL-B, 50ns | 24.7/28.8 | 25.5/29.7 | 25.5/NAN | 25.0/29.5 | NAN/NAN | NAN/NAN |
| CDL-D, 20ns | 19.3/20.7 | 19.6/21.2 | 20.1/22.4 | 19.7/21 | 26/NAN | 19.7/21.4 |
| CDL-D, 30ns | 19.3/20.7 | 19.6/21.3 | 20.1/22.4 | 19.8/21 | 26/NAN | 19.8/21.2 |

Table 03: SINR in dB achieving PDSCH/PUSCH BLER of 10% /1% for CP-OFDM and 960 kHz with different CP length

| CP type | MCS22@CPE | MCS24@CPE | MCS26@ICI-3 |
| --- | --- | --- | --- |
| NCP | 17.7/20 | 21/27.4 | 24.9/NAN |
| ECP | 17.3/19 | 19.6/21.8 | 22.2/24.8 |

Table 04: SINR in dB achieving PDSCH/PUSCH BLER of 10% /1% for CP-OFDM with optional phase noise model and MCS22 when only CPE compensation is used

| Channel | 120 kHz@400 MHz | 240 kHz@400 MHz | 480 kHz@400 MHz | 960 kHz@400 MHz |
| --- | --- | --- | --- | --- |
| CDL-B, 20ns | 18.4/21.3 | 17.3/19.8 | 16.4/19 | 15.7/18.2 |
| CDL-D, 20ns | 13.1/13.9 | 12.9/13.6 | 12.6/13.3 | 12.3/12.9 |

Table 05: SINR in dB achieving PDSCH/PUSCH BLER of 10% /1% for CP-OFDM and different PTRS patterns at MCS22

| Channel | Rel-15 PTRS | Block-PTRS | Block PTRS with special sequence |
| --- | --- | --- | --- |
| CDL-B, 20ns | 17.7/21.9 | 17.8/22.6 | 17.2/21.5 |
| CDL-D, 20ns | 13.9/14.8 | 14.2/15.4 | 13.6/14.5 |

Table 06: SINR in dB achieving PUSCH BLER of 10% /1% for DFT-s-OFDM

| MCS | Channel | 120 kHz @400 MHz | 240 kHz @400 MHz | 480 kHz @400 MHz | 960 kHz @400 MHz |
| --- | --- | --- | --- | --- | --- |
| 7 | CDL-B, 20ns | 3.0/4.3 | 2.4/4.4 | 3.2/5.0 | 2.8/4.7 |
|  | CDL-B, 50ns | 3.0/4.4 | 2.9/5.1 | 2.8/5.6 | 3.3/5.5 |
|  | CDL-D, 20ns | -0.4/0.2 | -0.2/0.5 | -0.5/0.3 | -0.1/0.4 |
|  | CDL-D, 30ns | -0.4/0.2 | -0.2/0.5 | -0.5/0.3 | -0.1/0.4 |
| 16 | CDL-B, 20ns | 12.1/13.7 | 11.1/13.8 | 12.2/15.4 | 12.5/15.5 |
|  | CDL-B, 50ns | 12.6/14.9 | 11.6/14.4 | 12.2/15.2 | 14.1/16.1 |
|  | CDL-D, 20ns | 7.0/7.6 | 7.5/8.1 | 7.3/8.0 | 8.1/8.6 |
|  | CDL-D, 30ns | 7.0/7.6 | 7.5/8.1 | 7.3/8.0 | 8.1/8.5 |
| 22 | CDL-B, 20ns | 20.9/NAN | 17.4/20.6 | 19.1/22.7 | 18.4/21.4 |
|  | CDL-B, 50ns | 21.4/NAN | 18.0/21.2 | 18.8/22.2 | 19.7/21.9 |
|  | CDL-D, 20ns | 14.7/NAN | 13.4/14.4 | 13.2/14.3 | 13.4/13.8 |
|  | CDL-D, 30ns | 14.7/NAN | 13.4/14.4 | 13.2/14.3 | 13.4/13.8 |

Table 07: SINR in dB achieving PRACH preamble misdetection probability of 1% and corresponding false alarm probability 1‰

| Format | Channel | 120 kHz | 240 kHz | 480 kHz | 960 kHz |
| --- | --- | --- | --- | --- | --- |
| A1 | CDL-B, 20ns | -6.920/1‰ | -7.820/1‰ | -7.829/1‰ | -8.234/1‰ |
|  | CDL-B, 50ns | -7.842/1‰ | -8.258/1‰ | -8.652/1‰ | -8.917/1‰ |
|  | CDL-D, 20ns | -6.740/1‰ | -6.740/1‰ | -6.740/1‰ | -6.728/1‰ |
|  | CDL-D, 30ns | -6.740/1‰ | -6.740/1‰ | -6.740/1‰ | -6.728/1‰ |
| A2 | CDL-B, 20ns | -9.836/1‰ | -10.860/1‰ | -10.739/1‰ | -11.030/1‰ |
|  | CDL-B, 50ns | -10.777/1‰ | -11.079/1‰ | -11.558/1‰ | -11.955/1‰ |
|  | CDL-D, 20ns | -9.529/1‰ | -9.523/1‰ | -9.526/1‰ | -9.526/1‰ |
|  | CDL-D, 30ns | -9.529/1‰ | -9.523/1‰ | -9.526/1‰ | -9.526/1‰ |

- Appendix B.

Table 01. System level evaluation results for indoor scenario A (no-LBT and omni-directional LBT)

| Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 2690.3 | 1015.6 | 222.3 | 2527.9 | 881.7 | 158.8 |
|  | 50%ile | 6635.8 | 4431.6 | 3385.4 | 6137.7 | 4068.8 | 3062.5 |
|  | 95%ile | 9477.6 | 9469.3 | 9404.3 | 8949.0 | 8837.5 | 8628.6 |
|  | mean | 6381.8 | 4819.0 | 3818.9 | 5943.5 | 4404.3 | 3439.1 |
| DL delay (s) | 5%ile | 0.0228 | 0.0228 | 0.0229 | 0.0241 | 0.0244 | 0.0250 |
|  | 50%ile | 0.0325 | 0.0485 | 0.0634 | 0.0352 | 0.0530 | 0.0698 |
|  | 95%ile | 0.0801 | 0.2059 | 0.7516 | 0.0867 | 0.2492 | 1.0180 |
|  | mean | 0.0396 | 0.0743 | 0.1751 | 0.0430 | 0.0882 | 0.2161 |
| UL UPT (Mbps) | 5%ile | 2817.4 | 1027.9 | 304.2 | 2606.0 | 839.2 | 185.6 |
|  | 50%ile | 6949.9 | 4798 | 3802.7 | 6527.9 | 4358.4 | 3372.2 |
|  | 95%ile | 10637 | 10118 | 9111.1 | 9982.3 | 9243.3 | 8536.7 |
|  | mean | 6771.0 | 5046.4 | 4165.3 | 6320.3 | 4602.4 | 3726.3 |
| UL delay (s) | 5%ile | 0.0203 | 0.0211 | 0.0236 | 0.0216 | 0.0225 | 0.0251 |
|  | 50%ile | 0.0311 | 0.0448 | 0.0564 | 0.0331 | 0.0493 | 0.0634 |
|  | 95%ile | 0.0764 | 0.2034 | 0.5975 | 0.0837 | 0.2500 | 0.8877 |
|  | mean | 0.0375 | 0.0728 | 0.1613 | 0.0410 | 0.0867 | 0.2029 |
| Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
| 𝜌DL |  | 99.64% | 99.49% | 97.37% | 99.69% | 99.44% | 96.69% |
| 𝜌UL |  | 99.68% | 99.45% | 97.89% | 99.65% | 99.32% | 96.91% |
| BO |  | 12.94% | 42.34% | 57.72% | 13.81% | 45.13% | 60.60% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; BS to UE: InH open office channel. |  |  |  |  |  |  |  |

Table 02. System level evaluation results for indoor scenario A (directional LBT and receiver assisted LBT)

| Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 2507.1 | 874.7 | 154.7 | 2587.9 | 994.5 | 242.7 |
|  | 50%ile | 6129.2 | 4038.4 | 3031.0 | 6170.7 | 4181.1 | 3187.9 |
|  | 95%ile | 8956.3 | 8827.5 | 8625.3 | 8959.2 | 8852.4 | 8690.1 |
|  | mean | 5931.7 | 4382.4 | 3432.0 | 5991.0 | 4516.8 | 3582.2 |
| DL delay (s) | 5%ile | 0.0241 | 0.0244 | 0.0250 | 0.0241 | 0.0244 | 0.0248 |
|  | 50%ile | 0.0351 | 0.0530 | 0.0707 | 0.0352 | 0.0515 | 0.0673 |
|  | 95%ile | 0.0858 | 0.2411 | 0.9853 | 0.0838 | 0.2237 | 0.7022 |
|  | mean | 0.0427 | 0.0857 | 0.2087 | 0.0424 | 0.0819 | 0.1971 |
| UL UPT (Mbps) | 5%ile | 2606.0 | 836.3 | 181.8 | 2698.8 | 938.8 | 275.2 |
|  | 50%ile | 6528.8 | 4351.0 | 3369.3 | 6621.1 | 4490.5 | 3537.3 |
|  | 95%ile | 9985.1 | 9250.1 | 8531.0 | 9990.1 | 9469.4 | 8631.4 |
|  | mean | 6317.1 | 4594.3 | 3724.6 | 6396.9 | 4750.6 | 3868.7 |
| UL delay (s) | 5%ile | 0.0216 | 0.0230 | 0.0252 | 0.0216 | 0.0224 | 0.0248 |
|  | 50%ile | 0.0330 | 0.0494 | 0.0634 | 0.0329 | 0.0481 | 0.0605 |
|  | 95%ile | 0.0824 | 0.2490 | 0.8331 | 0.0815 | 0.2332 | 0.7032 |
|  | mean | 0.0405 | 0.0880 | 0.1989 | 0.0402 | 0.0808 | 0.1831 |
| Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
| 𝜌DL |  | 99.62% | 99.41% | 96.81% | 99.69% | 99.47% | 96.90% |
| 𝜌UL |  | 99.66% | 99.33% | 96.98% | 99.66% | 99.34% | 97.21% |
| BO |  | 13.82% | 45.20% | 60.57% | 13.81% | 44.89% | 60.35% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; BS to UE: InH open office channel. |  |  |  |  |  |  |  |

Table 03. System level evaluation results for indoor scenario C (no-LBT and omni-directional LBT)

| Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 2712.1 | 1271.5 | 585.9 | 2478.7 | 1136.6 | 474.6 |
|  | 50%ile | 6542.7 | 4481.1 | 4029.6 | 6091.8 | 4458.6 | 3595.8 |
|  | 95%ile | 9478.1 | 9472.7 | 9459.5 | 8954.3 | 8848.7 | 8715.9 |
|  | mean | 6383.1 | 5130.7 | 4346.5 | 5945.0 | 4722.5 | 3913.9 |
| DL delay (s) | 5%ile | 0.0228 | 0.0228 | 0.0228 | 0.0241 | 0.0244 | 0.0247 |
|  | 50%ile | 0.0329 | 0.0442 | 0.0534 | 0.0354 | 0.0481 | 0.0598 |
|  | 95%ile | 0.0792 | 0.1646 | 0.3375 | 0.0865 | 0.1870 | 0.4128 |
|  | mean | 0.0396 | 0.0636 | 0.1125 | 0.0427 | 0.0706 | 0.1309 |
| UL UPT (Mbps) | 5%ile | 3004.4 | 1470.8 | 801.5 | 2748.7 | 1271.5 | 551.6 |
|  | 50%ile | 6947.7 | 5450.4 | 4476.4 | 6508.5 | 4898.4 | 3944.6 |
|  | 95%ile | 10637 | 10594 | 9402.1 | 10014 | 9792.6 | 8676.5 |
|  | mean | 6835.8 | 5534.8 | 4759.1 | 6376.1 | 5058.1 | 4232.4 |
| UL delay (s) | 5%ile | 0.0203 | 0.0204 | 0.0227 | 0.0215 | 0.0220 | 0.0248 |
|  | 50%ile | 0.0311 | 0.0394 | 0.0481 | 0.0331 | 0.0438 | 0.0544 |
|  | 95%ile | 0.0719 | 0.1422 | 0.2501 | 0.0786 | 0.1671 | 0.3530 |
|  | mean | 0.0366 | 0.0573 | 0.0951 | 0.0395 | 0.0661 | 0.1228 |
| Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
| 𝜌DL |  | 99.69% | 99.52% | 98.91% | 99.65% | 99.44% | 98.71% |
| 𝜌UL |  | 99.80% | 99.54% | 99.01% | 99.79% | 99.47% | 98.53% |
| BO |  | 12.75% | 38.75% | 52% | 13.60% | 41.32% | 55.23% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, one operator, MCOT=5ms; BS to UE: InH open office channel. |  |  |  |  |  |  |  |

Table 04. System level evaluation results for indoor scenario C (directional LBT and receiver assisted LBT)

| Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 2487.3 | 1128.1 | 462.6 | 2558.2 | 1209.5 | 561.7 |
|  | 50%ile | 6069.5 | 4452.3 | 3588.1 | 6119.3 | 4564.6 | 3801.1 |
|  | 95%ile | 8955.1 | 8838.9 | 8732.4 | 8959.9 | 8860 | 8761.1 |
|  | mean | 5931.7 | 4711.4 | 3892.5 | 5995.4 | 4826.2 | 4078.2 |
| DL delay (s) | 5%ile | 0.0241 | 0.0244 | 0.0247 | 0.0241 | 0.0243 | 0.0246 |
|  | 50%ile | 0.0355 | 0.0482 | 0.0599 | 0.0353 | 0.0471 | 0.0565 |
|  | 95%ile | 0.0867 | 0.1884 | 0.4249 | 0.0834 | 0.1738 | 0.3608 |
|  | mean | 0.0427 | 0.0705 | 0.1315 | 0.0420 | 0.0669 | 0.1214 |
| UL UPT (Mbps) | 5%ile | 2786.3 | 1266.3 | 558.5 | 2864.9 | 1353.5 | 619.5 |
|  | 50%ile | 6510.4 | 4914.7 | 3943.5 | 6562.9 | 5096.8 | 4128.0 |
|  | 95%ile | 10006 | 9819.6 | 8623.7 | 10020 | 9805.7 | 8768.0 |
|  | mean | 6376.3 | 5050.6 | 4229.1 | 6443.4 | 5180.5 | 4374.5 |
| UL delay (s) | 5%ile | 0.0216 | 0.0219 | 0.0249 | 0.0215 | 0.0220 | 0.0244 |
|  | 50%ile | 0.0331 | 0.0437 | 0.0545 | 0.0329 | 0.0422 | 0.0520 |
|  | 95%ile | 0.0775 | 0.1666 | 0.3475 | 0.0754 | 0.1562 | 0.3143 |
|  | mean | 0.0395 | 0.0662 | 0.1225 | 0.0388 | 0.0622 | 0.1122 |
| Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
| 𝜌DL |  | 99.64% | 99.44% | 98.71% | 99.65% | 99.44% | 98.78% |
| 𝜌UL |  | 99.79% | 99.48% | 98.55% | 99.79% | 99.49% | 98.72% |
| BO |  | 13.60% | 41.31% | 55.25% | 13.64% | 41.33% | 55.20% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; BS to UE: InH open office channel. |  |  |  |  |  |  |  |

Table 05. System level evaluation results for outdoor scenario B (1 site, no-LBT and omni-directional LBT)

| Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1717.8 | 571.6 | 114.3 | 1650.1 | 465.4 | 82.3 |
|  | 50%ile | 4271.6 | 3191.9 | 2307.5 | 3986.2 | 2871.9 | 1985.4 |
|  | 95%ile | 7548.5 | 7296.0 | 7053.1 | 7115.3 | 6746.6 | 6462.0 |
|  | mean | 4543.2 | 3467.8 | 2746.8 | 4242.6 | 3131.2 | 2434.7 |
| DL delay (s) | 5%ile | 0.0584 | 0.0292 | 0.0306 | 0.0301 | 0.0317 | 0.0329 |
|  | 50%ile | 0.0506 | 0.0673 | 0.0919 | 0.0540 | 0.0746 | 0.1069 |
|  | 95%ile | 0.1236 | 0.3558 | 1.0587 | 0.1294 | 0.4282 | 1.3218 |
|  | mean | 0.0589 | 0.1096 | 0.2430 | 0.0631 | 0.1278 | 0.3009 |
| UL UPT (Mbps) | 5%ile | 2771.4 | 861.9 | 223.1 | 2582.3 | 694.0 | 115.5 |
|  | 50%ile | 6935.2 | 4863.5 | 3648.8 | 6475.1 | 4332.3 | 3097.7 |
|  | 95%ile | 10570 | 9542.9 | 9058.7 | 9824.7 | 8894.9 | 8501.6 |
|  | mean | 6574.7 | 5028.7 | 4045.6 | 6142.3 | 4546.0 | 3540.6 |
| UL delay (s) | 5%ile | 0.0204 | 0.0223 | 0.0238 | 0.0219 | 0.0239 | 0.0252 |
|  | 50%ile | 0.0311 | 0.0443 | 0.0586 | 0.0333 | 0.0497 | 0.0692 |
|  | 95%ile | 0.0779 | 0.2433 | 0.7830 | 0.0830 | 0.2974 | 1.0580 |
|  | mean | 0.0385 | 0.0772 | 0.1719 | 0.0416 | 0.0912 | 0.2261 |
| Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
| 𝜌DL |  | 99.66% | 99.07% | 96.74% | 99.61% | 98.85% | 95.26% |
| 𝜌UL |  | 99.80% | 99.40% | 97.19% | 99.78% | 99.22% | 96.05% |
| BO |  | 15.86% | 47.41% | 61.76% | 16.92% | 50.47% | 64.82% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; 1 site deployment. |  |  |  |  |  |  |  |

Table 06. System level evaluation results for outdoor scenario B(1 site, directional LBT and receiver assisted LBT)

| Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1654.3 | 479.9 | 77.5 | 1642.2 | 468.6 | 81.5 |
|  | 50%ile | 3980.2 | 2843.3 | 1936.8 | 3978.3 | 2878.8 | 2035.3 |
|  | 95%ile | 7110.6 | 6722.2 | 6445.0 | 7060.1 | 6709.4 | 6435.9 |
|  | mean | 4227.9 | 3119.4 | 2413.8 | 4207.0 | 3152.7 | 2464.7 |
| DL delay (s) | 5%ile | 0.0302 | 0.0318 | 0.0330 | 0.0305 | 0.0318 | 0.0331 |
|  | 50%ile | 0.0542 | 0.0752 | 0.1089 | 0.0542 | 0.0742 | 0.1041 |
|  | 95%ile | 0.1289 | 0.4167 | 1.3588 | 0.1307 | 0.4221 | 1.3624 |
|  | mean | 0.0632 | 0.1275 | 0.3056 | 0.0633 | 0.1284 | 0.2997 |
| UL UPT (Mbps) | 5%ile | 2581.5 | 679.19 | 117.2 | 2580.8 | 701.0 | 133.3 |
|  | 50%ile | 6478.3 | 4323.7 | 3093.7 | 6486.2 | 4413.9 | 3213.7 |
|  | 95%ile | 9838.3 | 8912.6 | 8516.8 | 9880.6 | 8910.9 | 8504.7 |
|  | mean | 6138.7 | 4537.8 | 3550.8 | 6165.1 | 4589.3 | 3604.5 |
| UL delay (s) | 5%ile | 0.0219 | 0.0240 | 0.0253 | 0.0219 | 0.0236 | 0.0252 |
|  | 50%ile | 0.0333 | 0.0498 | 0.0691 | 0.0333 | 0.0488 | 0.0666 |
|  | 95%ile | 0.0837 | 0.3032 | 1.0907 | 0.0833 | 0.2950 | 1.0840 |
|  | mean | 0.0416 | 0.0923 | 0.2282 | 0.0415 | 0.0890 | 0.2253 |
| Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
| 𝜌DL |  | 99.61% | 98.86% | 95.08% | 99.61% | 98.84% | 95.25% |
| 𝜌UL |  | 99.79% | 99.18% | 96.03% | 99.78% | 99.23% | 96.39% |
| BO |  | 16.93% | 50.45% | 64.81% | 16.97% | 50.58% | 64.90% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; 1 site deployment. |  |  |  |  |  |  |  |

Table 07. System level evaluation results for outdoor scenario B (7 site, no-LBT and omni-directional LBT)

| Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1456.9 | 909.7 | 229.5 | 1294.1 | 749.5 | 180.6 |
|  | 50%ile | 4252.2 | 3809.4 | 2996.6 | 3962.8 | 3390.4 | 2687.2 |
|  | 95%ile | 7472.9 | 7389.9 | 7142.7 | 7027.2 | 6877.8 | 6682.8 |
|  | mean | 4460.8 | 3887.5 | 3258.4 | 4144.6 | 3558.5 | 2947.9 |
| DL delay (s) | 5%ile | 0.0288 | 0.0293 | 0.0303 | 0.0306 | 0.0314 | 0.0325 |
|  | 50%ile | 0.0510 | 0.0588 | 0.0722 | 0.0548 | 0.0654 | 0.0807 |
|  | 95%ile | 0.1440 | 0.2349 | 0.5771 | 0.1557 | 0.2664 | 0.6798 |
|  | mean | 0.0625 | 0.0904 | 0.1567 | 0.0678 | 0.1022 | 0.1808 |
| UL UPT (Mbps) | 5%ile | 2603.7 | 1349.6 | 346.4 | 2288.2 | 1147.3 | 248.2 |
|  | 50%ile | 6936.5 | 5628.5 | 4358.9 | 6455.4 | 5204.2 | 3836.5 |
|  | 95%ile | 10518 | 9156.1 | 9053.4 | 9688.7 | 8694.4 | 8515.9 |
|  | mean | 6452.0 | 5610.1 | 4637.2 | 6013.1 | 5151.1 | 4177.2 |
| UL delay (s) | 5%ile | 0.0209 | 0.0235 | 0.0238 | 0.0224 | 0.0248 | 0.0252 |
|  | 50%ile | 0.0312 | 0.0385 | 0.0492 | 0.0339 | 0.0421 | 0.0556 |
|  | 95%ile | 0.0869 | 0.1573 | 0.4317 | 0.0944 | 0.1796 | 0.5169 |
|  | mean | 0.0414 | 0.0597 | 0.1131 | 0.0447 | 0.0675 | 0.1317 |
| Arrival rate (files/s) |  | 0.4 | 0.8 | 1.4 | 0.4 | 0.8 | 1.4 |
| 𝜌DL |  | 98.79% | 98.59% | 96.37% | 98.68% | 98.39% | 95.55% |
| 𝜌UL |  | 99.27% | 99.14% | 96.83% | 99.26% | 98.88% | 95.81% |
| BO |  | 16.04% | 35.34% | 57.44% | 17.11% | 35.40% | 60.35% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; 7 site deployment. |  |  |  |  |  |  |  |

Table 08. System level evaluation results for outdoor scenario B (7 site, directional LBT and receiver assisted LBT)

| Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1286.7 | 750.8 | 179.2 | 1385.6 | 743.9 | 198.2 |
|  | 50%ile | 3934.1 | 3258.1 | 2643.7 | 3926.1 | 3299.0 | 2705 |
|  | 95%ile | 7085.9 | 6762.7 | 6671.5 | 6938.1 | 6748.1 | 6514 |
|  | mean | 4109.5 | 3485.6 | 2934.1 | 4110.0 | 3520.0 | 2933.3 |
| DL delay (s) | 5%ile | 0.0301 | 0.0315 | 0.0317 | 0.0306 | 0.0316 | 0.0327 |
|  | 50%ile | 0.0548 | 0.0656 | 0.0798 | 0.0546 | 0.0643 | 0.0787 |
|  | 95%ile | 0.1482 | 0.2616 | 0.6793 | 0.1527 | 0.2618 | 0.6499 |
|  | mean | 0.0672 | 0.1009 | 0.1808 | 0.0670 | 0.0996 | 0.1757 |
| UL UPT (Mbps) | 5%ile | 2126.9 | 1082.2 | 245.9 | 2285.2 | 1091.7 | 274.1 |
|  | 50%ile | 6214.7 | 5064.6 | 3816.0 | 6363.7 | 5151.7 | 3903.9 |
|  | 95%ile | 8849.0 | 8607.6 | 8455.1 | 9595.1 | 8640.3 | 8491.6 |
|  | mean | 5873.7 | 5085.0 | 4157.8 | 5947.3 | 5124.8 | 4216.9 |
| UL delay (s) | 5%ile | 0.0242 | 0.0248 | 0.0263 | 0.0223 | 0.0247 | 0.0252 |
|  | 50%ile | 0.0343 | 0.0423 | 0.0576 | 0.0336 | 0.0417 | 0.0544 |
|  | 95%ile | 0.0975 | 0.1876 | 0.5191 | 0.0916 | 0.1835 | 0.5068 |
|  | mean | 0.0458 | 0.0686 | 0.1359 | 0.0445 | 0.0681 | 0.1290 |
| Arrival rate (files/s) |  | 0.4 | 0.8 | 1.4 | 0.4 | 0.8 | 1.4 |
| 𝜌DL |  | 98.45% | 98.83% | 95.85% | 98.63% | 98.37% | 95.49% |
| 𝜌UL |  | 99.13% | 98.80% | 95.93% | 99.31% | 98.89% | 96.01% |
| BO |  | 17.51% | 35.84% | 61.15% | 17.32% | 35.93% | 60.93% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; 7 site deployment. |  |  |  |  |  |  |  |

Table 09. System level evaluation results for indoor scenario A (InH mixed, no-LBT and omni-directional LBT)

| Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 346.9 | 150.2 | 62.9 | 339.6 | 145.6 | 60.7 |
|  | 50%ile | 1141.5 | 764.6 | 597.5 | 1077.0 | 726.0 | 578.8 |
|  | 95%ile | 1851.6 | 1848.6 | 1841.6 | 1786.7 | 1768.5 | 1751.1 |
|  | mean | 1105.3 | 846.8 | 701.5 | 1057.6 | 805.6 | 669.1 |
| DL delay (s) | 5%ile | 0.0346 | 0.0353 | 0.0404 | 0.0361 | 0.0371 | 0.0421 |
|  | 50%ile | 0.0543 | 0.0811 | 0.1034 | 0.0574 | 0.0858 | 0.1080 |
|  | 95%ile | 0.2901 | 0.7812 | 1.7303 | 0.3024 | 0.8198 | 1.7773 |
|  | mean | 0.1062 | 0.2269 | 0.3583 | 0.1086 | 0.2320 | 0.3627 |
| UL UPT (Mbps) | 5%ile | 279.7 | 123.7 | 46.8 | 271.3 | 108.8 | 37.5 |
|  | 50%ile | 1071.6 | 753.1 | 590.5 | 1018 | 697.7 | 554.5 |
|  | 95%ile | 1973.5 | 1969.3 | 1943.5 | 1886.5 | 1865.4 | 1825.6 |
|  | mean | 1076.8 | 846.3 | 701.9 | 1021.9 | 792.7 | 659.9 |
| UL delay (s) | 5%ile | 0.0325 | 0.0346 | 0.0391 | 0.0342 | 0.0366 | 0.0415 |
|  | 50%ile | 0.0573 | 0.0823 | 0.1054 | 0.0603 | 0.0897 | 0.1121 |
|  | 95%ile | 0.4021 | 1.1329 | 2.2083 | 0.4133 | 1.2521 | 2.4032 |
|  | mean | 0.1378 | 0.2777 | 0.4186 | 0.1446 | 0.2981 | 0.4457 |
| Arrival rate (files/s) |  | 0.25 | 0.6 | 0.8 | 0.25 | 0.6 | 0.8 |
| 𝜌DL |  | 99.37% | 97.46% | 94.91% | 99.33% | 97.43% | 94.77% |
| 𝜌UL |  | 98.15% | 96.48% | 93.69% | 98.11% | 96.23% | 93.12% |
| BO |  | 20.19% | 45.88% | 58.88% | 21.01% | 47.36% | 59.9% |
| Additional report/notes: BW = 400MHz, FTP3 traffic file size = 8Mbyte, Fixed CW = 127, two operator, MCOT=5ms; BS to UE: InH mixed office channel. |  |  |  |  |  |  |  |

Table 10. System level evaluation results for indoor scenario A (InH mixed, directional LBT and receiver assisted LBT)

| Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 332.5 | 145.6 | 59.7 | 350.1 | 165.6 | 76.05 |
|  | 50%ile | 1072.7 | 729.1 | 573.7 | 1067.2 | 753.9 | 624.6 |
|  | 95%ile | 1787.5 | 1770.5 | 1751.1 | 1784.7 | 1770.2 | 1759.3 |
|  | mean | 1056.1 | 805.5 | 668.31 | 1057.5 | 821.0 | 699.1 |
| DL delay (s) | 5%ile | 0.0361 | 0.0370 | 0.0421 | 0.0361 | 0.0369 | 0.0406 |
|  | 50%ile | 0.0576 | 0.0856 | 0.1078 | 0.0583 | 0.0814 | 0.1000 |
|  | 95%ile | 0.3016 | 0.8349 | 1.7601 | 0.2941 | 0.6925 | 1.4098 |
|  | mean | 0.1104 | 0.2334 | 0.3629 | 0.1045 | 0.2196 | 0.3192 |
| UL UPT (Mbps) | 5%ile | 261.1 | 108.7 | 38.2 | 273.7 | 114.2 | 44.23 |
|  | 50%ile | 1016.5 | 699.4 | 557.0 | 1025.8 | 723.6 | 594.1 |
|  | 95%ile | 1885.7 | 1862.6 | 1819.1 | 1887.3 | 1867.5 | 1843.5 |
|  | mean | 1021.7 | 790.9 | 659.8 | 1032.6 | 810.4 | 684.9 |
| UL delay (s) | 5%ile | 0.0343 | 0.0367 | 0.0416 | 0.0342 | 0.0361 | 0.0404 |
|  | 50%ile | 0.0605 | 0.0890 | 0.1118 | 0.0599 | 0.0855 | 0.1039 |
|  | 95%ile | 0.4228 | 1.2325 | 2.3873 | 0.4007 | 1.0162 | 2.0620 |
|  | mean | 0.1452 | 0.2970 | 0.4438 | 0.1371 | 0.2713 | 0.4056 |
| Arrival rate (files/s) |  | 0.25 | 0.6 | 0.8 | 0.25 | 0.6 | 0.8 |
| 𝜌DL |  | 99.33% | 97.43% | 94.81% | 99.38% | 97.44% | 95.35% |
| 𝜌UL |  | 98.11% | 96.32% | 93.18% | 98.25% | 96.54% | 93.70% |
| BO |  | 21.04% | 47.4% | 59.8% | 21.24% | 47.41% | 59.83% |
| Additional report/notes: BW = 400MHz, FTP3 traffic file size = 8Mbyte, Fixed CW = 127, two operator, MCOT=5ms; BS to UE: InH mixed office channel. |  |  |  |  |  |  |  |

Table 11. System level evaluation results for indoor scenario A (receiver-only directional LBT)

| Cases |  | receiver-only directional LBT |  |  |
| --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 2809.9 | 1206.1 | 370.2 |
|  | 50%ile | 6586.1 | 4563.2 | 3591.4 |
|  | 95%ile | 9481.1 | 9475.2 | 9441.4 |
|  | mean | 6431.9 | 4943.0 | 3982.5 |
| DL delay (s) | 5%ile | 0.0228 | 0.0228 | 0.0229 |
|  | 50%ile | 0.0330 | 0.0471 | 0.0598 |
|  | 95%ile | 0.0783 | 0.1827 | 0.5443 |
|  | mean | 0.0395 | 0.0698 | 0.1610 |
| UL UPT (Mbps) | 5%ile | 2932.9 | 1139.2 | 356.9 |
|  | 50%ile | 7009.0 | 4947.9 | 3901.2 |
|  | 95%ile | 10621 | 10276 | 9203.9 |
|  | mean | 6838.1 | 5198.1 | 4277.9 |
| UL delay (s) | 5%ile | 0.0203 | 0.0207 | 0.0231 |
|  | 50%ile | 0.0310 | 0.0435 | 0.0549 |
|  | 95%ile | 0.0748 | 0.1881 | 0.5590 |
|  | mean | 0.0374 | 0.0685 | 0.1530 |
| Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 |
| 𝜌DL |  | 99.71% | 99.54% | 97.55% |
| 𝜌UL |  | 99.68% | 99.47% | 97.95% |
| BO |  | 12.96% | 42.14% | 57.46% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, two operator, MCOT=5ms; BS to UE: InH open office channel. No random back off before dl/ul data grant. |  |  |  |  |

Table 12. System level evaluation results for indoor scenario A (InH Mixed, receiver-only directional LBT)

| Cases |  | receiver-only directional LBT |  |  |
| --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 375.9 | 188.5 | 97.9 |
|  | 50%ile | 1146.4 | 822.5 | 677.0 |
|  | 95%ile | 1892.6 | 1890.1 | 1885.8 |
|  | mean | 1134.2 | 894.7 | 759.0 |
| DL delay (s) | 5%ile | 0.0339 | 0.0341 | 0.0380 |
|  | 50%ile | 0.0544 | 0.0750 | 0.0912 |
|  | 95%ile | 0.2647 | 0.5887 | 1.1616 |
|  | mean | 0.0975 | 0.1950 | 0.2849 |
| UL UPT (Mbps) | 5%ile | 289.3 | 130.9 | 53.9 |
|  | 50%ile | 1059.6 | 733.6 | 640.8 |
|  | 95%ile | 1973.2 | 1968.8 | 1957.5 |
|  | mean | 1090.8 | 863.8 | 733.9 |
| UL delay (s) | 5%ile | 0.0325 | 0.0340 | 0.0380 |
|  | 50%ile | 0.0558 | 0.0795 | 0.0960 |
|  | 95%ile | 0.3791 | 0.9621 | 1.8669 |
|  | mean | 0.1295 | 0.2605 | 0.3765 |
| Arrival rate (files/s) |  | 0.25 | 0.6 | 0.8 |
| 𝜌DL |  | 99.41% | 97.80% | 96.02% |
| 𝜌UL |  | 98.34% | 96.62% | 94.30% |
| BO |  | 20.3% | 45.43% | 58.06% |
| Additional report/notes: BW = 400MHz, FTP3 traffic file size = 8Mbyte, two operator, MCOT=5ms; BS to UE: InH mixed office channel. No random back off before dl/ul data grant |  |  |  |  |