3GPP TSG RAN WG1 #103-e	R1-2009617

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	Link budget analysis for FR1

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

Background

Link budget template for study of the baseline coverage is agreed in the post meeting e-mail discussion [102-e-Post-NR-CovEnh-02]. We provide analysis of link budget with MCL/MIL/MPL for each scenario.

Link budget analysis

The evaluation assumptions are in the excel file (updatedResults.xlsx) attached in the Tdoc. Table 1 and 2 shows analysis on MCL/MIL/MPL for each deployment scenarios.

Table 1: MCL/MIL/MPL for Urban scenario (4 GHz, TDD, DDSUU)

|  | PBCH | broadcast PDCCH | UE-specific PDCCH | msg2 PDSCH | msg4 PDSCH | PDSCH for eMBB | PRACH | msg3 PUSCH | Cell-specific PUCCH | PUCCH (22 bits) | PUSCH for eMBB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCL [dB] | 155.52 | 152.52 | 158.54 | 156.52 | 152.02 | 157.54 | 144.72 | 141.50 | 150.44 | 142.51 | 129.24 |
| MIL [dB] | 164.29 | 161.29 | 167.31 | 165.29 | 160.79 | 166.31 | 153.49 | 150.27 | 159.21 | 151.28 | 138.01 |
| MPL [dB] | 130.48 | 127.48 | 133.50 | 134.56 | 130.06 | 135.58 | 119.68 | 119.54 | 125.40 | 117.47 | 107.28 |

Table 2: MCL/MIL/MPL for Rural scenario (4 GHz, TDD, DDDSU)

|  | PBCH | broadcast PDCCH | UE-specific PDCCH | msg2 PDSCH | msg4 PDSCH | PDSCH for eMBB | PRACH | msg3 PUSCH | Cell-specific PUCCH | PUCCH (22 bits) | PUSCH for eMBB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCL [dB] | 154.02 | 152.02 | 158.04 | 156.52 | 152.02 | 157.54 | 148.72 | 142.00 | 149.94 | 142.51 | 137.99 |
| MIL [dB] | 158.02 | 156.02 | 162.04 | 160.52 | 156.02 | 161.54 | 152.72 | 146.00 | 153.94 | 146.51 | 141.99 |
| MPL [dB] | 137.07 | 135.07 | 141.09 | 142.89 | 138.39 | 143.91 | 131.77 | 128.37 | 132.99 | 125.56 | 124.36 |

We found that PUSCH for eMBB is the worst, and the Cell-specific PUCCH and PUCCH (22 bits) are also worse than other channels. Therefore, we propose,

Proposal 1: Coverage for UL channels at least including PUSCH for eMBB, msg3 PUSCH, PUCCH with 22 bits and Cell-specific PUCCH should be enhanced in Rel-17.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: Coverage for UL channels at least including PUSCH for eMBB, msg3 PUSCH, PUCCH with 22 bits and Cell-specific PUCCH should be enhanced in Rel-17.

References

- Chairman’s note at RAN1#102 e-Meeting, May 2020