3GPP TSG-RAN WG1 #103-e		R1-2009806

eMeeting, Oct 26th – Nov 13th, 2020

Source: 	Moderator (Ericsson)

Title:      	Summary#2 of Email discussion [103-e-NR-DSS-01]

Agenda item:	8.13.1

Document for:	Discussion and Decision

# 1 Introduction

This document summarizes the discussions for email thread [103-e-NR-DSS-01] under agenda item 8.13.1 on Cross-carrier scheduling (from Scell to Pcell) for the Rel17 WI on NR Dynamic spectrum sharing (DSS).

# 2. Discussion

## 2.1	Moderator Summary

Below is a short moderator summary based on tdocs [1-19] submitted for RAN1#103-e

- Search Space and DCI format handling when CCS from an SCell (sSCell) to PCell/PSCell is configured

  - Type 0/0A/1/2 CSS sets for any associated DCI formats

    - monitored only on PCell/PSCell – [1],[2],[4],[6],[7],[10],[11],[13],[14]

  - DCI formats 0_0 and 1_0 for scheduling PDSCH/PUSCH on PCell/PSCell in any associated SS set

    - monitored only on PCell/PSCell – [2],[3],[7],[10],[14],[16]

  - DCI formats 0_1,1_1,0_2,1_2 for scheduling PDSCH/PUSCH on PCell/PSCell in USS set(s)

    - Not monitored on PCell/PSCell and can be monitored only on sScell – [2],[3],[6?],[8],[11?],[19]

    - Can be monitored on both PCell/PSCell and the sSCell – [1],[7],[13],[14],[15],[16],[17?],[18]

    - Study further – [4],[5],[10]

  - DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set and DCI format 2_5

    - monitored on PCell/PSCell/sSell according to existing specifications – [1?],[2],[4],[6],[10],[18]

    - applicable to PCell/PSCell even when monitored on PCell/PSCell sSCell in some cases – [14]

  - DCI format 2_6 in type 3 CSS set

    - Monitored only on PCell/PSCell – [10]

    - Can be monitored on sSCell – [1],[6],

  - DCI formats 0_1,1_1 with SCell dormancy indication

    - Monitored only on PCell/PSCell –

    - Can be monitored on the sSCell – [1]

  - SS set overbooking and associated UE procedures

    - Applied only for PCell – [1],[6],[19]

    - Applied considering both PCell and sSCell – [14?]

- Scheduling framework when CCS from an SCell (sSCell) to PCell/PSCell is configured

  - Activation/deactivation of sSCell

    - Discussed in [2],[4],[6],[7],[9],[18]

  - Dormancy/non-dormancy for sSCell

    - Discussed in [2],[4],[18]

  - No out of order scheduling between a) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on sSCell

    - Discussed in [1],[18],[19]

  - No simultaneous rx/tx of unicast PDSCH/PUSCH on PCell/PSCell scheduled from PCell/PSCell and unicast PDSCH/PUSCH on PCell/PSCell scheduled from sSCell

    - Discussed in [18],[19]

  - Simultaneous rx of PDCCH on sSCell used for scheduling PCell/PCell and PDCCH for self-scheduling on PCell/PSCell

    - Discussed in [5]

  - PDCCH for initial transmission and retransmission can be on different cells for same TB

    - Discussed in [1],[5]

- BD/CCE limit handling when CCS from an SCell (sSCell) to PCell/PSCell is configured

  - Use multi-TRP for BD/CCE handling as starting point – [1]

  - PCell is counted as a scheduled cell for both the PCell and sSCell – [6]

  - BD/CCE limits are counted over both scheduling cells (PCell and SCell) and determined for scheduled PCell, overbooking is allowed [14]

  - Both PCell self-scheduling and SCell to PCell are considered – [18]

  - Per-scheduling-CC limit is determined as (per-CC BD or CCE limit) – (max of the number of BDs or CCEs at a slot across all the slots on the PCell/PSCell) – [19]

  - Study further – [2],[10],[11],[13],

- Other aspects

  - Further details on CCS configuration

    - CrossCarrierSchedulingConfig details – [2],[3],[10].[11]

  - Dynamic switching of PDCCH candidates/SS sets/SS groups between PCell/PSCell and sSCell – [13],[14],[15],[17]

  - Further details on search space handling

    - UE is not configured with complete USS in P(S)cell for self-scheduling/only parts of USS are moved – [2],[10]

    - Light SS linking – [2]

    - SS set linking between PCell/PSCell and sSCell by using same index – [4],[10]

    - Use n_CI for SS hashing – [19]

    - Independent config. of parameters related to PDCCH monitoring occasions on sSCell for scheduling PCell/PSCell and for scheduling sSCell – [19]

    - Configuration of individual DCI format(s) (e.g. only DCI format 0-1, only DCI format 1-1) – [18]

    - CIF = 0 used for sSCell or PCell? – [11],[19]

  - sSCell can schedule dynamic and CG PDSCH/PUSCH – [2]

  - Multi/single-TRP operation should remain independent on the PCell and the SCell – [6]

  - Is SCell scheduling PCell configured per UE, per BWP or per SS? – [11]

  - Reuse Rel16 preparation time between sSCell PDCCH and PCell/PSCell PDSCH/PUSCH – [19]

  - Clarify whether PCell and/or sSCell can be unlicensed cell(s) – [16]

  - Clarify allowed #UL DCI and #DL DCI combination – [16]

  - DCI size alignment clarifications – [12]

  - Impact of BWP change on sScell on Scell to Pcell scheduling – [6]

  - SCell to PCell/PSCell scheduling has no impact on PUCCH – [6],[14]

  - Clarify that one sSCell per CG can be configured (in case wording in previous agreement is not clear) – [9]

  - BFR operations/mechanism for the sSCell should be studied – [9]

## 2.1	Proposals for discussion

Below are some proposals for discussion

### Proposal 1

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

Companies are requested to indicate their view about the above proposal in the Table below

| Company Name | support/not support | Comments (Proposal 1) |
| --- | --- | --- |
| Intel | support | Type0/0A/1/2 CSS sets are necessary for UE with single cell operation on ‘PCell’. Specifically, they are used before a CA UE enters CA mode. It is possible ‘SCell’ has its own Type0/0A/1/2 CSS sets, since ‘SCell’ can be PCell for some other UEs. However, this is not related to ‘SCell scheduling PCell’ under current scope of WI. |
| Qualcomm | Support |  |
| CATT | Support |  |
| vivo | support | Type0/0A/1/2 CSS sets remains on PCell/PSCell as NR Rel-15/16 |
| Nokia | Support | This approach is aligned to the current specifications and limit the specification change needs. |
| InterDigital | Support |  |
| Samsung | Support | This is the main usage of scheduling combination (a) from RAN1#102e (“self-scheduling on PCell/PSCell is allowed”) |
| ZTE | Support | We are generally fine with the proposal. Just to make it clear, we propose to clarify that Type 0/0A/2 CSS is only allowed in PCell while Type 1 CSS is allowed in both PCell and PSCell. Thus, we propose the following. ### Proposal 1 - When CCS from an SCell (sSCell) to PCell/PSCell is configured, - UE monitors Type 0/0A/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell and not on the sSCell - UE monitors Type 1 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell |
| LG | Support | Agree with ZTE’s proposal for more clarity. |
| DOCOMO | Support |  |
| Apple | Support |  |
| ETRI | Support |  |
| Xiaomi | Support |  |
| OPPO | Support | Type0/0A/1/2 CSS sets keep on PCell/PSCell as NR Rel-15/16 |
| Huawei, HiSilicon | support | It is not reasonable to use SCell PDCCH to schedule the PCell/PSCell cell-specific information transmission, since the PCell/PSCell PDCCHs for scheduling cell-specific information transmission should still be transmitted for the legacy UEs so that the PCell/PSCell PDCCH in these CSS sets can’t be offloaded. |
| Ericsson | support | The impact on PCell operation should be reduced as much as possible when SCell to PCell scheduling is configured. |
| Lenovo, Motorola Mobility | Support |  |

Revised proposal below, based on above discussion

#### Proposal 1v2

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

  - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell and PSCell

### Proposal 2

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors DCI formats 0_0 and 1_0 for scheduling PDSCH/PUSCH on PCell/PSCell (in any associated SS set(s)) only on the PCell/PSCell and not on the sSCell

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 2) |
| --- | --- | --- |
| Intel | Support | We are considering two sources of DCI 0_0/1_0 on PCell/PSCell: - a DCI format in CSS, which has same DCI size as DCI 0_0/1_0 can be used for unicast scheduling - a USS containing only DCI 0_0/1_0 may be configured on PCell |
| Qualcomm | Not support | We do not see the need of supporting a USS set on the PCell/PSCell when the UE is configured to monitor USS set(s) on the SCell for cross-carrier scheduling from the SCell to the PCell/PSCell. Therefore, we propose the following. - When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors DCI formats 0_0 and 1_0 for scheduling PDSCH/PUSCH on PCell/PSCell (in any associated CSS set(s)) only on the PCell/PSCell and not on the sSCell |
| CATT | Not support | We support Qualcomm’s update. The motivation of supporting a USS on the scheduled P(s)Cell is far from being justified. The intention of supporting CCS from Scell to PCell is to offload the PDCCH overhead on the scheduled PCell. If a USS can be configured on the PCell for self-scheduling, there is no reason to further configure CCS from SCell to PCell. |
| vivo |  | We understand the key point of this proposal is not to allow DCI 0_0 and 1_0 from sScell to PCell/PSCell scheduling. If this is the correct understanding, we support it; otherwise we agree with QC’s update. |
| Nokia | Support | DCI formats 0_0/1_0 do not carry the CIF field, there is no need to monitor them in the scheduling SCell for cross-carrier scheduling. In other words, DCI formats 0_0/1_0 are monitored for self-scheduling in PCell in CSS and USS, and for self-scheduling in the SCell in the USS. |
| InterDigital | Support | We think that USS on PCell is needed when the SCell quality drops. |
| Samsung |  | For CSS, support (UE anyway monitors non-Type3 CSS on PCell). For USS, consider together with P4. Suggest to resolve P4 first. |
| ZTE | Support | Our understanding is that the legacy UE behaviors of DCI format 0-0 and DCI format 1-0 are the same as before. Thus, DCI format 0-0 and DCI format 1-0 in both CSS and USS should still be kept in PCell. Regarding Qualcomm’s comment above, fallback DCI on PCell/PSCell should be used to schedule PDSCH/PUSCH in case fallback scheduling is needed. Because there are many restrictions only using fallback DCI with C-RNTI in CSS (For example, only aggregation level 4/8/16 can be used for Type 0/0A/2 CSS; Type 1 CSS can be only used if the UE has not been provided a Type 3 CSS or a USS and the UE has received a C-RNTI; Type 3 CSS can be used if this optional search space is configured and only configured on PCell.) From network vendor’s perspective, we see the need to kept fallback DCIs in USS in PCell/PSCell. |
| LG | Need discussion | We think that USS associated with fallback DCI may need to be monitored on at least one of P(S)Cell and SCell, with consideration of potential PDCCH blocking/congestion in UE-common CSS resources in case of scheduling fallback DCI. |
| DOCOMO |  | We have similar view as Samsung. |
| Apple | In principle support | We think the fall back DCI is only monitored in SpCell |
| ETRI | Support |  |
| Xiaomi | Support |  |
| OPPO | Support | We share view with Nokia. |
| Huawei, HiSilicon | support | Since there is no carrier indicator field in DCI format 0_0/1_0, when CCS from an SCell to PCell/PSCell and self-scheduling on the SCell are configured, a UE specific DCI format 0_0/1_0 on the SCell cannot indicate the scheduled carrier index. |
| Ericsson | support | The impact on PCell operation should be reduced as much as possible when SCell to PCell scheduling is configured. Regarding QC suggestion “…UE monitors DCI formats 0_0 and 1_0 for scheduling PDSCH/PUSCH on PCell/PSCell in CSS set(s)) only on the PCell/PSCell and not on the sSCell”, we would like to better understand how DCI 0_0,1_0 for scheduling PDSCH/PUSCH on PCell/PSCell using USS should then be handled. i.e., a) DCI 0_0,1_0 for scheduling PDSCH/PUSCH on PCell/PSCell using USS is not supported? or b) DCI 0_0,1_0 for scheduling PDSCH/PUSCH on PCell/PSCell is monitored on sSCell USS by adding CIF for these DCI formats? or c) some other option? |
| Lenovo, Motorola Mobility | Support | To avoid additional specification impact, we prefer to keep the legacy setting to not allow cross-carrier scheduling with DCI format 0_0/1_0. |

Revised proposal below, based on above discussion

#### Proposal 2v2

- When CCS from sSCell to PCell/PSCell is configured,

  - for ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’ from CSS set(s), UE monitors them only on CSS set(s)) of PCell/PSCell and not on CSS set(s) of the sSCell

  - FFS how to handle ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’ from USS set(s), including (but not limited to) below alternatives

    - Alt1: UE monitors them only on USS set(s)) of PCell/PSCell and not on USS set(s) of the sSCell

    - Alt2: UE monitors them only on USS set(s)) of sSCell and not on USS set(s) of the PCell/PSCell

    - Alt3: UE does not monitor ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’ from USS set(s)

- Note: The SCell configured with CCS to PCell/PSCell is referred to as ‘sSCell’

| Company Name | support/not support | Comments (Proposal 2v2) |
| --- | --- | --- |
| Moderator |  | Revisions in v2 capture comments from some companies to limit the current discussion for CSS case and discuss USS handling later. |
| Intel | support | Regarding the Alt3 of FFS point, does it intend to say UE monitors DCI format 0_0/1_0 scheduling PUSCH/PDSCH in neither PCell/PSCell nor sSCell? To make a full list of all possible alternatives, we could also consider another one: - Alt4: UE does monitor ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on both PCell/PSCell and sSCell We think it is preferable to maintain legacy behavior, i.e. DCI 0_0/1_0 in CSS can be used to schedule unicast PDSCH/PUSCH. Then, since UE anyway detects DCI 0_0/1_0 on PCell, there is no/limited complexity to support DCI 0_0/1_0 on USS of PCell/PSCell. Specifically, for initial DL BWP or default DL BWP on PCell, it is quite reasonable to assume that a USS of DCI 0_0/1_0 can be configured for reliable operation. Our preference is Alt1 or Alt4. |
| ZTE |  | We are generally fine with the proposal. Regarding Alt.2, we are not sure whether it is practical or not because there is no CIF field in fallback DCI format. If proponents have solutions to address this issue, some more clarification is needed. Otherwise, we prefer to delete Alt.2 for now. |
| Apple | Generally fine | But when CCS is configured for SCell to scheduled SpCell. We should not assume we agree that CSS can be configured on sSCell or USS can be configured on SpCell. |
| Qualcomm |  | It would be good to combine this topic with the topic under proposal 6 since they are not fully orthogonal. For example, selection of Alt.1 – Alt.3 under Proposal 2v2 will impact on the “FFS impact on BD/CCE limit handling” under Proposal 6. |
| Xiaomi | Support |  |
| CATT | Fine with the proposal | We are fine with the proposal as the FFS point is general. Our preference is alt.3. |
| Samsung |  | If Alt.2 is possible for the USS, then we don’t see why this is not possible for the CSS. If Alt.2 is removed, we’re OK with the proposal. |
| LG | Support |  |
| DOCOMO | Support | We agree with Qualcomm. |
| Huawei | Support |  |
| ETRI | Support | We agree with Qualcomm. |
| InterDigital | Support |  |
| Nokia | Support | We can defer the debate on the 0_0/1_0 in USS to a later stage |

Revised proposal below, based on above discussion

#### Proposal 2v3

- When CCS from sSCell to PCell/PSCell is configured,

  - for ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’ from CSS set(s), UE monitors them only on CSS set(s)) of PCell/PSCell and not on CSS set(s) of the sSCell

  - FFS how to handle ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’ from USS set(s)

- Note: The SCell configured with CCS to PCell/PSCell is referred to as ‘sSCell’

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 2v3) |
| --- | --- | --- |
| vivo | Support |  |
| Qualcomm |  | For the 2nd sub-bullet, our preference is that the UE does not monitor these DCI formats from USS set(s). To accommodate this option, we suggest revising it to “FFS whether/how to handle …”. |
| LG | Support |  |
| ZTE | Support |  |
| Intel | Support |  |
| InterDigital | Support |  |
| CATT |  | Share the same view as Qualcomm. |
| Ericsson | Support | We prefer to not include the change suggested by QC for the FFS point |
| Samsung |  | It is preferable to handle CSS and USS together. We see no point agreeing for the CSS and having the USS as FFS. If the same statement as for the CSS can be made for USS, we support the proposal. Otherwise, we do not. |
| DOCOMO |  | We have similar view as Qualcomm. |
| ETRI | Support |  |

### Proposal 3

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set configured on Pcell/PSCell/sSell, and applicability of the information in the DCI formats is as per Rel15/Rel6 framework

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 3) |
| --- | --- | --- |
| Intel | Support | Our understanding is the support of DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 on a SCell is already supported in Rel-15. Therefore, it can be used on sSCell |
| Qualcomm | Need a clarification | It is better to clarify that a UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set configured on either of Pcell/PSCell or sSell. |
| CATT | Support |  |
| vivo | Need updates | It seems 2_5 introduced in NR Rel-16 is missed in the proposal. Besides, 2_6 is already supported in Pcell/PSCell and whether to extend to sScell can be FFS. Combined with QC’s update, we suggest the proposal updated below: - When CCS from an SCell (sSCell) to Pcell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4, 2_5 in Type 3 CSS set configured on either of Pcell/PSCell or sSell and 2_6 in Pcell/PSCell, and applicability of the information in the DCI formats is as per Rel15/Rel6 framework - FFS: whether/how to support DCI 2_6 in sScell |
| Nokia | Support | It is desirable to have at least the CSS type 3 for power control of the Pcell configurable in the SCell (DCI format 2_2/2_3) so that all the normal operations of an ongoing connection can be handled via the SCell. We see no obvious reasons why any of the DCI format, search space or Pcell related limitations would need to be revisited. |
| InterDigital | Support |  |
| Samsung | Need clarification | What additional to Rel-16 is to be agreed? Is this for DCI format 2_6? |
| ZTE | Support | Two comments, 1. It seems DCI format 2_5 can also be included in the above proposal. 2. Similar comment as Qualcomm. We prefer not to use “either …. Or” here, as it seems that these DCI formats are not precluded to be configured on both Pcell/PSCell and sSCell. ### Proposal 3 When CCS from an SCell (sSCell) to Pcell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 or 2_5 in Type 3 CSS set configured on Pcell/PSCell or/sSell, and applicability of the information in the DCI formats is as per Rel15/Rel6 framework |
| LG | Support | Also fine with vivo’s update. |
| DOCOMO | Support |  |
| Apple | Need discussion | We also have 2_5 and 2_6. Also similar concern as Qualcomm |
| ETRI | Support |  |
| Xiaomi | Support |  |
| OPPO | Support | Fine with vivo’s update |
| Huawei, HiSilicon | support | In current specification, Type3-PDCCH CSS set can be configured in both in a Pcell/PSCell and an SCell. According to the PDCCH loads of a Pcell/PSCell and an SCell, gNB can select the preferred cell to configure Type3-PDCCH CSS set and the RNTIs needs to be monitored. Thus, there is no need to change this. |
| Ericsson | support | We do not see the need to change anything from Rel15/Rel16 framework for these DCI formats when sSCell to Pcell/PSCell scheduling is configured. |
| Lenovo, Motorola Mobility | Support |  |

Revised proposal below, based on above discussion

#### Proposal 3v2

- When CCS from an SCell (sSCell) to Pcell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set configured on Pcell/PSCell/sSell [Pcell/PSCell or sSell], and applicability of the information in the DCI formats is as per Rel15/Rel6 framework

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

Revised proposal based on discussion in GTW session below (note, updated v2 to v3)

#### Proposal 3v3

- Define sScell as the Scell configured with CCS to Pcell/Pscell

- When CCS for the sScell to Pcell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set configured on Pcell/PSCell [/ vs. or] sScell, and applicability of the information in the DCI formats is as per Rel-15/Rel-16 framework

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

Suggest below updates to the version discussed in GTW (explanation of changes in moderator comments below)

#### Proposal 3v4

- When CCS from sSCell to Pcell/PSCell is configured, UE monitors DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 in Type 3 CSS set configured on Pcell/PSCell [/ vs. or] sSCell, and applicability of the information in the DCI formats is as per Rel-15/Rel-16 framework

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

- Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

Companies are requested to indicate their view about the above proposal 3v4 in the Table below

| Company Name | support/not support | Comments (Proposal 3v4) |
| --- | --- | --- |
| Moderator |  | Made some updates to version discussed in GTW session. - Moved ‘sSCell’ terminology related explanation to a Note as the term is already used in previous agreement with same meaning - Updated “When CCS for the sScell to Pcell/PSCell is configured” to “When CCS from sScell to Pcell/PSCell is configured” as the formulation ‘CCS for the sScell ..’ can be potentially confusing. |
| Intel | support | Regarding wheter DCI 2_6 can be configured on sSCell, it depends on how to enable PS/dormancy operation under DSS, which needs to be clarified first. |
| ZTE |  | It seems companies have different understandings on whether Type 3 CSS set can be configured on Pcell and SCell at the same time. Our understanding is that this is allowed by the current spec. However, if we want to avoid the Rel-15/Rel-16 spec discussion/clarification here, maybe we could try toe following proposal. Proposal: When CCS from an SCell (sSCell) to Pcell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as Rel-15/Rel-16. FFS: DCI format 2_5 and 2_6. |
| Apple |  | DCI format 2_x can be configured in SpCell. So we prefer to have sSCell either removed, or completely in bracket ([/ vs. or sSCell]) |
| Qualcomm | Support | OK with the FL proposal. |
| Xiaomi | Support |  |
| CATT |  | Support ZTE’s version. |
| Samsung |  | We understand that the proposal is already possible in Rel-16. If so, no need for an agreement and we therefore do not support the proposal. If not, what is different from Rel-16 needs to be outlined. |
| LG |  | Our understanding for this proposal is that regardless of whether CCS from sCell to P(S)Cell is configured or not, UE’s monitoring and applicability for DCI format 2_0, 2_1, 2_2, 2_3, and 2_4 are the same as Rel-15/16, but FFS for DCI format 2_5 and 2_6. If this is the case, we support the proposal 3v4. |
| DOCOMO | Support |  |
| Huawei | Support |  |
| ETRI | Support |  |
| InterDigital | Support |  |
| Nokia |  | Support ZTE version |
| Moderator |  | 3v4 seems to be OK for most companies. So, suggest we agree to it and resolve the square bracket in next meeting. Regarding ZTE proposed revision, in my view, it is perhaps better to at least attempt to clarify the expected handling one way or other. |
| vivo | Support |  |
| Ericsson | Support | We are ok with current formulation of 3v4 |
| Samsung2 |  | Support ZTE’s version |

Further updates to 3v4 below based on discussion on [103-e-NR-DSS-01] email thread

#### Proposal 3v5 (for conclusion)

- When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

- Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

### Discussion point 4

- For USS handling, below options should be further discussed

  - Option 1

    - When CCS from an SCell (sSCell) to PCell/PSCell is configured, regarding DCI formats 0_1,1_1,0_2,1_2 that schedule PDSCH/PUSCH on PCell/PSCell, UE does not monitor them on PCell/PSCell USS set(s), and monitors them only on the sSCell USS set(s)

  - Option 2

    - When CCS from an SCell (sSCell) to PCell/PSCell is configured, regarding DCI formats 0_1,1_1,0_2,1_2 that schedule PDSCH/PUSCH on PCell/PSCell, UE can monitor them on both PCell/PSCell USS set(s) and sSCell USS set(s)

Companies are requested to indicate their view about the above options in the Table below. If companies prefer to study further, please indicate FFS and the aspects to be considered.

| Company Name | Preferred option | Comments (Discussion point 4) |
| --- | --- | --- |
| Intel | Option 2 | We prefer to allow the flexibility to configure a USS on PCell/PSCell or sSCell. By this way, operator can select a most proper configuration based for a deployment. Configuration of a USS on PCell/PSCell or sSCell could be per BWP. In fact, it is possible that the NR BW and LTE BW are only partially overlapped. In this case, it is possible that, a first BWP is overlapped with LTE hence coordination is needed, while a second BWP is not overlapped with LTE. For the latter case, it is preferable to allow USS configured on PCell. Even for the overlap case, it doesn’t mean that PCell cannot transmit PDCCH. In fact, UE anyway needs to monitor DCI 0_0/1_0 and DCI 2_x on PCell. Therefore, it doesn’t introduce new behavior to monitor a USS on PCell. The choice of configuration of a USS or not on PCell can be left to implementation. |
| Qualcomm | Option 1 | So far, for a given scheduled cell, there was only one scheduling cell that is configured by higher-layer ignaling semi-statically. We do not think we need to change this principle. The reason why a UE configured with cross-carrier scheduling from an SCell to the PCell/PSCell has to monitor PDCCH on the PCell/PSCell is because of UE-common functions and fallback operation using CSS set(s). This does not mean we should enable USS on the PCell/PSCell. Allowing USS on the PCell/PSCell will cause further trouble if the subcarrier-spacings are different between PDCCH on the PCell/PSCell and PDCCH on the scheduling SCell. |
| CATT | Option 1 | We don’t see the necessity of supporting USS configuration on the scheduled P(S)Cell. The case here is similar to the current cross carrier scheduling scheme in some regard, wherein UE only monitoring USS on the scheduling cell. gNB has the freedom to configure the combination of scheduling cell and scheduled cell, and any proper search space on the scheduling cell. The flexibility is already fully respected from the current mechanism. As we commented on proposal 2, it doesn’t make sense to configure USS on the scheduled cell considering it can already been scheduled by a USS on the scheduling cell. It will cause more efforts of monitoring PDCCH on different cells in vain. |
| Vivo | Option 1 | In our view, there is no use case to monitor USS simultaneously from Pcell/Pscell and sScell. It makes the handling of DCI size budget, BD/CCE budget more complicated. |
| Nokia | Option 2 | When cross carrier scheduling from SCell to Pcell, (for formats 0_1,1_1,0_2,1_2) UE monitors USS on scheduling SCell. UE can be configured to monitor USS on PCell as well. We do not see a need to have any restriction on the PCell USS. It can be left to the implementation to configure USS on SCell and/or PCell. This also serves as a fall-back for the case when UE runs out of coverage on SCell. In situations when scheduling cell fall-back happens, gNB may switch SearchSpace group and within 2ms move the majority of USS PDCCH candidates from Scell to Pcell. R16 SS group switching feature may be used for this purpose. |
| InterDigital | Option 2 | Having USS only on SCell can create reliability issue for downlink control channels that schedule Pcell. For example, the radio link quality of SCell can drop while the Pcell radio link quality is still good. In that case, the UE cannot be scheduled on Pcell using USS and RRC reconfiguration is needed to configure USS on Pcell. |
| Samsung | Discuss further | We understand the concerns for complexity of option 2. However, if the PCell is viewed as another scheduled cell for the scheduling sSCell, Rel-16 can be directly re-used. Although the per-cell limits on BDs/CCEs may not apply (resulting total of BDs/CCEs can be larger), we don’t think that is a problem because it does not affect UE complexity. Otherwise, if the per cell limits on BDs/CCEs need to be kept, option 1 is of course simpler. But then there is no ‘D’ in DSS. At least, if option 1 is selected, self/cross-carrier scheduling should be BWP-specific, not cell-specific. |
| ZTE | Option2 | Option 1 may be simpler while Option 2 can offer higher flexibility for network implementation. Both alternatives are feasible, more studies are needed for these two alternatives. Since USS with corresponding DCI formats are configured by RRC, Option 2 supports dynamic switching of the PDCCH for PCell scheduling. When the UE is close to gNB, the scheduling PDCCH is transmitted on sSCell. When the UE moves to cell edge area, the PDCCH carrying non-fallback DCI can reside in PCell (usually low frequency and better for coverage) to guarantee the performance. While if Option 1 is supported, this switching will rely on RRC reconfiguration which is not flexible. |
| LG | Option 2 |  |
| DOCOMO | Discuss further | In the current specifications, scheduling cell for a serving cell is semi-statistically configured by schedulingCellInfo. Therefore, when PDCCH capacity of P(S)Cell becomes sufficient and NW would like to change from cross-carrier scheduling to self-carrier scheduling for the P(S)Cell, RRC reconfiguration is necessary. Option2 doesn’t have the concern. On the other hand, if option1 is selected, we may need to consider enhancement to address the above point (e.g. dynamic switching of scheduling cell for P(S)Cell). |
| Apple | Option 1 | When SCell is configured to schedule PCell, the main purpose is to schedule non-fallback DCI. Then we should maintain the same Rel-15 principle as much as possible, i.e. only one scheduling cell can schedule a scheduled cell. The reason we agree simultaneous PCell self-scheduling is due to the fall back and support of initial acquisition etc. |
| ETRI | Option 2 | Option 2 may prevent potential throughput loss when PDCCH reception from sSCell is suddenly disabled (due to e.g., link failure, dynamic activation/deactivation). |
| Xiaomi | Option 1 | In our view, there is no need to monitor USS simultaneously from Pcell/Pscell and sScell. It makes the handling of DCI size budget, BD/CCE budget more complicated. |
| OPPO | Option 2 | It is not necessary to restrict USS configuration in sSCell. Moreover, there is reliability and coverage issue for sSCell. However, to avoid exceeding BD/CCE budget, USS configuration can be configured in either PCell/PSCell or sSCell. It is left to implementation. To cover flexible configuration in implementation, we update option 2 - Option 2 - When CCS from an SCell (sSCell) to PCell/PSCell is configured, regarding DCI formats 0_1,1_1,0_2,1_2 that schedule PDSCH/PUSCH on PCell/PSCell, UE can monitor them on both PCell/PSCell USS set(s) and/or sSCell USS set(s) |
| Huawei, HiSilicon | Option 2 | It is preferred to monitor USS set(s) on the SCell and CSS set(s) on the PCell/PSCell, which means there will be anyway two carriers used for scheduling the PCell/PSCell. If the USS can be only configured in SCell in this case, there may be benefits for UE power saving; if the USS can be configured on both PCell/PSCell and SCell, more scheduling opportunity can be available at the cost of UE power saving. From network flexibility point of view, it may be beneficial to allow the latter as one configuration option, which can be turned ON/OFF based on e.g. UE capability and network configuration. |
| Ericsson | Option 2 | The impact on PCell operation should be reduced as much as possible when SCell to PCell scheduling is configured. Non-fallback DCI formats such as 0_1,1_1 are the normal mechanism of scheduling data on a cell. From NW perspective, Option 2 allows broad configuration of sSCell to PCell scheduling and the sSCell PDCCH resources can be opportunistically used to reduce NR PDCCH load on DSS carrier. With Option 1, this is not possible and NW has to upfront make a decision to limit or not the normal data scheduling for PCell/PSCell (i.e., during configuration of sSCell to PCell scheduling), and this imposes a trade-off for configuring sSCell to PCell scheduling which we do not prefer. From UE procedures perspective, since CCS with different SCS is supported in Rel16, our expectation is the Rel16 framework can be mostly reused. Also, if the total PDCCH blind decoding budget is not changed, there should not be much UE complexity impact with this. |
| Lenovo, Motorola Mobility | Option 2 (fine also with FFS) | In our view, there could be advantageous of allowing such monitoring: for instance, - DCI format x_2 for PCell scheduling can be configured to be monitored with 0 bits for carrier indicator (saving URLLC DCI bits). - in case of PCell and sSCell having different SCS, the resulting UE PDSCH reception preparation time may not be desirable for URLLC operation (e.g., DCI format(s) x_2) |

### Proposal 5

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, CA activation/deactivation of the sSCell is supported

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 5) |
| --- | --- | --- |
| Intel |  | We are fine with the proposal. However, it seems the proposal is not as basic as the other 4 proposals. It is fine to leave this issue for further study. |
| Qualcomm |  | Agree with Intel. |
| CATT |  | Share similar views with Intel. |
| Vivo |  | Agree with Intel. In addition, dormancy behavior of sSCell should also be discussed for future study. |
| Nokia | Support | We agree with Vivo. |
| InterDigital |  | We agree with Intel |
| Samsung |  | Support in principle. This also relates to P4 and having self/cross-carrier scheduling be BWP specific (e.g. when sSCell moves to dormant, PCell becomes self-carrier scheduled) – can discuss together with P4. In addition, above proposals 1 ~ 4 require corresponding updates with this proposal 5, e.g.: Proposals 1 ~ 4: “When CCS from an SCell (sSCell) to PCell/PSCell is configured activated, ….” |
| ZTE | Support | We are ok with the proposal and we are also fine to either clarify it in this meeting or in the future meeting. |
| LG |  | Agree with vivo. |
| DOCOMO |  | We agree with Intel. |
| Apple | Need discussion | First of all, it is related to whether non-fallback DCI is only monitored on sSCell which is our preferred solution. In that case, if NW uses MAC-CE to deactivate the sSCell, UE cannot receive non-fallback DCI scheduled data on PCell, i.e., UE can only be scheduled by fallback DCI. This is partly like to deactivate the PCell. |
| ETRI | Support | We support the proposal, and also agree with Intel. |
| Xiaomi |  | We agree with Intel. |
| OPPO | Support | We agree with Intel and vivo |
| Huawei, HiSilicon | support | When USS set(s) are configured in PCell/PSCell and SCell, CA activation/deactivation of the SCell can be supported obviously. When USS is configured only on the SCell and not on the PCell/PSCell, fallback mechanism can be considered. |
| Ericsson | support | CA activation/deactivation should be applicable for sSCell as well. |
| Lenovo, Motorola Mobility | FFS | We prefer to leave this issue FFS for now. |

Based on inputs for discussion point 4 and Proposal 5, following proposal can be considered

### Proposal 6

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

- FFS impact of sSCell activation/deactivation and sSCell dormancy

- FFS impact on BD/CCE limit handling

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 6) |
| --- | --- | --- |
| Moderator |  | Intention of above proposal 6 is mainly to set up some framework for discussion in next meeting and not to initiate further discussion between Alt x vs. Alt y. So, requesting comments on clarification of above alternatives etc. with the goal of enabling ‘full proposals’ to be submitted by proponents for next meeting. |
| Intel | Support | OK for down selection at next meeting. Clarification on Alt 2-2 & 2-3, ‘simultaneously’ means CORESETs on PCell/PSCell and sSCell are overlapped with at least one symbol with high SCS between PCell/PSCell and sSCell, right? |
| ZTE |  | We are generally fine with the proposal. Since the configuration/activation/deactivation of sSCell scheduling PCell may have impact on these alternatives, we would like to add an FFS for this proposal (for all the alternatives). With the following FFS, it seems the FFS point in Alt.2-2 can be deleted. FFS configuration/activation/deactivation of sSCell scheduling PCell. |
| Apple |  | The first thing we need to discuss is whether USS can be configured on both the SpCell and sSCell. |
| Qualcomm |  | (1) As commented to Proposal 2v2, the proposal here should be jointly discussed with handling of DCI 0_0/1_0 on USS set. (2) We think it is important to clarify that the following basic principles are kept unchanged (regardless of which alternative we will pick-up). If any alternative aims at changing any of them, it should be clarified as part of the alternative. - PDCCH overbooking monitored on SCell is not supported - When the PCell/PSCell and the scheduling SCell use different numerologies, the PDSCH reception preparation time (in TS38.214 Section 5.5) between the PDCCH on the scheduling SCell and the PDSCH on the PCell/PSCell is ensured - OOO scheduling/HARQ and time-overlapping PDSCHs or PUSCHs on the PCell/PSCell are not allowed (3) There could be additional issues due to different numerologies between the PDCCH on the PCell/PSCell and on the sSCell. It would be good to add one FFS “FFS impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell”. |
| Xiaomi | Support |  |
| CATT |  | We share the same views with Apple. |
| Samsung | Not support | For Alt.1 and Alt.2, there is no need to have CCS configuration as a condition, i.e. the “When CCS from sSCell to PCell/PSCell is configured …” is not needed. There is no motivation/reason for Alt. 2.3 and it seems it has not been proposed by anyone. |
| LG | Support, in general | Our understanding of Alt 2 is that {P(S)Cell, sSCell, both P(S)Cell and sSCell} can be configured to monitor non-fallback DCI formats. In this sense, following editorial change is suggested: - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded) We prefer to remove Alt 2-1, 2-2, and 2-3 and to discuss further details of Alt 1 and 2 next meeting. |
| DOCOMO |  | We have similar view as Apple. |
| Huawei |  | Support Alt1 and Alt2-1 |
| ETRI | Support | Also fine with LG’s update. |
| InterDigital | Support |  |
| Nokia |  | Tend to agree with Samsung that the CSS set configuration is not a necessary condition when outlining the options Agree with the editorial change suggested by LG. The UE can be configured to monitor USS on Pcell/SCell and/or sSCell, when CCS from sSCell. We support Alt 2-2 |

Based on above discussion, revised proposal 6v2 below

#### Proposal 6v2

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’

  - Whether PDCCH overbooking on sSCell is supported or not supported

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

| Company Name | support/not support | Comments (Proposal 6v2) |
| --- | --- | --- |
| Moderator |  | Please check version 6v2 with some updates to reflect the comments received so far. Lingyang – I think we can consider ‘simultaneously’ = ‘at the same time’ for the time being. Fred – on (1), modified the FFS point on BD/CCE handling to hopefully reflect your comment on fallback DCI formats. On (2), added the overbooking part as FFS. The other parts, i.e., preparation time, out of order scheduling etc. are more generic than just the search space discussion. So, added them as separate proposals 7 and 8. If OK with everyone we can conclude in this meeting. On (3), added the FFS. Haitong, Wanglei, Masa -- whether USS can be configured on both the SpCell and sSCell or not is covered by Alt 1 vs. Alt 2 discussion point. Youngbum – removed “When CCS from sSCell to PCell/PSCell is configured …” form Alt 1 and Alt 2 per your comments. I think Alt 2-3 was discussed in [16]. Seonwook, Karri – I added ‘/or’ for Alt 2. |
| vivo |  | For Alt. 2-2, suggest to remove the “e.g. “ part in FFS. First, SS group switching is a feature only applied in unlicensed band currently. Second, it is not clear how to reuse the BWP switching within a carrier for USS switching between carriers. |
| Qualcomm |  | Looks good overall. One minor comment on PDCCH overbooking under the 3rd main bullet: we prefer to generalize it, i.e., delete “on sSCell” as follows. Whether PDCCH overbooking on sSCell is supported or not supported For Alt. 2-2, agree with vivo. In addition, “dynamic” should be deleted for the time being. |
| LG |  | Support Alt 2 and Alt 2-1. |
| ZTE |  | We support Alt.2 and Alt.2-1. From our perspective, for Alt.2-1, the function of SCell scheduling PCell can still be activated/deactivated. We would propose to add an FFS for Alt.2-1. - Alt 2-1: - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - FFS: activation/deactivation of SCell scheduling PCell |
| Intel |  | We support the FL proposal. It sounds impossible to do down selection in this meeting. Anyway, our preference is alt 2-1 which is a super set of Alt 2-2. In case Alt 2-1 is not agreeable, Alt 2-2 is definitely beneficial. We already commented to discussion point 4. A DL BWP may be configured with USS due to 1) the BWP doesn’t overlap with LTE (i.e. NR cell and LTE cell are partially overlap in frequency); 2) if the DL BWP is initial BWP or default BWP, it may be preferred to use self-scheduling. |
| InterDigital |  | We prefer Alt. 2-2 and Alt. 2-1 |
| CATT |  | We are generally fine with the updated proposal. For alt 2-1, 2-2, 2-3, we think it is better to leave it open as the benefits is not clear. We think the sub-bullet of alt 2 is sufficient. For the FFS part, we are not clear about the last bullet. Is it about BD/CCE calculation or something else? It’s better to provide some examples in order to better understands the issue. |
| Ericsson |  | It would be good to clarify the last bullet as CATT also suggested -- “Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell” |
| Samsung | Support |  |
| DOCOMO |  | We are generally fine with the FL proposal. For Alt 2-2, we are OK with vivo’ suggestion. We can discuss switching mechanism in next meeting. |
| ETRI | Support | Our preference is Alt. 2-1. We also agree with vivo’s suggestion. |

Further updates to 6v2 below based on discussion on [103-e-NR-DSS-01] email thread

#### Proposal 6v3

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

        - FFS activation/deactivation of scheduling from sSCell to PCell/PSCell

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

Further updates to 6v3 below based on discussion on [103-e-NR-DSS-01] email thread

#### Proposal 6v4

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

        - FFS activation/deactivation of scheduling from sSCell to PCell/PSCell

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

    - Alt 2-4:

      - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell

  - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a configured USS set in PCell/PSCell))

Further updates to 6v4 below based on discussion on [103-e-NR-DSS-01] email thread

#### Proposal 6v5

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded)

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

        - FFS activation/deactivation of scheduling from sSCell to PCell/PSCell

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

    - Alt 2-4:

      - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell

  - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a configured USS set in PCell/PSCell))

### Proposal 7 (for conclusion)

- When the PCell/PSCell and sSCell use different numerologies, the PDSCH reception preparation time between the PDCCH on the sSCell and the PDSCH on the PCell/PSCell is applied (i.e., as specified in TS38.214 Section 5.5).

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 7) |
| --- | --- | --- |
| Moderator |  | My understanding is this already covered by current Rel16 framework, hence conclusion should be ok. |
| vivo | Support |  |
| Qualcomm | Support |  |
| LG | Support |  |
| ZTE | Support |  |
| Intel | Support |  |
| InterDigital | Support |  |
| CATT |  | Although we are fine with the proposal, we are not clear about the point here considering it can already be covered by the current specification. |
| Ericsson | Support | Our view also is that this is already covered by existing specification. |
| Samsung |  | Agree with CATT – no need for an agreement |
| ETRI | Support | Seems already clear in the current specification. |

### Proposal 8

- When CCS from sSCell to PCell/PSCell is configured,

  - Out of order scheduling between a) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on sSCell is not allowed

  - Simultaneous reception of a) unicast PDSCH on PCell/PSCell scheduled from PCell/PSCell and b) unicast PDSCH on PCell/PSCell scheduled from sSCell is not allowed

  - Simultaneous transmission of a) PUSCH on PCell/PSCell scheduled from PCell/PSCell and b) PUSCH on PCell/PSCell scheduled from sSCell is not allowed

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 8) |
| --- | --- | --- |
| Moderator |  | This based on QC comments for proposal 6 and also discussion in [1],[18],[19] |
| vivo |  | Fine with the 1st bullet. For the 2nd and 3rd, it should be clarified what is simultaneous reception/transmission. It means partial or full overlapping PDSCH/PUSCH in time domain? Is this the correct understanding for the proposal? |
| Qualcomm |  | Looks good. We would like to add one FFS point regarding different numerologies between PDCCH on the PCell/PSCell and that on the sSCell, same as for Proposal 6v2. This is mainly for the 1st sub-bullet (out-of-order). - FFS: Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell |
| ZTE |  | Similar comment as vivo. Clarification of “simultaneous” is needed. |
| Intel |  | We support FL proposal. Adding FFS proposed by QC is also OK |
| InterDigital |  | We are fine with QC suggestion. |
| CATT | Support |  |
| Ericsson | Support | QC proposed FFS is not clear to us as also commented on similar bullet for 6v2. So, more clarification would be needed for it. |
| Samsung |  | For the 1st sub-bullet, need clarification whether or not the following is supported: - Initial transmission of PDSCH/PUSCH on P(S)Cell is scheduled by PDCCH on P(S)Cell and retransmission of the PDSCH/PUSCH on the P(S)Cell is scheduled by PDCCH on sSCell, and vice versa |
| ETRI | Support |  |

Further updates to Proposal 8 below based on discussion on [103-e-NR-DSS-01] email thread

#### Proposal 8v2

- When CCS from sSCell to PCell/PSCell is configured,

  - Out of order scheduling between a) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on sSCell is not allowed

    - FFS: Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Simultaneous reception of a) unicast PDSCH on PCell/PSCell scheduled from PCell/PSCell and b) unicast PDSCH on PCell/PSCell scheduled from sSCell is not allowed

  - Simultaneous transmission of a) PUSCH on PCell/PSCell scheduled from PCell/PSCell and b) PUSCH on PCell/PSCell scheduled from sSCell is not allowed

Further updates to Proposal 8v2 below based on discussion on [103-e-NR-DSS-01] email thread

#### Proposal 8v3

- When CCS from sSCell to PCell/PSCell is configured,

  - Out of order scheduling between a) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on sSCell is not allowed

    - FFS: Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Simultaneous reception of a) unicast PDSCH on PCell/PSCell scheduled from PCell/PSCell and b) unicast PDSCH on PCell/PSCell scheduled from sSCell is not allowed

  - Simultaneous transmission of a) PUSCH on PCell/PSCell scheduled from PCell/PSCell and b) PUSCH on PCell/PSCell scheduled from sSCell is not allowed

  - Note: Simultaneous implies full/partial time overlapping

Further updates to Proposal 8v3 below based on discussion on [103-e-NR-DSS-01] email thread

#### Proposal 8v4

- When CCS from sSCell to PCell/PSCell is configured,

  - Out of order scheduling between a) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH/PUSCH on PCell/PSCell scheduled by PDCCH on sSCell is not allowed

    - FFS: Whether/how to handle impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Simultaneous reception of a) unicast PDSCH on PCell/PSCell scheduled from PCell/PSCell and b) unicast PDSCH on PCell/PSCell scheduled from sSCell is not allowed

  - Simultaneous transmission of a) PUSCH on PCell/PSCell scheduled from PCell/PSCell and b) PUSCH on PCell/PSCell scheduled from sSCell is not allowed

  - Note: Simultaneous implies full/partial time overlapping

# 3 Conclusions

Following conclusions and agreements were made in RAN1#103-e based on discussions in [103-e-NR-DSS-01] email thread

Based on discussion for Proposal 1, following was agreed

Agreements:

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

  - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell/PSCell

Based on discussion for Proposal 3, following conclusion was made

Conclusion

- When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

- Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

Based on discussion for Proposal 6, following was agreed

Agreements:

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded)

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

        - FFS activation/deactivation of scheduling from sSCell to PCell/PSCell

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

    - Alt 2-4:

      - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell

  - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a configured USS set configured for scheduling of in PCell/PSCell)

Based on discussion for Proposal 7, following conclusion was made

Conclusion

- When the PCell/PSCell and sSCell use different numerologies, the PDSCH reception preparation time between the PDCCH on the sSCell and the PDSCH on the PCell/PSCell is applied (i.e., as specified in TS38.214 Section 5.5).

# 4 References

- R1-2007579	Discussion on SCell PDCCH scheduling P(S)Cell PDSCH or PUSCH	Huawei, HiSilicon

- R1-2007695	Discussion on Scell scheduling Pcell	vivo

- R1-2007839	Disucssion on cross-carrier scheduling from Scell to Pcell	CATT

- R1-2008062	Discussion on cross-carrier scheduling from SCell to Pcell	LG Electronics

- R1-2008110	Discussion on cross-carrier scheduling from SCell to Pcell	Spreadtrum Communications

- R1-2008195	Cross carrier scheduling from SCell to Pcell	Samsung

- R1-2008284	Discussion on cross-carrier scheduling from Scell to Pcell	OPPO

- R1-2008451	Views on Rel-17 DSS SCell scheduling PCell	Apple

- R1-2008695	Discussion on cross-carrier scheduling from SCell to PCell	ASUSTeK

- R1-2008830	Discussion on Cross-Carrier Scheduling from SCell to PCell	ZTE

- R1-2009003	On SCell scheduling PCell transmissions	Intel Corporation

- R1-2009023	Cross-carrier scheduling from SCell to Pcell	ETRI

- R1-2009040	Discussion on Cross-carrier scheduling from SCell to Pcell	Xiaomi

- R1-2009046	Cross-carrier scheduling from SCell to Pcell	Nokia, Nokia Shanghai Bell

- R1-2009085	Search space monitoring to support SCell scheduling PCell 	InterDigital, Inc.

- R1-2009110	Cross-carrier scheduling (from Scell to Pcell)	Lenovo, Motorola Mobility

- R1-2009195	Discussion on cross-carrier scheduling enhancements for NR DSS	NTT DOCOMO, INC.

- R1-2009206	Enhanced cross-carrier scheduling for DSS	Ericsson

- R1-2009277	Views on cross-carrier scheduling from an SCell to the PCell/PSCell	Qualcomm Incorporated

# 5 Annex A – Agreements from previous meetings

## Agreements from RAN1#102-e

Agreements:

- Following scheduling combinations are allowed/not allowed when cross-carrier scheduling from an SCell to PCell/PSCell is configured

  - self-scheduling on PCell/PSCell is allowed

  - cross-carrier scheduling from PCell/PSCell to another SCell is not allowed

  - self-scheduling on the ‘SCell used for scheduling PCell/PSCell’ is allowed

  - cross-carrier scheduling from the ‘SCell used for scheduling PCell/PSCell’ to another serving cell is allowed

  - cross-carrier scheduling from another serving cell to the ‘SCell used for scheduling PCell/PSCell’ is not allowed

- FFS: Search space and DCI format handling for the allowed cases above

Agreements:

- Configuring 2 or more Scells to schedule the PCell/PSCell is not allowed