3GPP TSG-RAN WG1 #103-e		R1-2009810

e-Meeting, Oct 26- Nov 13, 2020

Source: 	Moderator (Ericsson)

Title:      	Summary of Email discussion [103-e-NR-MRDC-CA-01]

Agenda item:	7.2.10

Document for:	Discussion and Decision

# 1 Introduction

This document provides summary of email discussion [103-e-NR-MRDC-CA-01] on following issues discussed during preparation phase of RAN1#103-eMeeting

[103-e-NR-MRDC-CA-01] Email discussion/approval on Dormancy and Unaligned CA on the following until 10/29 with potential CRs by 11/5 – Ravi (Ericsson)

- Dormancy Topic 1: Corrections to 38.213 related to SCell dormancy indication in DCI format 2_6

- Dormancy Topic 3: Corrections to 38.213 related to Case 2 dormancy indication

- Dormancy Topic 4: Corrections to 38.213 related to switching time during dormant/non-dormant BWP transition

- Dormancy Topic 5: Corrections to 38.212

- Unaligned CA Issue 1: whether the single offset duration limitation is applied to per CG or across CGs in case of NR-DC

- Unaligned CA Issue 2: Determination of Sync/Async NR-DC in the context of unaligned frame boundary CA

- Unaligned CA Issue 3: update of spec. TS 38.214 section 5.2.1.5.1 on aperiodic CSI-RS timing when the triggering PDCCH and the CSI-RS have the same numerology, to align with the agreement.

# 2. Discussion

### 2.1 Dormancy Topic 1

Please provide your input to below questions Q1-Q3 on this topic, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to TP1 from R1-2007578?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 1, Q1) |
| --- | --- | --- |
| OPPO | No | The behavior seems be sufficient by text in [10, TS 38.133], which state no receiving and transmission during the switching delay. |
| ZTE | No | It seems this is more like something nice to have. We prefer not to have this TP in Rel-16. Without this enhancement, if the BWP switching delay is longer than the gap between DCI 2-6 and the start of DRX on, UE may finish the BWP switching after the start of the DRX on. However, this is not an issue at all from our perspective. |
| LG | No | Same reason as above companies. |
| CATT | No | This issue has been discussed for a few times in Rel-16 UE power saving and got rejected every time. UE could monitor PDCCH of SCell after completion of BWP switching and does not need to monitor SCell at the beginning of the DRX ON. |
| Nokia, NSB | No | The reason is that current agreements are clear on linkage of WUS and Dormancy indication in DCP “The value of minimum time gap is decoupled with SCell dormancy indication.” |
| Intel | No | Share the views from OPPO and ZTE. It can be handled by implementation |
| MTK | Yes | We are OK to agree to TP1. |
| Qualcomm | No | The CR seems to mandate that dormant BWP switch should occur right at the start of the On duration. There is no need to have such a restriction. Ignoring the dormancy indication will even worsen the dormant BWP switching delay. |
| Huawei, HiSilicon | Y | It is not mandating where the BWP switch should occur, rather to leave more flexibility. |
| Ericsson | No | The value of minimum time gap is decoupled with SCell dormancy indication and it should remain as such – we prefer to not add more scheduling restrictions on transmission of WUS. |
| Samsung | No | Similar view with other companies. This is not essential. |
| vivo | No | We think the UE behavior is clear and expect gNB to configure properly. |

#### Question 2

Q2. Is it OK to agree to the TP in Proposal 4 of R1-2007737?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 1, Q2) |
| --- | --- | --- |
| OPPO | Yes | We are OK for that, we also propose the change in a set of changes, which the major part is not agreed. But the correction is still acceptable. |
| ZTE | Yes | This change was firstly approved in previous meetings. However, seems that it was missing somehow. |
| LG | Yes |  |
| CATT | Yes |  |
| Nokia, NSB | Yes | This is aligned with active time indication of Dormancy |
| Intel | No |  |
| MTK | Yes | Same view as Nokia |
| Qualcomm | No | This is a minor issue. No need to change spec given that the spec already mentioned “active DL BWP”. |
| Huawei, HiSilicon | Y | ok with the TP |
| Ericsson | Yes, with an update… | Suggested update: “the UE sets the active DL BWP to the indicated active DL BWP if UE detects a DCI format 2_6 that includes a SCell dormancy indication field,” |
| NEC | Yes | We are OK as it is. |
| Samsung | No | We think specification clear enough. It is already mentioned that ‘1’ value indicates ‘active DL BWP’. |
| vivo | Yes |  |

#### Question 3

Q3. Is it OK to agree to the draft CR in R1-2008113?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 1, Q3) |
| --- | --- | --- |
| OPPO | Yes | Can be agreed with Q2 |
| ZTE | Yes | The second change is the same as that in Question2. |
| LG | Yes |  |
| CATT | Yes and no | Only the 2nd change “the UE sets the active DL BWP to the indicated active DL BWP” is needed, which is same as that in Question 2 |
| Nokia, NSB | No | The first change is not needed, and second change is handled in Q2 |
| Intel | Yes |  |
| MTK | Yes | We see no harm to add the clarification. |
| Qualcomm | No | This is a minor issue. |
| Huawei, HiSilicon | Y | Can be combined with the x7737 above. |
| Ericsson | See comment | First change is not needed. Second change is handled in Q2. |
| NEC | Y | We are OK not to have the first change. |
| Samsung | No | We think specification clear enough. |
| vivo | No | Agree with the 2nd change, which is the same as in Q2. The 1st change is not essential. |

### 2.2 Dormancy Topic 3

Please provide your input to below questions Q1-Q3 on this topic, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to draft CR2 of R1-2007737?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 3, Q1) |
| --- | --- | --- |
| OPPO | Yes | Ok to clarify. |
| ZTE | Yes | We are supportive of this CR. Without this CR, the UE behavior for HARQ-ACK for case 2 dormancy indication is not complete. |
| LG | Yes |  |
| CATT | Yes |  |
| Nokia, NSB | Partially | The CASE 2 DCI is called by Aris “UE considers the DCI format 1_1 as indicating SCell dormancy” Therefore, “without scheduling a PDSCH reception” is not necessary, and increases the length of specification. Otherwise it is OK to “indicating SCell dormancy together“ to places discussing SPS PDSCH release in sub-clause 9 |
| Intel | Yes |  |
| MTK | Yes | We see no harm to add the clarification. |
| Qualcomm | Yes | We are fine with the CR. |
| Huawei, HiSilicon | N | Change#18 may not be needed. For others, if they are all agreeable, then a better way may be to add a header description somewhere that PDCCH indicating SCell dormancy without scheduling PDSCH is similar to PDCCH indicating SPS release. |
| Ericsson | Yes | OK to add “without scheduling a PDSCH reception” as it is clearer. |
| Samsung | Partially | We do not think a redundant lengthy wording “without scheduling a PDSCH reception” is needed. |
| vivo | Yes |  |

#### Question 2

Q2. Is it OK to agree to draft CR3 of R1-2007737?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 3, Q2) |
| --- | --- | --- |
| OPPO | No | The feedback should not be taken into priority procedure. No need to include. |
| ZTE | Yes | Currently, DCI format 1-1 is allowed to be configured with priority indicator field and dormancy indication at the same time. It is not clear whether the HARQ-ACK for DCI format 1-1 indicating SCell dormancy without scheduling PDSCH should be put in the first HARQ-ACK codebook or the second HARQ-ACK codebook. Basically, there are following three alternatives to address this issue. No matter which way is adopted, a conclusion/CR is needed. Alt.1. Based on the priority indicator; Alt.2. Low/High priority by default. Alt.3. UE is not expected to be configured with priority indicator for the SCell dormancy indication. Our preference is Alt.1. Thus, we are supportive of this CR. |
| LG | Want to see more discussion | Not sure if the HARQ priority should be handled for dormancy indication |
| CATT | Yes | We think it is a valid issue and the proposal seems to be the most straightforward solution. |
| Nokia, NSB | Yes, but | If we allow this for Dormancy Case 2 DCI, same should apply to DL SPS release or TYPE-CB trigger which does not scheduled PDSCH. We suggest the following TP a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority |
| Intel | Want to see more discussion | The real issue is not HARQ-ACK feedback, but is how to handle priority for dormancy indication, if HARQ priority & URLLC can be configured simultaneously. As ZTE commented, there are multiple alternatives, we need to decide on an alternative first. |
| MTK | Yes | We see the change reasonable. Also ok with Nokia’s suggestion. |
| Qualcomm | No | We do not see a need for this CR. Otherwise, we may also need to consider whether a similar CR is needed for SPS release PDCCH. |
| Huawei, HiSilicon | Maybe | It might require some further discussion whether the features specified for URLLC (not concerning power consumption) will be useful for dormancy indication as well. |
| Ericsson | May be | The corresponding behavior for SPS release needs to be clarified before further discussing this CR. The coversheet of the draft CR says “Based on the current TS38.213, HARQ-ACK priority determination for the corresponding PDSCH reception and the corresponding SPS release has been specified”. However, this does not seem to be common understanding given comments so far. |
| Samsung | FFS | We think further discussion is needed. We need to first discuss whether or not to apply the prioritization for the HARQ-ACK feedback for SCell dormancy indication. |
| vivo | Maybe | We agree this is a valid issue but not sure if the proposed solution is the best way. |

#### Question 3

Q3. Is it OK to agree to draft CR in R1-2008566?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 3, Q3) |
| --- | --- | --- |
| OPPO | NO | The change is not needed. Base station should not indicate the SCell dormancy if it not configure the dormant BWP at all. |
| ZTE |  | We don’t have a strong view on this. We believe that network won’t transmit SCell dormancy indication if no SCell is configured with dormant BWP. From this perspective, it seems this CR is not needed. But if companies want to put it in the specification, we won’t object to do so. |
| LG | No | Agree with OPPO |
| CATT | No | This is not an essential correction since all SCells supporting SCell dormancy should be configured with dormant BWP. |
| Nokia, NSB | No | This is non-essential; it can avoid some misdetections, when UE falsely detects “all zero“ RA, but if no dormancy BWP is configured, then UE would treat that DCI as inconsistent. |
| Intel | No | Section 10.3 is for dormancy operation, so it is already clear |
| MTK | Yes | We see no harm to add the clarification. |
| Qualcomm | No | There seems no need to clarify the error case. This spec text is also related to NR-U oneshot HARQ-ACK. If both cases are considered, the spec text will become very complicated. In the meanwhile, usage of the DCI fields is already clear. |
| Huawei, HiSilicon | Perhaps N, or | For the section above the part being changed, there is the following If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, It would be good to use a similar structure/text for the part that proposed to be changed, e.g. If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1 and DCI format 1_1 includes a SCell dormancy indication field, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if |
| Ericsson | Yes | We prefer to have the CR. The CR avoids the unnecessary change of interpretation of FDRA bit sequence between Rel15 UEs and Rel16 UEs without dormant BWP. Also, without the change, the spec seems to allow use of DCI 1_1 to transition UE to dormant BWP inside active time even when the UE is configured only with firstOutsideActiveTimeBWP-Id. HW proposed change would not be clear since earlier text is for case 1 dormancy indication and it cannot be directly used in this part. |
| Samsung | No | This is not essential. |

### 2.5 Dormancy Topic 4

Please provide your input to below questions Q1 and Q2 on this topic, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to the TP in section 2 of R1-2008145?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 4, Q1) |
| --- | --- | --- |
| OPPO | No | For power saving purpose, cancel the last transmission would be beneficial to UE. But, no need to clarify, could left for implementation. |
| ZTE | No | The same issue happens in Rel-15. For example, if DCI based BWP switching happens, Rel-15 doesn’t specify anything related to the UL cancellation. From this perspective, it seems not necessary to specify UL cancellation for Rel-16 dormant BWP change. |
| LG | No | Can be left for implementation |
| CATT | No. | We understand the concern of on-going UL process during BWP switching. However, this is not for SCell dormancy only. |
| Nokia, NSB | No | We are not fully sure this CR is really needed. - PUCCH cannot be configured on a Scell which is configured with Dormant DL BWP – so no PUCCH - If UE transmits still SRS on Scell short time after dormancy trigger DCI, not big deal - gNB should not schedule CFRA PRACH or PUSCH on Scell if wants to send Scell to dormancy, but if it did, what is the value-add to say that the UE is not expected to cancel the scheduled transmission? It does not seem to require the UE to transmit either, so that would not seem to be able to rely on the presence or the absence of the PUSCH. Due to this the gNB should avoid scheduling a PUSCH, given that there maybe a chance that PUSCH ( that may or may not get transmitted ) would carry UCI. CG PUSCH is not in gNB power and piggybacking could cause trouble here as discussed in 3). Maybe scope could be reduced to CG-PUSCH. |
| Intel | No | Up to implementation as Rel-15 BWP switching |
| MTK | No | The argument is valid but not limited to dormant BWP. May need to modify R15 spec also if companies see the necessity. |
| Qualcomm | No | This can be considered as UE implementation. Network should expect that the UL transmission overlapping with BWP switch is corrupted. |
| Ericsson | No | Since UE is not required to transmit or receive during the BWP switching, we do not see a need for this. |
| Samsung | Yes | We think this is needed. We agree with other companies comments that this is a general issue occurring even for Rel-15 BWP switching. Actually, we think this is a missing point for Rel-15 BWP design. Unlike Rel-15 BWP switching, since the SCell dormancy switching can be happen in any symbols in a slot without restriction, the situation is more problematic. Having this CR is good for both UE and network to handle the unexpected interference from residual UL transmission not being cancelled. |
| vivo | No | We agree with previous comments that this is the same case as in Rel-15 UL BWP switching. |

#### Question 2

Q2. Is it OK to agree to the TP corresponding to Proposal 1 in R1-2008203?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 4, Q2) |
| --- | --- | --- |
| OPPO | NO | Usually, the delay defined in 133 is clear. |
| ZTE |  | We don’t have a strong view for this. Either way is fine. |
| LG | No strong view |  |
| CATT | No | Current text is clear enough without additional text. |
| Nokia, NSB | Yes |  |
| Intel | No | The operation as clarify by the TP is the default. It should be clear even without the TP. |
| MTK |  | We have no strong view in this. |
| Qualcomm | No | The duration should be defined and specified in TS 38.133. |
| Huawei, HiSilicon | Maybe N | We think the current specification is fine. |
| Ericsson |  | We agree that delay duration (already specified in 38.133) and reference point for start of the delay should be both clear. Perhaps this is also handled in RAN4 spec? |
| Samsung | No | The duration is defined in TS 38.133. |
| vivo | No | We agree with previous comments that definition in 38.133 seems clear. |

### 2.5 Dormancy Topic 5

Please provide your input to question Q1 below, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to draft CR1 of R1-2007737?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 5, Q1) |
| --- | --- | --- |
| OPPO | No | The first and last can means the configured order. |
| ZTE | Yes | The issue arises when discontinuous group index for SCell dormancy is configured by network. Basically, two approaches can be considered. Approach#1: Specify in RAN2 that only continuous group index for SCell dormancy is allowed. This may need LS exchange between RAN1 and RAN2. Approach#2: The SCell dormancy indication field corresponds to the ascending order of the group index. This is a pure RAN1 solution and is in line with what we did for BWP indication (as shown below). - Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL BWPs configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as bits, where - if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id; - otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;. We prefer approach#2 and thus we are supportive of this CR. |
| LG | Yes | Clarification seems necessary |
| CATT | Yes | It can simply add DomancyGroupID-r16 “MSB to LSB of the bitmap corresponding to the first to last configured SCell group DormancyGroupID-r16” |
| Nokia, NSB | No | We prefer that RAN2 maintains indices of Dormancy Scell-groups in contiguous manner, similarly as for BWPs of a cell. Send LS to RAN2 and RAN2 fix is simple. |
| Intel | Yes | The TP has the least overhead |
| MTK | Yes | Approach#2 by ZTE seems valid. |
| Qualcomm | No | It is common understanding that “first to last” corresponds to lowest ID to highest ID. |
| Huawei, HiSilicon | Ok |  |
| Ericsson | Yes | It is good to align the common understanding among companies, and clarify in RAN1 spec. |
| Samsung | Yes | We are supportive of this CR. |
| vivo | No | We agree with OPPO and Qualcomm. |

### 2.6 Unaligned CA Issue 1

Please provide your input to question Q1 below, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to below proposal (based on FL proposal 1 in [15])?

- Clarify the below RAN1#99 agreement that at most single non-zero offset duration can be configured within a CG in case of DC in the unaligned CA configuration.

Agreements:

At most single non-zero offset duration (independent on SCS) can be configured among CCs in the unaligned CA configuration.

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Unaligned CA Issue 1, Q1) |
| --- | --- | --- |
| ZTE | Yes | For Asynchronous NR-DC, the baseband and RF processing are almost independent in each CG. Thus, it makes sense to apply one single slot offset per CG. For Synchronous NR-DC, all Rel-16 UEs are required to support non-SFN-sync NR-DC. From our perspective, non-SFN-sync NR-DC is similar to have different slot offset between MCG and SCG. Thus, it seems UE is possible to support one slot offset per CG. |
| LG | Fine with FL proposal | We are fine with either single offset per CG or across CGs |
| CATT | Yes |  |
| Nokia, NSB | Yes |  |
| Intel | Yes |  |
| MTK | No | We prefer single offset across CGs to stick to the original RAN1 agreement. |
| Qualcomm | No | The agreement was clearly one non-zero offset per UE, not per CG. The agreement said “in the unaligned CA configuration”. But all the agreements were equally for CA and DC. Note that none of the agreements mentioned DC at all, even though the WI was for both CA and DC. Therefore, RAN1 should either endorse that all CA agreements equally apply to DC or declare that the feature is not applicable to DC. What RAN1 should not do, which we would find objectionable, is to start making new sets of agreements, like the FL proposal. There can be one non-zero offset, and if it exists, it is between the CGs. The same goes for PUCCH groups. One non-zero offset per UE applies. |
| Huawei, HiSilicon | Y |  |
| Ericsson | Yes | As we mentioned in our contribution, we are OK with one slot offset per UE if the slot offset is defined to be within a cell group. |
| CMCC | Yes | This thread is moderated by Ravi, so I can present the view on behalf of CMCC now. Recall the procedure of the unaligned CA discussion, we all know the reason that we make such kind of agreement is focusing on accommodating up to 2-band CA, which of course make it applicable to per CG. From this point of view, we don’t think it a new agreement. However, we understand the concern from the opponents, we are ok to introduce different UE capability for with DC and without DC. |
| Samsung | Yes |  |
| vivo | No | From UE implementation perspective, introducing up to one non-zero CA offset across all the cell groups has a comparable complexity level with asynchronous NR-DC supported in Rel-16. On the other hand, supporting more than one non-zero CA offsets for a UE, e.g., different non-zero offsets between cell groups in asynchronous NR-DC, would significantly increase the UE implementation complexity. Thus, we prefer at most one non-zero CA offset across cell groups can be configured for a UE supporting capability 18-7 (i.e., “CA with non-aligned frame boundaries”). If more than one non-zero offsets are required, a separate UE capability can be discussed. |

### 2.6 Unaligned CA Issue 2

Please provide your input to question Q1 below, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to below proposal (based on FL proposal 2 in [15])?

- Synchronous or asynchronous NR-DC is determined according the PCell and PSCell.

- LS can be sent to RAN4 to confirm this.

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Unaligned CA Issue 2, Q1) |
| --- | --- | --- |
| ZTE | Yes | We are supportive of the above proposal. Since the synchronous and asynchronous NR-DC requirements are specified before the introduction of unaligned frame boundary, it is preferred to determine synchronous or asynchronous NR-DC before applying the slot offset in MCG and SCG. From our perspective, determining synchronous or asynchronous NR-DC before applying the slot offset in MCG and SCG is the same as determining according to the PCell and PSCell. Thus, we support the above proposal. To make it clear, maybe we can fine-tune the proposal as below. - If unaligned frame boundary CA is configured in either CG, Synchronous or asynchronous NR-DC is determined according the PCell and PSCell. - LS can be sent to RAN4 to confirm this. |
| LG | Yes |  |
| CATT | Yes |  |
| Nokia, NSB | Yes |  |
| Intel | Needs further clarifications | Fail to see the real difference between a first case is that PCell/PSCell are synchronized and a second case that other two cells of the two CGs are synchronized. - For the second case, all cells of two CGs have fixed time offset or zero time offset too. The time offset doesn’t drift with time. Therefore, it seems OK to consider this is sync NR-DC, if the first case is considered as sync DC. - On the contrary, if people have concern on the slot offset of a cell in a CG, i.e. not all cells of two CGs are exactly synchronized, we may consider both two cases as asynchronized DC. |
| MTK | Yes with ZTE’s modification | We see ZTE’s modification to be more clear. |
| Qualcomm | No | There can be one non-zero offset per UE. If all offsets are zero, or there is a non-zero offset that complies with the unaligned CA/DC definitions, it is sync DC. Otherwise it is async. |
| Huawei, HiSilicon | Y |  |
| Ericsson | Yes | OK with ZTE modifications. |
| CMCC | Yes | OK with ZTE modifications. |
| Samsung | Yes |  |
| vivo | Yes | OK with ZTE modifications. |

### 2.6 Unaligned CA Issue 3

Please provide your input to question Q1 below, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to the TP corresponding to Proposal 1 of R1-2008504?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Unaligned CA Issue 3, Q1) |
| --- | --- | --- |
| ZTE | No | It seems this TP is not necessary. Based on the following description from TS38.331, it has been specified that aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 is the slot offset between he slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. Based on this, it seems the current spec is clear. aperiodicTriggeringOffset, aperiodicTriggeringOffset-r16 Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. For aperiodicTriggeringOffset, the value 0 corresponds to 0 slots, value 1 corresponds to 1 slot, value 2 corresponds to 2 slots, value 3 corresponds to 3 slots, value 4 corresponds to 4 slots, value 5 corresponds to 16 slots, value 6 corresponds to 24 slots. For aperiodicTriggeringOffset-r16, the value indicates the number of slots. The network configures only one of the fields. When neither field is included, the UE applies the value 0. |
| LG | Need further check |  |
| CATT | No | Agree with ZTE’s comments. |
| Nokia, NSB | Yes |  |
| Intel | Yes | Assuming offset X is configured by high layer assuming no slot offset, the TP is needed to CA with unaligned frame boundary |
| MTK | Yes | We agree with ZTE that in can be seen from 38.331. However, it is not clear if we only specify the timing effect of ca-SlotOffset in 38.214 g20 spec 5.2.1.5.1a (ACSI-RS with different SCS): - The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset but not in 38.214 g20 spec 5.2.1.5.1 (ACSI-RS with same SCS). It creates confusion for Layer-1 implementation. We think adding the Proposal 1 of R1-2008504 makes current spec more clear and consistent. |
| Huawei, HiSilicon | Y |  |
| Ericsson | Yes |  |
| CMCC | Yes |  |
| Samsung | Yes |  |
| vivo | No | Agree with ZTE’s comments. |

# 3 Conclusions

As conclusion of discussion on [103-e-NR-MRDC-CA-01] email thread for Q2 of Dormancy Topic 1 and Q1 of Dormancy Topic 3, following CR to 38.213 was agreed

R1-2009755 (Rel-16, TS38.213, CR0177) Corrections for SCell dormancy indication               Moderator (Ericsson), ZTE

For Dormancy Topic 5, after discussion on [103-e-NR-MRDC-CA-01] email thread, following was conclusion was made

Conclusion:

In description of SCell dormancy indication in 38.212, “…MSB to LSB of the bitmap corresponding to the first to last configured SCell group...” implies that the MSB to LSB of the bitmap correspond to the first to last configured SCell group in ascending order of DormancyGroupID.

As conclusion of discussion on [103-e-NR-MRDC-CA-01] email thread for Unaligned CA Issue 2, following CR to 38.214 was agreed

R1-2009756 (Rel-16, TS38.214, CR0153) Corrections for A-CSI triggering with unaligned CA               Moderator (Ericsson)

# 4 References

- R1-2007578	Remaining issues on CA	Huawei, HiSilicon

- R1-2007737	Remaining Issues of SCell Dormancy, Single Tx and Unaligned Frame Boundary ZTE

- R1-2008113	Correction to dormancy indication outside Active Time	NEC

- R1-2008145	Remaining issues for Scell dormancy	Samsung

- R1-2008203	Remaining issues on dormancy	Nokia, Nokia Shanghai Bell

- R1-2008275	BWP switching delay issues and Scell dormancy		OPPO

- R1-2008504	Remaining issues on Rel-16 carrier aggregation	MediaTek Inc.

- R1-2008566	Draft CR to 38.213 on corrections for SCell dormancy	Ericsson

- R1-2008613	Remaining issue on SCell dormancy	Qualcomm Incorporated

- R1-2007806	Remaining issues on inter-band CA with unaligned frame boundary	CATT

- R1-2008504	Remaining issues on Rel-16 carrier aggregation	MediaTek Inc.

- R1-2008567	Maintenance for other MR-DC topics	Ericsson

- R1-2008680	Remaining issues on MR-DC and CA enhancements	vivo

- R1-20xxxxx	Summary of maintenance issues for SCell Dormancy	Moderator (Ericsson)

- R1-20xxxxx	FL summary on support of unaligned frame boundary for R16 NR inter-band	Moderator (CMCC)